3GPP TSG-RAN WG4 Meeting #111	R4-2409055

# Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

Source:	Ericsson

Title:	Core maintenance paper related to deriveSSB-IndexFromCell and deriveSSB-IndexFromCellInter-r17

Agenda item:	5.2.6.3

Document for:	Approval

# 1	Introduction

In WF [1], it is stated:

… Issue 1-1: Whether to change the reference point of the tolerance for deriveSSB-IndexFromCell and deriveSSB-IndexFromCellInter-r17 Agreement: - Do not change the existing tolerance requirement of deriveSSB-IndexFromCell/Inter-r17. - FFS whether need to clarify the applicable scenario for deriveSSB-IndexFromCell/Inter-r17 - FFS: E.g., UE does not expect the deriveSSB-IndexFromCell/Inter-r17 to be set as true when RTD from UE between serving and neighbour cell is large than X - Other wording of applicability rules are not precluded. …

In this contribution we analyze issue 1-1, whether to change the reference point of the tolerance for deriveSSB-IndexFromCell and deriveSSB-IndexFromCellInter-r17.

# 2	Discussion

In RAN4#110bis we have the agreement:

“Do not change the existing tolerance requirement of deriveSSB-IndexFromCell/Inter-r17”.

This agreement is fine for Ericsson. A fundamental change of this nature should preferably not be done in maintenance phase of WI.

Observation 1: 	A fundamental change of whether to change the reference point of the tolerance for deriveSSB-IndexFromCell and deriveSSB-IndexFromCellInter-r17 should preferably not be done in maintenance phase of WI.

Observation 2: 	The agreement “Do not change the existing tolerance requirement of deriveSSB-IndexFromCell/Inter-r17” is fine for Ericsson.

Regarding:

“FFS: E.g., UE does not expect the deriveSSB-IndexFromCell/Inter-r17 to be set as true when RTD from UE between serving and neighbour cell is large than X”.

This issue is a consistency rule regarding parameter setting. Such a consistency rule is up to the operator parameter setting and should not be part of the standard.

Observation 3: 	The rule “UE does not expect the deriveSSB-IndexFromCell/Inter-r17 to be set as true when RTD from UE between serving and neighbour cell is large than X”, is a consistency rule regarding parameter setting. Such a consistency rule is up to the operator parameter setting and should not be part of the standard.

Proposal 1: 	A consistency rule like “UE does not expect the deriveSSB-IndexFromCell/Inter-r17 to be set as true when RTD from UE between serving and neighbour cell is large than X”, is not needed.

# 3 	Summary

Observation 1: 	A fundamental change of whether to change the reference point of the tolerance for deriveSSB-IndexFromCell and deriveSSB-IndexFromCellInter-r17 should preferably not be done in maintenance phase of WI.

Observation 2: 	The agreement “Do not change the existing tolerance requirement of deriveSSB-IndexFromCell/Inter-r17” is fine for Ericsson.

Observation 3: 	The rule “UE does not expect the deriveSSB-IndexFromCell/Inter-r17 to be set as true when RTD from UE between serving and neighbour cell is large than X”, is a consistency rule regarding parameter setting. Such a consistency rule is up to the operator parameter setting and should not be part of the standard.

Proposal 1: 	A consistency rule like “UE does not expect the deriveSSB-IndexFromCell/Inter-r17 to be set as true when RTD from UE between serving and neighbour cell is large than X”, is not needed.

# References

[1] 		R4-2406428, WF on RRM requirements for NR ATG, CMCC