3GPP TSG-RAN WG4 Meeting #111	R4-2409060

# Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

Source:	Ericsson

Title:	UE RF requirements for type 4a/4b capable FWA UE for EN-DC/NR-CA

Agenda item:	10.7.2.1

Document for:	Approval

# 1	Introduction

In RAN4#110bis the following WF was agreed [1]:

… # Topic #2: Rel-19 Type-4 ### Sub-topic 2-2: UE RF requirements for Type 4 EN-DC/NR-CA < Issue 2-2-1: Power imbalance requirements and REFSENS > Way Forward: - Check 25dB power imbalance and 1dB REFSENS Relaxation and conclude them in the next meeting. < Issue 2-2-2: How to capture the power imbalance and REFSENS requirements > Way Forward: - Continue further discussion in the next meeting. < Issue 2-2-3: How to capture the other power imbalance related requirements > Way Forward: - Continue further discussion in the next meeting. < Issue 2-2-4: DL frequency separation > Way Forward: - Check and conclude the following requirement in the next meeting. - No limitation on DL maximum frequency separation is needed for type 4 UE. - For DL minimum frequency separation for type 2 and type 4 UE, 80MHz is used. < Issue 2-2-5: The number of CCs > Way Forward: - Consider the conclusion of Type 2 discussion in RAN4#110bis. - Continue further discussion with the following baselines in the next meeting. - Non-collocated EN-DC - B42: multiple contiguous CCs up to four, collocated - n77/n78: one CC - Non-collocated NR-CA - n77/n78: Non-contiguous two CCs, non-collocated - If needed, check operators’ preference before the next May meeting. …

In this contribution we discuss issues related to Sub-topic 2-2: UE RF requirements for Type 4 EN-DC/NR-CA.

# 2	Discussion

## 2.1 	Issue 2-2-1:  Power imbalance requirements and REFSENS

In release 18 we have specified the following for EN-DC in TS 38-101-3:

… Table 7.10B.3-1: Test parameters for FDD-FDD or TDD-TDD inter-band EN-DC operation with overlapping or partially overlapping DL bands Test configurations; Carriers; Rx Power in transmission bandwidth configuration (dBm); channel bandwidth; Frequency relationship (Center of BWanother Relative to edge of BWwanted). 1; Wanted carrier; REFSENS + 1; BWwanted ≤ BWanother; < max (5/2* BWanother, 50MHz). Another wanted carrier with overlapping DL bands; Power of wanted carrier + 25. 2; Wanted carrier; REFSENS + 1; BWwanted > BWanother. Another wanted carrier with overlapping DL bands; Power of wanted carrier + 25 – 10*log10(BWwanted /BWanother). 3; Wanted carrier; REFSENS + 1; NA; ≥ max (5/2* BWanother, 50MHz). Another wanted carrier with overlapping DL bands; Power of wanted carrier + 25. NOTE 1: For NR carrier, the transmitter shall be set to 24dB below PCMAX_L,f,c,NR at the minimum uplink configuration specified in Table 7.3.2-3 [2] with PCMAX_L,f,c,NR as defined in clause 6.2B.4. NOTE 2: For E-UTRA carrier, the transmitter shall be set to 24 dB below PCMAX_L_E-UTRA,c at the minimum uplink configuration specified in Table 7.3.1-2 [4] with PCMAX_L_E-UTRA,c as defined in clause 6.2B.4 for single carrier. NOTE 3: BWwanted is the channel bandwidth of wanted carrier. BWanother is the channel bandwidth of another wanted carrier with overlapping DL bands NOTE 4: REFSENS is the reference sensitivity level or two antenna port in Table 7.3.2-1a or in Table 7.3.2-1b of TS 38.101-1, or in Table 7.3.1-1 of TS 36.101. NOTE 5: The minimum requirements apply only when there is non-simultaneous Rx/Tx operation between E-UTRA and NR carriers for the EN-DC combinations in Table 7.10B.3-2. This restriction applies also for these carriers when applicable EN-DC configuration is part of a higher order EN-DC configuration. NOTE 6: The combination is not used alone as fall back mode of other band combinations in which UL in Band 42 is not used.. …

In a similar way we have, for release-18, specified for CA in TS 38.101-1:

… Table 7.10A.2-1: Power imbalance parameters for intra-band non-contiguous CA Test configurations; Carriers; Rx Power in transmission bandwidth configuration (dBm); channel bandwidth; Center of BWanother Relative to edge of BWwanted. 1; Wanted carrier; REFSENS NOTE 4 + 1; BWwanted ≤ BWanother; < max (5/2* BWanother, 50MHz. Another wanted carrier; Power of wanted carrier + 25. 2; Wanted carrier; REFSENS NOTE 4 + 1; BWwanted > BWanother. Another wanted carrier; Power of wanted carrier + 25 – 10*log10(BWwanted /BWanother). 3; Wanted carrier; REFSENS NOTE 4 + 1; NA; ≥ max (5/2* BWanother, 50MHz). Another wanted carrier; Power of wanted carrier + 25. NOTE 1: The transmitter shall be set to 24dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 with PCMAX_L,f,c as defined in clause 6.2A.4. NOTE 2: BWwanted is the channel bandwidth of wanted carrier. BWanother is the channel bandwidth of another wanted carrier with 25 dB power imbalance. NOTE 3: It’s allowed to use one of test configurations to verify the RX power imbalance requirement for type 2 UE. NOTE 4: REFSENS is the reference sensitivity level for two antenna port in Table 7.3.2-1b. NOTE 5: The minimum requirements only apply for non simultaneous Rx/Tx between all carriers for TDD intra-band non-contiguous CA band combinations in Table 7.10A.2-2. …

Observation 1: For baseline approved specifications we have agreed a power imbalance of 25 dB for UE Type 2.

For the type 4a and 4b UE architectures we have no LNA sharing, similar to the type 2 architecture. It is also reasonable to make type 4 UE compatible with a network with type 2 UE

| 2 | 1 | 2 | 4 total | 2 | 2 | 2Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Reuse of baseline architecture restricted to 2Rx/band but need 2LO frequencies |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4a | 1 | 4 | 6 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 25dB full range | Requires 6 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 4b | 1 | 4 | 8 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Requires 8 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |

Observation 2: Type 4a and 4b UE architectures we have no LNA sharing, similar to the type 2 architecture.

Based on these observations, no LNA sharing and compatibility with networks deployed with Type 2 UE it is fair to propose that the power imbalance for Type 4a and 4b is 25 dB.

Proposal 1: RF power imbalance requirements for NRCA and ENDC is 25 dB.

## 2.2 	Issue 2-2-2:  How to capture the power imbalance and REFSENS requirements

In current specification CA and EN-DC we use REFSENS as defined by the reference sensitivity level for two antenna port in Table 7.3.2-1b.

… Table 7.10A.2-1: Power imbalance parameters for intra-band non-contiguous CA Test configurations; Carriers; Rx Power in transmission bandwidth configuration (dBm); channel bandwidth; Center of BWanother Relative to edge of BWwanted. 1; Wanted carrier; REFSENS NOTE 4 + 1; BWwanted ≤ BWanother; < max (5/2* BWanother, 50MHz. Another wanted carrier; Power of wanted carrier + 25. 2; Wanted carrier; REFSENS NOTE 4 + 1; BWwanted > BWanother. Another wanted carrier; Power of wanted carrier + 25 – 10*log10(BWwanted /BWanother). 3; Wanted carrier; REFSENS NOTE 4 + 1; NA; ≥ max (5/2* BWanother, 50MHz). Another wanted carrier; Power of wanted carrier + 25. NOTE 1: The transmitter shall be set to 24dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 with PCMAX_L,f,c as defined in clause 6.2A.4. NOTE 2: BWwanted is the channel bandwidth of wanted carrier. BWanother is the channel bandwidth of another wanted carrier with 25 dB power imbalance. NOTE 3: It’s allowed to use one of test configurations to verify the RX power imbalance requirement for type 2 UE. NOTE 4: REFSENS is the reference sensitivity level for two antenna port in Table 7.3.2-1b. NOTE 5: Void.. … Table 7.10A.2-1: Power imbalance parameters for intra-band non-contiguous CA Test configurations; Carriers; Rx Power in transmission bandwidth configuration (dBm); channel bandwidth; Center of BWanother Relative to edge of BWwanted. 1; Wanted carrier; REFSENS NOTE 4 + 1; BWwanted ≤ BWanother; < max (5/2* BWanother, 50MHz. Another wanted carrier; Power of wanted carrier + 25. 2; Wanted carrier; REFSENS NOTE 4 + 1; BWwanted > BWanother. Another wanted carrier; Power of wanted carrier + 25 – 10*log10(BWwanted /BWanother). 3; Wanted carrier; REFSENS NOTE 4 + 1; NA; ≥ max (5/2* BWanother, 50MHz). Another wanted carrier; Power of wanted carrier + 25. NOTE 1: The transmitter shall be set to 24dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 with PCMAX_L,f,c as defined in clause 6.2A.4. NOTE 2: BWwanted is the channel bandwidth of wanted carrier. BWanother is the channel bandwidth of another wanted carrier with 25 dB power imbalance. NOTE 3: It’s allowed to use one of test configurations to verify the RX power imbalance requirement for type 2 UE. NOTE 4: REFSENS is the reference sensitivity level for two antenna port in Table 7.3.2-1b. NOTE 5: Void.. …

When it comes to 4 RX in existing specification we have, for REFSENS:

… when the UE is verified with 4 Rx antenna ports, it shall be verified against those requirements by applying the resulting REFSENS value derived from the requirement in Table 7.3.2-2 with 4 Rx antenna ports tested. … For UE(s) equipped with 4 Rx antenna ports, reference sensitivity for 2Rx antenna ports in Table 7.3.2-1a and in Table 7.3.2-1b shall be modified by the amount given in ΔRIB,4R in Table 7.3.2-2 for the applicable operating bands. For operating band frequency range ≤ 1 GHz, the 4Rx operation is primarily for FWA form factor, and when 4Rx operation is supported by handheld UE, ∆RIB,4R as indicated in Table 7.3.2-2 NOTE 2 is applied. Table 7.3.2-2: Four antenna port reference sensitivity allowance ΔRIB,4R Operating band; ΔRIB,4R (dB). n5, n8, n13, n26, n28, n71, n85, n105; -2.71. n5, n8, n28, n71, n20, n26; -2.42. n1, n2, n3, n25, n30, n40, n7, n34, n38, n39, n41, n66, n70; -2.7. n48, n77, n78, n79, n104; -2.2. NOTE 1: When 4 Rx operation is supported by FWA form factor NOTE 2: When 4Rx operation is supported by handheld UE.. …

Since existing specification already have a 4 RX REFSENS, based on 2 RX REFSENS table and an additive modification term ΔRIB,4R , we can reuse that.

Observation 3: existing specification already have a 4 RX REFSENS based on 2 RX REFSENS table and an additive modification term ΔRIB,4R.

Proposal 2: Use existing 4 RX REFSENS as a specification of REFSENS for Type 4.

Regarding how to modify REFSENS for Inter-band EN-DC configurations with multiple contiguous E-UTRA CCs in one band, please refer to contribution [2]

# 3 	Summary

Observation 1: For baseline approved specifications we have agreed a power imbalance of 25 dB for UE Type 2.

Observation 2: Type 4a and 4b UE architectures we have no LNA sharing, similar to the type 2 architecture.

Proposal 1: RF power imbalance requirements for NRCA and ENDC is 25 dB.

Observation 3: existing specification already have a 4 RX REFSENS based on 2 RX REFSENS table and an additive modification term ΔRIB,4R.

Proposal 2: Use existing 4 RX REFSENS as a specification of REFSENS for Type 4.

# References

[1] 		R4-2406627, WF on NonCol_intraB_ENDC_NR_CA, KDDI.