# 3GPP TSG-RAN WG4 Meeting # 111                                  R4-2409082

# Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

# Title: 	Simulation results for multiple PRACH

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.17.2

# Document for:	Discussion

# 1 Introduction

In RAN #102 meeting[1], the revised work item on Revised WID on further NR coverage enhancements was approved.

This contribution updates our simulation results for multiple PRACH transmission based on the conclusion last meeting.

# 2	Simulation assumption

It was pointed in RAN4 #110 meeting[2] that the TDD pattern and PRACH interval need to be checked.

PRACH repetition number for BS performance requirements for Multiple PRACH transmission Way forward for the next meeting: - Companies to further check the simulation assumptions including TDD pattern, PRACH repetition interval for both FR1 and FR2. - For the repetition number, keep the previous meeting agreement before simulation assumption agreed.

Table 2-1 lists the simulation assumption including some assumption agreed in [2] and [3].

Table 2-1 Simulation assumption for multiple PRACH

| Parameters | Values |
| --- | --- |
| Frequency range | FR2-1 |
| Sequence length | 139 |
| Preamble format | A2, B4, C2 |
| Repetition number | 2 |
| Antenna configuration | 1X2 |
| Channel model | AWGN, TDLA30-300 low for FR2-1 AWGN, TDLC300-100 low for FR1 |
| Frequency offset | Fading channel FR2-1 4000Hz AWGN 0 Fading channel FR1 400Hz |
| SCS | FR2-1 120kHz, FFS FR2-1 60kHz FR1 15kHz and 30kHz |
| Test metric | Missing detection 1% False alarm 0.1% |
| TDD pattern | 15kHz, 60kHz, 120kHz: 3D1S1U 30kHz: 7D1S2U |
| Timing tolerance(AWGN) | 15kHz: 0.52 us 30kHz: 0.26 us 60kHz: 0.13 us 120kHz: 0.07 us |
| Timing tolerance(fading channel) | 15kHz: 2.03 us 30kHz: 1.77 us 60kHz: 0.28 us 120kHz: 0.22 us |
| Ncs | 15kHz: 23 30kHz: 46 60kHz: 69 120kHz: 69 |
| PRACH interval | Option 1 15kHz, 60kHz, 120kHz: 5 slots 30kHz: 1 slot |
|  | Option 2 15kHz, 30kHz, 120kHz : 10ms |
|  | Option 3 120kHz:10 slots |

# 3 Simulation results

Table 2-2 shows the simulation results for FR1 and the miss detection curves are appended in the Annex. Table 2-3 shows the simulation results for FR2-1 and the miss detection curves are appended in the Annex.

Table 2-2 Simulation results for FR2-1 for different preamble format with repetition number = 2, 15kHz SCS and 30kHz SCS

| index | SCS | Preamble format | Repetition interval | Channel mode | Operating SNR |
| --- | --- | --- | --- | --- | --- |
| 1 | 15kHz | A2 | 5 slots | AWGN w/o FO | -18.5 |
| 2 | 15kHz | A2 | 5 slots | TDLC300-100 low w/ 400Hz FO | -11.1 |
| 3 | 15kHz | B4 | 5 slots | AWGN w/o FO | -21.7 |
| 4 | 15kHz | B4 | 5 slots | TDLC300-100 low w/ 400Hz FO | -13.0 |
| 5 | 15kHz | C2 | 5 slots | AWGN w/o FO | -18.6 |
| 6 | 15kHz | C2 | 5 slots | TDLC300-100 low w/ 400Hz FO | -11.2 |
| 7 | 30kHz | A2 | 1 slot | AWGN w/o FO | -18.5 |
| 8 | 30kHz | A2 | 1 slot | TDLC300-100 low w/ 400Hz FO | -12.9 |
| 9 | 30kHz | B4 | 1 slot | AWGN w/o FO | -21.7 |
| 10 | 30kHz | B4 | 1 slot | TDLC300-100 low w/ 400Hz FO | -17.1 |
| 11 | 30kHz | C2 | 1 slot | AWGN w/o FO | -18.6 |
| 12 | 30kHz | C2 | 1 slot | TDLC300-100 low w/ 400Hz FO | -12.7 |
| 13 | 15kHz | A2 | 10 ms | TDLC300-100 low w/ 400Hz FO | -11.2 |
| 14 | 15kHz | B4 | 10 ms | TDLC300-100 low w/ 400Hz FO | -14.7 |
| 15 | 15kHz | C2 | 10 ms | TDLC300-100 low w/ 400Hz FO | -11.3 |
| 16 | 30kHz | A2 | 10 ms | TDLC300-100 low w/ 400Hz FO | -11.5 |
| 17 | 30kHz | B4 | 10 ms | TDLC300-100 low w/ 400Hz FO | -16.0 |
| 18 | 30kHz | C2 | 10 ms | TDLC300-100 low w/ 400Hz FO | -11.5 |

Table 2-3 Simulation results for FR2-1 for different preamble format with repetition number = 2, 120kHz SCS

| index | SCS | Preamble format | Repetition interval | Channel mode | Operating SNR |
| --- | --- | --- | --- | --- | --- |
| 1 | 120kHz | A2 | Option 1 | AWGN w/o FO | -18.6 |
| 2 | 120kHz | A2 | Option 1 | TDLA30-300 low w/ 4000Hz FO | -11.3 |
| 3 | 120kHz | B4 | Option 1 | AWGN w/o FO | -21.7 |
| 4 | 120kHz | B4 | Option 1 | TDLA30-300 low w/ 4000Hz FO | -13.8 |
| 5 | 120kHz | C2 | Option 1 | AWGN w/o FO | -18.6 |
| 6 | 120kHz | C2 | Option 1 | TDLA30-300 low w/ 4000Hz FO | -11.3 |
| 7 | 120kHz | A2 | 10 ms | TDLA30-300 low w/ 4000Hz FO | -11.7 |
| 8 | 120kHz | B4 | 10 ms | TDLA30-300 low w/ 4000Hz FO | -14.3 |
| 9 | 120kHz | C2 | 10 ms | TDLA30-300 low w/ 4000Hz FO | -11.7 |
| 10 | 120kHz | A2 | 10 slots | TDLA30-300 low w/ 4000Hz FO | -11.2 |
| 11 | 120kHz | B4 | 10 slots | TDLA30-300 low w/ 4000Hz FO | -14.0 |
| 12 | 120kHz | C2 | 10 slots | TDLA30-300 low w/ 4000Hz FO | -11.2 |

# 4	Reference

[1]  RP-233558 Revised WID on further NR coverage enhancements, China Telecom

[2]  R4-2406137, WF on NR_cov_enh2_demod, China Telecom

# Annex

A-1 Miss detection curve for A2 preamble, AWGN, 120kHz SCS and 2 repetition with option 1 repetition interval

A-2 Miss detection curve for B4 preamble, AWGN, 120kHz SCS and 2 repetition with option 1 repetition interval

A-3 Miss detection curve for C2 preamble, AWGN, 120kHz SCS and 2 repetition with option 1 repetition interval

A-4 Miss detection curve for A2 preamble, TDLA30-300, 120kHz SCS and 2 repetition with option 1 repetition interval

A-5 Miss detection curve for B4 preamble, TDLA30-300, 120kHz SCS and 2 repetition with option 1 repetition interval

A-6 Miss detection curve for C2 preamble, TDLA30-300, 120kHz SCS and 2 repetition with option 1 repetition interval

A-7 Miss detection curve for A2 preamble, AWGN, 15kHz SCS and 2 repetition with option 1 repetition interval

A-8 Miss detection curve for B4 preamble, AWGN, 15kHz SCS and 2 repetition with option 1 repetition interval

A-9 Miss detection curve for C2 preamble, AWGN, 15kHz SCS and 2 repetition with option 1 repetition interval

A-10 Miss detection curve for A2 preamble, AWGN, 30kHz SCS and 2 repetition with option 1 repetition interval

A-11 Miss detection curve for B4 preamble, AWGN, 30kHz SCS and 2 repetition of option 1 repetition interval

A-12 Miss detection curve for C2 preamble, AWGN, 30kHz SCS and 2 repetition of option 1

A-13 Miss detection curve for A2 preamble, TDLC300-100, 15kHz SCS and 2 repetition of option 1

A-14 Miss detection curve for A2 preamble, TDLC300-100, 30kHz SCS and 2 repetition of option 1

A-15 Miss detection curve for B4 preamble, TDLC300-100, 15kHz SCS and 2 repetition of option 1

A-16 Miss detection curve for B4 preamble, TDLC300-100, 30kHz SCS and 2 repetition of option 1

A-17 Miss detection curve for C2 preamble, TDLC300-100, 15kHz SCS and 2 repetition of option 1

A-18 Miss detection curve for C2 preamble, TDLC300-100, 30kHz SCS and 2 repetition of option 1

A-19 Miss detection curve for A2 preamble, TDLC300-100, 15kHz SCS and 2 repetition of option 2

- A-20 Miss detection curve for B4 preamble, TDLC300-100, 15kHz SCS and 2 repetition with option 2 repetition

A-21 Miss detection curve for C2 preamble, TDLC300-100, 15kHz SCS and 2 repetition of option 2

A-22 Miss detection curve for A2 preamble, TDLC300-100, 30kHz SCS and 2 repetition of option 2

A-23 Miss detection curve for B4 preamble, TDLC300-100, 30kHz SCS and 2 repetition of option 2

A-24 Miss detection curve for C2 preamble, TDLC300-100, 30kHz SCS and 2 repetition of option 2

A-25 Miss detection curve for A2 preamble, TDLA30-300, 120kHz SCS and 2 repetition of option 2

A-26 Miss detection curve for B4 preamble, TDLA30-300, 120kHz SCS and 2 repetition of option 2

A-27 Miss detection curve for C2 preamble, TDLA30-300, 120kHz SCS and 2 repetition of option 2

A-28 Miss detection curve for A2 preamble, TDLC300-100, 120kHz SCS and 2 repetition of option 3

A-29 Miss detection curve for B4 preamble, TDLC300-100, 120kHz SCS and 2 repetition of option 3

A-30 Miss detection curve for C2 preamble, TDLC300-100, 120kHz SCS and 2 repetition of option 3