# 3GPP TSG-RAN WG4 Meeting # 111						R4-2409106

# Fukuoka, Japan, 20th  –24th  May, 2024

Source:		Ericsson

Title:			Other NTN UE RF impact

Agenda item:		10.15.2.2

Document for:		Approval

# Introduction

In this paper, we present our general overview on NTN UE RF impact other than Redcap.

# Discussion

In WID[1], the objectives relating to the RF are quoted below:

- Study and specify if beneficial downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4]

- Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth.

- Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements.

- Study and if needed specify solutions, including link level enhancements for FR1-NTN (e.g. for PDCCH, PDSCH) and/or system level enhancements for FR1-NTN and/or FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint.

- Notes for this objective:

  - SSB channel enhancement is not considered

  - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE

  - NGSO to be considered in priority: LEO Set-1 @ 600 km

  - Rel-18 network energy saving techniques should be considered as baseline in the system level study

- Uplink Capacity/Throughput Enhancement for FR1-NTN [RAN1, RAN2, RAN4]

- Study then specify, if beneficial, DFT-s-OFDM PUSCH enhancements via Orthogonal Cover Codes (OCC)

  - Determine the achievable capacity improvement to be targeted taking into account realistic impairments (e.g. Doppler, time variation, phase distortion, etc)

  - Specify necessary signalling, if needed

  - Update RF requirements accordingly, if needed

  - Note: The study can consider orthogonal cover codes across OFDM symbols, across slots, and/or within an OFDM symbol.

  - Note: the study phase is targeted to be completed by RAN#104

- Notes for this objective:

  - The enhancement is not targeting improvements/impacts of MU-MIMO capability

  - The enhancement is not targeted to PUSCH DMRS

  - No enhancement for initial access

  - Enhancements to PRACH are not in scope.

  - This feature may be applicable for UEs operating in terrestrial networks based on a common design

For DL coverage enhancement objective, in our companion paper [3], we identify the time uncertainty for the beam switching between cell active/inactive time. This time uncertainty may be caused by either the network synchronization time error for transparent payload or the beam switching delay. As the beam switching impact both for DL and UL, it is a question on the UE behavior for data transmission/reception during the beam switching together with the time uncertainty. RAN4 could wait more discussion on this issue before investigation on the UE impact.

UE RF may be impacted due to the beam switching time uncertainty and switching delay.

For UL coverage enhancement objective, the main concern is the OCC feature impact on the UE RF, the latest RAN1 agreement is below, we think RAN4 can start the evaluation once RAN1 reach some conclusions or down-selection.

- Agreement

Support OCC for PUSCH in Rel-19 NR NTN:

- At least PUSCH with Type A repetition

  - FFS PUSCH without Type A repetition for intra-symbol and/or inter-symbol cases

- At least code length 2 or 4, FFS code length 8

- FFS: number of RBs

- Potential OCC techniques listed below are for further down-selection:

  - Inter-slot time-domain OCC with PUSCH repetition Type A

  - Inter-symbol(s) time domain OCC

  - Intra-symbol pre-DFT-s OCC (comb-like structure as in PUCCH format 4)

  - Combinations of OCC techniques

- TBoMS for OCC techniques is FFS

Wait RAN1 reach conclusions on OCC feature before RAN4 start to evaluate the RF impact

# Conclusions

In this contribution, we present our general overview for NTN UE RF impact other than Redcap with below proposal:

Observation 1 UE RF may be impacted due to the beam switching time uncertainty and switching delay.

Proposal-1: Wait RAN1 reach conclusions on OCC feature before RAN4 start to evaluate the RF impact

# References

- RP-240801, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo etc

- R4-2406610, WF on NR_NTN_Ph3_UERF, Qualcomm Inc

- R4-240xyz, SAN RF impact overview, Ericsson