# 3GPP TSG-RAN WG4 Meeting # 111						R4-2409108

# Fukuoka, Japan, 20th  –24th  May, 2024

Source:		Ericsson

Title:			RedCap UE RF impact on HD-FDD

Agenda item:		10.15.2.1

Document for:		Approval

# Introduction

In this paper, we present our general view on the RedCap UE RF remaining issue.

# Discussion

For RedCap receiver requirements, the one Rx and HD-FDD support needs to be added, in last meeting, RAN4 agrees to reuse the TN Redcap scaling factor for NTN in WF[3], but no the HD-FDD related agreement.  For Rel-17 RedCap UE operating at HD-FDD mode , a general scaling factor of 0.8 dB tightening with exception for some bands[2]. The exception is mainly because of comparison with the tightest TDD band REFSENS and set no tightening if REFSENS lower than -100 dBm. The agreement is quoted below[2].

[Agreement:]

Agree on the framework of Option 1a

-	Option 1a: per band exception and selected band for different scaling factor as below: [Apple]

The HD-FDD 5MHz REFSENS tightening from FD-FDD is proposed as in the table below.

| FD-FDD 5MHz REFSENS | HD-FDD REFSENS Tightening | Bands |
| --- | --- | --- |
| ≤ -100 dBm | 0 dB | n1, n18, n24, n70 |
| > -100 dBm and ≤ -99 dBm | 0.5 dB | n30, n65, n66, n74 |
| > -99 dBm | 0.8 dB | n2, n3, n5, n7, n8, n12, n13, n14, n20, n25, n26, n28, n71, n85 |

-	HD-FDD REFSENS for channel BW wider than 5 MHz can be calculated by REFSENS(5MHz) + 10log10(n x NRB/25), where NRB is the maximum transmission bandwidth configuration with n=1 for 15kHz SCS and n=2 for 30kHz SCS.

Such agreement could apply for the NTN band also, with only concern that the NTN RedCap UE operating at low wanted signal due to the large distance to the satellite. Therefore, it may be good to at least be tightened with 0.5 dB. The REFSENS of NTN band of 2 RX is quoted below.

Table 7.3.2-1: Two antenna port reference sensitivity QPSK PREFSENS for FDD bands

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
|  | 15 | -99.5 | -96.3 | -94.5 | -93.8 |  |  |  |  |  |  |
| n256 | 30 |  | -96.6 | -94.6 | -94.0 |  |  |  |  |  |  |
|  | 60 |  | -97.0 | -94.9 | -94.2 |  |  |  |  |  |  |
|  | 15 | -100.0 | -96.8 | -95.0 | -93.8 |  |  |  |  |  |  |
| n255 | 30 |  | -97.1 | -95.1 | -94.0 |  |  |  |  |  |  |
|  | 60 |  | -97.5 | -95.4 | -94.2 |  |  |  |  |  |  |
|  | 15 | -99.5 | -96.3 | -94.5 |  |  |  |  |  |  |  |
| n254 | 30 |  | -96.6 | -94.6 |  |  |  |  |  |  |  |
|  | 60 |  | -97.0 | -94.9 |  |  |  |  |  |  |  |
| NOTE：The transmitter shall be set to PUMAX as defined in clause 6.2.4 of 3GPP TS 38.101-1 [5]. |  |  |  |  |  |  |  |  |  |  |  |

Consider the at least 0.5 dB tightening for the HD-FDD NTN band in general.

# Conclusions

In this contribution, we present our general overview with Redcap UE RF below proposal:

Proposal-1: Consider the at least 0.5 dB tightening for the HD-FDD NTN band in general.

# References

- RP-234078, New WID: Non-Terrestrial Networks (NTN) for NR Phase 3 , Huawei

- MeetingReport__2022-01-29_06h03_UTC_R2.zip

- R4-2406610, WF on NR_NTN_Ph3_UERF, Qualcomm Inc