3GPP TSG-RAN WG4 Meeting #111	R4-2409138

Fukuoka, Japan, May 20 – May 24, 2024

Source:	Nokia

Title:	On multi-Rx RRM test case design

Agenda item:	7.3.2.1

Document for:	Discussion

Introduction

Proposals for the performance part based on RAN4#110bis meeting contributions were captured in WF in [1]. In this contribution we discuss some aspects of the test case design and the necessary test cases further.

Test cases

Number of probes in RRM test cases

Issue 2-3: Number of probes in RRM test cases

- FFS following proposals by taking agreements in the last meeting and testability into consideration.

  - Option 1:

    - It is suggested to verify the dual to dual active TCI state switching from [RS 1, RS 2] to [RS 1, RS3] under the assumption of 3 active probes.

  - Option 2:

    - Define a dual-to-dual TCI test case for m-DCI, where the UE needs to switch both the TCI states i.e. [RS1, RS3], to [RS2, RS4], with [RS1, RS3] and [RS2, RS4] each forming beam pairs.

    - When less than four probes are used, the test equipment should emulate different DL transmit beams by transmitting different signals with different power and delay.

  - Option 3:

    - For TCI state switching TC for mDCI, target TCI states are beam pair for simultaneous reception. Before TCI state switching, PDCCH/PDSCH with different TCI states are non-overlapped in time domain.

  - Option 4:

    - RAN4 not to test dual-to-dual active TCI state switching in R18

  - Option 5:

    - RAN4 to introduce new 2 AoA setup with 3 active probes for multi-RX tests.

In the last meeting, various options were discussed and proposed. All of these options, consider three probes as a baseline. In the following section we propose our views to each of these options.

Option 1 proposes a test case where the UE is tested for dual to dual TCI test case switch keeping one RS common. This test case would cover a scenario where the UE will switch only one TCI state and hence in this case the legacy TCI state switch delay requirements will apply, which we believe does not need to be tested.

When the UE needs to switch only one TCI state while the other one is common i.e. [RS1, RS2], to [RS1, RS3] then legacy delay requirements apply. There is no need to test this scenario again.

Options 2 and 4 address the dual to dual TCI state switch case and the proposals put forth by companies on this topic.

Option 4 proposes to not test this scenario. However, in a m-DCI scenario, dual to dual TCI state switch would be a common occurrence and hence we believe it is important that test cases are defined for this scenario.

Since dual to dual TCI state switch in m-DCI would be a common occurrence, test case needs to be defined for this scenario.

Option 2 on the other hand will cover the case where the UE has to switch both the TCI states and it is here that the UE conformance to TCI state switch delay requirements defined for DCI based dual TCI state switch for m-DCI needs to be tested.

This of course will pose a challenge in testing using three probes. In our view this needs to be tested using four probes which would be close to replicating an on field/network scenario. However, using more than three probes presented challenges based on the OTA SI discussions. If less than four probes are used, the test equipment would have to emulate different DL transmit beams by transmitting different signals with different power and delay, so that the T/F synchronization step during TCI switching is verified. RAN4 should discuss how to implement the test case with such approach.

Define a dual-to-dual TCI test case for m-DCI, where the UE needs to switch both the TCI states i.e. [RS1, RS3], to [RS2, RS4], with [RS1, RS3] and [RS2, RS4] each forming beam pairs.

When less than four probes are used, the test equipment should emulate different DL transmit beams by transmitting different signals with different power and delay.

Option 5 proposes introducing two new AoA setup with three active probes. This is certainly required for carrying out m-Rx testing. However, this also poses a practical challenge in terms of testing. The m-TRP TCI state switch delay RRM requirements will apply only when the UE is configured with groupBaseBeamReporting-R17 and the UE can receive the target dual TCI states simultaneously. This then implies that even in case of single to dual TCI state switch, UE has to receive from one TCI state while it measures and reports that it can receive simultaneously from two other TCI states. Hence, it can be assumed that all the three probes need to be within the UE spherical coverage during the test. Since the spherical coverage for m-Rx will require the UE to pass the spherical coverage at a declared 2 AoA separation, UE at one time can receive from two probes. How to conduct the test with three probes without any significant interruption during test time needs to be discussed.

Since the direction and placement of the probes in the anechoic chamber during testing is fixed, the UE can receive only those beams which are within its spherical coverage.

Spherical coverage for m-Rx will require the UE to pass the spherical coverage at a declared 2 AoA separation.

How to conduct the dual TCI state switch delay requirements using three probes which are within the UE’s spherical coverage during the test duration needs to be discussed.

Test cases for TCI state switch

During RAN4 #110-bis the following issue was open for discussion regarding TCI switching test cases [1]

Issue 2-7: Test case(s) for dual TCI state switching for m-DCI

- Further discuss test cases for m-DCI after conclusion of issue 2-3

  - TC Y1: DCI based TCI state switch for m-DCI

    - Active TCI state list update for m-DCI is included during the test.

  - TC Y2: MAC-CE based dual TCI state switch for m-DCI

Since TCI state switching requirements have been defined for m-DCI scenarios and given the fact that this mode of deployment will very likely be often implemented in the network, we believe that test cases for dual MAC-CE based TCI state switching for m-DCI need to be defined.

Test cases for DCI based and MAC-CE dual TCI state switch for m-DCI need to be defined.

Issue 2-13: Test setup for dual TCI state switching

- FFS following proposal

  - Option 1:

    - Reuse the configuration and test requirement of legacy L1-RSRP and single TCI state switching test case as baseline.

    - Three AoAs (AoA0, AoA1 and AoA2) for different SSB indices (SSB 0, 1 and 2)

    - No DRX is configured.

    - Test requirement are set on time duration to check the expected behaviour of dual TCI state switching with correct delay requirement.

    - Note: UE does not indicate its preference for single-RX in UE assistance information throughout the test.

The current agreed test case for DCI based TCI state switch in s-DCI tests the UE conformance to delay requirements when the UE switches from RS1 to RS1 + RS2. In such a case, the existing SSB configuration defined for single TCI state switching test case can be reused. In case of AoA, 2 AoAs are sufficient but these need to be setup for simultaneous reception.

However, in case more than 2 SSBs are used (for example if it is agreed to define a TCI state switch delay test case where the UE switches from RS 1 + RS 2 to RS 1 + RS 3), then additional SSB configuration and the corresponding TRS resource set need to be defined for these tests. Also a 3 AoA setup for simultaneous reception has to be specified.

We have described all of the above configurations in our draft CR [2]

DCI based TCI state switch test case where the UE switches from RS1 to RS1+RS2 can reuse the existing SSB configuration already specified.

For 2 AoA setup, even if two AoAs are used, these need to be setup for simultaneous reception.

Introduce new 2 AoA setup for simultaneous reception.

For dual TCI state switch test cases, where UE needs to switch from RS1+RS2 to RS1+RS3, three AoAs need to be defined and setup for simultaneous reception.

Introduce new 3 AoA setup for simultaneous reception in dual-to-dual TCI switching test cases.

For dual TCI state switch test cases with 3 AoA setup, where UE needs to switch from RS1+RS2 to RS1+RS3, three SSBs will be required. Additional SSB and corresponding TRS resource set configuration need to be defined for testing.

We have proposed such test configurations in our company CR R4-2408688. However, the new setup for 3 AoAs is only needed if RAN4 agrees to define test cases that require this setup.

Conclusion

The following observations and proposals were made in this contribution:

Observation 1: When the UE needs to switch only one TCI state while the other one is common i.e. [RS1, RS2], to [RS1, RS3] then legacy delay requirements apply. There is no need to test this scenario again.

Proposal 1: Since dual to dual TCI state switch in m-DCI would be a common occurrence, test case needs to be defined for this scenario.

Proposal 2: Define a dual-to-dual TCI test case for m-DCI, where the UE needs to switch both the TCI states i.e. [RS1, RS3], to [RS2, RS4], with [RS1, RS3] and [RS2, RS4] each forming beam pairs.

Proposal 3: When less than four probes are used, the test equipment should emulate different DL transmit beams by transmitting different signals with different power and delay.

Observation 2: Since the direction and placement of the probes in the anechoic chamber during testing is fixed, the UE can receive only those beams which are within its spherical coverage.

Observation 3: Spherical coverage for m-Rx will require the UE to pass the spherical coverage at a declared 2 AoA separation.

Proposal 4: How to conduct the dual TCI state switch delay requirements using three probes which are within the UE’s spherical coverage during the test duration needs to be discussed.

Proposal 5: Test cases for DCI based and MAC-CE dual TCI state switch for m-DCI need to be defined.

Observation 4: DCI based TCI state switch test case where the UE switches from RS1 to RS1+RS2 can reuse the existing SSB configuration already specified.

Observation 5: For 2 AoA setup, even if two AoAs are used, these need to be setup for simultaneous reception.

Proposal 6: Introduce new 2 AoA setup for simultaneous reception.

Observation 6: For dual TCI state switch test cases, where UE needs to switch from RS1+RS2 to RS1+RS3, three AoAs need to be defined and setup for simultaneous reception.

Proposal 7: Introduce new 3 AoA setup for simultaneous reception in dual-to-dual TCI switching test cases.

Observation 7: For dual TCI state switch test cases, where UE needs to switch from RS1+RS2 to RS1+RS3, three SSBs will be required. Additional SSB and corresponding TRS resource set configuration needs to be defined for testing.

- References

- R4- 2406365, WF on core maintenance and performance part for multi-Rx WI, Vivo, Ericsson, RAN4#110-bis meeting,

- R4-2408688, Draft CR for TC for mRx S-DCI DCI-based TCI state switch and related configurations