rform3GPP TSG-RAN WG4 Meeting #111	R4-2409144

Fukuoka, Japan, May 20th – 24th, 2024

Source:	Nokia

Title:	Discussion on measurements without gaps

Agenda item:	7.5.2

Document for:	Discussion

Introduction

This paper presents Nokia’s views on measurements without gaps in Rel 18.

Discussion

This paper will focus on the last remaining open item, DRX periods colliding with interruptions.

Interruptions with DRX ON

In the last RAN4 meeting, the following was open regarding DRX [1]

### Sub-topic 1-1 DRX Issue 1-1-1: Misalignment between DRX-on duration and SMTC for NFG measurements - Proposals - Option 1: UE measures only within SMTC occasions once every DRX cycle and no interruption is allowed. - Option 1a: interruption can be allowed during the time extended due to drx-inactivityTimer. - Option 2: UE measures within SMTC occasions when DRX cycle is larger than 320ms and no interruption is allowed; but when DRX cycle is equal or smaller than 320ms interruption is allowed once every DRX cycle according to Tcycle, i. - Option 2a: 160ms instead of 320ms. And interruption can be allowed during the time extended due to drx-inactivityTimer. - Option 3: UE measurements can affect DRX-on duration. Interruption is allowed once every DRX cycle, and it is according to Tcycle,i. - Option 5: Interruptions may be allowed during DRX ON duration under the following conditions: - if DRX ON duration is smaller or equal to [5 or 10] ms and DRX cycle is larger than [40 or 80ms], - Interruptions are not allowed in the DRX ON duration, excluding the time extended due to drx-inactivityTimer - Otherwise - interruptions are allowed, except for the last slot containing PDCCH in the DRX ON duration, excluding the time extended due to drx-inactivityTimer - Option 6: Interruptions may be allowed during DRX ON duration under the following conditions: - if DRX ON duration is smaller or equal to [5 or 10] ms and DRX cycle is larger than [40 or 80ms], - Interruptions are not allowed in the DRX ON duration, excluding the time extended due to drx-inactivityTimer - Otherwise - interruptions are allowed Issue 1-1-2: Aligned DRX-on duration and SMTC for NFG measurements and DRX cycle is larger than 320ms - Proposals - Option 1: UE does not measure within SMTC occasions and no interruption is expected. - Option 2: Interruption is always allowed, and it is according to Tcycle,i. Issue 1-1-3: Aligned DRX-on duration and SMTC for NFG measurements and DRX cycle is equal or shorter than 320ms - Proposals - Option 1: Interruption is always allowed, and it is according to Tcycle,i. Issue 1-1-4: Consider DRX-on duration length as conditions to allow interruptions - Proposals - Option 1: if DRX ON duration is larger than 10 ms, interruptions are allowed, except for the last slot containing PDCCH in the DRX ON duration. Issue 1-1-5: Consider other DRX cycle length threshold as conditions to allow interruptions - Proposals - Option 1: consider 40ms instead of 320ms. Issue 1-1-6: Consider whether there is interruption on PDCCH reception as conditions to allow interruptions - Proposals - Option 1: - if it is not possible to place an interruption without affecting first and last PDCCH occasion in the DRX ON duration, it is not allowed to cause interruptions during DRX ON durations. - Interruptions during inactivity time periods allowing both interruptions and PDCCH allocations shall be placed so that the last PDCCH occasion in the inactivity time is not-interrupted. Issue 1-1-7: Scaling factor 1.5 when DRX is configured and equal or smaller than 320ms - Background - 1.5 is to address frequent measurements in legacy releases when DRX cycle length is smaller than 320ms. - Proposals - Option 1: Apply 1.5 to measurement and interruption requirements when DRX is configured and equal or smaller than 320ms.

In the issue list, issue 1-1-1, 1-1-4, 1-1-5 and 1-1-6 are considered related. These issues are summarized in tables below.

In this paper we will discuss the impact of interruptions colliding with different periods in the defined DRX operation.

During DRX operation, there are 4 defined periods:

- Drx-LongCycle: Maximum time between the UE monitoring PDCCH during DRX.

- DRX On-duration: the period the UE is monitoring PDCCH during DRX operation.

- DRX-InactivityTimer: Every time a device is scheduled during On-Duration, the DRX-InactivityTimer is started during which the UE continues to monitor PDCCH. Every time the UE is scheduled, the timer is restarted.

- Drx-ShortCycle: If the device is scheduled in the On-duration, the device will again schedule an On-duration period after a Drx-ShortCycle instead waiting for the next Drx-LongCycle occasion. The Drx-ShortCycle is an integer part of the Drx-LongCycle.

Of these periods, interruptions colliding with On-duration or DRX-InactivityTimer may cause the UE to miss out on a PDCCH scheduling the UE for uplink and/or downlink resources, which are then wasted and causing a potential delay before the UE again monitors PDCCH of either the long or short DRX cycle.

An interruption during the On-duration or while the DRX-InactivityTimer is running may cause the UE to miss out on PDCCH scheduling of uplink and/or downlink resources.

Missing a PDCCH will waste the scheduled resources from a NW perspective and cause a delay in data transfer of up to the Drx-LongCycle.

When discussing DRX cycles as in spec and in the WF, it is assumed to be the Drx-LongCycle.

The time values for the periods can be seen in Table 1.

-- ASN1START -- TAG-DRX-CONFIG-START DRX-Config ::= SEQUENCE { drx-onDurationTimer CHOICE { subMilliSeconds INTEGER (1..31), milliSeconds ENUMERATED { ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60, ms80, ms100, ms200, ms300, ms400, ms500, ms600, ms800, ms1000, ms1200, ms1600, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 } }, drx-InactivityTimer ENUMERATED { ms0, ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60, ms80, ms100, ms200, ms300, ms500, ms750, ms1280, ms1920, ms2560, spare9, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1}, drx-HARQ-RTT-TimerDL INTEGER (0..56), drx-HARQ-RTT-TimerUL INTEGER (0..56), drx-RetransmissionTimerDL ENUMERATED { sl0, sl1, sl2, sl4, sl6, sl8, sl16, sl24, sl33, sl40, sl64, sl80, sl96, sl112, sl128, sl160, sl320, spare15, spare14, spare13, spare12, spare11, spare10, spare9, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1}, drx-RetransmissionTimerUL ENUMERATED { sl0, sl1, sl2, sl4, sl6, sl8, sl16, sl24, sl33, sl40, sl64, sl80, sl96, sl112, sl128, sl160, sl320, spare15, spare14, spare13, spare12, spare11, spare10, spare9, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 }, drx-LongCycleStartOffset CHOICE { ms10 INTEGER(0..9), ms20 INTEGER(0..19), ms32 INTEGER(0..31), ms40 INTEGER(0..39), ms60 INTEGER(0..59), ms64 INTEGER(0..63), ms70 INTEGER(0..69), ms80 INTEGER(0..79), ms128 INTEGER(0..127), ms160 INTEGER(0..159), ms256 INTEGER(0..255), ms320 INTEGER(0..319), ms512 INTEGER(0..511), ms640 INTEGER(0..639), ms1024 INTEGER(0..1023), ms1280 INTEGER(0..1279), ms2048 INTEGER(0..2047), ms2560 INTEGER(0..2559), ms5120 INTEGER(0..5119), ms10240 INTEGER(0..10239) }, shortDRX SEQUENCE { drx-ShortCycle ENUMERATED { ms2, ms3, ms4, ms5, ms6, ms7, ms8, ms10, ms14, ms16, ms20, ms30, ms32, ms35, ms40, ms64, ms80, ms128, ms160, ms256, ms320, ms512, ms640, spare9, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 }, drx-ShortCycleTimer INTEGER (1..16) } OPTIONAL, -- Need R drx-SlotOffset INTEGER (0..31) }

Table 1: DRX-config taken from R17. As observed, the on-duration can be quite short, whereas the inactivity timer cannot be sub-millisecond.

In RAN4#109, it was identified that DRX configuration should be also considered for the interruption ratio. When DRX is configured the measurement cycles of the UE are typically extended for power saving, where the UE is not expected to measure every SMTC occasion in the case of DRX is configured. In that case the UE can choose one or more SMTC occasions for performing measurements within the DRX cycle, which are not necessarily aligned with the DRX ON duration of the DRX cycle. It is also possible that the SMTC configuration is such that the DRX ON duration is never overlapping with the SMTC. The DRX duration may also be as short as 1 ms, and if we consider the interruption length values that are under discussion, any interruption during the DRX ON would have high impact on the UE throughput performance. DRX ON duration can be as short as 1/32 ms.

drx-onDurationTimer can be as small as 1/32 ms

Interruption lengths of 0.25 to 1 ms during short DRX ON duration would have extremely large impact on UE throughput, since it could cover the whole DRX ON duration.

Figure 1: Example of every Drx-ShortCycle being scheduled.

Figure 1 shows one example of DRX behavior. In this figure the UE is configured with the Drx-LongCycle and Drx-ShortCycle. The UE is always required to be available during the DRX ON duration due to the long cycle, but the ON durations on Drx-ShortCycles are only activated if the UE receives PDCCH in the last DRX ON duration. As can be seen in Figure 1, every Drx-ShortCycle can be scheduled and therefore, the reminder of the document will assume Drx-ShortCycle, if configured, and else Drx-LongCycle for the discussion. This is of course only valid if the Drx-LongCycle is >= than 4 times the minimum of the Drx-ShortCycle, meaning Drx-LongCycle larger or equal to 8ms.

In this case, if the On-duration is configured at 1ms, there will be 1ms left between potential On-duration periods. This is exactly enough for an interruption.

Figure 2 Diagram representing DRX with ON duration, inactivity timer, long and short DRX cycle times.

When DRX is configured, every time the UE is scheduled during the DRX ON period, the active part of the DRX cycle is extended by the DRX inactivity timer. Safeguarding the DRX ON duration, together with the extensions due to the inactivity timer from interruptions can avoid extremely long latencies.

If the UE causes an interruption during the DRX ON period it could miss the PDCCH transmission. If an interruption, which is not a scheduling restriction period on the NW side, is colliding with a PDCCH occasion, especially during long DRX cycles, the device will lose the opportunity for UL and DL grants. If the PDCCH occasion is the only PDCCH occasion during the on-period of the long DRX period, the device is not reachable until next on-period of the DRX configuration, which will result in a long latency for retransmission of data.

The effect of missing PDSCH is not as severe as missing PDCCH due to an interruption. If the UE receives PDCCH it will be aware of the extension of the activity time and could receive the retransmission. If the UE misses the reception of PDCCH due to an interruption, it will not be aware that the DRX ON is extended due to the DRX inactivity timer and might miss data retransmissions. Additionally, since the DRX cycle can be configured as multiples of the SMTC periodicity, it is possible that in some more extreme cases the UE will systematically place an interruption on top of the DRX ON duration. That could even cause that one DL grant would be delayed for several DRX cycles.

Figure 3 Example of impact of interruptions during DRX ON with SMTC periodicity configured with same periodicity as DRX cycle.

Figure 3 shows one example of the impact of interruptions during DRX ON. In this example the SMTC and DRX have the same periodicity, and the DRX ON is located at a position where the UE is systematically causing interruptions during DRX ON. If there would be no interruptions, the UE would be able to receive data during or shortly after the first DRX ON, which means that the data latency would be about 1 DRX cycle in wort case. Here the UE loses the PDCCH scheduling the UE for DL grants in 2 consecutive DRX cycles. For each PDCCH lost on DRX ON, the network may attempt several PDSCH retransmissions during the DRX inactivity timer. As a result, the total latency is increased from less than 1 DRX cycle to between 2 and 3 DRX cycles. Additionally, the network is also wasting its resources when the UE is not available.

Interruptions on PDCCH on the DRX cycle can cause the UE to experience a further delay in UL and DL grants as long as the drx-LongCycle, which can be configured from 10 ms to 10 s.

The impact of interruption is more severe on PDCCH than for PDSCH during DRX activity time.

If the minimum cycle length is configured to be small, there may not be sufficient time for interruptions between on-durations. In that case, the only option is to allow interruptions during DRX ON durations.

If DRX cycles are very small, there may not be sufficient time between DRX ON durations to allow for interruptions and therefore, interruptions should be allowed during DRX ON durations if DRX cycle are very small.

However, if there is a sufficiently long DRX cycle length, the DRX ON duration should be protected against interruptions to reduce the likelihood of missed opportunities for UL and DL grants, causing further delay in data transfer, DL and UL.

Another concern is if the DRX ON duration is very long. As can be seen in Table 1, the DRX ON duration can be from sub-milliseconds to 1.6 seconds. That is a potential long time to have a second RF path enabled seen from a power consumption viewpoint.

Therefore, a compromise is suggested, that if the DRX ON duration is very long and if it is possible to place an interruption between first and last PDCCH in the DRX ON duration, interruptions are allowed to be placed within the DRX On duration.

RAN4 to consider rules of when interruptions are allowed in when DRX is configured depending on the DRX ON duration, and DRX cycle configuration.

Interruptions may be allowed during DRX ON duration under the following conditions:

  - if DRX ON duration is smaller than or equal to 10 ms and the DRX cycle is larger than 40 ms:

    - Interruptions are not allowed in the DRX ON duration.

  - Otherwise:

    - Interruptions are allowed in the DRX ON duration, except for the last slot containing PDCCH in the DRX ON duration.

Requirements with dual connectivity

The current requirements for interruptions due to measurements without gaps are captured in the NR Standalone clause in 38.133. It is not clear in the current state of the specification whether the interruption requirements also apply when the UE is configured with NR-DC, EN-DC, NE-DC.

In the latest spec version, interruption requirements are only defined for SA carrier aggregation scenario.

Interruption requirements in 8.2.2.2.19 shall also apply for NR-DC, EN-DC, and NE-DC considering that operations in one cell group do not impact operations on another cell group.

Definition of measurements without gaps with interruptions

In RAN4 #106, the following agreement was reached regarding interruption behavior

Issue 1-5-1: Condition for intra-frequency requirements without gaps < Agreement >: - When the target SSB is completely contained in active BWP of UE or the active downlink BWP is initial BWP, the intra-frequency measurement should be without gap regardless of the NeedForGaps’ status reporting. - When the target SSB is outside active BWP (not initial BWP), the intra-frequency measurement will be - without gap without interruption, if UE reports ‘[no gap no interruption:TBD]’ for the intra-band; - without gap with interruption, if UE reports ‘[no gap with interruption:TBD]’ for the intra-band

Even though we have reached this agreement in RAN4 #106, this was not captured in the specification, and should be considered for the definition of the RRC core requirements.

Agreement from RAN4#106 that intra-frequency measurements without gaps don’t cause interruption when target SSB is completely contained withing the active BWP of the UE is not captured in the latest specification.

Capture in the specification the agreement from RAN4#106 that intra-frequency measurements without gaps don’t cause interruption when target SSB is completely contained withing the active BWP of the UE.

Scenarios for measurements without gaps

In the CRs agreed in the last meetings, some text was added regarding HST and FR2-2. In our view, RAN4 has never actively discussed the applicability of measurements without gaps with interruptions for HST and FR2-2. Those scenarios differ from the usual FR1 and FR2-1 discussion. For HST scenario, mobility is a very important issue, and extension of measurement delays due to the minimum measurement cycle can affect the system performance. Additionally, in FR2-2 beam sweeping factor is increased from 8 to 12. That means that measurement delays are already extended when operating in frequencies above 52.6 GHz, and RAN4 didn’t discuss whether this would be a problem to set a lower bound on measurement cycle for the scenario with extended beam sweeping scaling factor.

Since this work item is officially closed, we think that the best solution is that all the references regarding HST and FR2-2 in combination with measurements without gaps and interruptions are removed from 38.133 and 36.133.

RAN4 didn’t discuss whether HST and FR2-2 should be considered for supporting Rel 18 measurements without gaps with interruptions.

Rel 18 measurements without gaps with interruptions do not apply for HST.

Rel 18 measurements without gaps with interruptions do not apply for operation above 52.6 GHz.

Conclusion

In the paper, the following Observations and Proposals were made:

- An interruption during the On-duration or while the DRX-InactivityTimer is running may cause the UE to miss out on PDCCH scheduling of uplink and/or downlink resources.

Missing a PDCCH will waste the scheduled resources from a NW perspective and cause a delay in data transfer of up to the Drx-LongCycle.

drx-onDurationTimer can be as small as 1/32 ms

Interruption lengths of 0.25 to 1 ms during short DRX ON duration would have extremely large impact on UE throughput, since it could cover the whole DRX ON duration.

Interruptions on PDCCH on the DRX cycle can cause the UE to experience a further delay in UL and DL grants as long as the drx-LongCycle, which can be configured from 10 ms to 10 s.

The impact of interruption is more severe on PDCCH than for PDSCH during DRX activity time.

If DRX cycles are very small, there may not be sufficient time between DRX ON durations to allow for interruptions and therefore, interruptions should be allowed during DRX ON durations if DRX cycle are very small.

- RAN4 to consider rules of when interruptions are allowed in when DRX is configured depending on the DRX ON duration, and DRX cycle configuration.

Interruptions may be allowed during DRX ON duration under the following conditions:

  - if DRX ON duration is smaller than or equal to 10 ms and the DRX cycle is larger than 40 ms:

    - Interruptions are not allowed in the DRX ON duration.

  - Otherwise:

    - Interruptions are allowed in the DRX ON duration, except for the last slot containing PDCCH in the DRX ON duration.

In the latest spec version, interruption requirements are only defined for SA carrier aggregation scenario.

Interruption requirements in 8.2.2.2.19 shall also apply for NR-DC, EN-DC, and NE-DC considering that operations in one cell group do not impact operations on another cell group.

Agreement from RAN4#106 that intra-frequency measurements without gaps don’t cause interruption when target SSB is completely contained withing the active BWP of the UE is not captured in the latest specification.

Capture in the specification the agreement from RAN4#106 that intra-frequency measurements without gaps don’t cause interruption when target SSB is completely contained withing the active BWP of the UE.

RAN4 didn’t discuss whether HST and FR2-2 should be considered for supporting Rel 18 measurements without gaps with interruptions.

Rel 18 measurements without gaps with interruptions do not apply for HST.

Rel 18 measurements without gaps with interruptions do not apply for operation above 52.6 GHz.

- References

- R4-2406442, WF on NR_MG_enh2_part2, Intel

- R4-2405798, Discussion on measurements without gaps, Nokia, Nokia Shanghai Bell