3GPP TSG-RAN WG4 Meeting #111	R4-2409164

# Fukuoka, Japan, May 20 – 24, 2024

Source:	Nokia

Title:	Simulation results for PDSCH for eRedCap

Agenda item:	7.21.2.1

Document for:	Information

Introduction

The work item on enhanced support for reduced capability NR devices (eRedCap) [1] has progressed and the core requirement part was finalized in RAN4. Thus, demodulation performance requirements for UE from eRedCap are being discussed. The agreed WF in RAN4 #110bis [2] contains agreements on UE performance requirements for eRedCap. Among them the PDSCH performance needs to be evaluated due to the restricted number of PRBs or restricted peak rate, respectively, for eRedCap UE.

This contribution contains our ideal PDSCH results for alignment based on the agreed simulation assumptions [2]. The results do not contain any RF/BB impairments. It is proposed to include them in the simulation summary document for eRedCap.

Simulation Results

Ideal PDSCH simulation results for alignment are provided in this section.

PDSCH performance for UE not supporting eRedCapNotReducedBB-BW-r18 (FG 48-1)

Results for FDD/HD-FDD SCS=15 kHz are shown in Table 1 and for TDD SCS 30 kHz in Table 2.

| Modulation | Modulation | Code rate | Rank | PRB size | TBS | Propagation | Received antennas | SNR @70%of max TP(dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | MCS4 in Table 1 | 1/3 | Rank 1 | 25 | 1928 | TDLB100-400 | 1Rx | 1.7 |
|  |  |  |  |  |  |  | 2Rx | -2.3 |
| 16QAM | MCS13 in Table 1 | 0.47 | Rank 1 | 25 | 6272 | TDLC300-100 | 1Rx | 9.5 |
|  |  |  |  |  |  |  | 2Rx | 5.0 |
| 64QAM | MCS19 in Table 1 | 0.50 | Rank 1 | 25 | 9992 | TDLA30-10 | 1Rx | 12.6 |
| 64QAM | MCS19 in Table 1 | 0.50 | Rank 2 | 12 | 9480 | TDLA30-10 | 2Rx | 16.8 |

Table 1: PDSCH results for eRedCap FG 48-1 with FDD/HD-FDD SCS=15kHz

| Modulation | Modulation | Code rate | Rank | PRB size | TBS | Propagation | Received antennas | SNR @70%of max TP(dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | MCS4 in Table 1 | 1/3 | Rank 1 | 12 | 928 | TDLB100-400 | 1Rx | 1.9 |
|  |  |  |  |  |  |  | 2Rx | -2.2 |
| 16QAM | MCS13 in Table 1 | 0.47 | Rank 1 | 12 | 3104 | TDLC300-100 | 1Rx | 9.8 |
|  |  |  |  |  |  |  | 2Rx | 5.4 |
| 64QAM | MCS19 in Table 1 | 0.50 | Rank 1 | 12 | 4736 | TDLA30-10 | 1Rx | 12.2 |
| 64QAM | MCS19 in Table 1 | 0.50 | Rank 2 | 6 | 4736 | TDLA30-10 | 2Rx | 15.3 |

Table 2: PDSCH results for eRedCap FG 48-1 with TDD SCS=30kHz

PDSCH performance for UE supporting eRedCapNotReducedBB-BW-r18 (FG 48-2)

Results for FDD/HD-FDD SCS=15 kHz are shown in Table 3 and for TDD SCS 30 kHz in Table 4.

| Modulation | Modulation | Code rate | Rank | PRB size | TBS | Propagation | Received antennas | SNR @70%of max TP(dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | MCS4 in Table 1 | 1/3 | Rank 1 | 52 | 3904 | TDLB100-400 | 1Rx | 1.1 |
|  |  |  |  |  |  |  | 2Rx | -2.6 |
| 16QAM | MCS13 in Table 1 | 0.47 | Rank 1 | 40 | 9992 | TDLC300-100 | 1Rx | 9.7 |
|  |  |  |  |  |  |  | 2Rx | 5.2 |
| 64QAM | MCS19 in Table 1 | 0.50 | Rank 1 | 25 | 9992 | TDLA30-10 | 1Rx | 12.6 |
| 64QAM | MCS19 in Table 1 | 0.50 | Rank 2 | 12 | 9480 | TDLA30-10 | 2Rx | 16.8 |

Table 3: PDSCH results for eRedCap FG 48-2 with FDD/HD-FDD SCS=15kHz

| Modulation | Modulation | Code rate | Rank | PRB size | TBS | Propagation | Received antennas | SNR @70%of max TP(dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | MCS4 in Table 1 | 1/3 | Rank 1 | 51 | 3840 | TDLB100-400 | 1Rx | 1.1 |
|  |  |  |  |  |  |  | 2Rx | -2.6 |
| 16QAM | MCS13 in Table 1 | 0.47 | Rank 1 | 20 | 4992 | TDLC300-100 | 1Rx | 9.5 |
|  |  |  |  |  |  |  | 2Rx | 5.3 |
| 64QAM | MCS19 in Table 1 | 0.50 | Rank 1 | 12 | 4736 | TDLA30-10 | 1Rx | 12.2 |
| 64QAM | MCS19 in Table 1 | 0.50 | Rank 2 | 6 | 4736 | TDLA30-10 | 2Rx | 15.3 |

Table 4: PDSCH results for eRedCap FG 48-2 with TDD SCS=30kHz

- References

| [1] | RP-223544, “Revised WID on Enhanced support of reduced capability NR devices”, Ericsson, RAN #98-e |
| --- | --- |
| [2] | R4-2406052, "Way Forward for [110bis][330] NR_redcap_enh_demod ", Ericsson |