3GPP TSG-RAN WG4 Meeting #111                                        R4-2409174

Fukuoka, Japan, May 20 - 24, 2024

Source: 	Huawei, HiSilicon

Title: 	On 6Rx SRS antenna switching requirements

Agenda Item:	10.1.1.4.3

Document for:	Approval

Introduction

During RAN4#110bis, FR1 6Rx UE RF requirements have been discussed. Following conclusion regarding ΔTRxSRS can be found in [1].

- Topic 2: SRS antenna switching and ΔTRxSRS - Sub-topic 2-1: SRS antenna switching configurations Way forward: Further discuss the following options with a focus on 1tr6 and t2r6 for RAN4#111. - Option 1: Preclude t3r6 and t4r6 SRS antenna switching configuration for 6Rx in Rel-19 and update the WID objective in the next plenary. - Option 2: Preclude 3T6R AS-SRS from 6 Rx ∆TRxSRS requirements discussion in Rel-19 and update the WID, while inform RAN1 to start the work on enabling 4T6R AS-SRS, while RAN2 can wait for future inputs from RAN1 and RAN4. - Option 3: RAN4 discussion on 6 Rx ∆TRxSRS requirements should focus on 1T6R and 2T6R before RAN1 conclusion on how to support 4T6R can be available. - Option 4: To specify the ∆TRxSRS for 6Rx, it is necessary to analyse the 6Rx SRS antenna switching IL below scenarios. - t1r6 - t2r6 - t4r6 - t1r6-t2r6 - t1r6-t2r6-t4r6 - t2r6-t4r6 - t1r6-t4r6 - Sub-topic 2-2: ΔTRxSRS values Way forward: Further discuss the ∆TRxSRS requirements.

In this contribution, we would like to share our views regarding this topic.

Discussion

While RAN1 will consider introducing 3T6R/4T6R SRS for antenna switching in Rel-19 as asked in the approved LS [2], we provide our preference on the prioritized 1T6R and 2T6R ΔTRxSRS requirements.

As illustrated in our contribution about REFSENS requirements [3], following aspects should be carefully considered at least for handheld UE:

- Due to form factor limitation, physical antenna layout in order to accommodate increased Rx number could lead to compromised performance on:

  - RF isolation

  - Antenna efficiency

And it was shared by some companies that the aforementioned implementation difficulties could be more serious for higher frequency like n104.

Observation: Ultra high frequency band like n104 may need further check for the ΔTRxSRS requirements.

Therefore, our intuitive idea is to reuse what have been specified for 8 Rx and FFS on the higher frequency band n104. So we have the following proposal.

Proposal: Adopt the following framework regarding ∆TRxSRS requirements.

- When antenna switching SRS capability is indicated as 't1r6' or ‘t2r6’:

  - When the device is capable of power class 3 or power class 5 or power class 1.5 in the band, or when the device is capable of power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating txDiversity-r16.

    - For the bands whose FUL_high is lower than the FUL_low of n79, the value of ∆TRxSRS is [4] dB.

    - For the bands whose FUL_high is lower than the FUL_low of n104, the value of ∆TRxSRS is [5.5] dB.

    - For the bands whose FUL_high is higher than the FUL_low of n104, the value of ∆TRxSRS is [TBD] dB.

- When antenna switching SRS capability is indicated as 't1r6-t2r6':

  - When the device is capable of power class 3 or power class 5 or power class 1.5 in the band, or when the device is capable of power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating txDiversity-r16.

    - For the bands whose FUL_high is lower than the FUL_low of n79, the value of ∆TRxSRS is [4.5] dB.

    - For the bands whose FUL_high is lower than the FUL_low of n104, the value of ∆TRxSRS is [6] dB.

    - For the bands whose FUL_high is higher than the FUL_low of n104, the value of ∆TRxSRS is [TBD] dB.

Conclusion

In this contribution we discussed on the 6Rx SRS antenna switching requirements. We have the following observation and proposal:

Observation: Ultra high frequency bands e.g. n104 may need further check for the ΔTRxSRS requirements.

Proposal: Adopt the following framework regarding ∆TRxSRS requirements.

- When antenna switching SRS capability is indicated as 't1r6' or ‘t2r6’:

  - When the device is capable of power class 3 or power class 5 or power class 1.5 in the band, or when the device is capable of power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating txDiversity-r16.

    - For the bands whose FUL_high is lower than the FUL_low of n79, the value of ∆TRxSRS is [4] dB.

    - For the bands whose FUL_high is lower than the FUL_low of n104, the value of ∆TRxSRS is [5.5] dB.

    - For the bands whose FUL_high is higher than the FUL_low of n104, the value of ∆TRxSRS is [TBD] dB.

- When antenna switching SRS capability is indicated as 't1r6-t2r6':

  - When the device is capable of power class 3 or power class 5 or power class 1.5 in the band, or when the device is capable of power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating txDiversity-r16.

    - For the bands whose FUL_high is lower than the FUL_low of n79, the value of ∆TRxSRS is [4.5] dB.

    - For the bands whose FUL_high is lower than the FUL_low of n104, the value of ∆TRxSRS is [6] dB.

    - For the bands whose FUL_high is higher than the FUL_low of n104, the value of ∆TRxSRS is [TBD] dB.

- References

- R4-2406585, “WF on requirements for 6Rx”, AT&T, 3GPP TSG-RAN4 Meeting #110bis, Changsha, China, April 15-19, 2024.

- R4-2406718, “LS on 3T6R and 4T6R antenna switching SRS”, 3GPP TSG-RAN4 Meeting #110bis, Changsha, China, April 15-19, 2024.

- R4-2409172, “On 6Rx reference sensitivity requirements for FR1 UE”, Huawei, HiSilicon, 3GPP TSG-RAN4 Meeting #111, Fukuoka, Japan, May 20-25, 2024.