3GPP TSG-RAN WG4 Meeting # 111						R4-2409211

Fukuoka Meeting, May 20th – May 24th, 2024

Title:	TP to TR 38.898: Addition of PC2 for DC_(n)40AA

Source:	Nokia, nbn

Agenda item:	6.15.2

Document for:	Approval

# 1	Introduction

This is a TP to TR 38.898 to add PC2 for DC_(n)40AA. DC_(n)40AA PC3 is completed as a fallback R4-2408453.

************************************* Start of TP*****************************************

## 5.x	DC_(n)40

### 5.x.1	Configurations

Table 5.x.1-1: Intra-band contiguous EN-DC configurations

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_(n)40AA7 | DC_(n)40AA6,7 | Yes6 |
| NOTE 6:Only single switched UL is supported NOTE 7: The PC2 Uplink EN-DC configuration supported in Table 6.2B.1.1-1 is applicable to the same EN-DC configuration. |  |  |

### 5.x.2	Maximum output power

Table 5.x.2-1: Maximum output power for EN-DC (continuous sub-blocks)

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_(n)40AA3 | 26 | +2/-3 | 23 | +2/-3 |
| NOTE 3: Only single switched UL is supported. |  |  |  |  |

### 5.x.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is not needed due to higher power UL DC. Since DC_(n)40AA is to support only single switched UL, there should be no MSD interference issues occurred. Hence, additional MSD is not needed.

### 5.x.4	∆TIB and ∆RIB values

ΔTIB,c and ΔRIB,c are not applicable for intra-band contiguous EN-DC, so this section is omitted.

************************************* End of TP*****************************************

# References