3GPP TSG-RAN4 Meeting #111		R4-2409231

Fukuoka, Japan, May 20th – 24th, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 4554 | rev |  | Current version: | 17.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_redcap-Core) eDRX requirements for CG-SDT for non-RedCap UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap -Core |  |  |  |  |  | Date: |  |  | 2024-05-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | eDRX requiremens for non-RedCap UE in RRC INACTIVE state were introduced in R4-2321628. In this CR the CG-SDT requirements in INACTIVE mode are updated to take into account the eDRX support. The basic requirements are reused from corresonding RedCap CGT-SDT section in clause 5.2B.3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | TA validation formula is updated to take into account the eDRX configuration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear or no requirements when the UE shall perform measuremnets when configured with eDRX. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

### 5.5.3	TA validation requirements

When cg-SDT-RSRP-ChangeThreshold [TS 38.331] is configured for TA validation based on the RSRP change criterion according to clause 5.8.2 in [TS 38.321], the UE is allowed to transmit using CG-SDT using the timing derived using the latest available NTA value as specified in subclause 7.1 provided that

-	the first RSRP (RSRP1) measurement and the second RSRP (RSRP2) measurements used in the TA validation are valid measurements and,

-	timing alignment validation for transmission using CG-SDT is valid according to the validation criteria in clause 5.8.2 in [TS 38.321].

RSRP1 and RSRP2 are considered valid provided that the conditions in Table 5.5.3-1 are met for FR1 when UE is not configured with any eDRX cycle by serving gNB or core network.

RSRP1 and RSRP2 are considered valid provided that the conditions in Table 5.5.3-2 are met for FR2-1 regardless of whether the UE is configured with eDRX cycle by serving gNB or core network (AMF).

RSRP1 and RSRP2 are considered valid provided that the conditions in Table 5.5.3-3 are met for FR1 when UE is configured:

-	only with RAN eDRX cycle (TeDRX-RAN) by the serving gNB, or

-	with both TeDRX-RAN by serving gNB and CN eDRX cycle (TeDRX-CN) by the core network (AMF)

Table 5.5.3-1 Valid measurement for FR1 for UE not configurd with eDRX cycle

| Measurement | FR1 |
| --- | --- |
| RSRP1 | (T1 – min(640ms, M1*TDRX)) ≤ T1’ ≤ (T1 + min(640ms, M1*TDRX)) |
| RSRP2 | (T2 – min(640ms, M1*TDRX)) ≤ T2’ ≤ T2 |

Table 5.5.3-2 Valid measurement for FR2-1

| Measurement | FR2-1 |
| --- | --- |
| RSRP1 | (T1 – max(480ms, 8*SMTC periodicity)) ≤ T1’ ≤ (T1 + max(480ms, 8*SMTC periodicity)) |
| RSRP2 | (T2 – max(480ms, 8*SMTC periodicity)) ≤ T2’ ≤ T2 |

Table 5.5.3-3 Valid measurement for FR1 for UE configured with eDRX_cycle

| Measurement | FR1 |
| --- | --- |
| RSRP1 | (T1 – min(640ms, M1* TeDRX-RAN)) ≤ T1’ ≤ (T1 + min(640ms, M1* TeDRX-RAN)) |
| RSRP2 | (T2 – min(640ms, M1* TeDRX-RAN)) ≤ T2’ ≤ T2 |

If at least one of RSRP1 and RSRP2 is considered to be invalid based on the above conditions, then the UE shall not validate the CG-SDT using RSRP1 and RSRP2 and shall not transmit using CG-SDT. Additionally, for the initial CG-SDT transmission, the UE shall not transmit in an CG-SDT occasion that occurs more than 640 ms after T2.

Where:

-	T1 is the time when

-	RRCRelease with CG-SDT configuration (TS 38.331 [2]) is received, or

-	the latest TA is received if TA is received while in RRC_INACITVE state.

-	T1’ is the time when the UE has completed RSRP1.

-	T2 is the time when the UE performs TA validation as defined in clause 5.27.2 in (TS 38.321 [7]) for transmission using CG-SDT.

-	T2’ is the time when the UE has completed RSRP2.

-	TDRX is the DRX cycle length in ms.

-	M1 the scaling factor as defined in clause 4.2.2.2.

<End of Change 1>