3GPP TSG-RAN WG4 Meeting #111	R4-2409276

Fukuoka, Japan, 20 – 24 May, 2024

Title: 	additional simulation results for RedCap positioning

Source: 	Huawei, HiSilicon

Agenda item:	7.12.2.4

Document for:	Discussion

Introduction

Simulation assumption for evaluating the link level performance of PRS measurement with FH by RedCap UE is approved in RAN4#108 [1]. In RAN4#109 we provided our simulation results for 2RX UE [2]. In this paper we will provide our simulation results for 1Rx UE with FH.

Discussion

In this paper, we present our initial simulation results for

- RSTD and UE Rx-Tx

- FR1 (1RX is not applicable for FR2)

- under AWGN and fading channels

- with 1 and 4 samples

All the results are for 1RX.

Simulation results for RSTD

FR1

Table 1: RSTD error for 4-sample, Es/Iot of (-6,-13)dB, AWGN, FR1

| Parameter | Value |  |  | error |
| --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num per hop | RB num in total | 90% |
|  | 15 | 52 | 268 | 60 |
|  | 30 | 52 | 272 | 27.6 |
|  | 60 | 24 | 132 | 23.5 |

Table 2: RSTD error for 4-sample, Es/Iot of (-6,-10)dB, TDL-A, FR1

| Parameter | Value |  |  | error |
| --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num per hop | RB num in total | 90% |
|  | 15 | 52 | 268 | 103 |
|  | 30 | 52 | 272 | 112 |
|  | 60 | 24 | 132 | 999 |

Table 3: RSTD error for 1-sample, Es/Iot of (-3,-6)dB, AWGN, FR1

| Parameter | Value |  |  | error |
| --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num per hop | RB num in total | 90% |
|  | 15 | 52 | 268 | 58.4 |
|  | 30 | 52 | 272 | 30.4 |
|  | 60 | 24 | 132 | 36.3 |

Simulation results for UE Rx-Tx

FR1

Table 7: UE Rx-Tx error for 4-sample, Es/Iot of -13dB and -3dB, AWGN, FR1

| Parameter | Value |  |  |  | error |
| --- | --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | Es/Iot (dB) | SCS (kHz) | RB num per hop | RB num in total | 90% |
|  | -3 | 15 | 52 | 268 | 13.4 |
|  |  | 30 | 52 | 272 | 9.4 |
|  |  | 60 | 24 | 132 | 22.5 |
|  | Es/Iot (dB) | SCS (kHz) | RB num per hop | RB num in total | 90% |
|  | -13 | 15 | 52 | 268 | 69.2 |
|  |  | 30 | 52 | 272 | 35.7 |
|  |  | 60 | 24 | 132 | 35.7 |

Table 8: UE Rx-Tx error for 4-sample, Es/Iot of -10dB and -3dB, TDL-A, FR1

| Parameter | Value |  |  |  | error |
| --- | --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | Es/Iot (dB) | SCS (kHz) | RB num per hop | RB num in total | 90% |
|  | -3 | 15 | 52 | 268 | 51 |
|  |  | 30 | 52 | 272 | 32 |
|  |  | 60 | 24 | 132 | 24.6 |
|  | Es/Iot (dB) | SCS (kHz) | RB num per hop | RB num in total | 90% |
|  | -10 | 15 | 52 | 268 | 95 |
|  |  | 30 | 52 | 272 | 68 |
|  |  | 60 | 24 | 132 | 999 |

Table 9: UE Rx-Tx error for 1-sample, Es/Iot of -6dB and 0dB, AWGN, FR1

| Parameter | Value |  |  |  | error |
| --- | --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | Es/Iot (dB) | SCS (kHz) | RB num per hop | RB num in total | 90% |
|  | 0 | 15 | 52 | 268 | 12 |
|  |  | 30 | 52 | 272 | 10 |
|  |  | 60 | 24 | 132 | 21.5 |
|  | Es/Iot (dB) | SCS (kHz) | RB num per hop | RB num in total | 90% |
|  | -6 | 15 | 52 | 268 | 49 |
|  |  | 30 | 52 | 272 | 26 |
|  |  | 60 | 24 | 132 | 49 |

Conclusions

It is suggested to take above simulation results into account when defining accuracy requirements for 1RX RedCap UE with FH.

Reference

- R4-2314460, Simulation assumptions for RedCap positioning with FH, Ericsson

- R4-2319997, Simulation results for PRS measurement with FH, Huawei, HiSilicon