3GPP TSG-RAN WG4 Meeting #111                                R4-2409302

Fukuoka, Japan, 20-24 May, 2024

Source:	 KDDI, LGE, Samsung

Title:	TP for TR 38.898: DC_1-3_n41

Agenda item:			6.15.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.898 to include DC_1-3_n41 according to the request in [1].

The corresponding 3-band PC3 combos have already been specified.

The 2-band PC2 DL_1A_n41A with UL_1A_n41A and DL_3A_n41A with UL_3A_n41A have already been specified in [2].

There is no pending fallbacks.

2. Reference

[1] RP-240241 Revised WID on High power UE (power class m with 1<m<3) for a single FR1 band in UL of Dual Connectivity (DC) combinations of x bands (x=1,2,3, 4 for y=1 or x=1, 2 for y=2) LTE inter-band CA (xDL/1UL) and y bands NR inter-band CA (yDL/1UL)

[2] 3GPP TS 38.101-3 V18.5.1 (2024-03)

# 3. Text Proposal

<Start of Text Proposal>s

### 5.x   DC_1-3_n41

#### 5.x.1	Configuration for DC

Table 5.x.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A_n41A14 DC_1A-3C_n41A14 | DC_1A_n41A14 DC_3A_n41A14 DC_3C_n41A14 |
| NOTE1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 14: Minimum requirements for PC2 are applicable for this uplink EN-DC configuration in this downlink/uplink EN-DC configuration. |  |

#### 5.x.2	Maximum output power for DC

Since the maximum output power requirements for DC_1_n41 and DC_3_n41 have already been specified, this section can be omitted.

#### 5.x.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Based on co-existence studies of DC_1_n41, DC_3_n41 own Rx impact of the 3rd band is the followings. 6th order IMD generated by dual uplink of band 1 and band n41 impacts the Rx of band 3. Similarly, 6th order IMD generated by dual uplink of band 3 and band n41 impacts the Rx of band 1. However, the MSD for IMD6 need not be defined.

#### 5.x.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC.

<End of Text Proposal>