3GPP TSG-RAN WG4 Meeting #111	R4-2409348

Fukuoka, Japan, 20 – 24 May 2024

Source:	Ericsson, BT plc

Title:	TP for 38.899 to add PC2 and PC1.5 UL to CA_n1-n28-n78

Agenda item:			6.17.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for 38.899 to add PC2 and PC1.5 UL to CA_n1-n28-n78.

# 2. Text Proposal

# ---Start of changes---

## . 6.x	CA_n1-n28-n78

### 6.x.1	Configurations

Table 6.x.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n28A-n78A | n787,9 CA_n1A-n28A CA_n1A-n78A7 CA_n28A-n78A7 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 202 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20 | 1 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 2 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n28A-n78(2A) | n787,9 CA_n78(2A)7 CA_n1A-n28A CA_n1A-n78A7 CA_n28A-n78A7 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.x.2	Maximum output power

Table 6.x.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n1-n78 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n28-n78 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.x.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.x.3.1	Power class 2 case a, b

Based on calculation, IMD5 of dual UL CA_n1-n78 fall into n28 DL, IMD3 of dual UL CA_n28-n78 falls into n1 DL.

Coexistence analysis for UL n78 and CA_n78(2A) for lower order fallbacks are performed in R4-2409353.

MSD for PC2 UL CA is calculated from MSD for PC3 as follows:

If the input signal increases by 3 dB, the IMD3 interference power increases by 3*3=9 dB.

MSD due to interference power is given by

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have

If is increased by dB, then is given by

Table 6.25.3-1 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n28-n78 | n1 | N/A | 5 | N/A | 2150 | 24.6 | FDD | IMD3 |
|  | n28 | 740 | 5 | 25 | 795 | N/A | FDD | N/A |
|  | n78 | 3630 | 10 | 50 | 3630 | N/A | TDD | N/A |
|  | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n28 | N/A | 5 | N/A | 794 | 17.2 | FDD | IMD5 |
|  | n78 | 3352 | 10 | 50 | 3352 | N/A | TDD | N/A |

### 6.25.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

---End of changes---

# Reference

[1] 		RP-240355, Revised WID HPUE_FR1_TDD_NR_CADC_SUL_R18 RAN#103