3GPP TSG-RAN WG4 Meeting #111 	R4-2409371

Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024

Agenda Item:	7.12.2.2

Source: 	Ericsson

Title:	On SL positioning performance issues

Document for:	Discussion

# Introduction

The latest agreed WF is in [17].

In this contribution, we discuss issues related to SL positioning performance.

# Discussion

## Side conditions for SL PRS measurements in RAN4 requirements

Side conditions need to be defined for core requirements and accuracy requirements. The agreement from [17] is:

Agreements:

- SL RSTD:

  - Option 1: (0, -6) dB for reference and another anchor UE

  - Option 2: (-3, -6) dB for reference and another anchor UE

- SL UE Rx-Tx and SL RSRP(P):

  - FFS -6 dB,

  - FFS -3 dB

- SL AoA and SL RTOA (for core requirements only):

  - -6 dB

Side conditions for SL RSTD, SL Rx-Tx, SL PRS-RSRP, and SL PRS-RSRPP are still open.

- Proposal 1 (side conditions): For SL RSTD, the side conditions assumed in RAN4 requirements are: ≥0 dB for the reference link and ≥-6 dB for the measured link.

- Proposal 2 (side conditions): For SL Rx-Tx, the requirements are defined down to -6 dB.

- Proposal 3 (side conditions): Given that SL PRS-RSRP can be configured with other SL-PRS based measurements, the requirements for SL PRS-RSRP can be defined down to -6 dB.

- Proposal 4 (side conditions): Given that SL PRS-RSRPP can be configured with other SL-PRS based measurements, the requirements for SL PRS-RSRPP can be defined down to -6 dB.

## Measurement accuracy requirements

The core requirements for SL positioning measurements were defined for S=1 and S=4 samples. The accuracy requirements should be defined for the same numbers of samples.

- Proposal 5 (measurement accuracy requirements): Two sets of accuracy requirements are to be defined for SL positioning measurements, i.e., for:

  - 1 sample (>48 PRBs), and

  - 4 samples (≤48 PRBs).

- Proposal 6 (measurement accuracy requirements): The measurement accuracy requirements can be specified in the following format:

| Accuracy | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | SL-PRS Ês/Iot | SL-PRS SCS | SL-PRS bandwidth Note 1 | Number of samples, S | Io Note 2 range |  |  |
|  |  |  |  |  | NR operating band groups Note 3 | Minimum Io | Maximum Io |
| Tc Note 4 | dB | kHz | RB |  |  | dBm/SCS | dBm/BWChannel |
| TBD | (SL-PRS Ês/Iot)ref ≥ 0 dB (SL-PRS Ês/Iot)i ≥ -6 dB | 15 | ≥ 24 | ≥ 4 | NR_TDD_FR1_B | -126.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_J | -122.5 | -50 |
| TBD |  |  | >48 | ≥ 1 | Note 5 | Note 5 | Note 5 |
| TBD TBD |  | 30 | ≥ 24 | ≥ 4 | NR_TDD_FR1_B | -123.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_J | -119.5 | -50 |
| TBD |  |  | >48 | ≥ 1 | Note 5 | Note 5 | Note 5 |
| TBD |  | 60 | ≥ 24 | ≥ 4 | NR_TDD_FR1_B | -120.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_J | -116.5 | -50 |
| TBD |  |  | >48 | ≥ 1 | Note 5 | Note 5 | Note 5 |
| NOTE 1: Minimum SL-PRS bandwidth, which is the minimum of the SL-PRS bandwidths of the reference resource and the measured neighbour resource i. NOTE 2: Io is assumed to have constant EPRE across the bandwidth. NOTE 3: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 4: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 5: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the SL-PRS bandwidth of the smallest PRB number for the corresponding SCS. |  |  |  |  |  |  |  |

## Test cases for SL positioning measurements

- Proposal 7 (test cases):SL positioning test cases are defined for 3 general NR Uu configurations:

| NR Uu configuration | Description |
| --- | --- |
| 1 | NR Uu: 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | NR Uu: 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | NR Uu: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

- Proposal 8 (test cases):SL positioning test cases are defined for 3 general NR SL configurations:

| NR SL configuration | Description |
| --- | --- |
| 1 | SL: 15 kHz, 10 MHz bandwidth, TDD duplex mode |
| 2 | SL: 30 kHz, 10 MHz bandwidth, TDD duplex mode |
| 3 | SL: 30 kHz, 20 MHz bandwidth, TDD duplex mode |

RAN4 needs to decide the number of SL UEs in the test cases.

- Proposal 9 (test cases): The number of SL UEs in the measurement delay test cases for SL positioning is 4:

  - 1 target UE,

  - 3 anchor UEs (1 reference anchor UE and 2 other anchor UEs).

- Proposal 10 (test cases): The number of SL UEs in the measurement accuracy test cases for SL positioning is 3:

  - 1 target UE,

  - 2 anchor UEs (1 reference anchor UE and 1 other anchor UE).

RAN4 needs to also decide the SL-PRS configuration in relation to the number of samples in the test cases. The test cases can be specified for both 1 and 4 samples, which can be done in the same tables/section.

- Proposal 11 (test cases): For each SL positioning measurement type, measurement delay test cases are defined for both numbers of samples (can be in the same section):

  - 1 sample, and

  - 4 samples.

- Proposal 12 (test cases): For each SL positioning measurement type (for which accuracy requirements are defined), measurement accuracy test cases are defined for both numbers of samples (can be in the same section):

  - 1 sample, and

  - 4 samples.

- Proposal 13 (test cases): RAN4 to discuss whether to define test cases for additional path measurements.

# Summary

The following have been proposed in the current contribution:

- Proposal 1 (side conditions): For SL RSTD, the side conditions assumed in RAN4 requirements are: ≥0 dB for the reference link and ≥-6 dB for the measured link.

- Proposal 2 (side conditions): For SL Rx-Tx, the requirements are defined down to -6 dB.

- Proposal 3 (side conditions): Given that SL PRS-RSRP can be configured with other SL-PRS based measurements, the requirements for SL PRS-RSRP can be defined down to -6 dB.

- Proposal 4 (side conditions): Given that SL PRS-RSRPP can be configured with other SL-PRS based measurements, the requirements for SL PRS-RSRPP can be defined down to -6 dB.

- Proposal 5 (measurement accuracy requirements): Two sets of accuracy requirements are to be defined for SL positioning measurements, i.e., for:

  - 1 sample (>48 PRBs), and

  - 4 samples (≤48 PRBs).

- Proposal 6 (measurement accuracy requirements): The measurement accuracy requirements can be specified in the following format:

| Accuracy | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | SL-PRS Ês/Iot | SL-PRS SCS | SL-PRS bandwidth Note 1 | Number of samples, S | Io Note 2 range |  |  |
|  |  |  |  |  | NR operating band groups Note 3 | Minimum Io | Maximum Io |
| Tc Note 4 | dB | kHz | RB |  |  | dBm/SCS | dBm/BWChannel |
| TBD | (SL-PRS Ês/Iot)ref ≥ 0 dB (SL-PRS Ês/Iot)i ≥ -6 dB | 15 | ≥ 24 | ≥ 4 | NR_TDD_FR1_B | -126.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_J | -122.5 | -50 |
| TBD |  |  | >48 | ≥ 1 | Note 5 | Note 5 | Note 5 |
| TBD TBD |  | 30 | ≥ 24 | ≥ 4 | NR_TDD_FR1_B | -123.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_J | -119.5 | -50 |
| TBD |  |  | >48 | ≥ 1 | Note 5 | Note 5 | Note 5 |
| TBD |  | 60 | ≥ 24 | ≥ 4 | NR_TDD_FR1_B | -120.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_J | -116.5 | -50 |
| TBD |  |  | >48 | ≥ 1 | Note 5 | Note 5 | Note 5 |
| NOTE 1: Minimum SL-PRS bandwidth, which is the minimum of the SL-PRS bandwidths of the reference resource and the measured neighbour resource i. NOTE 2: Io is assumed to have constant EPRE across the bandwidth. NOTE 3: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 4: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 5: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the SL-PRS bandwidth of the smallest PRB number for the corresponding SCS. |  |  |  |  |  |  |  |

- Proposal 7 (test cases):SL positioning test cases are defined for 3 general NR Uu configurations:

| NR Uu configuration | Description |
| --- | --- |
| 1 | NR Uu: 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | NR Uu: 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | NR Uu: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

- Proposal 8 (test cases):SL positioning test cases are defined for 3 general NR SL configurations:

| NR SL configuration | Description |
| --- | --- |
| 1 | SL: 15 kHz, 10 MHz bandwidth, TDD duplex mode |
| 2 | SL: 30 kHz, 10 MHz bandwidth, TDD duplex mode |
| 3 | SL: 30 kHz, 20 MHz bandwidth, TDD duplex mode |

- Proposal 9 (test cases): The number of SL UEs in the measurement delay test cases for SL positioning is 4:

  - 1 target UE,

  - 3 anchor UEs (1 reference anchor UE and 2 other anchor UEs).

- Proposal 10 (test cases): The number of SL UEs in the measurement accuracy test cases for SL positioning is 3:

  - 1 target UE,

  - 2 anchor UEs (1 reference anchor UE and 1 other anchor UE).

- Proposal 11 (test cases): For each SL positioning measurement type, measurement delay test cases are defined for both numbers of samples (can be in the same section):

  - 1 sample, and

  - 4 samples.

- Proposal 12 (test cases): For each SL positioning measurement type (for which accuracy requirements are defined), measurement accuracy test cases are defined for both numbers of samples (can be in the same section):

  - 1 sample, and

  - 4 samples.

- Proposal 13 (test cases): RAN4 to discuss whether to define test cases for additional path measurements.

# References

[1] RP-230328, Revised WID on Expanded and Improved NR Positioning, Intel Corp, March, 2023.

[2] R4-2303190, WF on Expanded and Improved NR Positioning RRM requirements, Ericsson, March, 2023.

[3] R1-2301950, Summary #1 on Measurements and reporting for SL positioning, vivo, March 2023.

[4] R1-2301951, Summary #2 on Measurements and reporting for SL positioning, vivo, March 2023.

[5] R1-2301952	Summary #3 on Measurements and reporting for SL positioning, vivo, March 2023.

[6] R4-2306351, WF on RRM requirements for NR sidelink positioning and carrier phase positioning, CATT, April 2023.

[7] R4-2310163, WF on NR Positioning – SL positioning and CPP, CATT, May 2023.

[8] R4-2310077, Simulation assumptions for SL positioning, CATT, May 2023.

[9] R4-2314343, Ad-hoc minutes on R18 NR Positioning RRM requirements, Ericsson, Aug. 2023.

[10] R4-2314354, WF on R18 NR positioning – SL positioning and Carrier Phase Positioning, CATT, Aug. 2023.

[11] R4-2314358, LS on SL positioning and CPP measurements report mapping, Aug. 2023.

[12] R4-2312724, On SL positioning, Ericsson, Aug. 2023.

[13] RP-232670, Revised WID on Expanded and Improved NR Positioning, Intel Corporation, CATT, MediaTek, Sep. 2023.

[14] R4-2317380, WF on R18 NR positioning – SL positioning and Carrier Phase Positioning, CATT, Oct. 2023.

[15] R4-2321524, WF on R18 NR positioning – SL positioning and Carrier Phase Positioning, CATT, Nov. 2023.

[16] R4-2403368, WF on R18 NR positioning – SL positioning and Carrier Phase Positioning, CATT, March 2024.

[17] R4-2406384, WF on SL positioning and carrier phase positioning, CATT, Apr. 2024.