3GPP TSG-RAN WG4 Meeting # 111	R4-2409381

Fukuoka City, Fukuoka, JP, May 20 – May 24, 2024

Agenda Item: 7.19.2.2

Source: MediaTek Inc.

Title: Discussion on enhanced unified TCI state switching test cases for MIMO evo

Document for: Discussion

- Introduction

According to WF [1], there’re one open issues about s-DCI mTRP test cases. In this paper, we provide our view for the test case design.

- Discussion

2.1 Test cases for s-DCI mTRP

Issue 3-3-1: TCs design for s-DCI mTRP cases: < Agreement> : TC1; separate DL TCI state switch. TC2; separate UL TCI state switch. TC3; Joint TCI state switching. - Add the Note for the UE should pass which tests in the test purpose. Such as: UE can pass only one test or two tests depends on the UE capability.

Last RAN4 meeting agreed to define test case for s-DCI mTRP cases. As clarified last meeting, joint TCI state is the default capability for unified TCI state. There are only two types of UEs. One is supporting joint TCI state only. Another is supporting both joint and separate TCI state. For UE supporting joint TCI state only, there is no divergence that they only need to pass TC3, i.e., the test case for joint TCI state. For UE supporting both joint and separate TCI states, more discussion is needed for which test(s) UE needs to pass. In our understanding, the joint TCI state switching requirement for DL is the same as separate DL TCI state switching requirement. Ad the joint TCI state switching requirement for UL is the same as separate UL TCI state switching requirement. In this sense, UE does not need to pass separate TCI state switch TCs. RAN4 could only define test case for joint TCI state switching. There’s no need to define test case for separate TCI state switching.

Proposal 1: Do not define test cases of separate TCI state switching for both DL and UL in sDCI mTRP.

- Summary

In this paper, the discussion of R18 multi-Rx chains is provided. We have the following proposal:

Proposal 1: Do not define test cases of separate TCI state switching for both DL and UL in sDCI mTRP.

- Reference

[1] R4-2406503, WF on RRM requirements for NR_MIMO_evo_DL_UL, Samsung, RAN#110-bis