3GPP TSG-RAN WG4 Meeting #111 	R4-2409401

Fukuoka, Japan, May 20 – 24, 2024

Source: 	Huawei, HiSilicon

Title: 	Expected EIRP requirements

Agenda Item:	10.2.2

Document for:	Approval

1 Introduction

In the last meeting the Expected EIRP core requirements were discussed and a WF [1] was approved.

In this paper we discuss the open issues from that WF.

2 Discussion on core requirements

2.1	Definition of vertical and horizontal

The WF agreed:

  - Issue 2-3: New definition for vertical and horizontal for U6GHz EIRP mask

Agreement:

- To use the following figure as stating point;

There 2 points to raise here:

- If this definition is required in core specification?

- How to represent the diagram and relate the existing BS based coordinate system and the geographical system.

Firstly on point 1.

Currently the core specification does not include any definitions of coordinate systems, the requirements specify directions and over what range of directions requirements are to be met, however how these directions are defined i.e. the coordinate system is not explained in the core specification but in the conformance specification.

It is certainly possible to explain the coordinate system in the core, however as it requires knowledge and description of some of the declared parameters a more natural fit in the specifications was the conformance.

Expected EIRP requirement are not with respect to the BS coordinate system but the horizon, it is important that this distinction is clear and that the 2 coordinate systems can be described together.

As such describing the geographical (horizon) coordinate system in the core requirement without the BS coordinate system would cause confusion.

The horizon (and angles relative to the horizon) are a universal concept which really require no clarification anyway, we do not need to define (or redefine) the horizon.

Whilst for the Expected EIRP requirement it will be necessary to make some definitions of mechanical tilt and to define mechanical tilt it is necessary to use the physical parameters of the BS and the coordinate system(s) a similar issue occurs with beam directions, this is handled by referring to them in the core but only making the declarations in the conformance specifications.

It is stated that the BS meets the requirements for “all foreseen modes of operation” this includes the range of declared operating parameters made in the conformance requirements which in general are not explicitly referenced in the core. As this statement was made when the physical installation parameters were not relevant to the requirements it could possibly be strengthened for the Expected EIRP requirement to include all foreseen modes of operation and installation parameters.

Proposal 1: It is not necessary to define the horizon or a coordinate system relative to it in the core specification.

Proposal 2: The statement covering all foreseen modes of operation could be strengthened to better cover the installation parameters to:

all foreseen modes of operation and installation parameters

On point 2, it is necessary to include definitions and if possible diagrams to explain:

- The coordinate systems (both with respect to BS and also the horizon)

- The relationship between them

The diagram in the WF is a good starting point for the geographical (horizon based) system, θL and θH represent the mechanical tilt extremes, however its not clear how this is aligned with the existing BS referenced coordinate system. Which is described in 38.141-2 sub-clause 4.14

Radiated requirements are stated in terms of electromagnetic characteristics (e.g. EIRP and EIS) at certain angles with respect to the base station. To be able to declare radiated characteristics part of radiated requirements a reference coordinate system is required. The reference coordinate system is should be associated to an identifiable physical feature on the base station enclosure. The location of the origin and the orientation of the reference coordinate system are for the base station manufacturer to declare.

The reference coordinate system is created of a Cartesian coordinate system with rectangular axis (x, y, z) and spherical angles () as showed in figure 4.14-1.

Figure 4.14-1: Reference coordinate system

In this case θ=90 is the vertical plane of the BS enclosure rather than at right angles to the horizon (as shown in the proposes horizon diagram).

Observation 1: θ=0 is NOT the horizon in our coordinate system

In the TR 37.941 there are some spatial description diagrams which show the BS enclosure and could be of some use, for example figure 6.3.4.2-2

If this could be combined with the proposed horizon diagram then this would show the complete picture and relationships.

For example:

The is the angle between the projection of the vector in the x/y plane and the horizon is the mechanical tilt.

Proposal 3: The coordinate system diagram and the geographical (horizon) diagram should be combined to make it clear the reference points for θ and ϕ.

In the core regulatory requirement the symbols θL and θH are used to designate the range of vertical angles above the horizon. The use of θ in this context is confusing as θ is used to define the BS local coordinate system. A different symbol is required, possibly a capital theta (Θ) could be used to denote a different angle, or a subscript that can be used to correctly identify the angle description such as θHL and θHH Where the additional H in the subscript can be used to identify t as an angle from the horizon.

Proposal 4: in the EEIRP requirement table use θHL and θHH and define the symbols in clause 3 as above the horizon.

2.2	Clause in TS

The WF had the following options:

  - Issue 2-4: Which sub-clause to capture the requirement the requirement

- Option 1: clause 9.2 for BS EIRP

- Option 2: clause 9.7 for unwanted emission mask

- Option 3: add new clause for EIRP emission mask

We think each of these can be made to work, however as both option 1 and option 2 have potential conflicts with existing definitions in those sections option 3 might be the best.

Proposal 5: Option 3: add new clause for EIRP emission mask

2.3	The applicability of the expected EIRP mask

The WF agreed:

  - Issue 2-1: The applicability of band for the EIRP mask requirement

Agreement:

- The applicable band is n104

- Further discuss the applicable freq range within band n104.

The expected e.i.r.p. is a new regulatory requirement that is defined to protect the FSS (Earth-to-space). In Radio Regulation (Edition of 2020) and the Final Acts WRC-23, the service of FSS (Earth-to-space) is allocated up to 7 075 MHz, as shown below.

Resolution 220 (WRC-23) “considering h) that existing satellite networks of the fixed-satellite service (FSS) (Earth-to-space) are used within the frequency band 6425-7075 MHz, or parts thereof, and their characteristics may

evolve in the future” explains that existing satellite networks of the fixed-satellite service (FSS) (Earth-to-space) are used within the frequency band 6425-7075 MHz, or parts thereof, under the FSS (Earth-to-space) allocation, and the characteristics of the existing satellite networks that operating on the frequency band 6 425-7 075 MHz, or parts thereof, may evolve in the future.

Those contributions submitted to WRC-23 that include proposals of technical condition for protection of FSS (Earth-to-space) propose to impose the condition to 6425 – 7075 MHz, or parts thereof, and no input contribution proposes to impose the condition for protection of FSS (Earch-to-space) to above 7075 MHz.

As a conclusion, WRC-23 defines the expected EIRP to be applied to up to 7075 MHz.

Proposal 6: to define the expected EIRP to be applied to up to 7075 MHz

Proposal 7: to endorse the draft CR to 38.104 as proposed in annex

3 Overview on conformance testing

From the regulatory requirement there are a number of points that need clarifying within RAN4 before a conformance requirement can be written.

- Necessary declarations of steering parameters and horizon reference.

- Beam steering profiles/test vectors to ensure sufficient averaging

- Angular step size within Azimuth and Elevation to ensure accuracy Expected EIRP measurement

- Measurement uncertainty.

Each of these does not to be solved immediately and can wait for the conformance work to start but as way of an introduction to the tasks each is briefly discussed.

3.1	Declarations

Spatial declarations are discussed in paper [2]. However directional declarations, spatial coordinate systems and reference points are currently dealt with in the conformance specifications. We see this is the natural location for the modified declarations needed for the expected EIRP requirement

Proposal 8: Spatial declarations for Expected EIRP are made in the conformance requirement, no need for formal definitions in the core specification.

3.2	Beam steering profiles

For TRP emission measurements if beam sweeping is used it does not matter which direction the beam is pointed in as long as the wanted signal TRP is maximum. As all the power of the sphere is measured it does not matter in what direction the bema is pointed.

For Expected EIRP the same is not true as the vertical dimension is separated into angular bins. The requirement states that the beam is pointed in different directions over its beam steering range and that the directions have a uniform statistical angular distribution over this range (as opposed to a uniform UE location on the ground). During the regulatory analysis a number of different sub-sets of beam steering directions were proposed and some initial analysis was done to see if they were suitable.

In order to access if a sub-set of beam steering directions (or test vectors) are suitable we must first know what the ideal or correct result is? The requirement describes a statistical average based on random beam locations as such the “correct” result can never be described 100% as beam directions are random and as no upper limit is defined essentially there are potentially an infinite number of them. As close as can be achieved we must estimate the “correct” answer by averaging a very large number of samples until the error function is minimised. This brings up the issue of how an error function is formed based on the difference between 2 multi-dimensional average beam power distributions.

Once the method of analysis is agreed then potential test vectors can be tested against the statistically valid result.

Proposal 9: A means to identify a means to test the quality of potential conformance directional test vectors must be derived.

3.3	Measurement angular step size

As the expected EIRP measurement is similar to the TRP measurement in terms of averaging over the sphere a good place to start is the TRP minimum step size. In TS 38.141-2 annex I there are a number of grid measurement techniques described, however the simplest (but no necessarily the fastest) is the uniform angular grid of points. Using this technique the minimum step size can be identified by using the antenna dimensions or the specified minimum beam widths and they are:

∆ϕref=λλo BeWϕ

Δθref=λλo BeWθ

Other methods of reducing the number of test directions measured are described in Annex I but this is the simplest and is generally identified as giving the correct answer (within the required accuracy). Other methods often require assumptions and knowledge of the DUT. As such an equivalent to the uniform angular grid method should be developed for the Expected EIRP measurement (although of course other valid methods can be investigated).

The step size is agreed based on the allowable error in the TRP calculation or the TRP summation error (SETRP) which is 0.75dB in the MU calculations. It also must be noted that the SEEEIRP is effectively defined in the regulatory annex as being part of the 95% averaging uncertainty as such the SEEEIRP value used for the Expected EIRP measurement must be consistent with the value assigned to that uncertainty.

Observation 1: a suitable summation Error for the Expected EIRP MU calculation should be derived based on step size and averaging uncertainty.

3.4	Measurement uncertainty

Looking at the uniform angular step TRP method with a directional chamber such as the IAC. The measurement and calibration errors are based on each directional point measured.

As the SEEEIRP is effectively defined as part of the 95% averaging uncertainty in the regulation annex so until the work is done to identify that value we cannot say exactly what the total MU will be.

The test equipment accuracy for each point assumed in the TRP MU calculations are in the order of 1-1.3dB resulting in a total MU (including the SE) of 1.4-1.5dB. However the accuracy of the test equipment when measuring the large wanted signals is better than when measuring small signals. It much also be considered that the wanted signal is present in the chamber at the same time and hence the dynamic range of the equipment may limit the available smallest signal level that can be measured. As such the accuracy of the test equipment may be closer to the unwanted emissions measurements or possible the absolute ACLR measurement (2.2-2.7dB).

Observation 2: A test equipment MU analysis is required but MU value per directional point may be closer to the unwanted emissions level accuracy rather than the wanted signal TRP accuracy.

4	Summary

This paper looked at the open issues highlighted for the core requirement for the Expected EIRP requirement in the WF [1]. It makes the following observations and proposals

Proposal 1: It is not necessary to define the horizon or a coordinate system relative to it in the core specification.

Proposal 2: The statement covering all foreseen modes of operation could be strengthened to better cover the installation parameters to:

Observation 1: θ=0 is NOT the horizon in our coordinate system

Proposal 3: The coordinate system diagram and the geographical (horizon) diagram should be combined to make it clear the reference points for θ and ϕ.

Proposal 4: in the EEIRP requirement table use θHL and θHH and define the symbols in clause 3 as above the horizon.

Proposal 5: Option 3: add new clause for EIRP emission mask

Proposal 6: to define the expected EIRP to be applied to up to 7075 MHz

Proposal 7: to endorse the draft CR to 38.104 as proposed in annex

Proposal 8: Spatial declarations for Expected EIRP are made in the conformance requirement, no need for formal definitions in the core specification.

Proposal 9: A means to identify a means to test the quality of potential conformance directional test vectors must be derived.

Observation 1: a suitable summation Error for the Expected EIRP MU calculation should be derived based on step size and averaging uncertainty.

Observation 2: A test equipment MU analysis is required but MU value per directional point may be closer to the unwanted emissions level accuracy rather than the wanted signal TRP accuracy.

References

[1]	R4-2406088 Way Forward for [110bis][311] NR_BS_RF, ZTE

[2] R4-2405422, Spatial reference definitions for expected EIRP mask, Huawei

Draft CR to 38.104

< Start change >

3.2	Symbols

For the purposes of the present document, the following symbols apply:

	Percentage of the mean transmitted power emitted outside the occupied bandwidth on the assigned channel

BeWθ,REFSENS	Beamwidth equivalent to the OTA REFSENS RoAoA in the θ-axis in degrees. Applicable for FR1 only.

BeWφ,REFSENS	Beamwidth equivalent to the OTA REFSENS RoAoA in the φ-axis in degrees. Applicable for FR1 only.

BWChannel	BS channel bandwidth

BWChannel_CA	Aggregated BS Channel Bandwidth, expressed in MHz. BWChannel_CA = Fedge,high- Fedge,low.

BWChannel,block	Sub-block bandwidth, expressed in MHz. BWChannel,block = Fedge,block,high- Fedge,block,low.

BWConfig	Transmission bandwidth configuration, where BWConfig = NRB x SCS x 12

BWContiguous	Contiguous transmission bandwidth, i.e. BS channel bandwidth for single carrier or Aggregated BS channel bandwidth for contiguously aggregated carriers. For non-contiguous operation within a band the term is applied per sub-block.

BWGB,low	The minimum guard band defined in clause 5.3.3 for lowest assigned component carrier

BWGB,high	The minimum guard band defined in clause 5.3.3 for highest assigned component carrier

f	Separation between the channel edge frequency and the nominal -3 dB point of the measuring filter closest to the carrier frequency

ΔfBE_offset	Separation between the edge of the last transmitted channel of the channels assigned for NR-U channel bandwidth and the nominal -3 dB point of the measuring filter closest to the carrier frequency

ΔFGlobal	Global frequency raster granularity

fmax	f_offsetmax minus half of the bandwidth of the measuring filter

ΔfOBUE	Maximum offset of the operating band unwanted emissions mask from the downlink operating band edge

ΔfOOB	Maximum offset of the out-of-band boundary from the uplink operating band edge

ΔFR2_REFSENS	Offset applied to the FR2 OTA REFSENS depending on the AoA

ΔminSENS	Difference between conducted reference sensitivity and minSENS

ΔOTAREFSENS	Difference between conducted reference sensitivity and OTA REFSENS

ΔFRaster	Channel raster granularity

Δshift	Channel raster offset for SUL

EISminSENS	The EIS declared for the minSENS RoAoA

EISREFSENS	OTA REFSENS EIS value

EISREFSENS_50M	Declared OTA reference sensitivity basis level for FR2 based on a reference measurement channel with 50MHz BS channel bandwidth

FFBWhigh	Highest supported frequency within supported operating band, for which fractional bandwidth support was declared

FFBWlow	Lowest supported frequency within supported operating band, for which fractional bandwidth support was declared

FC	RF reference frequency on the channel raster, given in table 5.4.2.2-1

FC,block,high	Fc of the highest transmitted/received carrier in a sub-block.

FC,block,low	Fc of the lowest transmitted/received carrier in a sub-block.

FC,low	The Fc of the lowest carrier, expressed in MHz.

FC,high	The Fc of the highest carrier, expressed in MHz.

FDL,low	The lowest frequency of the downlink operating band

FDL,high	The highest frequency of the downlink operating band

Fedge,low	The lower edge of Aggregated BS Channel Bandwidth, expressed in MHz. Fedge,low = FC,low - Foffset,low.

Fedge,high	The upper edge of Aggregated BS Channel Bandwidth, expressed in MHz. Fedge,high = FC,high + Foffset,high.

Fedge,block,low	The lower sub-block edge, where Fedge,block,low = FC,block,low - Foffset,low.

Fedge,block,high	The upper sub-block edge, where Fedge,block,high = FC,block,high + Foffset,high.

Ffilter	Filter centre frequency

Foffset,high	Frequency offset from FC,high to the upper Base Station RF Bandwidth edge, or from F C,block,high to the upper sub-block edge

Foffset,low	Frequency offset from FC,low to the lower Base Station RF Bandwidth edge, or from FC,block,low to the lower sub-block edge.

f_BE_offset	Separation between the edge of the last transmitted channel of the channels assigned for NR-U channel bandwidth and the centre of the measuring

f_offset	Separation between the channel edge frequency and the centre of the measuring

f_offsetmax	The offset to the frequency ΔfOBUE outside the downlink operating band

FREF	RF reference frequency

FREF-Offs	Offset used for calculating FREF

FREF,shift	RF reference frequency for Supplementary Uplink (SUL) bands

Fstep,X	Frequency steps for the OTA transmitter spurious emissions (Category B)

FUL,low	The lowest frequency of the uplink operating band

FUL,high	The highest frequency of the uplink operating band

GBChannel	Minimum guard band defined in clause 5.3.3

Ncells	The declared number corresponding to the minimum number of cells that can be transmitted by an BS type 1-H in a particular operating band

Physical resource block number

NRB	Transmission bandwidth configuration, expressed in resource blocks

NRB,high	Transmission bandwidth configuration for the highest assigned component carrier within a sub-block in CA

NRB,low	Transmission bandwidth configuration for the lowest assigned component carrier within a sub-block in CA

NREF	NR Absolute Radio Frequency Channel Number (NR-ARFCN)

NREF-Offs	Offset used for calculating NREF

NRXU,active	The number of active receiver units. The same as the number of demodulation branches to which compliance is declared for chapter 8 performance requirements

NRXU,counted	The number of active receiver units that are taken into account for conducted Rx spurious emission scaling, as calculated in clause 7.6.1

NRXU,countedpercell	The number of active receiver units that are taken into account for conducted RX spurious emissions scaling per cell, as calculated in clause 7.6.1

NTXU,counted	The number of active transmitter units as calculated in clause 6.1, that are taken into account for conducted TX output power limit in clause 6.2.1, and for unwanted TX emissions scaling

NTXU,countedpercell	The number of active transmitter units that are taken into account for conducted TX emissions scaling per cell, as calculated in clause 6.1

PEM,n50/n75,ind	Declared emission level for Band n50/n75; ind = a, b

PEM,n54,ind	Declared emission level for Band n54; ind = a, b, c, d, e, f

PEIRP,N	EIRP level for channel N

Pmax,c,AC	Maximum carrier output power measured per antenna connector

Pmax,c,cell	The maximum carrier output power per TAB connector TX min cell group

Pmax,c,TABC	The maximum carrier output power per TAB connector

Pmax,c,TRP	Maximum carrier TRP output power measured at the RIB(s), and corresponding to the declared rated carrier TRP output power (Prated,c,TRP)

Pmax,c,EIRP	The maximum carrier EIRP when the NR BS is configured at the maximum rated carrier output TRP (Prated,c,TRP)

Prated,c,AC	The rated carrier output power per antenna connector

Prated,c,cell	The rated carrier output power per TAB connector TX min cell group

Prated,c,FBWhigh	The rated carrier EIRP for the higher supported frequency range within supported operating band, for which fractional bandwidth support was declared

Prated,c,FBWlow	The rated carrier EIRP for the lower supported frequency range within supported operating band, for which fractional bandwidth support was declared

Prated,c,sys	The sum of Prated,c,TABC for all TAB connectors for a single carrier

Prated,c,TABC	The rated carrier output power per TAB connector

Prated,c,TRP	Rated carrier TRP output power declared per RIB

Prated,t,AC	The rated total output power declared at the antenna connector

Prated,t,TABC	The rated total output power declared at TAB connector

Prated,t,TRP	Rated total TRP output power declared per RIB

PREFSENS	Conducted Reference Sensitivity power level

SCSlow	Sub-Carrier Spacing for the lowest assigned component carrier within a sub-block in CA

SCShigh	Sub-Carrier Spacing for the highest assigned component carrier within a sub-block in CA

SSREF	SS block reference frequency position

Wgap	Sub-block gap or Inter RF Bandwidth gap size

θHL	Lowest angle above the Horizon in Expected EIRP vertical angle range

θHH	Highest angle above the Horizon in Expected EIRP vertical angle range

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AA	Antenna Array

AAS	Active Antenna System

ACLR	Adjacent Channel Leakage Ratio

ACS	Adjacent Channel Selectivity

AoA	Angle of Arrival

AWGN	Additive White Gaussian Noise

BS	Base Station

BW	Bandwidth

CA	Carrier Aggregation

CACLR	Cumulative ACLR

CPE	Common Phase Error

CP-OFDM	Cyclic Prefix-OFDM

CW	Continuous Wave

DFT-s-OFDM	Discrete Fourier Transform-spread-OFDM

DM-RS	Demodulation Reference Signal

EIS	Equivalent Isotropic Sensitivity

EIRP	Effective Isotropic Radiated Power

E-UTRA	Evolved UTRA

EVM	Error Vector Magnitude

FBW	Fractional Bandwidth

FR	Frequency Range

FRC	Fixed Reference Channel

FSS	Fixed Satellite Service

GSCN	Global Synchronization Channel Number

GSM	Global System for Mobile communications

HAPS	High Altitude Platform Station

ITU-R	Radiocommunication Sector of the International Telecommunication Union

ICS	In-Channel Selectivity

LA	Local Area

LNA	Low Noise Amplifier

MCS	Modulation and Coding Scheme

MR	Medium Range

NB-IoT	Narrowband – Internet of Things

NR	New Radio

NR-ARFCN	NR Absolute Radio Frequency Channel Number

OBUE	Operating Band Unwanted Emissions

OCC	Orthogonal Covering Code

OOB	Out-of-band

OSDD	OTA Sensitivity Directions Declaration

OTA	Over-The-Air

PRB	Physical Resource Block

PT-RS	Phase Tracking Reference Signal

QAM	Quadrature Amplitude Modulation

RB	Resource Block

RDN	Radio Distribution Network

RE	Resource Element

REFSENS	Reference Sensitivity

RF	Radio Frequency

RIB	Radiated Interface Boundary

RMS	Root Mean Square (value)

RoAoA	Range of Angles of Arrival

QAM	Quadrature Amplitude Modulation

RB	Resource Block

RX	Receiver

SCS	Sub-Carrier Spacing

SDL	Supplementary Downlink

SS	Synchronization Symbol

SSB	Synchronization Signal Block

SUL	Supplementary Uplink

TAB	Transceiver Array Boundary

TAE	Time Alignment Error

TDL	Tapped Delay Line

TX	Transmitter

TRP	Total Radiated Power

UCI	Uplink Control Information

UEM	Unwanted Emissions Mask

UTRA	Universal Terrestrial Radio Access

WA	Wide Area

ZF	Zero Forcing

< Next change >

9.9	OTA spatial emission above horizon

9.9.1	General

The OTA spatial emission above horizon requirement is a measure of the Expected EIRP radiated in ranges of vertical (perpendicular to the horizon) angles above the horizon. The OTA spatial emission above the horizon requirement is to ensure the protection of FSS (Earth-to-space) in the band 6425 – 7075MHz.

The requirement shall apply at each RIB supporting transmission in the appropriate frequency range of n104.

The requirement applies for all foreseen modes of operation and installation parameters.

NOTE: the OTA spatial emission above horizon requirement is specified with reference to the horizon, directional requirements are made with respect to the BS enclosures local coordinate system.

9.9.2	Minimum requirement for BS type 1-O

For BS type 1-O the Expected EIRP shall not exceed the levels shown in table 9.9.2-1.

Expected EIRP is defined as the average value of the EIRP, with the averaging being performed:

-	 Over horizontal angles from −180° to +180 ° and over the specified vertical angle range θHL ≤ θ < θHH in table 9.9.2-1

-	 With the BS randomly generating beam peak directions within the OTA peak directions set.

Table 9.9.2-1: Expected EIRP limits.

| Vertical angle range θHL ≤ θ < θHH Note 1. | Expected EIRP. (dBm/MHz) |
| --- | --- |
| 0° ≤ θ < 5° | 27 |
| 5° ≤ θ < 10° | 23 |
| 10° ≤ θ < 15° | 19 |
| 15° ≤ θ < 20° | 18 |
| 20° ≤ θ < 30° | 16 |
| 30° ≤ θ < 60° | 15 |
| 60° ≤ θ < 90° | 15 |
| Note1: Angles are specified with respect to the Horizon |  |

< End of change >