3GPP TSG-RAN WG4 # 111	R4-2409456

Fukuoka, Japan, May 20 – May 24, 2024

Agenda item:	7.19.3.1

Source:	MediaTek Inc.

Title:	Simulation results of NR MIMO evolution for downlink

Document for:	Discussion

# Introduction

In RAN#103, the status report of RAN1 led Rel-18 WI of NR MIMO evolution for downlink and uplink lists the current progress [1]. The latest WF of WI of NR MIMO evolution for downlink and uplink lists several open items with initial proposals [2]. This discussion document is showing simulation results for PMI prediction codebook and CJT codebook.

# Simulation parameters and results

In this chapter we are going to show simulation results of PMI prediction codebook in Chapter 2.1 and simulation results of CJT codebook in Chapter 2.2.

# 2.1 Rel-16/17 Type-II codebook refinement

# 2.1.1 Simulation parameters

In this chapter simulation configuration of PMI prediction tests are shown in Table 1. The configuration follows initial proposals given in WF [2]. We have simulated different parameter configurations to evaluate PMI prediction performance.

Table 1: Simulation configuration of PMI prediction.

| Channel | FDD: TDLA30-20 and TDLA30-30 TDD: TDLA30-10 and TDLA30-20 |
| --- | --- |
| Antenna correlation | XP medium |
| CSI-RS antenna ports | 16 with (N1, N2) = (4, 2), (O1, O2) = (4, 4) |
| Aperiodic CSI-RS period | 2 slots |
| CQI/RI/PMI delay | FDD: 8ms (8 slots) TDD: 6ms (12 slots) |
| paramCombination-Doppler-r18 | 7 (L=4, pυ=1/2, β=1/2) |
| Rank | 2 |
| MCS | 13 |
| K, N4, Q, m, d | K=4, N4=1, Q=1, m=d=2 (Rel-18 with prediction only) K=1, N4=1, Q=1, m=d=8 (Rel-16 reference) Type-I CB with random WB-i1 and SB-i2 (Random PMI reference) |
| Prediction delay | delta=1 |
| Beam steering | Dual cluster |

Note1: No PDSCH allocation in CSI-RS slots to keep constant PDSCH code rate
Note2: K is prediction filter length

Timing figures of FDD and TDD simulations are shown in Figures 1 – 2.

Figure 1: FDD timing of PMI prediction test.

Figure 2: TDD timing of PMI prediction test.

# 2.1.2 Simulation results

In this chapter the simulation results of different parameter combinations are shown in Figures 4 to 7. Corresponding gamma values are collected in Table 2. Left FDD figures have TDLA30-20 channel and right FDD figures have TDLA30-30 channel. All results use agreed FDD timing (Figure 1).

Figure 4: Simulation results of PMI prediction in FDD 4Rx in TDLA30-20 and TDLA30-30 channels.

Figure 5: Simulation results of PMI prediction in FDD 2Rx in TDLA30-20 and TDLA30-30 channels.

Left TDD figures have TDLA30-10 channel and right TDD figures have TDLA30-20 channel. All results use agreed TDD timing (Figure 2).

Figure 6: Simulation results of PMI prediction in TDD 4Rx in TDLA30-10 and TDLA30-20 channels.

Figure 7: Simulation results of PMI prediction in TDD 2Rx in TDLA30-10 and TDLA30-20 channels.

Table 2: Simulation results (gamma values) of PMI prediction with MCS13 Rank 2.

| Test | Rel-18 | Rel-16 |
| --- | --- | --- |
| FDD, 4Rx, TDLA-30ns-20Hz | 2.3 | 2.3 |
| FDD, 4Rx, TDLA-30ns-30Hz | 2.1 | 2.1 |
| FDD, 2Rx, TDLA-30ns-20Hz | 2.0 | 1.9 |
| FDD, 2Rx, TDLA-30ns-30Hz | 1.7 | 1.7 |
| TDD, 4Rx, TDLA-30ns-10Hz | 4.5 | 4.1 |
| TDD, 4Rx, TDLA-30ns-20Hz | 3.3 | 3.1 |
| TDD, 2Rx, TDLA-30ns-10Hz | 5.5 | 4.9 |
| TDD, 2Rx, TDLA-30ns-20Hz | 3.3 | 2.8 |

# 2.1.3 Simulation analysis

We have simulated the following configurations for performance comparison purpose

- Rel-18 with prediction: K=4, N4=1, Q=1, m=d=2

- Rel-16 reference: K=1, N4=1, Q=1, m=d=8

- Random PMI reference: Type-I CB with random WB-i1 and SB-i2

Rel-18 solution are simulated with prediction timing assumptions matching configured delta parameter timing. Our expectation of feasible test is where Rel-18 solution can outperform Rel-16 and random PMI references with reasonable margin and justify usage of new feature in such scenario. We can do following observations.

Observation #1: No prediction gains in FDD simulations probably due to too many allocated PDSCH slots.

Observation #2: Some prediction gains in TDD simulations with both doppler options.

Observation #3: Higher prediction gains in TDD simulations with 10Hz doppler compared to 20Hz.

# 2.2 Enhancements of CSI acquisition for CJT targeting FR1 and up to 4 TRPs

# 2.2.1 Simulation parameters

In this chapter simulation configuration of CJT tests are shown in Table 3. The configuration follows initial proposals given in WF [2].

Table 3: Simulation configuration of CJT.

| NTRP (Number of TRPs) | 2 |
| --- | --- |
| Channel | TDLA30-10 |
| Antenna correlation | XP high |
| K (numberOfCSI-RS-Resources) | 2 |
| CSI-RS periodicity | 5 slots (No PDSCH allocation in CSI-RS slots) |
| CQI/RI/PMI delay | 14ms (14 slots) |
| CSI-RS antenna ports | 8 with (N1, N2) = (4, 1), (O1, O2) = (4, 1) |
| codebookMode | Mode2 |
| paramCombination-CJT-L-r18 | 7 ({4, 4}) |
| paramCombination-CJT-r18 | 4 (pυ=1/4 and 1/8, β=1/2) |
| typeII-CJT-RI-Restriction-r18 | 0010 (Rank2) |
| MCS | 20 |
| Beam steering | Dual cluster |

# 2.2.2 Simulation results

In this chapter the simulation results of agreed CJT parameters and random PMI are shown in Figure 8. Corresponding gamma values and SNRs are collected in Table 4.

Figure 8: Simulation results of 4Rx and 2Rx CJT with MCS20 Rank2 in TDLA30-10 XP high.

Table 4: Gamma and SNR values of CJT 4Rx tests for different configurations.

|  | Gamma and SNR values of TDLA-30-10, XP high, 4Rx and 2Rx simulations |  |
| --- | --- | --- |
|  | 4Rx Rank-2 MCS20 | 2Rx Rank-2 MCS20 |
|  | eType-2 CJT {4,4} 4 | eType-2 CJT {4,4} 4 |
| γ | 2.32 | 2.43 |
| SNR | 6.9 dB | 10.1 dB |

# 2.2.3 Simulation analysis

We have simulated with agreed CJT parameters and random PMI. We can do observation from these results.

Observation #4: We see feasible gamma and SNR values in both 4Rx and 2Rx tests.

# Conclusion

In this paper we provided the view on the MIMO evolution for downlink. The following observations are made:

Observations of predicted PMI

Observation #1: No prediction gains in FDD simulations probably due to too many allocated PDSCH slots.

Observation #2: Some prediction gains in TDD simulations with both doppler options.

Observation #3: Higher prediction gains in TDD simulations with 20Hz doppler compared to 10Hz.

Observations of CJT

Observation #4: We see feasible gamma and SNR values in both 4Rx and 2Rx tests.

# Reference

- RP-240368, “Status Report: NR MIMO evolution for downlink and uplink”, Samsung

- R4-2406111, “WF on [110][323] NR_MIMO_evo_DL_UL_demod”, Samsung

- 3GPP TS 38.101-4 V18.3.0 (2024-03), “User Equipment (UE) radio transmission and reception; Part 4: Performance requirements (Release 18)”