# 3GPP TSG-RAN WG4 Meeting #111	R4-2409545

# Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024

Source: 	Huawei, HiSilicon

Title: 		Information on developments of ETSI TR 103 974 on the equivalence of measurement results with different OTA test methods

Agenda Item:	10.2.1

Document for:	Information

# Introduction

One of the objectives of the NR_BS_RF_req_evo work item [1] is related to the TRP test methodology and potential applicability improvements, and considered TRP test methods:

As the above objective belongs to the Performance part of the WI, this contribution is submitted for Information, to be discussed at later stage.

In this contribution, we provide initial description on the status of ETSI ERM WGRM work on the technical report ETSI TR 103 974 on equivalence of measurement results with different test methods, which includes TRP-related aspects.

# Discussion

Scope of the ETSI TR 103 974 different test methods for a specific RF requirement, across various standards (i.e. not BS-dedicated). One of the aims of that TR was how to ensure equivalent measurement results of a radiated power measurement while using different test sites (e.g. FAR, SAC, OATS).

The following OTA test methods were recognized so far in the TR:

the Open Area Test Site (OATS),

the Semi-Anechoic Chamber (SAC),

the Fully Anechoic Room (FAR),

the Absorber Lined OATS (ATS),

the Transverse Electro-Magnetic (TEM) cell,

the Reverberation Chamber (RC), and

the Near Field Scanner (NFS).

Figure 1: Exemplary test sites allocation to OTA requirements [3]

There was already an attempt to map available OTA test methods to the radiated requirements. However, it shall be clarified, that the work is still ongoing and there are still unresolved questions, e.g.

- More detailed evaluation on the selection of the preferred radiated test site(s),

- Evaluation of the case with multiple preferred solutions available, and potential guidance of further test method down-selection,

- Discussion on MU differences among test sites.

# Conclusions

It is suggested to follow progress on the ETSI TR 103 974 developments, and to consider its content in RAN4 discussions under NR_BS_RF_req_evo work item.

It is expected that ETSI TR 103 974 will be finalized within 2024.

# References

[1]		RP-240829	NR base station (BS) RF requirement evolution for FR1/FR2 and testing, WID, RAN#103

[2] 	On the equivalence of measurement results with different test methods, DTR/ERM-RM-284, ERM WGRM

[3]		ETSI TR 103 974	On the equivalence of measurement results with different test methods, v0.0.4, ERM WGRM, https://docbox.etsi.org/ERM/ERMRM/70-DRAFTS/WGRM284/ERM-RM-284v111_004.docx