# 3GPP TSG-RAN WG4 Meeting #111	R4-2409565

# Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.115-2 | CR |  | rev |  | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updated Draft CR to TS 38.115-2: Clauses 6.12~6.16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_netcon_repeater-Perf |  |  |  |  |  | Date: |  |  | 2024-04-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on Draft BigCR to TS 38.115-2 which was Endorsed in R4-2406136, in this Draft CR we provide further updates to clauses 6.12~6.16. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clauses 6.12~6.16 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing NCR requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

## 6.12	OTA reference sensitivity

### 6.12.1	Definition and applicability

The reference sensitivity power level REFSENS is defined as the EIS level at the centre of the quiet zone in the RX beam peak direction, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

### 6.12.2	Minimum requirement for NCR type 2-O

For WA class NCR type 2-O reference sensitivity level is specified the same as the Wide Area IAB-MT reference sensitivity level requirement in TS 38.174 [22], subclause 10.3.3.3.

For LA class NCR type 2-O  reference sensitivity level is specified the same as reference sensitivity power level for power class 3 in TS 38.101-2 [14], subclause 7.3.2.3.

This requirement applies to MT RIB only.

### 6.12.3	Test purpose

The test purpose is to verify that the IAB can meet the throughput requirement for a specified measurement channel at the EISREFSENS level and the range of angles of arrival within the OTA REFSENS RoAoA.

### 6.12.4	Method of test

#### 6.12.4.1	Initial conditions

Test environment: Normal, see annex B.2.

RF channels to be tested for single carrier:

B, M and T; see clause 4.9.1.

Directions to be tested:

-	OTA REFSENS receiver target reference direction (D.54),

-	OTA REFSENS conformance test directions (D.55).

#### 6.12.4.2	Procedure

1)	Place the NCR with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.2.1.

2)	Align the manufacturer declared coordinate system orientation of the NCR with the test system.

3)	Align the NCR with the test antenna in the declared direction to be tested.

4)	Ensure the polarization is accounted for such that all the power from the test antenna is captured by the NCR under test.

5)	Start the signal generator for the wanted signal to transmit:

-	The test signal as specified in clause 6.12.5.

6)	Set the test signal mean power so the calibrated radiated power at the Antenna Array coordinate system reference point is as specified in clause 6.12.5.

7)	Measure the throughput according to annex A.1 for each supported polarization.

9)	Repeat steps 3 to 9 for all OTA REFSENS conformance test directions of the NCR (D.55), and supported polarizations.

For multi-band capable FR1 and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carriers activated in the other band.

### 6.12.5	Test requirements

The FR1 EISREFSENS level is the conducted REFSENS requirement value offset by ΔOTAREFSENS. The test requirement is calculated from the EISREFSENS level offset by the EISREFSENS Test Tolerance specified in clause 4.1.

#### 6.12.5.1	Test requirements for NCR type 2-OW

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in the corresponding table and annex A.1 when the OTA test signal is at the corresponding EISREFSENS level and arrives from any direction within the OTA REFSENS RoAoA.

EISREFSENS levels are derived from a single declared basis level EISREFSENS_50M, which is based on a reference measurement channel with 50 MHz NCR-MT channel bandwidth. EISREFSENS_50M itself is not a requirement and although it is based on a reference measurement channel with 50 MHz NCR-MT channel bandwidth it does not imply that IAB-MT has to support 50 MHz IAB-MT channel bandwidth.

For Wide Area IAB-MT, EISREFSENS_50M is an integer value in the range -96 to -119 dBm. The specific value is declared by the manufacturer.

Table 6.12.5.1-1: Wide area FR2-1 OTA reference sensitivity requirement

| IAB-MT channel Bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 50, 100, 200 | 60 | G-FR2-A1-21 | EISREFSENS_50M + 3.3 + ΔFR2_REFSENS |
| 50 | 120 | G-FR2-A1-22 | EISREFSENS_50M + 3.3 + ΔFR2_REFSENS |
| 100, 200, 400 | 120 | G-FR2-A1-23 | EISREFSENS_50M + 3.3 + 3 + ΔFR2_REFSENS |
| NOTE 1: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full IAB-MT channel bandwidth. NOTE 2: The declared EISREFSENS_50M shall be within the range specified above. |  |  |  |

## 6.13	OTA maximum input level

### 6.13.1	Definition and applicability

The maximum input level is defined as the maximum mean power, for which the throughput shall meet or exceed the minimum requirements for the specified reference measurement channel.

This requirement applies at MT RIB only.

### 6.13.2	Minimum requirement

For LA class NCR type 2-O, the maximum input power is specified in TS 38.101-2 [x], clause 7.4.

### 6.13.3	Test purpose

Maximum input level tests ability to receive data with a given average throughput for a specified reference measurement channel, under conditions of high signal level, ideal propagation and no added noise.

### 6.13.4	Method of test

For test method, refer to TS 38.521-2 [x].

### 6.13.5	Test requirements

## 6.14	OTA adjacent channel selectivity

### 6.14.1	Definition and applicability

Adjacent Channel Selectivity (ACS) is a measure of a receiver's ability to receive a NR signal at its assigned channel frequency in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel. ACS is the ratio of the receive filter attenuation on the assigned channel frequency to the receive filter attenuation on the adjacent channel(s).

This requirement applies at MT RIB only.

### 6.14.2	Minimum requirement

For WA class NCR type 2-O, the ACS requirement is specified the same as the Wide Area IAB-MT ACS requirement in TS 38.174 [x], clause 10.5.1.4.

For LA class NCR type 2-O, the ACS requirement is specified the same as ACS requirement in TS 38.101-2 [x], clause 7.5.

### 6.14.3	Test purpose

Adjacent channel selectivity tests the ability to receive data with a given average throughput for a specified reference measurement channel, in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel, under conditions of ideal propagation and no added noise.

### 6.14.4	Method of test

For test method, refer to TS 38.521-2 [x].

### 6.14.5	Test requirements

## 6.15	OTA blocking characteristics

### 6.15.1	Definition and applicability

The blocking characteristic is a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency in the presence of an unwanted interferer on frequencies other than those of the spurious response or the adjacent channels, without this unwanted input signal causing a degradation of the performance of the receiver beyond a specified limit. The blocking performance shall apply at all frequencies except those at which a spurious response occurs.

This requirement applies at MT RIB only.

### 6.15.2	Minimum requirement

For WA class NCR type 2-O, the blocking requirement is specified the same as the Wide Area IAB-MT blocking requirement in TS 38.174 [x], clause 10.5.2.4.

For LA class NCR type 2-O, the blocking requirement is specified the same as blocking requirement in TS 38.101-2 [x], clause 7.6.

### 6.15.3	Test purpose

In-band blocking is defined for an unwanted interfering signal falling into the receive band or into the spectrum equivalent to twice the channel bandwidth below or above the receive band at which the relative throughput shall meet or exceed the minimum requirement for the specified measurement channels.

### 6.15.4	Method of test

For test method, refer to TS 38.521-2 [x].

### 6.15.5	Test requirements

## 6.16	OTA spurious emissions

### 6.16.1	Definition and applicability

The spurious emissions power is the power of emissions generated or amplified in a receiver. The spurious emissions power level is measured as TRP.

This requirement applies at MT RIB only.

### 6.16.2	Minimum requirement

For WA class NCR type 2-O, the OTA spurious emission requirement is specified the same as the Wide Area IAB-MT receiver spurious emission requirement in TS 38.174 [x], clause 10.7.3.2.

For LA class NCR type 2-O, the OTA spurious emission requirement is specified the same as receiver spurious emission requirement in TS 38.101-2 [xx], clause 7.9.

### 6.16.3	Test purpose

Test verifies whether spurious emissions meet the requirements described in clause 6.16.5. Excess spurious emissions increase the interference to other systems.

### 6.16.4	Method of test

For test method, refer to TS 38.521-2 [x].

### 6.16.5	Test requirements