3GPP TSG-RAN4 Meeting # 111	R4-2409576

# Fukuoka, JP, 20 - 24 May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 4576 | rev | - | Current version: | 17.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_pos_enh-Perf) CR to 38.133 Corrections to measurement reporting resolutions for RSTD and UE Rx-Tx measurements for Rel. 17 NR positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Perf |  |  |  |  |  | Date: |  |  | 2024-05-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Some measurement reporting resolutions for RSTD and UE Rx-Tx measurements are in []. |  |  |  |  |  |  |  |  |
| Summary of change: |  | [] around some reporting resolutions for RSTD and UE Rx-Tx measurements are removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Reporting resolution for RSTD and UE Rx-Tx measurements remain ambiguous. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.23.3.1, 10.1.23.3.2, 10.1.23.3.3, 10.1.25.3.1, 10.1.25.3.2, 10.1.25.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE 1

#### 10.1.23.3	Report mapping

##### 10.1.23.3.1	Absolute DL RSTD Measurement Reporting

The reporting range for the DL RSTD measurement is defined from -985024Tc to 985024Tc with the resolution step of 2kTc, where

Tc is defined in TS 38.211 [6],

kmin≤k≤kmax,

kmin=2 and kmax=5, when configured PRS resource of at least one of the reference cell and neighbor cell measured for the RSTD measurement is in FR1,

kmin=0 and kmax=5, when configured PRS resource of both the reference cell and neighbor cell measured for the RSTD measurement are in FR2,

k≥ timingReportingGranularityFactor [34] configured by LMF via LPP for the RSTD measurement.

The measurement report mapping for different k values are specified in Tables 10.1.23.3.1-1  10.1.23.3.1-6.

Table 10.1.23.3.1-1: Report mapping for k=0

| Reported Quantity Value, | Measured Quantity Value, | Unit |
| --- | --- | --- |
| RSTD_i | RSTD |  |
| RSTD_0000000 | RSTD < -985024 | Tc |
| RSTD_0000001 | -985024 RSTD < -985023 | Tc |
| RSTD_0000002 | -985023 RSTD < -985022 | Tc |
|  |  | … |
| RSTD_0985024 | -1 RSTD < 0 | Tc |
| RSTD_0985025 | 0 RSTD < 1 | Tc |
| … | … | … |
| RSTD_1970047 | 985022 RSTD < 985023 | Tc |
| RSTD_1970048 | 985023 RSTD < 985024 | Tc |
| RSTD_1970049 | 985024 RSTD | Tc |

Table 10.1.23.3.1-2: Report mapping for k=1

| Reported Quantity Value, | Measured Quantity Value, | Unit |
| --- | --- | --- |
| RSTD_i | RSTD |  |
| RSTD_000000 | RSTD < -985024 | Tc |
| RSTD_000001 | -985024 RSTD < -985022 | Tc |
| RSTD_000002 | -985022 RSTD < -985020 | Tc |
|  |  | … |
| RSTD_492512 | -2 RSTD < 0 | Tc |
| RSTD_492513 | 0 RSTD < 2 | Tc |
| … | … | … |
| RSTD_985023 | 985020 RSTD < 985022 | Tc |
| RSTD_985024 | 985022 RSTD < 985024 | Tc |
| RSTD_985025 | 985024 RSTD | Tc |

Table 10.1.23.3.1-3: Report mapping for k=2

| Reported Quantity Value, | Measured Quantity Value, | Unit |
| --- | --- | --- |
| RSTD_i | RSTD |  |
| RSTD_000000 | RSTD < -985024 | Tc |
| RSTD_000001 | -985024 RSTD < -985020 | Tc |
| RSTD_000002 | -985020 RSTD < -985016 | Tc |
|  |  | … |
| RSTD_246256 | -4 RSTD < 0 | Tc |
| RSTD_246257 | 0 RSTD < 4 | Tc |
| … | … | … |
| RSTD_492511 | 985016 RSTD < 985020 | Tc |
| RSTD_492512 | 985020 RSTD < 985024 | Tc |
| RSTD_492513 | 985024 RSTD | Tc |

Table 10.1.23.3.1-4: Report mapping for k=3

| Reported Quantity Value | Measured Quantity Value, | Unit |
| --- | --- | --- |
| RSTD_i | RSTD |  |
| RSTD_000000 | RSTD < -985024 | Tc |
| RSTD_000001 | -985024 RSTD < -985016 | Tc |
| RSTD_000002 | -985016 RSTD < -985008 | Tc |
|  |  | … |
| RSTD_123128 | -8 RSTD < 0 | Tc |
| RSTD_123129 | 0 RSTD < 8 | Tc |
| … | … | … |
| RSTD_246255 | 985008 RSTD < 985016 | Tc |
| RSTD_246256 | 985016 RSTD < 985024 | Tc |
| RSTD_246257 | 985024 RSTD | Tc |

Table 10.1.23.3.1-5: Report mapping for k=4

| Reported Quantity Value, | Measured Quantity Value, | Unit |
| --- | --- | --- |
| RSTD_i | RSTD |  |
| RSTD_000000 | RSTD < -985024 | Tc |
| RSTD_000001 | -985024 RSTD < -985008 | Tc |
| RSTD_000002 | -985008 RSTD < -984992 | Tc |
|  |  | … |
| RSTD_061564 | -16 RSTD < 0 | Tc |
| RSTD_061565 | 0 RSTD < 16 | Tc |
| … | … | … |
| RSTD_123127 | 984992 RSTD < 985008 | Tc |
| RSTD_123128 | 985008 RSTD < 985024 | Tc |
| RSTD_123129 | 985024 RSTD | Tc |

Table 10.1.23.3.1-6: Report mapping for k=5

| Reported Quantity Value, | Measured Quantity Value, | Unit |
| --- | --- | --- |
| RSTD_i | RSTD |  |
| RSTD_00000 | RSTD < -985024 | Tc |
| RSTD_00001 | -985024 RSTD < -984992 | Tc |
| RSTD_00002 | -984992 RSTD < -984960 | Tc |
|  |  | … |
| RSTD_30782 | -32 RSTD < 0 | Tc |
| RSTD_30783 | 0 RSTD < 32 | Tc |
| … | … | … |
| RSTD_61563 | 984960 RSTD < 984992 | Tc |
| RSTD_61564 | 984992 RSTD < 985024 | Tc |
| RSTD_61565 | 985024 RSTD | Tc |

##### 10.1.23.3.2	Differential Reporting for DL RSTD Measurement

A first DL RSTD measurement is reported by means of differential reporting, i.e. as RSTD, relative to a second DL RSTD measurement (RSTD2), provided that:

-	the absolute measured quantity value of the second DL RSTD measurement (RSTD2) is not larger than the absolute measured quantity value of the first DL RSTD measurement (RSTD1), i.e., RSTD=RSTD1-RSTD2≥0, and

-	the absolute value of the second DL RSTD measurement (RSTD2) is reported together with RSTD for the first DL RSTD measurement.

The reporting range for differential reporting RSTD of the first DL RSTD measurement is defined from 0 up to 8191Tc with the resolution step of 2kTc, where

Tc is defined in TS 38.211 [6],

kmin≤k≤kmax,

kmin=2 and kmax=5, when configured PRS resource of at least one of the reference cell and neighbor cell measured for the first RSTD measurement or second RSTD measurement is in FR1,

kmin=0 and kmax=5, when configured PRS resource of both the reference cell and neighbor cell measured for both of the first RSTD measurement and the second RSTD measurement are in FR2,

k≥ timingReportingGranularityFactor [34] configured by LMF via LPP for the RSTD measurement.

The measurement report mapping for different k values are specified in Tables 10.1.23.3.2-1  10.1.23.3.2-6.

Table 10.1.23.3.2-1: Report mapping for k=0

| Reported Quantity Value, DIFFRSTD_i | RSTD = RSTD1 RSTD2 | Unit |
| --- | --- | --- |
| DIFFRSTD_0000 | 0 RSTD < 1 | Tc |
| DIFFRSTD_0001 | 1 RSTD < 2 | Tc |
| DIFFRSTD_0002 | 2 RSTD < 3 | Tc |
|  |  | … |
| DIFFRSTD_8189 | 8189 RSTD < 8190 | Tc |
| DIFFRSTD_8190 | 8190 RSTD < 8191 | Tc |
| DIFFRSTD_8191 | 8191 RSTD | Tc |

Table 10.1.23.3.2-2: Report mapping for k=1

| Reported Quantity Value, DIFFRSTD_i | RSTD = RSTD1 RSTD2 | Unit |
| --- | --- | --- |
| DIFFRSTD_0000 | 0 RSTD < 2 | Tc |
| DIFFRSTD_0001 | 2 RSTD < 4 | Tc |
| DIFFRSTD_0002 | 4 RSTD < 6 | Tc |
|  |  | … |
| DIFFRSTD_4093 | 8186 RSTD < 8188 | Tc |
| DIFFRSTD_4094 | 8188 RSTD < 8190 | Tc |
| DIFFRSTD_4095 | 8190 RSTD | Tc |

Table 10.1.23.3.2-3: Report mapping for k=2

| Reported Quantity Value, DIFFRSTD_i | RSTD = RSTD1 RSTD2 | Unit |
| --- | --- | --- |
| DIFFRSTD_0000 | 0 RSTD < 4 | Tc |
| DIFFRSTD_0001 | 4 RSTD < 8 | Tc |
| DIFFRSTD_0002 | 8 RSTD < 12 | Tc |
|  |  | … |
| DIFFRSTD_2045 | 8180 RSTD < 8184 | Tc |
| DIFFRSTD_2046 | 8184 RSTD < 8188 | Tc |
| DIFFRSTD_2047 | 8188 RSTD | Tc |

Table 10.1.23.3.2-4: Report mapping for k=3

| Reported Quantity Value, DIFFRSTD_i | RSTD = RSTD1 RSTD2 | Unit |
| --- | --- | --- |
| DIFFRSTD_0000 | 0 RSTD < 8 | Tc |
| DIFFRSTD_0001 | 8 RSTD < 16 | Tc |
| DIFFRSTD_0002 | 16 RSTD < 24 | Tc |
|  |  | … |
| DIFFRSTD_1021 | 8168 RSTD < 8176 | Tc |
| DIFFRSTD_1022 | 8176 RSTD < 8184 | Tc |
| DIFFRSTD_1023 | 8184 RSTD | Tc |

Table 10.1.23.3.2-5: Report mapping for k=4

| Reported Quantity Value, DIFFRSTD_i | RSTD = RSTD1 RSTD2 | Unit |
| --- | --- | --- |
| DIFFRSTD_000 | 0 RSTD < 16 | Tc |
| DIFFRSTD_001 | 16 RSTD < 32 | Tc |
| DIFFRSTD_002 | 32 RSTD < 48 | Tc |
|  |  | … |
| DIFFRSTD_509 | 8144 RSTD < 8160 | Tc |
| DIFFRSTD_510 | 8160 RSTD < 8176 | Tc |
| DIFFRSTD_511 | 8176 RSTD | Tc |

Table 10.1.23.3.2-6: Report mapping for k=5

| Reported Quantity Value, DIFFRSTD_i | RSTD = RSTD1 RSTD2 | Unit |
| --- | --- | --- |
| DIFFRSTD_000 | 0 RSTD < 32 | Tc |
| DIFFRSTD_001 | 32 RSTD < 64 | Tc |
| DIFFRSTD_002 | 64 RSTD < 96 | Tc |
|  |  | … |
| DIFFRSTD_253 | 8096 RSTD < 8128 | Tc |
| DIFFRSTD_254 | 8128 RSTD < 8160 | Tc |
| DIFFRSTD_255 | 8160 RSTD | Tc |

##### 10.1.23.3.3	Additional Path Report Mapping for DL RSTD

The reporting range for the additional path reporting for an RSTD measurement is defined up to the range from -8175Tc to 8175Tc with the resolution step of 2kTc, where

Tc is defined in TS 38.211 [6],

kmin≤k≤kmax,

kmin=2 and kmax=5, when configured PRS resource of at least one of the reference cell and neighbor cell measured for the RSTD measurement is in FR1,

kmin=0 and kmax=5, when configured PRS resource of both the reference cell and neighbor cell measured for the RSTD measurement are in FR2,

k≥ timingReportingGranularityFactor [34] configured by LMF via LPP for the RSTD measurement.

The UE can report the timing of up to two additional paths with respect to the path timing determining the RSTD measurement.

A UE capable of  additionalPathsExtSupport-r17 can report the timing for a number additional paths, up to its capability, with respect to the path timing determining the RSTD measurement.

The report mappings for different k values are specified in Tables 10.1.23.3.3-1  10.1.23.3.3-6.

Table 10.1.23.3.3-1: Report mapping for k=0

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_00000 | path < -8175 | Tc |
| path_00001 | -8175 path < -8174 | Tc |
| path_00002 | -8174 path < -8173 | Tc |
|  |  | … |
| path_08175 | -1 path < 0 | Tc |
| path_08176 | 0 path < 1 | Tc |
| … | … | … |
| path_ 16349 | 8173 path < 8174 | Tc |
| path_ 16350 | 8174 path < 8175 | Tc |
| path_ 16351 | 8175 path | Tc |

Table 10.1.23.3.3-2: Report mapping for k=1

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_0000 | path < -8175 | Tc |
| path_0001 | -8175 path < -8173 | Tc |
| path_0002 | -8173 path < -8171 | Tc |
|  |  | … |
| path_4088 | -1 path < 1 | Tc |
| … | … | … |
| path_8174 | 8171 path < 8173 | Tc |
| path_8175 | 8173 path < 8175 | Tc |
| path_8176 | 8175 path | Tc |

Table 10.1.23.3.3-3: Report mapping for k=2

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_0000 | path < -8174 | Tc |
| path_0001 | -8174 path < -8170 | Tc |
| path_0002 | -8170 path < -8166 | Tc |
|  |  | … |
| path_2044 | -2 path < 2 | Tc |
| … | … | … |
| path_4086 | 8166 path < 8170 | Tc |
| path_4087 | 8170 path < 8174 | Tc |
| path_4088 | 8174 path | Tc |

Table 10.1.23.3.3-4: Report mapping for k=3

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_0000 | path < -8172 | Tc |
| path_0001 | -8172 path < -8164 | Tc |
| path_0002 | -8164 path < -8156 | Tc |
|  |  | … |
| path_1022 | -4 path < 4 | Tc |
| … | … | … |
| path_2042 | 8156 path < 8164 | Tc |
| path_2043 | 8164 path < 8172 | Tc |
| path_2044 | 8172 path | Tc |

Table 10.1.23.3.3-5: Report mapping for k=4

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_0000 | path < -8168 | Tc |
| path_0001 | -8168 path < -8152 | Tc |
| path_0002 | -8152 path < -8136 | Tc |
|  |  | … |
| path_511 | -8 path < 8 | Tc |
| … | … | … |
| path_1020 | 8136 path < 8152 | Tc |
| path_1021 | 8152 path < 8168 | Tc |
| path_1022 | 8168 path | Tc |

Table 10.1.23.3.3-6: Report mapping for k=5

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_000 | path < -8160 | Tc |
| path_001 | -8160 path < -8128 | Tc |
| path_002 | -8128 path < -8096 | Tc |
|  |  | … |
| path_256 | 0 path < 32 | Tc |
| … | … | … |
| path_509 | 8096 path < 8128 | Tc |
| path_510 | 8128 path < 8160 | Tc |
| path_511 | 8160 path | Tc |

END OF CHANGE 1

START OF CHANGE 2

#### 10.1.25.3	Report mapping

Absolute UE Rx-Tx measurement reporting in clause 10.1.25.3.1, differential reporting for UE Rx-Tx measurement in clause 10.1.25.3.2, and additional path report mapping for UE Rx-Tx measurement in clause 10.1.25.3.3 applies, regardless of number of samples used to measure PRS, to report:

-	TEG based measurement corresponding to UE reported Rx TEG in nr-UE-Rx-TEG-ID-r17 [34],

-	gap-based UE Rx-Tx measurement,

-	gapless UE Rx-Tx measurement,

-	UE Rx-Tx in RRC_INACTIVE state.

##### 10.1.25.3.1	Absolute UE Rx-Tx Measurement Report Mapping

The reporting range for the absolute UE Rx-Tx time difference measurement (TUE Rx-Tx) is defined from -985024Tc to 985024Tc with the resolution step of 2kTc, where:

Tc is defined in TS 38.211 [6],

kmin≤k≤kmax,

kmin=2 and kmax=5, when at least one of the PRS and the SRS resources configured for TUE Rx-Tx is in FR1,

kmin=0 and kmax=5, when both PRS and SRS resources configured for TUE Rx-Tx are in FR2,

k≥ timingReportingGranularityFactor [34] configured by LMF via LPP for the UE Rx-Tx time difference measurement.

The TUE Rx-Tx report mapping for k = 0, 1, 2, 3, 4, and 5 are specified in Tables 10.1.25.3.1-1, 10.1.25.3.1-2, 10.1.25.3.1-3, 10.1.25.3.1-4, 10.1.25.3.1-5, and 10.1.25.3.1-6, respectively.

Table 10.1.25.3.1-1: Absolute UE Rx-Tx time difference measurement report mapping for k=0

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_TIME_DIFFERENCE_0000 | TUE Rx-Tx < -985024 | Tc |
| RX-TX_TIME_DIFFERENCE_0001 | -985024  TUE Rx-Tx < -985023 | Tc |
| RX-TX_TIME_DIFFERENCE_0002 | -985023  TUE Rx-Tx < -985022 | Tc |
|  |  | … |
| RX-TX_TIME_DIFFERENCE_985024 | -1  TUE Rx-Tx < 0 | Tc |
| RX-TX_TIME_DIFFERENCE_985025 | 0  TUE Rx-Tx < 1 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_1970047 | 985022  TUE Rx-Tx < 985023 | Tc |
| RX-TX_TIME_DIFFERENCE_1970048 | 985023  TUE Rx-Tx < 985024 | Tc |
| RX-TX_TIME_DIFFERENCE_1970049 | 985024  TUE Rx-Tx | Tc |

Table 10.1.25.3.1-2: Absolute UE Rx-Tx time difference measurement report mapping for k=1

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_TIME_DIFFERENCE_0000 | TUE Rx-Tx < -985024 | Tc |
| RX-TX_TIME_DIFFERENCE_0001 | -985024  TUE Rx-Tx < -985022 | Tc |
| RX-TX_TIME_DIFFERENCE_0002 | -985022  TUE Rx-Tx < -985020 | Tc |
|  |  | … |
| RX-TX_TIME_DIFFERENCE_492512 | -2  TUE Rx-Tx < 0 | Tc |
| RX-TX_TIME_DIFFERENCE_492513 | 0  TUE Rx-Tx < 2 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_985023 | 985020  TUE Rx-Tx < 985022 | Tc |
| RX-TX_TIME_DIFFERENCE_985024 | 985022  TUE Rx-Tx < 985024 | Tc |
| RX-TX_TIME_DIFFERENCE_985025 | 985024  TUE Rx-Tx | Tc |

Table 10.1.25.3.1-3: Absolute UE Rx-Tx time difference measurement report mapping for k=2

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_TIME_DIFFERENCE_0000 | TUE Rx-Tx < -985024 | Tc |
| RX-TX_TIME_DIFFERENCE_0001 | -985024  TUE Rx-Tx < -985020 | Tc |
| RX-TX_TIME_DIFFERENCE_0002 | -985020  TUE Rx-Tx < -985016 | Tc |
|  |  | … |
| RX-TX_TIME_DIFFERENCE_246256 | -4  TUE Rx-Tx < 0 | Tc |
| RX-TX_TIME_DIFFERENCE_246257 | 0  TUE Rx-Tx < 4 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_492511 | 985016  TUE Rx-Tx < 985020 | Tc |
| RX-TX_TIME_DIFFERENCE_492512 | 985020  TUE Rx-Tx < 985024 | Tc |
| RX-TX_TIME_DIFFERENCE_492513 | 985024  TUE Rx-Tx | Tc |

Table 10.1.25.3.1-4: Absolute UE Rx-Tx time difference measurement report mapping for k=3

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_TIME_DIFFERENCE_0000 | TUE Rx-Tx < -985024 | Tc |
| RX-TX_TIME_DIFFERENCE_0001 | -985024  TUE Rx-Tx < -985016 | Tc |
| RX-TX_TIME_DIFFERENCE_0002 | -985016  TUE Rx-Tx < -985008 | Tc |
|  |  | … |
| RX-TX_TIME_DIFFERENCE_123128 | -8  TUE Rx-Tx < 0 | Tc |
| RX-TX_TIME_DIFFERENCE_123129 | 0  TUE Rx-Tx < 8 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_246255 | 985008  TUE Rx-Tx < 985016 | Tc |
| RX-TX_TIME_DIFFERENCE_246256 | 985016  TUE Rx-Tx < 985024 | Tc |
| RX-TX_TIME_DIFFERENCE_246257 | 985024  TUE Rx-Tx | Tc |

Table 10.1.25.3.1-5: Absolute UE Rx-Tx time difference measurement report mapping for k=4

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_TIME_DIFFERENCE_0000 | TUE Rx-Tx < -985024 | Tc |
| RX-TX_TIME_DIFFERENCE_0001 | -985024  TUE Rx-Tx < -985008 | Tc |
| RX-TX_TIME_DIFFERENCE_0002 | -985008  TUE Rx-Tx < -984992 | Tc |
|  |  | … |
| RX-TX_TIME_DIFFERENCE_61564 | -16  TUE Rx-Tx < 0 | Tc |
| RX-TX_TIME_DIFFERENCE_61565 | 0  TUE Rx-Tx < 16 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_123127 | 984992  TUE Rx-Tx < 985008 | Tc |
| RX-TX_TIME_DIFFERENCE_123128 | 985008  TUE Rx-Tx < 985024 | Tc |
| RX-TX_TIME_DIFFERENCE_123129 | 985024  TUE Rx-Tx | Tc |

Table 10.1.25.3.1-6: Absolute UE Rx-Tx time difference measurement report mapping for k=5

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_TIME_DIFFERENCE_0000 | TUE Rx-Tx < -985024 | Tc |
| RX-TX_TIME_DIFFERENCE_0001 | -985024  TUE Rx-Tx < -984992 | Tc |
| RX-TX_TIME_DIFFERENCE_0002 | -984992  TUE Rx-Tx < -984960 | Tc |
|  |  | … |
| RX-TX_TIME_DIFFERENCE_30782 | -32  TUE Rx-Tx < 0 | Tc |
| RX-TX_TIME_DIFFERENCE_30783 | 0  TUE Rx-Tx < 32 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_61563 | 984960  TUE Rx-Tx < 984992 | Tc |
| RX-TX_TIME_DIFFERENCE_61564 | 984992  TUE Rx-Tx < 985024 | Tc |
| RX-TX_TIME_DIFFERENCE_61565 | 985024  TUE Rx-Tx | Tc |

##### 10.1.25.3.2	Differential UE Rx-Tx Measurement Report Mapping

The reporting range for differential UE Rx-Tx time difference measurement (TUE Rx-Tx) is defined from 0 up to 8191Tc where:

TUE Rx-Tx = TUE Rx-Tx1 - TUE Rx-Tx2; where:

TUE Rx-Tx1 > TUE Rx-Tx2,

TUE Rx-Tx1 is the first absolute UE Rx-Tx time difference measurement,

TUE Rx-Tx1 is the second absolute UE Rx-Tx time difference measurement,

Tc is defined in TS 38.211 [6],

kmin≤k≤kmax,

kmin=2 and kmax=5, when at least one of the PRS and the SRS resources configured for TUE Rx-Tx is in FR1,

kmin=0 and kmax=5, when all the PRS and SRS resources configured for TUE Rx-Tx are in FR2,

k≥ timingReportingGranularityFactor [34] configured by LMF via LPP for the UE Rx-Tx time difference measurement.

The TUE Rx-Tx report mapping for k = 0, 1, 2, 3, 4, and 5 are specified in Tables 10.1.25.3.2-1, 10.1.25.3.2-2, 10.1.25.3.2-3, 10.1.25.3.2-4, 10.1.25.3.2-5, and 10.1.25.3.2-6, respectively.

Table 10.1.25.3.2-1: Differential UE Rx-Tx time difference measurement report mapping for k=0

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| DIFF_RX-TX_TIME_DIFFERENCE_0000 | 0  TUE Rx-Tx < 1 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_0001 | 1  TUE Rx-Tx < 2 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_0002 | 2  TUE Rx-Tx < 3 | Tc |
|  |  | … |
| DIFF_RX-TX_TIME_DIFFERENCE_8189 | 8189  TUE Rx-Tx < 8190 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_8190 | 8190  TUE Rx-Tx < 8191 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_8191 | 8191  TUE Rx-Tx | Tc |

Table 10.1.25.3.2-2: Differential UE Rx-Tx time difference measurement report mapping for k=1

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| DIFF_RX-TX_TIME_DIFFERENCE_0000 | 0  TUE Rx-Tx < 2 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_0001 | 2  TUE Rx-Tx < 4 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_0002 | 4  TUE Rx-Tx < 6 | Tc |
|  |  | … |
| DIFF_RX-TX_TIME_DIFFERENCE_4093 | 8186  TUE Rx-Tx < 8188 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_4094 | 8188  TUE Rx-Tx < 8190 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_4095 | 8190  TUE Rx-Tx | Tc |

Table 10.1.25.3.2-3: Differential UE Rx-Tx time difference measurement report mapping for k=2

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| DIFF_RX-TX_TIME_DIFFERENCE_0000 | 0  TUE Rx-Tx < 4 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_0001 | 4  TUE Rx-Tx < 8 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_0002 | 8  TUE Rx-Tx < 12 | Tc |
|  |  | … |
| DIFF_RX-TX_TIME_DIFFERENCE_2045 | 8180  TUE Rx-Tx < 8184 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_2046 | 8184  TUE Rx-Tx < 8188 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_2047 | 8188  TUE Rx-Tx | Tc |

Table 10.1.25.3.2-4: Differential UE Rx-Tx time difference measurement report mapping for k=3

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| DIFF_RX-TX_TIME_DIFFERENCE_0000 | 0  TUE Rx-Tx < 8 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_0001 | 8  TUE Rx-Tx < 16 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_0002 | 16  TUE Rx-Tx < 24 | Tc |
|  |  | … |
| DIFF_RX-TX_TIME_DIFFERENCE_1021 | 8168  TUE Rx-Tx < 8176 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_1022 | 8176  TUE Rx-Tx < 8184 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_1023 | 8184  TUE Rx-Tx | Tc |

Table 10.1.25.3.2-5: Differential UE Rx-Tx time difference measurement report mapping for k=4

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| DIFF_RX-TX_TIME_DIFFERENCE_0000 | 0  TUE Rx-Tx < 16 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_0001 | 16  TUE Rx-Tx < 32 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_0002 | 32  TUE Rx-Tx < 48 | Tc |
|  |  | … |
| DIFF_RX-TX_TIME_DIFFERENCE_509 | 8144  TUE Rx-Tx < 8160 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_510 | 8160  TUE Rx-Tx < 8176 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_511 | 8176  TUE Rx-Tx | Tc |

Table 10.1.25.3.2-6: Differential UE Rx-Tx time difference measurement report mapping for k=5

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| DIFF_RX-TX_TIME_DIFFERENCE_0000 | 0  TUE Rx-Tx < 32 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_0001 | 32  TUE Rx-Tx < 64 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_0002 | 64  TUE Rx-Tx < 96 | Tc |
|  |  | … |
| DIFF_RX-TX_TIME_DIFFERENCE_253 | 8096  TUE Rx-Tx < 8128 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_254 | 8128  TUE Rx-Tx < 8160 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_255 | 8160  TUE Rx-Tx | Tc |

##### 10.1.25.3.3	Additional Path Report Mapping for UE Rx-Tx Time Difference

The reporting range for the additional path reporting for an UE Rx-Tx time difference measurement is defined up to the range from -8175Tc to 8175Tc with the resolution step of 2kTc, where

Tc is defined in TS 38.211 [6],

kmin≤k≤kmax,

kmin=2 and kmax=5, when at least one of the PRS resource and SRS resource configured for the UE Rx-Tx time difference measurement is in FR1,

kmin=0 and kmax=5, when both of the PRS resource and SRS resource configured for the UE Rx-Tx time difference measurement is in FR2,

k≥ timingReportingGranularityFactor [34] configured by LMF via LPP for the UE Rx-Tx time difference measurement.

The UE can report the timing of up to two additional paths with respect to the path timing determining the UE Rx-Tx time difference measurement.

The UE capable of  additionalPathsExtSupport-r17 can report the timing of up to its supported number of additional paths with respect to the path timing determining the UE Rx-Tx measurement.

The report mappings for different k values are specified in Tables 10.1.25.3.3-1  10.1.25.3.3-6.

Table 10.1.25.3.3-1: Report mapping for k=0

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_00000 | path < -8175 | Tc |
| path_00001 | -8175 path < -8174 | Tc |
| path_00002 | -8174 path < -8173 | Tc |
|  |  | … |
| path_08175 | -1 path < 0 | Tc |
| path_08176 | 0 path < 1 | Tc |
| … | … | … |
| path_ 16349 | 8173 path < 8174 | Tc |
| path_ 16350 | 8174 path < 8175 | Tc |
| path_ 16351 | 8175 path | Tc |

Table 10.1.25.3.3-2: Report mapping for k=1

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_0000 | path < -8175 | Tc |
| path_0001 | -8175 path < -8173 | Tc |
| path_0002 | -8173 path < -8171 | Tc |
|  |  | … |
| path_4088 | -1 path < 1 | Tc |
| … | … | … |
| path_8174 | 8171 path < 8173 | Tc |
| path_8175 | 8173 path < 8175 | Tc |
| path_8176 | 8175 path | Tc |

Table 10.1.25.3.3-3: Report mapping for k=2

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_0000 | path < -8174 | Tc |
| path_0001 | -8174 path < -8170 | Tc |
| path_0002 | -8170 path < -8166 | Tc |
|  |  | … |
| path_2044 | -2 path < 2 | Tc |
| … | … | … |
| path_4086 | 8166 path < 8170 | Tc |
| path_4087 | 8170 path < 8174 | Tc |
| path_4088 | 8174 path | Tc |

Table 10.1.25.3.3-4: Report mapping for k=3

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_0000 | path < -8172 | Tc |
| path_0001 | -8172 path < -8164 | Tc |
| path_0002 | -8164 path < -8156 | Tc |
|  |  | … |
| path_1022 | -4 path < 4 | Tc |
| … | … | … |
| path_2042 | 8156 path < 8164 | Tc |
| path_2043 | 8164 path < 8172 | Tc |
| path_2044 | 8172 path | Tc |

Table 10.1.25.3.3-5: Report mapping for k=4

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_0000 | path < -8168 | Tc |
| path_0001 | -8168 path < -8152 | Tc |
| path_0002 | -8152 path < -8136 | Tc |
|  |  | … |
| path_511 | -8 path < 8 | Tc |
| … | … | … |
| path_1020 | 8136 path < 8152 | Tc |
| path_1021 | 8152 path < 8168 | Tc |
| path_1022 | 8168 path | Tc |

Table 10.1.25.3.3-6: Report mapping for k=5

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_000 | path < -8160 | Tc |
| path_001 | -8160 path < -8128 | Tc |
| path_002 | -8128 path < -8096 | Tc |
|  |  | … |
| path_256 | 0 path < 32 | Tc |
| … | … | … |
| path_509 | 8096 path < 8128 | Tc |
| path_510 | 8128 path < 8160 | Tc |
| path_511 | 8160 path | Tc |

END OF CHANGE 2