3GPP TSG-RAN WG4 Meeting#110bis                              R4-2409602

Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

Agenda item:	10.8.2.2

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on RF requirement for Inter-band CA for ATG UE in Rel-19

Document for:	Approval

Introduction

In the RAN#103 meeting, new WID on Enhancements for Air-to-ground network for NR has been approved and it’s expected to start the normative work from this April meeting. In this contribution, we would like to share some views on RF requirements for ATG UE supporting inter-band DL CA.

Issue 3-3-1: clarify the antenna type for each band for inter-band CA for further capability signaling definition - Proposals - Option 1: both following options are allowed (CMCC) - Option 1: Only one antenna type could be used for one CA inter-band combination. - Option 2: Both antenna types could be used for one CA inter-band combination. - Option 2: TBA Agreement: - FFS on the antenna type for ATG UE supporting inter-band CA. Issue 3-3-2: whether UL is only configured in band n3 for CA_n3_n39 - Proposals - Option 1: RAN4 should discuss the potential RF architecture for ATG UE supporting DL CA_n3-n39 and whether Uplink transmission is only configured in band n3 for ATG CA_n3-n39 (Huawei, CMCC) - Option 2: Agreement: - FFS on whether n39 can be transmitted for CA_n3_n39 Issue 3-3-3: BCS for DL CA_n3_n39 - Proposals - Option 1: (Apple) Table 2.2-2 BCS for CA_n3-n39 NR CA config; Uplink CA configuration or single uplink carrier10; NR Band; Channel bandwidth (MHz); BCS. CA_n3A-n39A; -; n3; 10, 15, 20, 25, 30, 35, 40, 45, 50; 0. n39; 10, 15, 20, 25, 30, 35, 40. - Option 2: TBA Agreement: - FFS on supported BCS for DL CA_n3_n39 Issue 3-3-4: Rx RF requirements for inter-band DL CA - Proposals - Option 1: (CMCC, Apple) - except for delta RIB, MSD requirements, per band requirements i.e the requirements in suffix J could be reused for ATG CA inter-band combination. - For delta RIB, MSD requirements, companies needs detailed analysis of whether current legacy UE requirements could be reused rather than directly reuse. - Option 2: (ZTE) - ΔRIB,c, in-band blocking, out of band blocking, spurious response, wide band intermodulation, spurious emissions could reuse legacy inter-band CA requirements - Narrow band blocking is not applicable Agreement: - FFS on the ATG Rx requirements for inter-band CA

Discussion

- 2.1. BCS for CA_n3+n39

- In general, BCS is related to the supported channel bandwidth of each constitute band in a inter-band CA band combination. In the current TS38.101-1 specification, several BCSs are defined for a certain inter-band CA band combination, the main reasons are aiming to include all the channel bandwidths in each constitute band in band combination but new channel bandwidth for a band requested by operators are introduced in different stages or different phases. Consequently, additional BCS is needed to include the new channel bandwidth. Moreover, to reduce the work load by introducing many/endless BCSs for a certain band combination, full-power BCS BCS4 and 5 are introduced.

- For CA_n3+n39, due the channel bandwidth in band n3 and band n39 are quite stable, so it is suggested to use BCS0 to include all the channel bandwidth of either n3 or n39 for CA_n3A-n39A. It should noted that only inter-band DL CA is supported.

- In addition, considering the DL frequency range of n3 is immediately closed to n39 UL, so it is proposed that only n3 is supported in the uplink.

- Considering the above, the BCS for inter-band DL CA_n3A-n39A is proposed in table 1.

Table 1.  Supported bandwidths per CA band combination of band n3+n39

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n39AX | - | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 35, 40 |  |
| NOTE X: Uplink is only in n3 for CA_n3-n39 |  |  |  |  |

- Proposal 1. The BCS for inter-band DL CA_n3A-n39A is proposed in table 1.

Table 1.  Supported bandwidths per CA band combination of band n3+n39

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n39AX | - | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 35, 40 |  |
| NOTE X: Uplink is only in n3 for CA_n3-n39 |  |  |  |  |

- 2.2. RF requirements for Inter-band CA

In this section, we provide some initial analysis on the overall requirement for ATG UE supporting inter-band DL CA and also provided the summary of proposals for it.

Table 2. Analysis and summary of proposals for ATG UE supporting inter-band DL CA

| The existing RF requirement for inter-band CA | ATG UE supporting inter-band CA |
| --- | --- |
| 6.2A.1.3 UE maximum output power for Inter-band CA | No impacts, to follow Rel-18 single carrier requirements |
| 6.2A.2.3 UE maximum output power reduction for Inter-band CA | No impacts, to follow Rel-18 single carrier requirements |
| 6.2A.3.1.3 UE additional maximum output power reduction for Inter-band CA | No impacts, to follow Rel-18 single carrier requirements |
| 6.2A.4.1.3 Configured transmitted power for Inter-band CA | No impacts, to follow Rel-18 single carrier requirements |
| 6.3A.1.3 Minimum output power for inter-band CA | No impacts, to follow Rel-18 single carrier requirements |
| 6.3A.2.3 Transmit OFF power for inter-band CA | No impacts, to follow Rel-18 single carrier requirements |
| 6.3A.3.3 Transmit ON/OFF time mask for inter-band CA | No impacts, to follow Rel-18 single carrier requirements |
| 6.3A.4.3 Power control for inter-band CA No requirements unique to CA operation are defined. | No impacts, to follow Rel-18 single carrier requirements |
| 6.4A.1.3 Frequency error for inter-band CA | No impacts, to follow Rel-18 single carrier requirements |
| 6.4A.2.3 Transmit modulation quality for inter-band CA | No impacts, to follow Rel-18 single carrier requirements |
| 6.5A.1.3 Occupied bandwidth for Inter-band CA | No impacts, to follow Rel-18 single carrier requirements |
| 6.5A.2.2.3 Spectrum emission mask for Inter-band CA | No impacts, to follow Rel-18 single carrier requirements |
| 6.5A.2.3.3 Additional spectrum emission mask for Inter-band CA | No impacts, to follow Rel-18 single carrier requirements |
| 6.5A.3.2.3 Spurious emissions for UE co-existence for Inter-band CA | No impacts, to follow Rel-18 single carrier requirements |
| 7.3A.2.3 Reference sensitivity power level for Inter-band CA | Reference sensitivity power level for Inter-band CA For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one NR band the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2, A.2.3.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 with parameters specified in Table 7.3.2-1, Table 7.3.2-2 and Table 7.3.2-3 modified in accordance with clause 7.3A.3.2. The reference sensitivity is defined to be met with all downlink component carriers active and one of the uplink carriers active. Exceptions to reference sensitivity are allowed in accordance with clause 7.3A.4. |
| 7.3A.3.2 ΔRIB,c for Inter-band CA | This needs further discussions for it. |
| 7.4A.3 Maximum input level for Inter-band CA | Maximum input level for Inter-band CA For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one NR band, the maximum input level is defined with the uplink active on the band(s) other than the band whose downlink is being tested. For NR CA configurations including an operating band without uplink band or an operating band with an unpaired DL part (as noted in Table 5.2-1), the requirements for all downlinks shall be met with the single uplink carrier active in each band capable of UL operation. The UE shall meet the requirements specified in clause 7.4J for each component carrier while all downlink carriers are active. The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexs A.3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1) for each component carrier. |
| 7.5A.3 Adjacent channel selectivity Inter-band CA | Adjacent channel selectivity Inter-band CA For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one NR band, the adjacent channel requirements are defined with the uplink active on the band(s) other than the band whose downlink is being tested. For NR CA configurations including an operating band without uplink operation or an operating band with an unpaired DL part (as noted in Table 5.2-1), the requirements for all downlinks shall be met with the single uplink carrier active in each band capable of UL operation. The ATG UE shall meet the requirements specified in clause 7.5J, for each component carrier while all downlink carriers are active. The throughput of each carrier shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1). |
| 7.6A.2.3 In-band blocking for Inter-band CA | To following the existing requirements defined in Clause 7.6A.2.3. |
| 7.6A.3.3 Out-of-band blocking for Inter-band CA | To following the existing requirements defined in Clause 7.6A.3.3. And no OOB exception for CA_n3-n39 |
| 7.6A.4.3 Narrow band blocking for Inter-band CA | N/A |
| 7.7A.3 Spurious response for Inter-band CA | To following the existing requirements defined in Clause 7.7A.3. |
| 7.8A.2.3 Wide band intermodulation for Inter-band CA | To following the existing requirements defined in Clause 7.8A.2.3. |
| 7.9A.3 Spurious emissions for Inter-band CA | To following the existing requirements defined in Clause 7.9A.3 |

Conclusions

In this contribution, we shared some initial views on the RF requirements for ATG UE supporting inter-band DL CA in Rel-19 and proposals are made as following:

- Proposal 1. The BCS for inter-band DL CA_n3A-n39A is proposed in table 1.

Table 1.  Supported bandwidths per CA band combination of band n3+n39

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n39AX | - | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 35, 40 |  |
| NOTE X: Uplink is only in n3 for CA_n3-n39 |  |  |  |  |

Proposal 2: for RF requirements for inter-band DL CA, refer to the proposals in table 2.

References

- RP-240839,New WID on Enhancements for Air-to-ground network for NR, Approved.

- R4-2321919,Big CR for TS 38.101-1 for NR ATG, Agreed.

- R4-2403864, CR for 38101-1 to update ATG related signaling name, Agreed

- R4-2406594, WF on Rel-19 ATG UE requirements, Approved.