3GPP TSG-RAN WG4 Meeting #111									                          R4- 2409661

Fukuoka, Japan, May 20 – 24, 2024

Agenda item:			10.8.2.1

Source:	Ericsson

Title:	Discussion on RF requirement of ATG UE intra-band contiguous CA

Document for:	Discussion

# Introduction

In 3GPP release 18, RAN4 has completed the standardization work for the air-to-ground (ATG) network WI, the main work focused on the FR1 single carrier scenario. To enhance the throughput to meet growing ATG capacity demand and fully utilize operators’ spectrum, the RAN plenary meeting #103 has approved a new WI on enhancements for the air-to-ground network for NR [1].

# Discussion

In the last meeting, some agreements were made addressing the general concerns and intra-band contiguous CA [2]. Part of the agreements are shown below:

  - Common requirements

Issue 3-1-1: whether maximum output power is declared per single carrier or per band(aggregated across all CCs) for inter-band CA and intra-band contiguous CA

Agreement:

- Reuse the existing Rel-18 capability for MOP for ATG UE in Rel-19

Issue 3-1-2: Tx RF requirements for intra-band contigous CA with one UL carrier and inter-band CA with one UL carrier.

Agreement:

- Reuse Tx RF requirements for ATG UE single CC operation

  - intra-band contiguous CA

Issue 3-2-1: more clarification for contiguous CA

Agreement:

- Only consider contiguous DL CA case for ATG UE intra-band CA.

- The example band combination for intra-band contiguous CA is CA_n79C

  - Intra-band Contiguous CA

The following sessions will discuss the RF requirements for ATG UE intra-band contiguous CA.

### BCS for DL CA_n79C

- BCS 0, 4, and 5 should be considered for CA_n79C

### ATG Rx requirements for intra-band CA

#### Reference sensitivity

Reference sensitivity could reuse the clauses defined in 7.3A

#### Maximum input level

The maximum input level requirement for the ATG UE was specified based on the link budget, and it differs from the TN UE. The maximum input level for ATG intra-band contiguous CA should be updated based on the Rel-18 requirement for ATG UE

Power in largest transmission bandwidth configuration CC, Plargest BW should be updated as -42 dBm for omni antenna and -30 dBm for array antenna.

- For intra-band contiguous CA, Power in largest transmission bandwidth configuration CC, Plargest BW should be updated as -42 dBm for omni antenna and -30 dBm for array antenna.

#### Adjacent channel selectivity

ACS test parameters for case 1 need to be specified, it could reuse the tables in 7.5A.1

ACS case 2 needs to be updated. Particularly Power in transmission bandwidth configuration, per CC needs to be scaled according to the ATG UE maximum input level. e.g.

Table x: Test parameters for intra-band contiguous CA with FDL_high <2700 MHz and FUL_high<2700 MHz, case 2

| Rx Parameter | Units | NR CA Bandwidth Class |  |
| --- | --- | --- | --- |
|  |  | B | C |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | -60.5 -48.5 | -57.5 -45.5 |
| PInterferer | dBm | -42 -30 | -42 -30 |

- ACS test parameters for case 1 need to be specified, it could reuse the tables in 7.5A.1, ACS case 2 needs to be updated. Particularly Power in transmission bandwidth configuration, per CC needs to be scaled according to the ATG UE maximum input level.

#### Blocking characteristic

Similar to the ATG UE requirement, for intra-band CA, the requirement defined in 7.6A.2.1 and 7.6A.3.1 could be reused.

#### Spurious response

Similar to the ATG UE requirement, for intra-band CA, the requirement defined in 7.7A.1 could be reused.

#### Intermodulation characteristics

For intra-band CA, the requirement defined in 7.8A could be reused.

- For reference sensitivity, blocking characteristic, spurious response, and intermodulation characteristics, the existing requirements defined in clause 7 could be reused.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	BCS 0, 4, and 5 should be considered for CA_n79C

Proposal 2	For intra-band contiguous CA, Power in largest transmission bandwidth configuration CC, Plargest BW should be updated as -42 dBm for omni antenna and -30 dBm for array antenna.

Proposal 4	For reference sensitivity, blocking characteristic, spurious response, and intermodulation characteristics, the existing requirements defined in clause 7 could be reused.

# References

- RP-240804, New WID on Enhancements for Air-to-ground network for NR.

- R4-2406594, WF on Rel-19 ATG UE requirements