3GPP TSG-RAN4 Meeting #111		R4-2409672

Fukuoka, Japan, May 20th – 24th, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 4586 | rev |  | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_SL_relay_enh-Perf) Applicability of SD-RSRP and SL-RSRP accuracy requirements in multipath scenario |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay_enh-Perf |  |  |  |  |  | Date: |  |  | 2024-05-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To define applicable conditions for a remote sidelink UE in multipath relay scenario to meet the SD-RSRP and SL-RSRP accuracies. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR makes additional changes to R4-2403336 which was endorsed at RAN4#110. The new changes are highlgithed. Following was agreed in R4-2406298, and this CR captures those changes as per worksplit: - Agreement: - Existing accuracies of SD-RSRP and SL-RSRP in clause 10.4.5 shall also apply for the remote UE in the multipath scenario provided that the remote UE: - is synchronised to the sidelink relay UE that is measured and - is in-coverage on the frequency used for sidelink if both the direct path and the SL on the indirect path are on the same frequency or - is out of coverage on the frequency used for sidelink if the direct path and the SL on the indirect path are on different frequencies.. |
| Consequences if not approved: |  | The SD-RSRP and SL-RSRP accuracy performance of the remote UE in multipath relay operation cannot be guaranteed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

### 10.4.5	Intra-Frequency Discovery Signal Measurement Accuracy Requirements

The requirements in this clause are applicable for a remote sidelink UE in U2N relay scenario provided that the remote UE:

-	is out of coverage on the frequency used for sidelink, and

-	that is synchronised to the sidelink relay UE that is measured.

The requirements in this clause are applicable for a remote sidelink UE in multipath relay scenario provided that the remote UE:

- is synchronised to the sidelink relay UE that is measured and

- is in-coverage on the frequency used for sidelink if both the direct path and the sidelink on the indirect path are on the same frequency

- is out of coverage on the frequency used for sidelink if the direct path and the sidelink on the indirect path are on different frequencies.

<End of Change 2>