3GPP TSG RAN WG4 Meeting #111	R4-2409710

Fukuoka, Japan, 20 May to 24 May 2024

Agenda Item:	7.8.5

Source: 	Ericsson

Title:	Discussion on RRM core requirement maintenance

Document for:	Discussion

# Introduction

In this contribution, we provide our views on the RRM core requirement maintenance.

# Discussion

Few open issues after the last meeting were listed below from the WF [1]. In this paper, we provide our views on capturing the time for SBI and MIB reading to be included in T∆ or not.

Agreement:

- For HO requirements, include the 2 additional SSB samples for SBI and MIB reading due to PBCH puncturing as part of T∆ instead of Tsearch.

  - For less than 5MHz requirement, include the additional time for T∆ in the spec.

- Further discuss whether to Capture in spec that in general the time for SBI and MIB reading is included in T∆.

In RAN4, in many of the WI discussions in the past, it was discussed that when should the UE perform SSB index reading and MIB reading while making measurements on SSB. As part of less than 5 MHz core maintenance, companies agreed that SBI and MIB reading are done as part of T∆ instead of Tsearch. To save RAN4 time and effort in future releases and requirements discussion, we suggest capturing this agreement in the specification.

- RAN4 to add following clarification in HO requirements section.

  - T∆ is time for fine time tracking, acquiring full timing information, SSB index reading and MIB reading of the target cell.

# Summary and Conclusion

In this contribution we have analysed RAN4 aspects for NR less than 5 MHz and made following proposals.

- RAN4 to add following clarification in HO requirements section.

  - T∆ is time for fine time tracking, acquiring full timing information, SSB Index reading and MIB reading of the target cell.

# References

- R4-2406329