3GPP TSG-RAN4 Meeting #111	R4-2410154

# Fukuoka, Japan, 20 – 24 May, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | 1 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on RRM requirements for PRS CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-05-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - The condition on single Tx chain is missing for requirements for PRS CA in RRC_IDLE. - The condition on single Tx chain is only applicable for aggregated measurement, but it is currently captured in clauses generic for all PRS measurement in RRC_CONN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the condition on single Tx chain for requirements for PRS CA in RRC_IDLE - Remove the condition on single Tx chain from clauses generic for all PRS measurement in RRC_CONN to clauses for aggregated measurement. Also, the exact clause number in RAN1 spec is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Core requirements for PRS CA are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5.2.6, 9.9.2.2, 9.9.2.10, 9.9.4.2, 9.9.4.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The draftCR is based on 38.133 18.5.0 and Big draftCR R4-2405983 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

4.5.2.6	Measurements Period Requirements with Bandwidth Aggregation

The requirements in this clause apply provided that UE receives requests from LMF to perform PRS measurement on aggregated positioning frequency layers (PFLs) via nr-DL-PRS-JointMeasurementRequestedPFL-List in NR-DL-TDOA-RequestLocationInformation.

After receiving both NR-DL-TDOA-ProvideAssistanceData message and NR-DL-TDOA-RequestLocationInformation message from the LMF via LPP [34], the UE shall be able to measure multiple (up to the UE capability specified in Clause 4.5.2.3) DL RSTD measurements, defined in TS 38.215 [4], during the measurement period TRSTD,Total defined as:

TRSTD,total= TRSTD, aggr+ TRSTD,non-aggr + Tmargin

Where:

-	TRSTD, aggr is the total measurement period for aggregated measurements, and

-	TRSTD,non-aggr is the total measurement period for non-aggregated measurements, and

-	When both TRSTD, aggr  and TRSTD,non-aggr are non-zero, Tmargin=maxTeffect,j, where the maximum Teffect,j is across all the PFL combination(s) and non-aggregated PFL(s) configured for positioning measurements; otherwise Tmargin is equal to zero.

TRSTD,non-aggr is zero if every resource set on every PFL is linked for aggregation to at least one other resource set on another PFL. Otherwise, TRSTD,non-aggr is as defined in clause 4.5.2.5 except that

-	only PFLs containing resource set(s) not linked to any other resource set(s) are considered in L

-	on each PFL i, only resource set(s) not linked to any other resource set(s) are considered in Tavailable_PRS,i, NPRS,islot and Lavailable_PRS,i

-	Nsample= 2 if the UE supports the capability of positioning measurements with reduced number of samples as indicated by supportedDL-PRS-ProcessingSamples-RRC-Inactive specified in TS 37.355 [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples.

TRSTD,aggr is zero if no resource sets on any PFL are linked for aggregation with other resource sets on other PFLs. Otherwise, TRSTD,aggr is defined as

TRSTD,aggr= m=1MTRSTD,aggr,m+ M-1*maxTeffect,aggr,m

where:

-	m is the index of PFL combination,

-	M is total number of PFL combinations,

-	Teffect,aggr,m is the periodicity of the PRS measurement in PFL combination m,

-	TRSTD,aggr,m is the measurement period for PRS RSTD measurement in PFL combination m as specified below.

TRSTD,aggr,m=Kcarrier,aggr*NRx,TEG,aggr,m*NRxBeam,aggr,m*NPRS,aggr,mslotNaggr,m'*LavailablePRS,aggr,mNaggr,m*Nsample-1*Teffect,aggr,m+Tlast,aggr,m

where:

-	Kcarrier,aggr is a scaling factor for PRS measurements in RRC_IDLE. If the UE supports parallelPRS-MeasRRC-Inactive-r17, Kcarrier_PRS= 1. Otherwise,

-	If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, Kcarrier_PRSequals to the sum of Kcarrier in 4.2.2.4 and one positioning layer.

-	If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, Kcarrier_PRS equals to the sum of Nlayer in 4.2.2.7 and one positioning layer.

-	NRx,TEG,m is the Rx TEG specific scaling factor:

-	NRx,TEG,m =1 if the UE is not configured by the LMF to measure a PRS resource with multiple Rx TEGs via measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 [34].

-	NRx,TEG,m is defined as follows if the UE is configured by the LMF with measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 [34] to perform measurement on same DL PRS resource of a TRP using different Rx TEGs in NR-DL-TDOA-RequestLocationInformation [34]:

-	NRx,TEG,m = P, if the UE is not capable of receiving same DL PRS resource simultaneously from multiple Rx TEGs, where P is the number of UE Rx TEGs that the UE is requested by LMF to measure the same DL-PRS Resource of a TRP indicated by measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 in [34], and in case ‘n0’ is indicated, P is the maximum number of Rx TEGs with which UE can support to measure the same PRS resource as reported in NR-UE-TEG-Capability.

-	NRx,TEG,m = PQ , if the UE is capable of receiving the same DL PRS resource simultaneously from multiple Rx TEGs, where Q is the number of UE Rx TEGs for measuring the same DL-PRS Resource simultaneously indicated by measureSameDL-PRS-ResourceWithDifferentRxTEGsSimul-r17 in [34].

-	NRxBeam,aggr,m is a scaling factor for PRS measurements with multiple Rx beams, and is defined as

-	NRxBeam,aggr,m = 1 if PFL combination m is in FR1,

-	NRxBeam,aggr,m is defined as follows if PFL combination m is in FR2

-	NRxBeam,aggr,m equals to the value as UE reported in supportedLowerRxBeamSweepingFactor-FR2 if the capability is reported by the UE for the band containing PFL combination m, and LMF indicates lowerRxBeamSweepingFactor-FR2 in NR-DL-TDOA-RequestLocationInformation,

-	NRxBeam,aggr,m equals to 8 otherwise

-	NPRS,aggr,mslot is the maximum number of DL PRS resources in PFL combination m configured in a slot, and only the PRS resources in resource set(s) linked to other resource set in PFL combination m are counted

-	Naggr,m' is the UE capability on maximum number of DL PRS resources that can be processed in a slot for PFL combination m as indicated by maxNumOfAggregatedDL-PRS-ResourcePerSlot specified in TS 37.355 [34].

-	LavailablePRS,aggr,m is the time duration of available PRS resources in PFL combination m to be measured during TPRS,aggr,m, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of LavailablePRS,aggr,m, only unmuted PRS resources in resource set(s) linked to other resource set in PFL combination m and that are not fully overlapped with other higher-priority DL signals/channels are considered.

-	Naggr,m is the UE capability on duration of DL PRS resources in ms for PFL combination m as indicated by prs-durationOfTwoPRS-BWA-ProcessingSymbolsN or prs-durationOfThreePRS-BWA-ProcessingSymbolsN specified in TS 37.355 [34].

-	Nsample is number of PRS measurement samples,

-	Nsample= 2 if the UE supports the capability of positioning measurements with reduced number of samples as indicated by reducedNumOfSampleInMeasurementWithPRS-BWA-RRC-IdleAndInactive specified in TS 37.355 [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples,

-	Nsample= 4 otherwise.

-	Teffect,aggr,m is the periodicity of the PRS measurement in PFL combination m,

Teffect,aggr,m=Taggr,mTavailable,PRS,aggr,m*Tavailable,PRS,aggr,m

-	Taggr,m is the UE capability on time for processing of DL PRS resources in ms for PFL combination m as indicated by prs-durationOfTwoPRS-BWA-ProcessingSymbolsT or prs-durationOfThreePRS-BWA-ProcessingSymbolsT specified in TS 37.355 [34].

-	Tavailable,PRS,aggr,m=LCMTPRS,aggr,m,TDRX, the least common multiple between TPRS,aggr,m and the DRX cycle length TDRX, where TPRS,aggr,m is the periodicity of DL PRS resource with muting on PFL combination m.

-	If more than one PRS periodicities are configured in PFL combination m, the least common multiple of PRS periodicities TperPRS with muting among all DL PRS resource sets that are linked to other resource set in PFL combination m, is used to derive TPRS,aggr,m, and for each applicable PRS resource set,

-	TperPRS with muting=Nmuting*TperPRS, is the PRS periodicity with muting per PRS resource, and

-	TperPRS is the periodicity of PRS resource set given by the higher-layer parameter DL-PRS-Periodicity, and

-	Nmuting is the scaling factor considering PRS resource muting. Nmuting=TmutingPRS*Lmuting, where 	TmutingPRS is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and Lmuting is the size of the bitmap b1.

-	Tlast,aggr,m is the measurement duration for the last PRS sample in PFL combination m, including the sampling time and processing time, Tlast,aggr,m=Taggr,m+Tavailable,PRS,aggr,m.

The requirements in this clause for aggregated measurements apply provided that the linked PRS resource sets on multiple PFLs for aggregated measurements satisfy all the conditions specified in clause 5.1.6.5.3 in TS 38.214 [26].

The time TRSTD,Total starts from the first DRX cycle containing the DL PRS resource(s) in the assistance data after both the NR-DL-TDOA-ProvideAssistanceData message and NR-DL-TDOA-RequestLocationInformation message are delivered from LMF to the UE via LPP [34].

Note:	No separate requirement on aggregated measurement based on TRSTD,non-aggr or on non-aggregated measurement based on TRSTD,aggr is applied.

If the DRX cycle is reconfigured during the RSTD measurement period, then the measurement period can be longer.

When PRS-RSRP is also reported by UE together with RSTD measurement based on aggregated DL-PRS resources from multiple-PFLs, PRS-RSRP are performed over the measurement period defined in 4.5.2.6.

When PRS-RSRPP is also reported by UE together with RSTD measurement based on aggregated DL-PRS resources from multiple-PFLs, PRS-RSRPP are performed over the measurement period defined in 4.5.2.6.

Longer RSTD measurement period is expected when there are collisions between PRS resources in any of the PFLs configured for aggregation and other higher-priority DL signals/channels.

If Kcarrier_PRS changes for any PFL or any PFL combination during the measurement period, the measurement period could be longer.

The measurement requirements do not apply for a PRS resource, if the PRS resource is across two sampling duration of N within duration Lavailable_PRS,i.

The measurement requirements do not apply for a PRS resource, if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

The requirements in clause 4.5.2.6 do not apply if the PRS configuration given by higher layer paramters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-DL-TDOA-ProvideCapabilities, and it is up to UE implementation which PRS resources are measured, subject to UE measurement capabilities.

If cell re-selection occurs while RSTD measurements are being performed, then the UE shall continue and complete the on-going RSTD measurements after the cell selection is completed. The RSTD measurement period can be longer.

If the RRC state transition occurs from RRC_IDLE to RRC_CONNECTED state during the RSTD measurement period then the UE shall continue the RSTD measurement in the RRC_CONNECTED state. The RSTD measurement period can be longer.

The UE shall meet the RSTD measurement accuracy requirements in clause 10.x.x.x.

<End of Change 1>

<Start of Change 2>

#### 9.9.2.2	Requirements Applicability

The requirements in clause 9.9.2 apply for periodic and triggered RSTD measurements, provided:

-	PRS-RSTD related side conditions given in clause 10.1.23 for FR1 and FR2 are fulfilled, for a corresponding Band.

<End of Change 2>

<Start of Change 3>

9.9.2.10	Measurements Period Requirements with Bandwidth Aggregation

When physical layer receives last of NR-DL-TDOA-ProvideAssistanceData message and NR-DL-TDOA-RequestLocationInformation message from LMF via LPP [34] with a request to perform measurement by aggregating PRS resources from multiple PFLs via nr-DL-PRS-JointMeasurementRequestedPFL-List, the UE shall be able to perform RSTD measurement by aggregating PRS resources from multiple PFLs (up to the UE capability specified in Clause 9.9.2.3) defined in TS 38.215 [4], during the measurement period TRSTD_aggregate, Total defined as:

TRSTD_aggregate, Total = Tnon_aggregate_RSTD + Taggregate_RSTD + Tmargin,

where,

Tnon_aggregate_RSTD is the total measurement period for RSTD measurement on PFLs that do not contain PRS resources for aggregation.

Calculation of Tnon_aggregate_RSTD is based on clause 9.9.2.5, such that Tnon_aggregate_RSTD is calculated by considering PRS resources that are not aggregated by UE, based on the configuration received from the LMF.

Tnon_aggregate_RSTD is equal to zero if UE is not configured to perform non-aggregated measurements by the LMF.

Tmargin is delay margin to account for delay between RSTD measurement performed by UE on PFLs that contain PRS resources for aggregation and RSTD measurement performed by UE on PFLs that do not contain PRS resources for aggregation. Tmargin is calculated as max(Teffect,i), by considering both aggregated PFLs and non-aggregated PFLs configured for positioning measurement. Tmargin is only applicable when UE is configured to perform measurements on PFLs that contain PRS resources for aggregation and on PFLs that do not contain PRS resources for aggregation.

If UE is only configured to perform measurements on PFLs that contain PRS resources for aggregation or if UE is only configured to perform measurements on PFLs that do not contain PRS resources for aggregation, then Tmargin = 0.

If UE is capable of performing latency reduced positioning measurements and is configured to perform latency reduced positioning measurement by LMF via reducedDL-PRS-ProcessingSamples-r17 [34], then Nsample = 2 shall be considered in calculations of Taggregate_RSTD and Tnon-aggregate_RSTD.

If UE is not capable to perform latency reduced positioning measurements or is not configured to perform latency reduced positioning measurement by LMF via reducedDL-PRS-ProcessingSamples-r17 [34], then Nsample = 4 shall be considered in calculations of Taggregate_RSTD and Tnon-aggregate_RSTD.

Taggregate_RSTD, total measurement period for RSTD measurements performed by UE by aggregating PRS resources from multiple PFLs as defined in TS 38.214 [26], is defined as:

Taggregate_RSTD=i=1GTRSTD_aggregate,i +(G-1)*maxTeffect_agg,i,

where

i is the index of effective PFL, corresponding to the group of PFLs containing linked PRS resource sets to be aggregated for RSTD measurement,

G denotes the number of effective PFLs that UE is configured to perform aggregated measurements on. G is configured by LMF and is provisioned to UE via nr-DL-PRS-AggregationInfo.

TRSTD_aggregate,i is the measurement period for PRS RSTD measurement in the effective PFL i as specified below:

TRSTD_aggregate,i=kmultiTEG_agg,i×CSSFPRS_agg,i×ceil( Kp,PRS_agg,i)×NRxBeam,i×NPRS_agg,islotNagg'Lavailable_PRS_agg,iNagg×Nsample_agg-1×Teffect_agg,i+Tlast_agg,i ,

where:

NRxBeam,i is the UE Rx beam sweeping factor.

In FR1, NRxBeam,i = 1.

In FR2, NRxBeam,i is equal to the value reported by the UE in supportedLowerRxBeamSweepingFactor-FR2 if the UE supports the capability for the band containing effective PFL i, and the LMF indicates lowerRxBeamSweepingFactor-FR2 inNR-DL-TDOA-RequestLocationInformation. NRxBeam,i is equal to 8, otherwise.

CSSFPRS_agg,i is the carrier-specific scaling factor for NR PRS-based positioning measurements in effective PFL i as defined in clause 9.1.5.2.

kmultiTEG_agg,i is the scaling factor for measurement of same PRS resource with multiple Rx TEGs.

kmultiTEG_agg,i = 1 if UE is not requested by LMF to measure a PRS resource with multiple Rx TEGs via measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 [34] in NR-DL-TDOA-RequestLocationInformation;

otherwise,

kmultiTEG_agg,i = NTEG,i, if UE is not capable of receiving same DL PRS resource simultaneously from multiple Rx TEGs, and

kmultiTEGagg,i=NTEG,ikTEG,simul,i if UE is capable of receiving the same DL PRS resource simultaneously from multiple Rx TEGs.

where

NTEG,i is the number of Rx TEGs with which UE is requested to measure aggregated PRS resource indicated via measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 [34] in NR-DL-TDOA-RequestLocationInformation, and in case ‘n0’ is indicated, NTEG,i is the maximum number of Rx TEGs with which UE can support to measure the same PRS resource as reported in NR-UE-TEG-Capability, and

kTEG,simul,i is the number of Rx TEGs UE can measure simultaneously which is reported via measureSameDL-PRS-ResourceWithDifferentRxTEGsSimul.

Kp,PRS_agg,i is a scaling factor for effective PFL i to be measured within the associated measurement gap pattern, which is defined as Kp,PRS_agg,i = Ntotal / Navailable for UE configured with concurrent measurement gap, and Kp,PRS_agg,i = 1 for UE not configured with concurrent measurement gap.

For a window W of duration max(TPRS_agg,i,  MGRP_max), where MGRP_max is the maximum MGRP across all configured per-UE MG and per-FR MG within the same FR as the effective PFL, and starting at the beginning of any associated gap occasions covering the PRS occasion:

-	Ntotal is the total number of associated gap occasions covering PRS occasions within the window, including both dropped and non-dropped instances of the associated measurement gap within the window, and

-	Navailable is the number of non-dropped associated gap occasions covering PRS occasions within the window W, after further accounting for MG collisions by applying the selected gap collision rule

-	Requirements do not apply if Navailable = 0.

NPRS_agg,islot is the maximum number of DL PRS resources in effective PFL i configured in a slot.

Lavailable_PRS_agg,i is the time duration of available PRS in the effective PFL i to be measured during Tavailable_PRS_agg,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lavailable_PRS_agg,i, only the PRS resources unmuted and fully or partially overlapped with MG and satisfying the conditions for PRS BW aggregation are considered.

Nsample_agg is the number of PRS RSTD measurement samples.

Tlast_agg,i is the measurement duration for the last PRS RSTD sample in the effective PFL i, including the sampling time and processing time. If all of the PRS resources to be measured are available in the same MG occasion during Tavailabe_agg, Tlast_agg,i = Tagg,i + MGL. Otherwise, Tlast_agg,i = Tagg,i + Tavailable_PRS_agg,i,

Teffect_agg,i is the periodicity of the PRS RSTD measurement in the effective PFL i defined as:

Teffect_agg,i = Tagg,iTavailable_PRS_agg,i×Tavailable_PRS_agg,i

where,

Tagg,i

Tavailable_PRS_agg,i= LCMTPRS_agg,i,MGRPi, the least common multiple between TPRS_agg,i and MGRPi. MGRPi is the repetition periodicity of the measurement gap applicable for measurement in the effective PFL i.

TPRS_agg,i is the periodicity of DL PRS resources meeting the bandwidth aggregation conditions with muting on effective PFL i. If more than one PRS periodicities are configured in effective PFL i, the least common multiple of PRS periodicities Tper_aggPRS with muting among all DL PRS resource sets in the effective PFL is used to derive TPRS_agg,i, where,

Tper_aggPRS with muting=Nmuting×Tper_aggPRS, is the PRS periodicity with muting per PRS resource configured for aggregation,

Tper_aggPRS is the periodicity of PRS resource sets given by the higher-layer parameter DL-PRS-Periodicity.

Nmuting is the scaling factor considering PRS resource muting. Nmuting=TmutingPRS_agg×Lmuting, where TmutingPRS_agg is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and Lmuting is the size of the bitmap b1.

-	Note: For the purpose of calculating TPRS_agg,i, only the PRS resources fully or partially covered by the MG and PRS resources that satisfy the conditions for PRS BW aggregation are considered.

{Nagg, Tagg,i}is UE capability combination per band to aggregate PRS resources from multiple PFLs within an effective PFL where Nagg is a duration of DL PRS symbols in ms corresponding to prs-durationOfTwoPRS-BWA-ProcessingSymbolsN-r18 or prs-durationOfThreePRS-BWA-ProcessingSymbolsN-r18 in TS 37.355 [34] processed every Tagg,i ms corresponding to prs-durationOfTwoPRS-BWA-ProcessingSymbolsT-r18 or prs-durationOfThreePRS-BWA-ProcessingSymbolsT in TS 37.355 [34] for a given maximum bandwidth supported by UE corresponding to maximumOfTwoAggregatedDL-PRS-Bandwidth or maximumOfThreeAggregatedDL-PRS-Bandwidth  in TS 37.355 [34].

Nagg' is UE capability for number of DL PRS resources that it can process in a slot as indicated by maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR1-r18 for FR1 and maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR2-r18 for FR2  specified in TS 37.355 [34].

The requirements in this clause for aggregated measurements apply provided that the linked PRS resource sets on multiple PFLs for aggregated measurements satisfy all the conditions specified in clause 5.1.6.5.3 in TS 38.214 [26].

Except for deferred MT-LR as defined in clause 4.1a.5 [TS 23.273], the time TRSTD_aggregate,Total starts from the first MG instance aligned with DL PRS resources in the assistance data for aggregation after both the NR-DL-TDOA-ProvideAssistanceData message and NR-DL-TDOA-RequestLocationInformation message are delivered from LMF to the physical layer of UE via LPP [34] including a request to perform measurement by aggregating PRS resources from multiple PFLs via nr-DL-PRS-JointMeasurementRequestedPFL-List.

For deferred MT-LR with other event than “Periodic Location” as defined in clause 4.1a.5.1 [TS 23.273], the time TRSTD_aggregate,Total starts from the first MG instance aligned with DL PRS resources in the assistance data after the associated event(s) occurs.

For deferred MT-LR with event “Periodic Location” as defined in clause 4.1a.5.1 [TS 23.273], the UE shall perform the aggregated RSTD measurement in each reporting period and activate the location report at the time when the periodic timer expires.

When PRS-RSRP is also reported by UE together with RSTD measurement based on aggregated DL-PRS resources from multiple-PFLs, PRS-RSRP are performed over the measurement period defined in 9.9.2.10.

When PRS-RSRPP is also reported by UE together with RSTD measurement based on aggregated DL-PRS resources from multiple-PFLs, PRS-RSRPP are performed over the measurement period defined in 9.9.2.10.

If during the measurement period of one or more effective PFLs, the MG pattern is reconfigured, the measurement period can be longer.

The measurement requirements in this clause apply, provided no PRS symbols are dropped during the measurement period TRSTD_aggregate,Total within measurement gaps due to collisions with other signals; otherwise, the measurement period can be longer.

If CSSF changes during the measurement period, the measurement period could be longer.

The measurement requirements do not apply for aggregated PRS resources, if the PRS resources to be aggregated are across two sampling duration of Nagg within duration Lavailable_PRS_agg,i.

The measurement requirements do not apply, if time span of the instance of the PRS resources to be aggregated (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability Nagg.

The requirements in clause 9.9.2.10 do not apply if the PRS configuration given by higher layer paramters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-DL-TDOA-ProvideCapabilities, and it is up to UE implementation which aggregated PRS resources are measured, subject to UE measurement capabilities.

If handover occurs while aggregated RSTD measurements are being performed, then the UE shall continue and complete the on-going aggregated RSTD measurements. The aggregated RSTD measurement period can be longer. The UE shall meet the RSTD measurement accuracy requirements in clause 10.1.23.x.

<End of Change 3>

<Start of Change 4>

#### 9.9.4.2 Requirements Applicability

The requirements in clause 9.9.4 apply for periodic and triggered UE Rx-Tx time difference measurements, provided:

-	UE Rx-Tx time difference measurement related side conditions given in clause 10.1.25 are met for a corresponding band.

-	UE Rx-Tx time difference measurement related side conditions given in clause 10.1.25.x are met for PRS aggregation from multiple PFLs.

-	SRS is configured on at least one of the PCell, PSCell and SCell.

-	The UE transmits SRS within [-160, 160] msec of at least one DL PRS resource of each of the TRPs in the assistance data.

<End of Change 4>

<Start of Change 5>

9.9.4.9	Measurements Period Requirements with Bandwidth Aggregation

When physical layer receives last of NR-Multi-RTT-ProvideAssistanceData message and NR-Multi-RTT-RequestLocationInformation message from LMF via LPP [34] with a request to perform measurement by aggregating PRS resources from multiple PFLs via nr-DL-PRS-JointMeasurementRequestedPFL-List, the UE shall be able to perform UE Rx-Tx time difference measurement by aggregating PRS resources from multiple PFLs (up to the UE capability specified in Clause 9.9.2.3) defined in TS 38.215 [4], during the measurement period TUE_RxTx_aggregated, Total defined as:

TUE_RxTx_aggregated, Total = Tnon-aggregate_RxTx + Taggregated_RxTx + Tmargin,

where,

Tnon_aggregate_RxTx is the total measurement period for UE Rx-Tx time difference measurement on PFLs that do not contain PRS resources for aggregation.

Calculation of Tnon_aggregate is based on clause 9.9.4.5, such that Tnon_aggregate_RxTx is calculated by considering PRS resources that are not aggregated by UE, based on the configuration received from the LMF.

Tnon_aggregate_RxTx is equal to zero if UE is not configured to perform non-aggregated measurements by the LMF.

Taggregate_RxTx is the total measurement period for UE Rx-Tx time difference measurements performed by UE by aggregating PRS resources from multiple PFLs as defined in TS 38.214 [26].

Tmargin is delay margin to account for delay between Rx-Tx measurement performed by UE on PFLs that contain PRS resources for aggregation and Rx-Tx measurement performed by UE on PFLs that do not contain PRS resources for aggregation. Tmargin is calculated as max(Teffect,i), by considering both aggregated PFLs and non-aggregated PFLs configured for positioning measurement. Tmargin is only applicable when UE is configured to perform measurements on PFLs that contain PRS resources for aggregation and on PFLs that do not contain PRS resources for aggregation.

If UE is only configured to perform measurements on PFLs that contain PRS resources for aggregation or if UE is only configured to perform measurements on PFLs that do not contain PRS resources for aggregation, then Tmargin = 0.

If UE is capable of performing latency reduced positioning measurements and is configured to perform latency reduced positioning measurement by LMF via reducedDL-PRS-ProcessingSamples-r17 [34], then Nsample = 2 shall be considered in calculations of Taggregate_RxTx and Tnon-aggregate_RxTx.

If UE is not capable to perform latency reduced positioning measurements or is not configured to perform latency reduced positioning measurement by LMF via reducedDL-PRS-ProcessingSamples-r17 [34], then Nsample = 4 shall be considered in calculations of Taggregate_RxTx and Tnon-aggregate_RxTx.

Taggregate_RxTx, total measurement period for UE Rx-Tx time difference measurements performed by UE by aggregating PRS resources from multiple PFLs as defined in TS 38.214 [26], is defined as:

Taggregate_RxTx=i=1GTRxTx_aggregate,i +(G-1)*maxTeffect_agg,i,

where

i is the index of effective PFL, corresponding to the group of PFLs containing linked PRS resource sets to be aggregated for UE Rx-Tx time difference measurement,

G denotes the number of effective PFLs that UE is configured to perform aggregated measurements on. G is configured by LMF and is provisioned to UE via nr-DL-PRS-AggregationInfo.

TRxTx_aggregate,i is the measurement period for UE Rx-Tx time difference measurement in the effective positioning frequency layer i as specified below:

TRxTx_aggregate,i=kmultiTEG_agg,i×CSSFPRS_agg,i×ceil( Kp,PRS_agg,i)×NRxBeam_agg,i×NPRS_agg,islotNagg'Lavailable_PRS_agg,iNagg×Nsample_agg-1×Teffect_agg,i+Tlast_agg,i ,

where:

NRxBeam,i is the UE Rx beam sweeping factor.

In FR1, NRxBeam,i = 1.

In FR2, NRxBeam,i is equal to the value reported by the UE in supportedLowerRxBeamSweepingFactor-FR2 if the UE supports the capability for the band containing effective positioning frequency layer i, and the LMF indicates lowerRxBeamSweepingFactor-FR2 in NR-Multi-RTT-RequestLocationInformation. NRxBeam,i is equal to 8, otherwise.

CSSFPRS_agg,i is the carrier-specific scaling factor for NR PRS-based positioning measurements in effective positioning frequency layer i as defined in clause 9.1.5.2.

kmultiTEG_agg,i is the scaling factor for measurement of same PRS resource with multiple Rx TEGs.

kmultiTEG,i = 1 if UE is not requested by LMF to measure a PRS resource with multiple Rx TEGs via measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 [34] or measureSameDL-PRS-ResourceWithDifferentRxTxTEGs-r17 [34] in NR-Multi-RTT-RequestLocationInformation;

otherwise,

kmultiTEG,i = NTEG,i, if UE is not capable of receiving same DL PRS resource simultaneously from multiple Rx TEGs, and

kmultiTEG,i=NTEG,ikTEG,simul,i if UE is capable of receiving the same DL PRS resource simultaneously from multiple Rx TEGs.

where

NTEG,i is the number of Rx TEGs with which UE is requested to measure aggregated PRS resource indicated via measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 or measureSameDL-PRS-ResourceWithDifferentRxTxTEGs-r17 [34] in NR-Multi-RTT-RequestLocationInformation, and in case ‘n0’ is indicated, NTEG,i is the maximum number of Rx TEGs with which UE can support to measure the same PRS resource as reported in NR-UE-TEG-Capability, and

kTEG,simul,i is the number of Rx TEGs UE can measure simultaneously which is reported via measureSameDL-PRS-ResourceWithDifferentRxTEGsSimul.

Kp,PRS_agg,i is a scaling factor for effective positioning frequency layer i to be measured within the associated measurement gap pattern, which is defined as Kp,PRS_agg,i = Ntotal / Navailable for UE configured with concurrent measurement gap, and Kp,PRS_agg,i = 1 for UE not configured with concurrent measurement gap.

For a window W of duration max(TPRS_agg,i,  MGRP_max), where MGRP_max is the maximum MGRP across all configured per-UE MG and per-FR MG within the same FR as the effective positioning frequency layer, and starting at the beginning of any associated gap occasions covering the PRS occasion:

-	Ntotal is the total number of associated gap occasions covering PRS occasions within the window, including both dropped and non-dropped instances of the associated measurement gap within the window, and

-	Navailable is the number of non-dropped associated gap occasions covering PRS occasions within the window W, after further accounting for MG collisions by applying the selected gap collision rule

-	Requirements do not apply if Navailable = 0.

NPRS_agg,islot is the maximum number of DL PRS resources in effective positioning frequency layer i configured in a slot.

Lavailable_PRS_agg,i is the time duration of available PRS in the effective positioning frequency layer i to be measured during Tavailable_PRS_agg,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lavailable_PRS_agg,i, only the PRS resources unmuted and fully or partially overlapped with MG and satisfying the conditions for PRS BW aggregation are considered.

Nsample_agg is the number of UE Rx-Tx time difference measurement samples.

Tlast_agg,i is the measurement duration for the last UE Rx-Tx time difference measurement sample in the effective positioning frequency layer i, including the sampling time and processing time. If all of the PRS resources to be measured are available in the same MG occasion during Tavailabe_agg, Tlast_agg,i = Tagg,i + MGL. Otherwise, Tlast_agg,i = Tagg,i + Tavailable_PRS_agg,i,

Teffect_agg,i is the periodicity of the UE Rx-Tx time difference measurement in the effective positioning frequency layer i defined as:

Teffect_agg,i = Tagg,iTavailable_PRS_agg,i×Tavailable_PRS_agg,i

where,

Tagg,i

Tavailable_PRS_agg,i= LCMTPRS_agg,i,MGRPi, the least common multiple between TPRS_agg,i and MGRPi. MGRPi is the repetition periodicity of the measurement gap applicable for measurement in the effective PRS frequency layer i.

TPRS_agg,i is the periodicity of DL PRS resources meeting the bandwidth aggregation conditions with muting on effective positioning frequency layer i. If more than one PRS periodicities are configured in effective positioning frequency layer i, the least common multiple of PRS periodicities Tper_aggPRS with muting among all DL PRS resource sets in the effective positioning frequency layer is used to derive TPRS_agg,i, where,

Tper_aggPRS with muting=Nmuting×Tper_aggPRS, is the PRS periodicity with muting per PRS resource configured for aggregation,

Tper_aggPRS is the periodicity of PRS resource sets given by the higher-layer parameter DL-PRS-Periodicity.

Nmuting is the scaling factor considering PRS resource muting. Nmuting=TmutingPRS_agg×Lmuting, where TmutingPRS_agg is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and Lmuting is the size of the bitmap b1.

Note: For the purpose of calculating TPRS_agg,i, only the PRS resources fully or partially covered by the MG and PRS resources that satisfy the conditions for PRS BW aggregation are considered.

{Nagg, Tagg,i}is UE capability combination per band to aggregate PRS resources from multiple PFLs within an effective PFL where Nagg is a duration of DL PRS symbols in ms corresponding to prs-durationOfTwoPRS-BWA-ProcessingSymbolsN or prs-durationOfThreePRS-BWA-ProcessingSymbolsN  in TS 37.355 [34] processed every Tagg,i ms corresponding to prs-durationOfTwoPRS-BWA-ProcessingSymbolsT or prs-durationOfThreePRS-BWA-ProcessingSymbolsT in TS 37.355 [34] for a given maximum bandwidth supported by UE corresponding to maximumOfTwoAggregatedDL-PRS-Bandwidth or maximumOfThreeAggregatedDL-PRS-Bandwidth in TS 37.355 [34].

Nagg' is UE capability for number of DL PRS resources that it can process in a slot as indicated by maxNumOfAggregatedDL-PRS-ResourcePerSlot specified in TS 37.355 [34].

The requirements in this clause for aggregated measurements apply provided that the linked PRS resource sets on multiple PFLs for aggregated measurements satisfy all the conditions specified in clause 5.1.6.5.3 in TS 38.214 [26].

Except for deferred MT-LR as defined in clause 4.1a.5 [TS 23.273], the time TUE_RxTx_aggregated,Total starts from the first MG instance aligned with DL PRS resources in the assistance data for aggregation after both the NR-Multi-RTT-RequestLocationInformation message and NR-Multi-RTT-ProvideAssistanceData message message are delivered from LMF to the physical layer of UE via LPP [34] including a request to perform measurement by aggregating PRS resources from multiple PFLs via nr-DL-PRS-JointMeasurementRequested.

For deferred MT-LR with other event than “Periodic Location” as defined in clause 4.1a.5.1 [TS 23.273], the time TUE_RxTx_aggregated,Total starts from the first MG instance aligned with DL PRS resources in the assistance data after the associated event(s) occurs.

For deferred MT-LR with event “Periodic Location” as defined in clause 4.1a.5.1 [TS 23.273], the UE shall perform the aggregated UE Rx-Tx time difference measurement in each reporting period and activate the location report at the time when the periodic timer expires.

When PRS-RSRP is also reported by UE together with UE RX-Tx measurement based on aggregated DL-PRS resources from multiple-PFLs, PRS-RSRP are performed over the measurement period defined in 9.9.4.9.

When PRS-RSRPP is also reported by UE together with UE Rx-Tx measurement based on aggregated DL-PRS resources from multiple-PFLs, PRS-RSRPP are performed over the measurement period defined in 9.9.4.9.

If during the measurement period of one or more effective positioning frequency layers, the MG pattern is reconfigured, the measurement period can be longer.

The measurement requirements in this clause apply, provided no PRS symbols are dropped during the measurement period TUE_RxTx_aggregated,Total within measurement gaps due to collisions with other signals; otherwise, the measurement period can be longer.

The requirements in clause 9.9.4.9 do not apply if the PRS configuration given by higher layer paramters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-Multi-RTT-ProvideCapabilities, and it is up to UE implementation which aggregated PRS resources are measured, subject to UE measurement capabilities.

When PSCell or SCell addition or release does not cause SRS reconfiguration during the measurement period, UE continues the UE Rx-Tx time difference measurement, and the measurement period requirements apply.

When PSCell or SCell addition or release causes SRS reconfiguration during the measurement period, UE shall restart the UE Rx-Tx time difference measurement after the SRS reconfiguration on the target cell is complete.

When SRS is reconfigured without serving cell change during the measurement period, UE shall restart the UE Rx-Tx time difference measurement after the SRS reconfiguration is complete. If UE uplink transmission timing changes due to the network-configured Timing Advance command during the UE Rx-Tx measurement period, then the UE Rx-Tx time difference measurement period is restarted after uplink transmission timing changes, and the UE Rx-Tx time difference measurement period requirements in this clause shall not apply.

When a serving cell change occurs during the measurement period, the UE shall continue and complete the UE Rx-Tx time difference measurement provided that the serving cell change does not impact SRS configuration for the UE Rx-Tx time difference measurement.

If UE uplink transmission timing changes due to the change in the NTA_offset defined in Table 7.1.2-2 during the UE Rx-Tx measurement period, then the UE Rx-Tx time difference measurement period is restarted after uplink transmission timing changes, and the UE Rx-Tx time difference measurement period requirements in this clause shall not apply.

If UE uplink transmission timing changes due to the UE autonomous timing adjustment defined in clause 7.1.2 during the UE Rx-Tx measurement period, then:

-	UE Rx-Tx measurement period requirements in this clause shall apply for a cell, which is also the downlink reference cell (defined in section 7.1.1) for SRS transmission.

-	UE Rx-Tx measurement period requirements in this clause shall not apply for a cell, which is not the downlink reference cell (defined in section 7.1.1) for SRS transmission. The UE Rx-Tx time difference measurement period may be restarted in such case.

<End of Change 5>