3GPP TSG-RAN WG4 Meeting #111	R4-2410193

Fukuoka, May 20 ‒ 24, 2024

Title: 	WF on SL positioning and carrier phase positioning

Agenda Item:	7.12.3

Source: 	CATT

Document for:	Approval

- Introduction

This WF includes the agreements and open issues discussed in topic summary for [111][213] NR_pos_enh2_part2.

Topic #1: Sidelink Positioning Requirements

Sub-topic 1-1 SL Positioning Core Requirements Maintenance (agenda 7.12.1.2)

Issue 1-1-1: Applicability of SL PRS measurement period requirements

Agreements:

    - The measurement requirements for SL positioning are applicable to a single SL carrier case. The FG 41-1-1a is not included as an applicability condition.

    - Measurement accuracy requirements for SL RSTD apply provided the time separation between the SL PRS resources from the target and reference is no larger than 160 ms

Issue 1-1-2: UE behaviour and the impact on SL-PRS measurement requirements when synchronization reference source change occurs at Tx side

Agreements:

    - It is a RAN4 understanding that UE performing measurements may not be aware on the synchronization source change at the Tx UE.

    - For synchronization reference source change occurring at Tx side, measurement accuracy requirements do not apply and no specific UE behaviour is defined.

      - Note: the agreement can be revisited if a RAN1/2 solution is introduced to inform the UE performing measurements on the synchronization source change at the Tx UE.

    - Send LS to RAN1/2 to check on whether they have introduced or are working on any solutions to inform an SL UE performing an SL positioning measurement (e.g., SL RSTD, SL Rx-Tx, and SL RTOA) about synchronization reference source change at a UE, which is transmitting SL-PRS for the measurement. Details of the LS can refer to R4-2410352.

Issue 1-1-3: End point of SL-PRS based RSTD measurement period requirements

Proposals:

  - Option 1:

    - Update the SL RSTD requirements to reflect that measurement period ends after the UE has measured SL PRS resources from at least two different Tx UEs.

Issue 1-1-4: Impact of Uu link connect

Proposals:

  - Option 1:

    - RAN4 not to define any impact of Uu link connection on the measurement period.

  - Option 2:

    - It is clarified in TS 38.133 for the SL-PRS based measurements in the introduction section 12A.1:

      - NOTE 3: When a UE in RRC_CONNECTED state is performing transmissions and/or reception for SL positioning operation, the UE shall meet all the requirements specified in Clause 6, assuming that UE has a dedicated RX/TX chain for the sidelink operation. Otherwise, the UE may interrupt the SL positioning measurements or SL-PRS transmissions in order to meet the measurement requirements specified in Clause 6.

Sub-topic 1-2 SL Positioning Performance Requirements (agenda 7.12.2.2)

Issue 1-2-1: SINR side conditions

Agreements:

  - SL RSTD: (0, -3) dB

  - SL Rx-Tx/RSRP/RSRPP: -3 dB

Issue 1-2-2: Margin for RF calibration

Agreements:

  - Define the RF calibration margin for SL RSTD measurements in FR1 using the following structure:

| PRS BW (RB number) |  |  | Margin (Tc) |
| --- | --- | --- | --- |
| SCS=15kHz | SCS=30kHz | SCS=60kHz |  |
| ≥ 48 | ≥ 24 | N/A | Z1 |
| ≥ 96 | ≥ 48 | ≥ 24 | Z2 |

  - Define the RF calibration margin for SL Rx-Tx measurements in FR1 using the following structure:

| [Min(SL PRS Rx BW, SL PRS Tx BW) (RB)] |  |  | Margin (Tc) |
| --- | --- | --- | --- |
| SCS = 15 kHz | SCS = 30 kHz | SCS = 60 kHz |  |
| ≥ 48 | ≥ 24 | N/A | 1 |
| ≥ 96 | ≥ 48 | ≥ 24 | 2 |

  - FFS the detailed values in the above tables.

Margins due to frequency drift (Y):

  - For SL RSTD, the existing margins due to frequency drift for DL PRS can be reused for SL PRS.

  - Consider time margins due to frequency drift to Rx-Tx time difference accuracy requirement if the UE reports the transmission timestamp of a SL PRS measurement

    - FFS the exact time margin values, e.g., the ones for RSTD could be reused.

Issue 1-2-3: Time margin for Rx-Tx time difference measurements when UE reports the transmission timestamp of a SL PRS

Resolved under issue 1-2-5.

Issue 1-2-4: Value of Nsample for 48 PRB SL-PRS BW

Resolved under issue 1-2-5.

Issue 1-2-5: Measurement accuracy requirements

Agreements:

  - Requirements are to be defined for 1 and 4 samples, depending on BW.

  - The accuracy requirements for 4 samples allow for non-coherent combining.

| Accuracy | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | SL-PRS Ês/Iot | SL-PRS SCS | SL-PRS bandwidth Note 1 | Number of samples, S | Io Note 2 range |  |  |
|  |  |  |  |  | NR operating band groups Note 3 | Minimum Io | Maximum Io |
| Tc Note 4 | dB | kHz | RB |  |  | dBm/SCS | dBm/BWChannel |
| TBD | (SL-PRS Ês/Iot)ref ≥ 0 dB (SL-PRS Ês/Iot)i ≥ -3 dB | 15 | 48 | ≥ 4 | NR_TDD_FR1_B |  |  |
|  |  |  |  |  | NR_TDD_FR1_J |  |  |
|  |  |  | >48 | ≥ 1 | Note 5 |  |  |
| TBD |  |  | ≥ 96 | ≥ 1 | Note 5 |  |  |
| TBD TBD |  | 30 | ≥ 24 | ≥ 4 | NR_TDD_FR1_B |  |  |
|  |  |  |  |  | NR_TDD_FR1_J |  |  |
| TBD |  |  | >48 | ≥ 1 | Note 5 |  |  |
| TBD |  | 60 | ≥ 24 | ≥ 4 | NR_TDD_FR1_B |  |  |
|  |  |  |  |  | NR_TDD_FR1_J |  |  |
| NOTE 1: Minimum SL-PRS bandwidth, which is the minimum of the SL-PRS bandwidths of the reference resource and the measured neighbour resource i. NOTE 2: Io is assumed to have constant EPRE across the bandwidth. NOTE 3: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 4: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 5: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the SL-PRS bandwidth of the smallest PRB number for the corresponding SCS. |  |  |  |  |  |  |  |

  - Further discuss the Io values in the table.

Issue 1-2-6: Test case configurations

Agreements:

  - RAN4 will not specify SL positioning test cases with additional path reporting

  - RAN4 will not specify SL positioning test cases with LoS/NLoS reporting (no corresponding RAN4 requirements to test)

  - RAN4 will not specify SL positioning test cases with Tx/Rx ARP-ID reporting (no corresponding RAN4 requirements to test)

  - NR Uu configurations

| NR Uu configuration | Description |
| --- | --- |
| 1 | NR Uu: 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | NR Uu: 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | NR Uu: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

  - SL PRS configurations

| NR SL configuration | Description |
| --- | --- |
| 1 | SL: 15 kHz, 10 MHz bandwidth, HD duplex mode |
| 2 | SL: 30 kHz, 10 MHz bandwidth, HD duplex mode |
| 3 | SL: 30 kHz, 20 MHz bandwidth, HD duplex mode |

  - The number of SL UEs in test cases

    - The number of SL UEs in the SL measurement delay test cases for SL positioning:

      - SL RSTD: 4 (1 target rx UE, 3 anchor tx UEs [1 reference anchor UE and 2 other anchor tx UEs)])

      - SL Rx-Tx, SL PRS-RSRP/RSRPP, SL AoA: 3 (1 target rx UE, 2 anchor tx UEs),

      - SL RTOA: [2] (1 target tx UE, 1 anchor rx UE).

    - The number of SL UEs in the SL measurement accuracy test cases for SL positioning:

      - SL RSTD: 3 (1 target rx UE, 2 anchor tx UEs [1 reference anchor UE and 1 other anchor tx UEs)])

      - SL Rx-Tx, SL PRS-RSRP/RSRPP: 3 (1 target rx UE, 2 anchor tx UEs).

  - All TX and RX UEs are in coverage and using gNB as sync reference.

Issue 1-2-7: Whether to verify accuracy of legacy measurements in SL PRS-RSRP(P) TCs

Proposals:

  - Option 1:

    - Consider the following rules:

      - If RAN1/2 confirms that the SL PRS-RSRP(P) is possible to be requested and/or reported standalone, the SL PRS-RSRP(P) accuracy is verified alone in separate section.

      - If RAN1/2 confirms that the SL PRS-RSRP(P) cannot be requested and/or reported standalone, the SL PRS-RSRP(P) accuracy should be verified with RSTD/RX-TX accuracy together in one section.

  - Option 2:

    - Verify accuracy requirements for SL PRS-RSRP together with SL Rx-Tx in the same section, but different test cases with and without SL PRS-RSRP.

    - Define test case to verify accuracy requirements for SL PRS-RSRPP in a separate section.

  - Option 3:

    - Support option 1a to define separate sections for testing SL PRS-RSRP/PRS-RSRPP, without verifying the accuracy of the other (SL RSTD/Rx-Tx) measurement, respectively.

Issue 1-2-8: Test case for additional path measurements

Agreements:

  - This issue resolved under issue 1-2-6 ("RAN4 will not specify SL positioning test cases with additional path reporting”).

Topic #2: Carrier Phase Positioning Requirements

Sub-topic 2-1 Carrier Phase Positioning Core Requirements Maintenance (agenda 7.12.1.2)

Issue 2-1-1: Clarification on the PRS measurement period requirements for DL RSCP/DL RSCPD

Agreements:

For UE configured for CPP measurement with legacy measurement with multiple PFLs, when LMF does not configure measurement time window(s) for a PFL

  - UE performs DL RSCPD/RSCP measurement on a single PFL that is common between the reference TRP and the target TRP.

  - Existing requirements for RSTD/UE Rx-Tx without time window apply.

For UE configured for CPP measurement with legacy measurement with multiple PFLs, when UE is configured with time window and does not support FG 41-2-3:

  - UE performs DL RSCPD/RSCP measurement on the indicated PFL by the network.

  - Existing legacy requirements without time window apply for both CP measurements and legacy measurements.

For UE configured for CPP measurement with legacy measurement with multiple PFLs, when UE is configured with time window and support FG 41-2-3, but does not support FG 41-2-8/41-2-9:

  - Existing legacy requirements without time window apply for legacy measurements.

  - CPP measurement requirements apply for CP measurements for the PRS resources associated to the time window have occasions only within the measurement time window.

For UE configured for CPP measurement with legacy measurement with multiple PFLs, when UE is configured with time window and support FG 41-2-3 and FG 41-2-8/41-2-9:

  - CPP measurement requirements apply for both legacy measurements and CP measurements.

FFS:

Whether and/or how to define the measurement period requirements for DL RSCPD/RSCP measurements when aperiodic time window is configured.

Issue 2-1-2: Measurement period requirements for DL RSCP/DL RSCPD with multiple PFLs configured

Proposals:

  - Option 1:

    - When multiple PFLs are configured in the assistance data and DL RSCPD is requested with RSTD, the measurement period is given by

TRSCPD with RSTD,Total=TRSCPD with RSTD,j+i=1,i≠jLTRSTD,i+ L-1*maxTeffect,i

      - where  j is the index of the indicated PFL, if provided by the LFM, otherwise the PFL  j is selected by UE implementation.

      - If an aperiodic time window is configured, the start of the measurement period coincides with the start of the time window.

    - When multiple PFLs are configured in the assistance data and DL RSCP is requested with UE Rx-Tx, the measurement period is given by

TRSCP with UERxTx, Total=TRSCP with UERxTx, j+i=1,i≠jLTUERxTx,i+ L-1*maxTeffect,i

      - where  j is the index of the indicated PFL, if provided by the LFM, otherwise the PFL  j is selected by UE implementation.

      - If an aperiodic time window is configured, the start of the measurement period coincides with the start of the time window.

  - Option 2:

    - When multiple PFLs are configured where RSCPD is supposed to be measured in PFL j, support option 1 with the following updates (taking RSCPD measurement as the example)

      - For UE supporting both FG 41-2-3 and FG 42-2-8，TRSCPD with RSTD, j is the measurement period for both RSCPD and RSTD in PFL j by taking time window into account.

      - For UE supporting FG 41-2-3 only, TRSCPD with RSTD, j=max(TRSCPD, j,TRSTD, j), where TRSCPD, j is the measurement period for RSCPD in PFL j by taking time window into account, and T RSTD, j is the legacy measurement period for RSTD in PFL j without time window.

      - For UE not supporting FG 41-2-2 or FG 41-2-8, TRSCPD with RSTD, j=TRSTD, j.

Issue 2-1-3: The impact of carrier frequency offset

Proposals:

  - Option 1:

    - The accuracy requirements for CP measurement apply provided that the two PRS resources for calculating RSCPD or relative RSCP are located in the same set of symbols after accounting for expected RSTD.

  - Option 2:

    - The carrier phase definition should be clarified to indicate the carrier phase is defined at a specific location within the slot.

    - Define a common reference time and refer the DL-PRS carrier phase measurements to this reference time by subtracting the phase rotation due to the carrier frequency offset in the time interval between the DL-PRS and the reference time for the carrier phase measurement.

    - Define the referred carrier phase difference as the difference between the referred carrier phase measurements.

    - Define the same common reference time for the UE and the PRU.

    - The UE and the PRU report either the referred carrier phase measurements or the carrier phase difference measurements computed using the referred carrier phase measurements.

  - Option 3:

    - RAN4 not to define enhancements to mitigate impact of carrier frequency offset on CPP measurements in Rel-18.

    - RAN4 to define an applicability condition for DL RSCPD accuracy requirements based on the time proximity between the target and reference PRS resources used to calculate the DL RSCPD measurement.

    - DL RSCPD accuracy requirements are defined and apply when the target and reference PRS resources are measured in the same time slot. FFS the maximum time separation between the resources within a slot.

    - RSCPD accuracy derived from RAN4 simulations applies assuming zero carrier frequency error at the UE and TRPs.

Issue 2-1-4: Update measurement period requirements for RSTD, UE Rx-Tx and PRE-RSRP(P) with time window configured

Background: Companies mention that the time window can be configured for legacy measurements with RSCPD/RSCP measurements not requested. Hence the measurement period requirements for legacy measurements need to be updated.

The issue has been addressed in RAN4#108bis meeting:

Agreements from RAN4#108bis meeting:

- It is RAN4 understanding that legacy UE positioning measurements performed within time window(s) without CPP configured by LMF is not within the scope of the Rel-18 WI.

Issue 2-1-5: UE behaviour due to AGC adjustment

Agreements:

  - RAN4 not to define UE behaviour related to AGC adjustment.

Sub-topic 2-2 Carrier Phase Positioning Performance Requirements (agenda 7.12.2.6)

Issue 2-2-1: Whether to verify the accuracy of legacy measurements in RSCPD/RSCP TCs

Proposals:

  - Option 1:

    - Verify both the accuracies of legacy measurements and CPP measurements in one TC with a 90% success rate to reflect UE’s real positioning performance in the deployment.

  - Option 2:

    - Not verify the accuracy requirements for legacy RSTD/Rx-Tx measurement in the RSCPD/RSCP TC.

Issue 2-2-2: Additional margins for frequency drift and RF calibration

Proposals:

  - Option 1: Define extra margin in requirements

    - FFS: need for extra simulations.

  - Option 2: Do not define extra margin in requirements

    - Do not update simulation assumptions for carrier phase measurement.

Further discuss in the next meeting, proponents are requested to bring results justifying the additional margin for Option 1.

Issue 2-2-3: Test configurations

Agreements:

Time window configuration in CPP TCs:

  - Offset: same as PRS resource offset

  - Duration: covering all PRS resources from all TRPs

  - For RRC_CONNECTED: Periodicity: 2 times of PRS resource periodicity

  - For RRC_INACTIVE: The periodicity of time window should be 2 times of LCM(DRX cycle, PRS periodicity)

    - Note: In the legacy configuration PRS periodicity is always smaller than DRX periodicity

Reference

- R4-2408010, Topic summary for [111][213] NR_pos_enh2_part2, CATT, RAN4#111.

- R4-2410133, Ad-hoc minutes for NR_pos_enh2 WI (Monday), Ericsson, RAN4#111.

- R4-2410134, Ad-hoc minutes for NR_pos_enh2 WI (Tuesday), Ericsson, RAN4#111.

- R4-2410135, Ad-hoc minutes for NR_pos_enh2 WI (Wednesday), Ericsson, RAN4#111.