3GPP TSG-RAN WG4 Meeting # 111									                  		R4-2410203

Fukuoka, Japan, May 20 – May 24, 2024

Source:	Ad-hoc chair (Samsung)

Title:	Ad-hoc minutes for [111][230] Reply_LS (Missing Test Parameters for RAN5)

Agenda item:	11.3

Document for:	Discussion and Approval

1		Introduction

This t-doc captures the ad-hoc discussion outcome on [111][230] Reply_LS (Missing Test Parameters for RAN5)

- Topic#2: missing relative angular offsets and UE gain-related parameters for different power classes

2		Open issues from moderator summary

Issue 1-1-1: what is the assumption for Gain difference Y and Z between fine beam and rough beam for PC1/5/6？

What essential for this discussion is the antenna elements to be considered for PC1/PC5/PC6, where the value ever considered are: 32, 16, 16. However, if use such values to define Y/Z in RRM, the gain differences seem to be too tighten, since a good UE may be equipped with even more antenna elements. Larger difference might be needed for PC1/5/6.

- Proposals

  - Proposal 1: (Samsung, Qualcomm, Huawei)

- Proposal 1: For the gain difference Y and Z between fine beam and rough beam for PC1/5/6, the square brackets can be removed.

- Table B.2.1.3.1-1: Gain difference Y between fine and rough beams, Rx beam peak direction

| Value "Y" in dB, for each UE power class |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 18 | 9.0 | 7.0 | FFS | 15.5 | 15.5 | FFS |

- Table B.2.1.3.2-1: Gain difference Z between fine and rough beams, Spherical coverage directions

| Value "Z" in dB, for each UE power class |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 18 | 9.0 | 7.0 | FFS | 15.5 | 15.5 | FFS |

- Agreement:

  - Proposal 1 confirmed with removing [ ] on the agreed values

Issue 1-1-2: what is the UE gain for PC1/5/6

- Proposals

  - Proposal 1: (Huawei, Qualcomm)

The Gmin and Gmax for PC 1/5/6 are defined as:

|  | UE Power class |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 7 |
| Minimum, dBi | 0 | FFS | -10 | FFS | -5 | FFS |
| Maximum, dBi | 57 | FFS | +20 | FFS | 57 | FFS |

  - Proposal 2: (Samsung)

- Table B.2.1.5.1-1: UE gain G, Rx beam peak direction

|  | UE Power class |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| Minimum, dBi | -22 | FFS | -10 | FFS | -22 | -22 | FFS |
| Maximum, dBi | +26 | FFS | +20 | FFS | +24 | +24 | FFS |

- Discussion

  - MTK: For proposal 1, maximum gain shall 48 based on the 144 antenna element.

  - HW: Same observation as MTK.

  - Anristru: 5+20log (#antenna element)+3 -> 10log ? The gain difference may have impact test ability issue and also MU.

  - QC: RSRP is power shall 20log instead of 10log.

  - Samsung: For low bound, we would like to check whether 2Y needed or not ?

  - Nokia: We have concern on maximum dBi, and this is pending on number of antenna elements. Maximum vs typical values?

  - QC: We want advanced UE still can pass the test cases.

Agreement:

- For minimum dBi:  value from proposal 1 agreed with [ ].

  - Note: Value derived based on minimum EIS delta compared to PC3)

- For maximum dBi:

  - Equation:

    - Option 1: 20log (# antenna element) +5dBi +3

      - FFS whether need to update the value for PC3 to align the equation

    - Option 2: 10log (# antenna element) +5dBi +3  (PC3 assumption)

  - Number of antenna elements

    - Option 1: 144 for PC1, 64 for PC5/6?

    - Option 2: aligned with the assumption from Y and Z

      - 64 for PC1, 36 for PC5/6

  - Note: Test ability issue also need to be considered for feasible values of Gmin and Gmax

Offline suggestion:

|  | UE Power class |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| Minimum, dBi | [0] | FFS | -10 | FFS | [-5] | [-5] | FFS |
| Maximum, dBi | +50 | FFS | +20 | FFS | +39 | +39 | FFS |

Reference

[1] R4-2408027	Topic summary for [111][230] Reply_LS, Moderator (Apple)