3GPP TSG-RAN WG4 Meeting #111 	R4-2410364

Fukuoka, Japan, 20 May – 24 May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | 1 | Current version: | 18.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for AoA set up for multi-rx and TC for GBBR measurement accuracy |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR2_multiRX_DL-Perf |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | New AoA setup are needed for defining the TC for UE supporitng multi-rx Dl simultaneous reception in the TS 38.133 |  |  |  |  |  |  |  |  |
| Summary of change: |  | New AoA setup for UE supporitng multi-rx Dl simultaneous reception is introduced |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The multi-rx feature is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (new) A.3.15.X1, (new) A.3.15.X2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | Y |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R4-2409705 |  |  |  |  |  |  |  |  |

<Start of Change #1>

### A.3.15.X1	Setup X1: 2 AoAs for simultaneous reception with QCL Type-D

There are 2 active probes in the test. The DL signals, and noise if applicable, transmitted from the two active probes.  The 2 AoAs [(AoA1 and AoA2)]  for simultaneous reception with different QCL-typeD is from the set of [qualified AoA pairs] according to the spherical coverage requirement for simultaneous reception from multiple directions as defined in clause 7.3K.3 of TS 38.101-2 for power class 3 supporting simultaneous reception from multiple directions, the angular separation between the directions (AoAs) of the 2 active probes is declared from Table 7.3K.3-1 in clause 7.3K.3 of TS 38.101-2 [19] and [shall not be changed for each test iteration].

[The relative angular offset between the directions of the AoA pair is based on the UE’s declared [AoA separation and UE] orientation as defined in clause 7.3K.3 of TS 38.101-2.]

[One of the AoAs (AoA1) needs to satisfy the spherical coverage requirement in Table 7.3.4.3-1 of TS 38.101-2].

Editor’s note: FFS how single AoA RF test and 2 AoA RF test could ensure this.

### A.3.15.X2	Setup X2: 3 AoAs for simultaneous reception with different QCL Type-D

There are 3 active probes in the test and the DL signals and noise are transmitted from the three active probes.

Out of the three AoA, one AoA [(AoA1)] is aligned to a direction which is from the set of directions corresponding to the EIS spherical coverage percentile of the DUT as defined in clause 7.3.4 of TS 38.101-2 [19] for UE power class 3 and other 2 AoAs [(AoA2, AoA3)] are from the set of [qualified AoA pairs] according to the spherical coverage requirement for simultaneous reception from multiple directions as defined in clause 7.3K.3 of TS 38.101-2 for power class 3 supporting simultaneous reception from multiple directions.

The relative angular offset between the directions of the AoA pair is based on the UE’s declared [AoA separation and UE] orientation as defined in clause 7.3K.3 of TS 38.101-2 and [shall not be changed for each test iteration].

<End of Change #1>