3GPP TSG-RAN WG4 Meeting # 111	R4-2410593

Fukuoka, Japan, 20th - 24th May 2024

Title:	TP for TR 36.718-02-01 Addition of CA_3-8-11 with 2UL

Source:	Softbank Corp.

Agenda item:	8.1.3.2

Document for:	Approval

1	Introduction

This contribution is a text proposal for TR 36.718-02-01[1] to include CA_3-8-11 with 2UL as requested in RAN4#109.

2. Reference

[1] TR36.718-02-01, LTE-A intra-band/inter-band Carrier Aggregation for x (x<=6) bands DL with y bands (y=1, 2) UL (Release 18) V0.0.6

[3] TR37.716-21-11, Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL) (Release 16) V16.1.0

Text Proposal

-- Start of TP –

### 5.4.x	CA_3-8-11

#### 5.4.x.1	Channel bandwidths per operating band for CA

Table 5.4.x.1-1: Inter-band CA operating bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
| 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
| 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |

Table 5.4.x.1-2: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_3A-8A-11A | CA_3A-8A CA_3A-11A | 3 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 11 |  |  | Yes | Yes |  |  |  |  |

#### 5.4.x.2	Co-existence studies

Coexistences for CA_3-8 and CA_3-11 were referred the DC_8-11_n3 with UL DC_8_n3 and UL DC_11_n3 which are already analysed in TR37.716-21-11[3]. The analysis result is that there is no additional intermodulation impact for the additional band receiver.

So this section can be omitted.

#### 5.4.x.3	∆TIB and ∆RIB values

Relaxation values for CA_3-8-11 have already been specified in TS 36.101.

#### 5.4.x.4	REFSENS Requirements

No additional REFSENS relaxation required compared to fallbacks.

-- End of TP --