3GPP TSG-RAN WG4 Meeting #111	R4-2410750

Fukuoka, Japan, 20-24 May, 2024

Title: 	WF on higher power UE

Agenda Item:	10.1.1.2

Source: 	Samsung

Document for:	Approval

1. General aspect

1.1 Duty cycle solution for SAR compliance

Online agreement: Duty cycle solution is considered for both intra-band CA and inter-band CA/EN-DC

2. PC1.5 for intra-band contiguous and non-contiguous UL CA

2.1 Assumed UE architecture(s)/parameters/methodology for MPR/A-MPR evaluation

Online agreement:

Agreement:

- Evaluate MPR/A-MPR numbers for contiguous CA first and then work on non-contiguous CA MPR/A-MPR afterwards.

- The intra-band PC1.5 ULCA R19 work focusses on UE implementing with two 26dBm PAs. Architectures requiring a 29dBm PA are not specified but can be implemented by fulfilling the requirements based on two 26dBm PAs.

- For intra-band contiguous ULCA w/ and w/o UL MIMO, the Architecture#2 is prioritized for MPR/A-MPR evaluation.

  - If time is allowed, architecture #1 can be evaluated.

| Architecture | Description | Indicated capability | Whether can support UL MIMO | Applicable cases |
| --- | --- | --- | --- | --- |
| #1 | 2x26 dBm PA + 2 LO with 100MHz BW | dualPA-Architecture | No | Contiguous CA |
| #2 | 2x26 dBm PA + 1 LO with 200MHz BW | TxD | Yes | Contiguous CA |

- For intra-band NC ULCA w/o UL MIMO, the Architecture#1 is prioritized for MPR/A-MPR evaluation.

  - FFS on Architecture #2

| Architecture | Description | Indicated capability | Applicable cases |
| --- | --- | --- | --- |
| #1 | 2x26 dBm PA + 2 LO with 100MHz BW | dualPA-Architecture | NC CA w/o UL MIMO |
| #2 | 2x26 dBm PA + 1 LO with 200MHz BW | TxD | NC CA w/o UL MIMO |

2.2 Whether to define separate MRP/A-MPR requirements for handheld UE and FWA respectively?

Way forward:  Yes, and assuming 20dB minimum antenna isolation for FWA and 10dB for handheld UE

2.3 A-MPR for n41/n77/n78/n79

Way forward:

  - A-MPR requirements are not needed for PC1.5 n77/n78/n79 intra-band ULCA

  - Evaluate A-MPR requirements for PC1.5 n41 intra-band UL CA

  - PCMAX,C

Proposals:

  - Proposal 1: Define PCMAX,c limitation for each component carrier considering PC1.5 UE architecture of intra-band UL CA. (LGE)

- For dualPA-architecture

  - PCMAX,C limitation  for each component carrier is 26dBm

- For TxD (dualTx)

  - PCMAX,CC1 = 26+10*log10( LCRB1*SCS1LCRB1*SCS1+LCRB2*SCS2)+3

  - PCMAX,CC2 = 26+10*log10( LCRB2*SCS2LCRB1*SCS1+LCRB2*SCS2)+3

  - Proposal 2: Changes to TS 38.101-1 for dual-PA architecture, inset new equation for PCMAX,c (Ericsson, more details refer to R4-2407721)

Way forward: FFS in future meetings

2.5 PCMAX

Proposals:

  - Option 1: (Skyworks)

  - MOP for 2Tx architectures (TxD w/wo UL MIMO) is: 29 dBm

  - MOP for Dual-PA architectures (one PA/CC, 2LO) is:

PCmax= 29 + 10*log(1/2*(1+Min(LCRB1*SCS1,LCRB2*SCS2)/Max(LCRB1*SCS1,LCRB2*SCS2))) dBm

  - Option 2: (Skyworks)

MPR for Dual-PA architectures (one PA/CC, 2LO) is modified by: 10*log(1/2*(1+Min(LCRB1*SCS1,LCRB2*SCS2)/Max(LCRB1*SCS1,LCRB2*SCS2))) dB

  - Option 3: (Captured in last meeting’s WF)

For R19 PC1.5 intra-band non-contiguous /contiguous ULCA with two 26dBm PAs and one PA per CC, the PCMAX is modified as follows to account for RB BW imbalances

PCmax=10*log(10^(26/10) + 10^((26-10*log(LCRB1*SCS1/(LCRB2*SCS2)))/10))

  - Option 4: For UEs indicating dualPA-Architecture (Architecture #1) the total configured output power PCMAX for the band combination is given by clause 6.2A.4.1.3 for inter-band CA in the current version of 38.101-1, the same applies for TxD (Ericsson)

Way forward: FFS in future meetings

2.6 PCMAX tolerance

Agreement: The PCMAX tolerance for uplink intra-band contiguous/non-contiguous CA are:

| PCMAX (dBm) | Tolerance TLOW(PCMAX) (dB) | Tolerance THIGH(PCMAX) (dB) |
| --- | --- | --- |
| 23 < PCMAX ≤ 29 | 3 | 2 |
| 21 ≤ PCMAX ≤ 23 | 2.0 |  |
| 20 ≤ PCMAX < 21 | 2.5 |  |
| 19 ≤ PCMAX < 20 | 3.5 |  |
| 18 ≤ PCMAX < 19 | 4.0 |  |
| 13 ≤ PCMAX < 18 | 5.0 |  |
| 8 ≤ PCMAX < 13 | 6.0 |  |
| -40 ≤ PCMAX < 8 | 7.0 |  |

3. 2-band Inter-band UL NR-CA/EN-DC with 2Tx and/or 3Tx

  - Requirements between handheld UE and FWA

Way forward:

  - For PC2 and PC1.5 2-band inter-band UL CA with 2Tx or 3Tx, only define one set of UE RF requirements for both handheld UE and FWA

  - For PC2 and PC1.5 2-band inter-band EN-DC with 2Tx or 3Tx, only define one set of UE RF requirements for both handheld UE and FWA.

3.2 Whether to collect configurations to derive general requirements?

Way forward:

  - General requirements can be specified including considerations of different duplex mode configurations:

  - For PC2, FDD+FDD, FDD+TDD, TDD+TDD with 2Tx and 3Tx

  - For PC1.5, [FDD+FDD,] FDD+TDD, TDD+TDD with 2Tx and 3Tx

3.3 3UL CC with 3Tx

Way forward: RAN4 to further check whether there is demand for 3 ULCC with 3Tx scenario (such as CA_nXA-nY(2A) and CA_ nXA-nYB), and whether it is desirable to be introduced in Rel-19.

3.4 Release independence

Way forward:

Discuss the release independence for 3Tx band combination for handheld UE and FWA at later stage of this WI

  - For FWA, only discuss the configurations that were not introduced in Rel-18

4. Increasing UE transmission power

4.1 The methodology

Online agreement: Use the higherPowerLimit-R17 and higherPowerLimtMRDC-R17 capability as the starting point.

4.2 Whether increasing power limit applicability requires ΔPPowerClass,CA /ΔPPowerClass,EN-DC =0

Adhoc agreement: The high-power limit feature (higherPowerLimit-R17/higherPowerLimtMRDC-R17) only applies with ΔPPowerClass,CA /ΔPPowerClass,EN-DC = 0 dB, which is aligned with the Rel-17 agreement

4.3 The scenarios to be considered in Rel-19

Proposals:

  - Proposal 1: (Samsung)

- For 2Tx, the following scenario can be considered for NR-CA/EN-DC.

| Indicated PC for A-B (2Tx in total) | PC for band A of A-B | PC for band B of A-B | From which release increasing high power limit feature supported | Note |
| --- | --- | --- | --- | --- |
| PC2 | PC3(FDD or TDD) | PC2 (FDD or TDD) | Support from Rel-19 | One CC on band A, 2CC on band B |
| PC2 | PC3(FDD or TDD) | PC2(FDD) | Support from Rel-19 | One CC per band |

- For 3Tx, the following scenarios may could be considered for NR-CA/EN-DC.

(Note the analysis is based on the implementation feasibility)

| Indicated PC for A-B (3Tx in total) | PC for band A of A-B (1Tx) | PC for band B of A-B (2Tx) | The total power (dBm) | Note |
| --- | --- | --- | --- | --- |
| PC2 | PC3 | PC2 | 27.8 | One CC per band One CC in band A, 2CC on band B (if this scenario would be introduced for 3Tx) |
| PC1.5 | PC3 | PC1.5 | 30.0 | One CC per band; For FWA only One CC in band A, 2CC on band B (if this scenario would be introduced for 3Tx); For FWA only |

  - Proposal 2: Consider PC3+PC2 and PC3+PC1.5 for 3Tx as scenarios for increasing power limit Rel-19. (vivo)

  - Proposal 3: (Qualcomm)

- higherPowerLimit-r17  is enabled for any standardized inter-band band combination. Cases where there is concern about exceeding local regulation are treated separately as exceptions.

- RAN4 to rely on network operators (carriers) to identify if local regulatory limits are exceeded when higherPowerLimit-r17 is enabled for their band combination.

- RAN4 to evaluate best method to identify corner case UL CA configuration (band combination + power class aggregation) where higherPowerLimit-r17 may NOT be used due to local regulation. Options:

    - Set up an NS case

    - Rely on PEMAX,CA, the value indicated by p-NR-FR1 or by p-UE-FR1

    - Maintain an exception list in 38.101-x for UL CA configurations where higherPowerLimit-r17 may NOT be used

- Remove references to power class or TR duplexing type from sections of the standard that enable use of higherPowerLimit-r17

  - Proposal 4: (Skyworks)

- Regardless of the down selected increased power cases for Release 19, to be future proof, the work should address PCmax equations in order to support:

- Power increase for 3 levels (0.5/1/1.8dB).

- Power boosting on one or two bands.

- Increased power with power boosting additionally.

- Increased power of 3dB should not be allowed and a higher band combination power class signalled instead.

  - FFS on how to deal with cases with 3dB increases above PC1.5

  - FFS if this rule applies to cases where the 3dB increase is the result of power boosting.

  - Proposal 5: The following power class configuration could be considered in Rel 19 for UE increasing high power limit. (Xiaomi)

- PC3 (TDD/FDD) +PC1.5 indicating PC1.5 with 3Tx

- PC2 (TDD with TxD) +PC3 (TDD/FDD) indicating PC2 with 3Tx

  - Proposal 6: Consider Option 3 as the scenarios of increasing higher power limit in Rel-19. (LGE)

- For PC2 2Tx inter-band NR CA and ENDC:

- Inter-band with intra-band UL CA in one of the bands

- For HPUE 3Tx inter-band NR CA and ENDC:

- PC2 band combination of PC3+PC2 with single carrier in each band.

- PC1.5 band combination of PC3+PC1.5 with single carrier in each band.

- PC1.5 band combination of PC2+PC1.5 with single carrier in each band.

- Note: Only PC3 is considered for LTE FDD in EN-DC

  - Proposal 7: Both FWA and handheld UE can support PC3+PC1.5 with increasing UE high power limit feature in Rel-19 if technical issues are not provided. (DCM)

  - Proposal 8: (Huawei)

- Consider the following list of scenarios for increasing UE transmission power limit as shown in Table 1 below. And Increasing the total Tx power limit beyond PC1.5 is only for FWA UEs, not for handheld UEs.

Table 1: A list of applicable scenarios for increasing UE transmission power limit

| CA power class | Power class configuration Band A + Band B |  | 2Tx | 3Tx |
| --- | --- | --- | --- | --- |
| PC3 | PC3 | PC5 | Completed in R18 | N/A |
| PC2 | PC2 | PC3 | Completed in R17 | R19 |
|  |  | PC5 | R19 | R19 |
| PC1.5 | PC1.5 | PC2 | N/A | R19 |
|  |  | PC3 | N/A | R19 |
|  |  | PC5 | N/A | R19 |

  - Proposal 9: (ZTE)

- To consider the following additional eligible PC2 2Tx inter-band NR CA and ENDC to enable increasing higher power limit in Rel-19:

- Inter-band with intra-band UL CA in one of the NR band

- To consider the following additional eligible HPUE 3Tx inter-band NR CA and ENDC with up to 3CC in UL bands to enable increasing higher power limit in Rel-19:

- PC2 band combination of PC3+PC2

- PC1.5 band combination of PC3+PC1.5

- PC1.5 band combination of PC2+PC1.5

Note: Only PC3 is considered for LTE FDD in EN-DC

  - Proposal 10: (CATT)

    - For 2Tx transmission, there is no new scenarios for increasing UE transmission power

    - For 3Tx transmission, there are three new scenarios for increasing UE transmission power as shown in the table below:

| Indicated PC for A-B (3Tx in total) | PC for band A of A-B (1Tx) | PC for band B of A-B (2Tx) | The total power (dBm) | Note |
| --- | --- | --- | --- | --- |
| PC2 | PC3 | PC2 | 27.8 | One CC per band |
| PC1.5 | PC3 | PC1.5 | 30.0 | One CC per band; For FWA only |
| PC1.5 | PC2 | PC1.5 | 30.8 | One CC per band; For FWA only |

Way forward: FFS in future meetings

4.4 MSD and SAR compliance

Way forward:

  - If higherPowerLimit-R17/higherPowerLimtMRDC-R17 capability is adopted for increasing UE transmission power, then taking same methodology as for higherPowerLimit-R17/ higherPowerLimtMRDC-R17 for SAR compliance and MSD. More specific,

- For a given band combination, no additional MSD requirements are specified in addition to the MSD requirements of its legacy power class(es)

- No new duty-cycle solution/mechanism for SAR compliance is considered

- P-MPR can be used for SAR mitigation