3GPP TSG-RAN WG4 Meeting #112													R4-2413424

Maastricht, Netherlands, 19 August – 23 August 2024

Agenda item:			5.28.3

Source:	Moderator (Ericsson)

Title:	Topic summary for [112][324] NR_redcap_enh_demod

Document for:	Information

# Introduction

This topic summary discusses the maintenance issues for CQI reporting in fading condition for eRedCap.

# Topic #1: Title

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2412549 | Ericsson | Observation 1: Throughput ratio of follow CQI and fixed median CQI is degraded due to the mismatch between CSI-RS BW and PDSCH BW. Observation 2: Throughput ratio of follow CQI and fixed median CQI is high enough compared with the Rel-17 RedCap CQI fading test, i.e., γ=1.05, even if CSI-RS BW is 24RB. Proposal 1: Apply the Rel-17 RedCap CQI reporting test in fading condition to Rel-18 eRedCap: - A CQI index not in the set {median CQI - 1, median CQI, median CQI + 1} shall be reported at least α% of the time, where α = 20%. - Throughput ratio of follow CQI and fixed median CQI shall be ≥ γ, where γ = 1.05 - Average BLER with follow CQI shall be greater than or equal to 0.02. Proposal 2: Set SNR=9/10dB and SNR=15/16dB for CQI reporting test in fading condition for FDD/HD-FDD/TDD with 1Rx, same as Rel-17 RedCap CQI reporting test in fading condition. Proposal 3: Set SNR=6/7dB for CQI reporting test in fading condition for FDD/HD-FDD/TDD with 2Rx, same as Rel-17 RedCap CQI reporting test in fading condition. |
| R4-2412757 | Huawei, HiSilicon | Proposal 1: RAN4 to define CQI requirements for eRedCap UE under fading channel. The test SNR and metric can be: - SNR=9/10dB and SNR=15/16dB for FDD/HD-FDD/TDD with 1Rx. - SNR=6/7dB for FDD/HD-FDD/TDD with 2Rx. - Requirements: - A CQI index not in the set {median CQI - 1, median CQI, median CQI + 1} shall be reported at least α% of the time where α = 20%. - Throughput ratio of follow CQI and fixed median CQI shall be ≥ γ, where γ = 1.05 - Average BLER with follow CQI shall be greater than or equal to 0.02. |
| R4-2413311 | Nokia, Nokia Shanghai Bell | Observation 1: Under static condition, the CQI metric > 90% is fulfilled in all cases. Thus, it is justified to specify CQI reporting requirements under static condition for eRedCap. Observation 2: Under fading condition, the CQI metric > alpha=20% is not fulfilled in all cases. Especially, for 1Rx UE it is not fulfilled for SNR values > 9 dB. Thereagainst, for 2Rx UE, the CQI metric is fulfilled. Proposal 1: RAN4 to remove CQI reporting requirements under fading condition for eRedCap, or to remove them at least for 1 Rx UE. |
| R4-2412550 | Ericsson | Simulation results summary |
| R4-2412551 | Ericsson, Huawei, HiSilicon | CR for 38.101-4: Correction of eRedCap demodulation and CSI reporting requirements |
| R4-2411394 | Apple | CR on PDSCH TDD Requirements for Enhanced Support of RedCap |

## Open issues summary

RAN4#111 concluded that RAN4#112 discusses whether the following Rel-17 RedCap CQI fading test is applicable for Rel-18 eRedCap UE or not: - SNR=9/10dB and SNR=15/16dB for FDD/HD-FDD/TDD with 1Rx. - SNR=6/7dB for FDD/HD-FDD/TDD with 2Rx. - Requirements: - A CQI index not in the set {median CQI - 1, median CQI, median CQI + 1} shall be reported at least α% of the time where α = 20%. - Throughput ratio of follow CQI and fixed median CQI shall be ≥ γ, where γ = 1.05 - Average BLER with follow CQI shall be greater than or equal to 0.02.

### Sub-topic 1-1 CQI reporting tests in fading condition

Issue 1-1-1: Whether to apply the CQI reporting tests in fading condition to eRedCap UEs.

- Proposals

  - Option 1 (Ericsson, Huawei): Define the CQI reporting requirements under fading condition for eRedCap.

  - Option 2 (Nokia): RAN4 to remove CQI reporting requirements under fading condition for eRedCap, or to remove them at least for 1 Rx UE.

- Recommended WF

  - Two companies propose to apply Rel-17 CQI fading tests to Rel-18 eRedCap. One company proposes not to define CQI test or only limit to 2Rx UE.

  - Moderator recommends:

    - Define the CQI reporting requirements under fading condition for 2Rx eRedCap UE (FDD/HD-FDD/TDD).

    - Discuss further for 1Rx eRedCap UE.

    - Revise CR (R4-2412551) according to the conclusion.

# Recommendation for Tdocs

| T-doc number | Suggested status | Comments |
| --- | --- | --- |
| R4-2412549 | Noted |  |
| R4-2412757 | Noted |  |
| R4-2413311 | Noted |  |
| R4-2412550 | Withdrawn? | To be withdrawn if no new simulation results in this meeting. |
| R4-2412551 | Revised | Revise according to the online discussion. |
| R4-2411394 |  | Collect comments. |