3GPP RAN WG 4 Meeting #112	R4-2414921

Maastricht, Netherlands 19- 24 August 2024

Agenda Item:	6.2.5

Source:	Spark NZ Ltd

Title:	Text Proposals on MIMO models and PA nonlinearity impacts and ACLR

Document for:	Approval

- Introduction

# There are two parts to this contribution and text proposals for each are provided. However, the PA non-linearity part also requires some discussion:

# MIMO models

# PA non linearity impacts and ACLR

# MIMO models

# 3GPP is working on a response to an LS from ITU-R WP 5D. Amongst other questions, there is a question on the impact of beamforming techniques (when considering simple multiuser MIMO (MU-MIMO) scenarios) on the radiated antenna gain from mobile base stations. The specific request from ITU-R WP 5D is as follows:

“Considering the system-level simulation work typically conducted in ITU-R WP 5D for sharing and compatibility studies using 3D modelling of antenna pattern, ITU-R WP 5D is seeking information on whether for implementation of IMT BS multiuser spatial beamforming techniques, such as zero-forcing (ZF) or minimum mean-square error (MMSE) based schemes, would be necessary to improve the assessment of interference and the accuracy of studies. If considered necessary, guidance on the process of deriving the necessary beamforming weights for the IMT AAS BS to compute its radiation pattern.”

The aim of this contribution is to provide the text proposals in how to calculate the beamforming matrices (known also as beamforming weights) when the mobile base station operates in the MU-MIMO mode using zero-forcing (ZF) and minimum mean square error (MMSE) (a.k.a. regularised zero-forcing) beamforming are used under the assumption that perfect channel state information (CSI) is available at the mobile base station and user equipments. This strong assumption is reasonable for the assessment of the impact of ZF and/or MMSE beamforming on the radiated gain levels of mobile base stations for co-existence analysis with other radio services. This is different from performance evaluation of specific beamforming techniques for optimisation of spectral efficiency/capacity or coverage.

Text proposals providing BF weights are given in this contribution.

ZF requires the computation of a pseudo inverse of the concatenation of channel matrices. It is important that the pseudo inverse is based on the correct formulation of the channel matrices as otherwise  the results will be misleading  The array response vmn  ( in the 3GPP AAS model) can be made to provide maxima at given angular directions by an appropriate choice of  the angles in wmn which can be viewed as additional phase terms in the array response.  It should be noted that pseudo inverse based on concatenated channels in eqn (1) that are in turn based on  wmn alone is flawed and will not provide an exact impact of ZF on the complete array response.

Simulations of array gain above the horizon  for an example of two users is provided. The results indicate that using the ZF the median value of the difference between the current approach in ITU R WP 5D and using Zero forcing via correct formulation of the pseudo inverse  is about 3 dB.  With more users this difference will further increase. During the Maastricht meeting the complexity of doing a pseudo inverse was discussed but performing a matrix inverse can  hardly be deemed as complex when it is just a single line command in Matlab.

There was also “agreement” amongst  the proponents who had submitted simulations and input contributions  to the Maastricht meeting that a common set of simulation assumptions be agreed . There have been discussions about the assumptions but they have not concluded though a draft simulation assumptions has been circulated to some delegates.

Proposal 1: The text proposals in this document be included in TR 38 922, section 7

Proposal 2ZF requires the computation of a pseudo inverse of the concatenation of channel matrices. It is important that the pseudo inverse is based on the correct formulation of the channel matrices as otherwise  the results will be misleading  The array response vmn  ( in the 3GPP AAS model) can be made to provide maxima at given angular directions by an appropriate choice of  the angles in wmn which can be viewed as additional phase terms in the array response.  It should be noted that pseudo inverse based on concatenated channels in eqn (1) that are in turn based on  wmn alone is flawed and will not provide an exact impact of ZF on the complete array response. : We could advise WP 5D on how to calculate the BF weights with ZF with correct formulation of the pseudo inverse and that the differences between the current approach ie using ITU R M 2101 and ZF cannot be ignored.

This text could be sent in an LS to WP 5D. Alternatively the RAN4 experts could agree on common simulation assumptions and resubmit the simulations to the RAN4 113 meeting so that a unified answer may be provided to WP 5D.

PA nonlinearity impacts and ACLR

The specific question from WP 5D is:

To establish a model for array correlation factor (r) roll-off to describe how the array factor collapses moving away from the centre frequency. If the beamforming performance degrades towards a single element radiation pattern.

There are adjacent services in the bands being considered for IMT 2030.

For example: for the case of 4400-4800 MHz, the adjacent band services are:

On the lower edge below 4400 MHz:

- Aeronautical radionavigation;

- Aeronautical mobile;

- Additional possible allocation of EESS (passive) as per Agenda Item 1.19 of WRC-27;

On the upper edge above 4800 MHz:

- Fixed service;

- Radio Astronomy;

- Radio navigation satellite;

For the case of 7125-8400 MHz, the adjacent band services are:

On the lower edge below 7125 MHz:

- Fixed

- Fixed Service Satellite (Earth-to-space)

On the upper edge above 8400 MHz:

- Space Research Service (space-to-Earth)

- Additional possible allocations of EESS (passive) as per Agenda Item 1.19 of WRC-27

- Earth Exploration Satellite

- Radiolocation

- Space Research (active).

For the case of 14.8-15.35 GHz, the adjacent band services are:

On the lower edge below 14.8 GHz:

- Fixed Service;

- Fixed Satellite Service (Earth-to-space)

- Radionavigation

On the upper edge above 15.35 GHz:

- Space research (passive)

- EESS (passive)

- Radio Astronomy

In particular, the frequency range 15.35 – 15.4 GHz is marked as a footnote 5.340 band, which is stating “All Emissions are Prohibited” with the exception of those provided by RR 5.511.

It is therefore important that out of band emissions are correctly modelled so that appropriate guidance is provided for the above question to WP 5D.

# 2. Array Performance in adjacent bands.

Out of band emissions are well understood but often for a single transmitter.

It is necessary to distinguish between a single element ACLR, and an array ACLR. In a non AAS system, where the radiation pattern is “constant” over a sector, the single element ACLR makes sense, since the received power ratio at any point is the same as the transmitted. However, with AAS, the desired signal experiences also an array/beamforming gain that might be different from the array/beamforming gain of the interfering signal received in the adjacent band; this later observation is also dependent if SU or MU MIMO is used (ie the array may send distortions in unintended directions).

When an array in AAS base stations is used, the 3GPP specifications provide a method to compute the array ACLR (albeit in a TRP mode) [16]:

“The ACLR (CACLR) absolute basic limits in table 6.6.3.2-2 + X, 6.6.3.2-2a + X (where X = 9 dB) or the ACLR (CACLR) basic limit in table 6.6.3.2-1, 6.6.3.2-2a or 6.6.3.2-3, whichever is less stringent, shall apply.”

However, the above method is array pattern agnostic and does not account for beamforming variations due to MU MIMO.

Observation 1: Is the method to compute an array ACLR as defined currently in 3GPP specification still correct? If not, what improvements are needed?

Recent published results, discussed/summarized in this report, show that that array ACLR is spatially sensitive and is also dependent if SU or MU MIMO functionality is utilized. There is thus a need to accurately model the ACLR when AAS is used.

As part of additional information requested by ITU-R WP 5D establish a model for array correlation factor (r) roll-off to describe how the array factor collapses moving away from the centre frequency. If the beamforming performance degrades towards a single element radiation pattern, then the frequency when the continuity of array features is regarded as degraded, will also need to be determined.

To address the SI, the questions posed may be devolved into topics below:

- Establish a model for array correlation factor-ie the correlation of the in-band signal with the out of band signal.

Observation 2a:  There is no definition of an array correlation factor. Is it the correlation of the in-band signal with out of band signal. But the in-band signal, Pinband (θ,φ),   is spatially sensitive and as discussed below the out of band signal , P oob (θ,φ),  may also be spatially sensitive.  Then the array correlation  is also spatially sensitive may be defined as:

r (θ, φ) = E (Pinband (θ,φ) PT oob (θ,φ) ) ………………………….(1)

- How does the array factor collapse when moving away from the centre frequency?

Observation 2b: This also means that r (θ, φ) defined above is a function of frequency separation from the band centre. In this case best to denote this as:

r f(θ, φ)………………………………………………………………..(2)

Where f is the frequency separation from band centre

- If the beamforming performance degrades towards a single element radiation pattern, then the frequency when the continuity of array features is regarded as degraded, will also need to be determined.

Observation 3: In order to do this, we would therefore need to characterise and investigate the constituent components of P oob (θ,φ) as a function of f

Proposal 3: There is a need to define array correlation factor so that it may be computed. Equation (1) may be used.

Proposal 4: Define the constituent components of P oob (θ,φ) and their dependence on  f

Proposal 5: There is a need to investigate array ACLR and see that its computation method in the standards is still the correct way.

The above topics are interrelated but need to be separately discussed as well. The discussion below addresses topics 2 and 3 respectively.

As an aside, let us denote the three bands under study as:

- Band 1: 4400-4800 MHz

- Band 2: 7125-8400 MHz

- Band 3: 14.8-15.35 GHz.

Array performance in adjacent bands is governed by:

- Array performance at different frequencies arising due to adjacent bands.

- Modelling of the impact of PA non linearities

- Modelling of band pass filters

- Putting all of the above together to obtain an end-to-end model

Additionally, one must also consider if the array is supporting a single user or multi-user systems.

Array performance at adjacent band frequencies.

Assume that the lower and upper adjacent bands have a bandwidth of 100 MHz each respectively.  Arrays in a band are designed at the centre frequency of a given band ie vertical and horizontal spacing is determined by the centre frequency of the band which for example in band 1 is 4600 MHz shown in Table xxxxx-1 below. Likewise for other bands. The lower and upper adjacent bands can be simulated via a proxy array with different spacings e.g for band 1, the lower adjacent band is 4300-4400 MHz with centre frequency 4350 MHz. Therefore, for band 1, changing the vertical spacing to 0.74 lambda and horizontal spacing to 0.53 lambda will effectively simulate the lower adjacent band. Likewise for the upper adjacent band (0.66, 0.474) will simulate the upper adjacent band.

Table xxxxx-1: Array performance in adjacent bands

| Band | Centre Frequency | 0.7 Lambda, 0.5 Lambda (cm) | Centre freq of Lower adj band | Centre freq of Upper adj band | 0.7 Lambda, 0.5 Lambda for lower adj band | 0.7 Lambda, 0.5 Lambda for upper adj band |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 4600 MHz | 4.565/3.26 | 4350 MHz | 4850 MHz | 4.827/3.45 (0.74, 0.53) | 4.33/3.09 (0.66, 0.474) |
| 2 | 7762.5 MHz | 2.7/1.93 | 7075 MHz | 8450 MHz | 2.96/2.118 (0.767, 0.55) | 2.48/1.773 (0.64, 0.46) |
| 3 | 15.075 GHz | 1.39/0.99 | 14.75 GHz | 15.40 GHz | 1.42/1.012 (0.715, 0.511) | 1.36/0.97 (0.68, 0.49) |

The following parameter/metrics are impacted [1] when the inter element spacing is changed:

- Nulls

- Maxima

- Half power points

- Minor lobe maxima

- First null beamwidth

- Half power beamwidth

- First side lobe beamwidth

Exact expressions for the above are given below from [1] for a ULA along the z axis and for omni directional antennas. Considering the exact formulas below in Table xxxxx-2 and Table xxxxx-3, the changes due to inter element spacing in the proxy array will result in some changes to the values of the above metrics. See Figure xxxxx-1, Figure xxxxx-2 and Figure xxxxx-3 for an eight element ULA for 0.5 lambda, 0.7 lambda and 0.8 lambda inter element spacing respectively. However these frequency/wavelength adjustments are no different to what is present today in systems that have a wide operating bandwidth and antennas are designed at the centre frequency of the band). Note that the results below are not for an array of sub arrays [9] and 3GPP antenna element patterns, but the trends will be similar for the array of sub array case as well.

Table xxxxx-2: Broadside array antenna pattern equations

| Nulls, Maxima, Half-Power points and Minor Lobe maxima for Broadside Arrays |  |
| --- | --- |
| Nulls | θn = cos-1 (±nN λd), n = 1,2,3…., n≠N, 2N,3N.. |
| Maxima | θm = cos-1 (±m λd), m = 0,1,2,3…., |
| Half Power Points | θh = cos-1 (±1.391λπ Nd), πdλ ≪ 1 |
| Minor Lobe Maxima | θs= cos-1 (±λ2d 2s+1N), s = 1,2,3, πλd ≪ 1 |

Table xxxxx-3: Broadside array beamwidth equations

| Beamwidths for Uniform Amplitude Broadside Arrays |  |
| --- | --- |
| First Null beamwidth (FNBW) | θn=π2-cos-1λNd |
| Half Power beamwidth (HPBW) | θh≅2π2-cos-11.391λπNd πdλ ≪ 1 |
| First sidelobe beamwidth (FSLBW) | θs≅2π2-cos-13λ2Nd πdλ ≪ 1 |

Figure xxxxx-1:  Eight element ULA response with 0.5 lambda spacing

Figure xxxxx-2:  Eight element ULA response with 0.7 lambda spacing

Figure xxxxx-3:  Eight element ULA response with 0.8 lambda spacing

It is also to be noted that in case of band 2 where the band is very wide (operating bandwidth 1275 MHz) proxy antenna inter element spacings for a carrier at the lower end of the band will almost have a similar impact as the lower adjacent band. Likewise for the upper end of the band.

The above inferences will also remain valid for the second adjacent band.

The above observations will remain valid even if the array is made of sub arrays and the notion that an array may drop down to a sub array in adjacent bands is not valid.

Observation 4:  The change in array performance due to a different vertical and horizontal spacing arising from the lower and upper adjacent bands is not large enough to warrant a conclusion that the array drops down to a single element in adjacent bands.

Proposal 6: An array will not drop down to a single element in adjacent bands. Nor its performance will drop down to a sub array.

Modelling of PA non linearities

Non linearities in hardware in the RF path cause signal distortions- both in band and out of band-(adjacent bands). The nonlinear components in the RF path consist of power amplifiers that are the main cause of nonlinear distortions but other components such as D/A converters, mixers, beam forming circuitry may also contribute to the nonlinear distortions. When antenna arrays are deployed, multiple users may be served by beam forming either in the analog domain or both in the analog and digital domain.  Digital beamforming is commonly employed in today’s systems- such as zero forcing, MMSE [5]. This results in the placing of nulls in the path of all users except the desired user, but this operation may also cause out of band distortions to fall in unintended spatial directions.

The modelling of PA non linearities has received a lot of interest in the published literature [see 8,10-14 and references therein]. PA behavioural models can be classified as:

- Volterra series based,

- Artificial neural networks,

- Table look up methods,

- Models with linear and nonlinear memory, with and without memory etc.

Ref [8] gives a very good comparison of PA behavioural models in terms of metrics like identification complexity, adaptation complexity, running complexity etc. Measurements on some PAs and the accuracy of the models are presented in [12]. Some commonly used models use polynomials or memory polynomials [10, 14], here PA units are modelled via 9th order polynomials whose coefficients are obtained from RF measurements. These papers conclude that the received signal with massive MIMO consists of a desired term with linear gain of the array and also a nonlinear distortion term. Additionally,  if the linear response coefficients of the different parallel PAs (in the massive MIMO unit) are different (and they do have small differences), then even the linear signal terms are not fully coherently combining at the receiver. In case of the nonlinear distortion terms, this effect is even more evident as here the phase characteristics of the constituent terms representing the component PAs are different. These two effects manifest in the decrease of the desired instantaneous SINR. When pre-coded multiuser MIMO is used, [13,14] derive analytical models for three different precoder types and for each case derives the received signal. The later now consists of a linear useful signal part and several distortion terms arising from multiuser interference and nonlinear distortion. The conclusion of this paper is:

Observation 5:   The choice of a PA behavioural model validated by measurements is needed. Ref [10,11] provide a good framework to model these non-linearities via polynomial models.

Observation 6: The differences in the linear responses of the parallel PAs have a large impact on the system performance. The nonlinear terms do not necessarily adopt the digital predistortion as easily and efficiently. Therefore, the impact of nonlinear power amplifiers is nontrivial both in band and out of band. However, to do this we would need a model for the differences in the PA models in an AAS.

Proposal 7:  Power Amplifier non linearities should be carefully modelled when considering adjacent channel impacts when MIMO arrays are used.  The choice of a PA model should be agreed (say via polynomial models), then it may be validated by measurements and also agree on a model for the differences in the parameters for the distributed PAs.

Proposal 8: We may assume for the sake of simplicity there are no differences in the PA model parameters in a distributed PA AAS.

Adjacent band impacts are however not the main focus of [10], but they are considered in [8,10,13]. These papers give the spatial distribution of out of band radiation in the presence of non-linear amplifiers that are modelled by orthogonal polynomials- a special case of the more general Volterra series as described in [8]. Refs [8,13] derive an expression for array ACLR when linear precoding is used with MIMO arrays and a spatial distribution of the ACLR is derived for 1, 2 and 10 users respectively. In particular, [13] develops a framework for rigorous analysis of the spatial characteristics of nonlinear distortion from arrays both in band and out of band. The framework is validated via measurements on a GaN class AB amplifier [15]. It is shown that in the immediate adjacent bands, the third order distortion terms are significant. For a two user MU MIMO case it is shown that the desired signal is beamformed in two directions but the adjacent band distortion signal (arising from PA non linearities) is not beamformed in the same direction as the desired signal. Consideration of this will be important in evaluating the impact of adjacent band victims due to PA non linearities in the operating band.

Observation 7: PA non linearities may result in the array beamforming in different spatial directions than the desired signal. The impact of Power Amplifier non linearities when MIMO arrays are used vary with single user and multiuser systems. In multiuser beamforming case the distortion resulting from non-linearities is beamformed into distinct spatial directions that are different from the desired signal direction.

Band Pass filters:

The filters modelled in some references [1, 4 ,9] use a root raised cosine filter with an excess bandwidth of 0.22.  Also, the adjacent channel has the same bandwidth as the operating bandwidth. This is clearly not the case, for all the bands under consideration the operating bandwidths are much larger than the carrier bandwidth. The adjacent channel bandwidth may be say 100 MHz. However, the framework in [15] can be used to change the bandwidth of the BPF to reflect the operating bandwidth. However, we must specify the bandwidth of the BPF and a model for the BPF.

Observation 8: It is important to specify a bandwidth and a model for the BPF.

Proposal 9: A model for the BPF needs to be agreed

SU and MU MIMO

The distortion impacts need to be analysed with assumptions regarding MU MIMO. Ref [13] shows that if K is the number of served users and L is the number of significant channel taps (given by the channel model), the distortions are beamformed in many directions and these scale with O(K3L2) and some of these directions will be out of band. Therefore, it is important to consider SU and MU MIMO and precoder type.

The paper also observes that the array ACLR and antenna ACLR are different quantities. It derives a ccdf of array ACLR for 1, 2 and 10 users respectively. When the number of users increases, the array ACLR becomes almost isotropic as the channel is now beamforming in many directions.

Observation 9:  Array ACLR and antenna ACLR are different quantities.  As given in observation 1, there is a need to determine if the computation of array ACLR as given in the specifications is still accurate.  Furthermore, when the number of users increases, the array ACLR becomes almost isotropic as the channel is now beamforming in many directions.  Therefore, out of band distortions are also influenced by MU MIMO; this needs the numbers of simultaneous users.

Proposal 10: MU-MIMO precoder type and numbers of simultaneous users need to be agreed and could impact out of band distortions to fall in un-intended directions.

In summary, to address the question posed by WP 5D, attention to the questions below is needed.

Array performance in adjacent bands is governed by:

- Array performance at different frequencies arising due to adjacent bands.

The discussion in this document shows that the array continues to perform in adjacent bands and does not drop down to a single element or that of a sub array.

- Modelling of the impact of PA non linearities

The discussion in this document shows that a model of PA non linearities via a memory less polynomial model may be simple to use. It may also be assumed that in a distributed PA systems used in an AAS, all PAs have the same model parameters.

- Modelling of band pass filters

It could be assumed that band pass filters are raised cosine but excess bandwidth needs to be agreed

- SU and MU MIMO

The discussion in this document shows that Adjacent channel distortions are also space sensitive and also dependent if MU MIMO features are used.

# Summary of MIMO models, PA nonlinearity impacts and ACLR

We now make the following proposals:

Proposal 1: The text proposals in this document be included in TR 38 922, section 7

Proposal 2:ZF requires the computation of a pseudo inverse of the concatenation of channel matrices. It is important that the pseudo inverse is based on the correct formulation of the channel matrices as otherwise  the results will be misleading  The array response vmn  ( in the 3GPP AAS model) can be made to provide maxima at given angular directions by an appropriate choice of  the angles in wmn which can be viewed as additional phase terms in the array response.  It should be noted that pseudo inverse based on concatenated channels in eqn (1) that are in turn based on  wmn alone is flawed and will not provide an exact impact of ZF on the complete array response. : We could advise WP 5D on how to calculate the BF weights with ZF with correct formulation of the pseudo inverse and that the differences between the current approach ie using ITU R M 2101 and ZF cannot be ignored.

This text could be sent in an LS to WP 5D. Alternatively the RAN4 experts could agree on common simulation assumptions and resubmit the simulations to the RAN4 113 meeting so that a unified answer may be provided to WP

Proposal 3: There is a need to define array correlation factor so that it may be computed. Equation (1) may be used.

Proposal 4: Define the constituent components of P oob (θ,φ) and their dependence on  f

Proposal 5: There is a need to investigate array ACLR and see that its computation method in the standards is still the correct way.

Proposal 6: An array will not drop down to a single element in adjacent bands. Nor its performance will drop down to a sub array.

Proposal 7:  Power Amplifier non linearities should be carefully modelled when considering adjacent channel impacts when MIMO arrays are used.  The choice of a PA model should be agreed (say via polynomial models), then it may be validated by measurements and also agree on a model for the differences in the parameters for the distributed PAs.

Proposal 8: We may assume for the sake of simplicity there are no differences in the PA model parameters in a distributed PA AAS.

Proposal 9: A model for the BPF needs to be agreed

Proposal 10: MU-MIMO precoder type and numbers of simultaneous users need to be agreed and could impact out of band distortions to fall in un-intended directions.

3.0 Text Proposals for Beamforming Models

### 7.1.4	Beamforming equations

An alternative to a single-user MIMO (a.k.a. point-to-point MIMO) system is MU-MIMO in which an antenna array at the mobile base station simultaneously serves a multiplicity of autonomous user equipments (UEs). These UEs can be single-antenna devices, in which case the multiplexing throughput gains of the MU-MIMO system are shared among the different UEs. A MU-MIMO system is more tolerant of the propagation environments than a single-user MIMO system. For instance, under line-of-sight dominant propagation conditions (implying strong levels of correlation across user channels), the multiplexing gains can disappear for a single-user MIMO system but are retained in the MU-MIMO case provided that the angular separation of the user equipments exceeds the Rayleigh resolution of the array.  The co scheduled UEs should lie in uncorrelated or weakly correlated positions. Considering the propagation conditions, , in dominant non-line-of-sight conditions, the multiplexing gains are naturally also retained for the MU-MIMO system. Keeping this in mind, it is worth noting that irrespective of the propagation conditions, the conclusions from standard MU-MIMO techniques like ZF and MMSE to serve multiple user equipments remain consistent. To this end, where necessary, we will assume the existence of a given propagation environment coupled with the appropriate model for generating the propagation channel impulse responses, without specifying or recommending a particular propagation model.

When the mobile base station employs multiuser spatial beamforming techniques like ZF, the antenna array produces spatial nulls in the direction of undesired users. Hence, assuming rank-1 transmission (in 3GPP terminology), if there are K users, then the array forms K analog beams towards the K users where retrieval of the single data streams will occur. Each of the K beams is given by the composite array radiation pattern, as given in Table 7.1.3-1. Yet the separation of users via analog beamforming (BF) alone is not adequate to sufficiently separate the K users signals. To do this, it is preferable to have the users adequately separated say in the azimuth. Spatial beamforming techniques are used in combination with analog beamforming. This will result in nulls in the undesired direction(s).

Consider the simple example below:

Suppose there are say three users communicating with a base station. The three users can be assumed to be located at \phi_{1} = -10 degrees, \phi_{2} = 0 degrees and \phi_{3} = +10 degrees in the azimuthal plane. One can also assume that the base station array (e.g., uniform planar configuration say of size (32x8) is able to steer three independent beams to the three user equipments via the composite radiation patterns generated via the array factor in Table 7.1.3-1 above, with the azimuth main lobe of each beam pointing towards the azimuth angles of -10, 0, and+10 degrees respectively, observed at a fixed elevation angle cut of, say, 90 degrees. Furthermore, one can assume that the base station can implement a multiuser beamforming technique which allows the base station to form nulls in certain undesired directions relative to a desired direction. Then, when serving the user at \phi_{2} = 0 degrees, the base station will form nulls in the direction of the other users (signifying the undesired directions), at \phi_{1}=-10 degrees and \phi_{3} = +10 degrees respectively.

When extrapolating this logic to the case when the base station is serving a higher number of users, e.g., eight users, located in eight different azimuth directions, allowing the base station to generate seven nulls in the direction of other users relative to the desired user in the azimuthal plane.

#### 7.1.4.1	ZF and MMSE-based beamforming for Rank-1 MU-MIMO Transmission

When spatial beamforming is used in conjunction with analogue beamforming, the beamforming weights may be estimated as below:

7.1.4.1.1	ZF based beamforming

Here, we consider ZF-based beamforming for two users, denoted by UE 1 and UE 2 served by a single base station (BS) within the same time-frequency resource. Large numbers of antennas enable the focussing and steering of energy in desired directions [3]. The BS is assumed to have MN antenna elements in total configured in a uniform planar array in the x-z plane in the standard three-dimensional (3D) local coordinate system. For the sake of simplicity, perfect channel estimation at both the BS and UEs are assumed. Both UEs are assumed to have a single antenna. UE 1 and UE 2 is assumed to be located at two discrete azimuth angles, ϕe-scan1 and ϕe-scan2, respectively, with say the same elevation angle, θ1=θ2. Considering this, the composite antenna array radiation pattern towards UE 1 and UE 2 can be obtained by following the mathematical expressions given in the Table 7.1.3-1 above, where the azimuth angles, ϕe-scan1 and ϕe-scan2, are used.  It is noted that the array factor in Table 7.1.3-1 is inspired from [1]. We derive  two RF matrices corresponding to UE 1 and 2 as FRF1 and FRF2.respectively. The size of the matrices FRF1 and FRF2is equal to MN x NtRF. where NtRF is the  number of  RF chains  at the base station. If every antenna has a transmitter then the size of the beamforming matrices FRF1 and FRF2 is MN x MN respectively.

Even though the UEs may be lying say in one dimension say the azimuth only their impulse response has both an azimuth and elevation component see the impulse response channel matrix in [2].

Note that the exact structure of h1and h2 depends on the radio propagation conditions at the time of transmission from the BS to both UEs. However, this notation is agnostic to the type of conditions and propagation mechanisms present in the channel responses and hence is preferred for use in this document.

Concatenating the equivalent channel responses into a total equivalent channel matrix of appropriate size by stacking the two equivalent user channels, one can write:

H= h1T   h2T T					(1)

Where    h1  and    h2  are the two equivalent channels and are in turn equal  h1 FRF1 and h2 FRF2  respectively.

Now the ZF-based beamforming to null the inter-user interference can be used on H. To this end, the ZF beamforming matrix, W, can be calculated as:

W=HHHHH-1.                                                              (2)

This will ensure that if the BS is transmitting the desired signal in the direction of UE 1, i.e., towards (ϕe-scan1,θ1), a null in the radiation pattern of the BS can be formed towards the unintended direction of UE 2, i.e., towards ϕe-scan2,θ2. We note that the ZF beamformer should be normalized to satisfy a total power constraint. There are two different ways to normalize the ZF beamformer, namely via a vector normalization or via a matrix normalization. Using the example of a vector normalization, the per-user digital beamforming weight vector specific to UE 1 and UE 2 can be written as w1and w2 forming the two columns of W. Normalizing these using the vector normalization method yields

w1normalized=w1FRF1 w1,                                                             (3)

and

w2normalized=w2FRF2 w2.                                                             (4)

This implies that for UE 1, multiplying the h1w1normalized=1,  while  h1w2normalized=0 and vice-versa for UE 2. Therefore, a null in the azimuth direction can be formed towards UE 2 while serving UE 1.

The array response vmn  ( in the 3GPP AAS model) can be made to provide maxima at given angular directions by an appropriate choice of  the angles in wmn which can be viewed as additional phase terms in the array response.  It should be noted that pseudo inverse based on concatenated channels in eqn (1) that are in turn based on  wmn alone is flawed and will not provide an exact impact of ZF on the complete array response.

We provide below example simulation results for using ZF with two UEs, both are in the azimuth at 0 and 10 degrees respectively and are at the same elevation with a 5 degrees down tilt (angle from the Z axis is 95 degrees). The antenna and other parameters are:

IMT deployment-related parameters

| Parameter | Value |
| --- | --- |
| Environment | Macro Urban |
| Carrier frequency | 6 GHz |
| Sectorization | 1 sector |
| Frequency reuse | 1 |

IMT base station beamforming antenna characteristics for IMT

| Parameter | Macro Urban |
| --- | --- |
| Antenna pattern | Refer to Section 5 of Recommendation ITU-R M.2101 |
| Element gain (incl. Ohmic loss) (dBi) | 5.5 |
| Horizontal/vertical 3 dB beamwidth of single element (degree) | 90º for H 90º for V |
| Horizontal/vertical front to back ratio (dB) | 30 for both H/V |
| Antenna polarization | Linear ±45º |
| Antenna array configuration (Row × Column) | 16 × 8 |
| Horizontal/Vertical radiating element spacing | 0.5 of wavelength for H 0.5 of wavelength for V |
| Array Ohmic loss (dB) | 2 |
| Base station maximum coverage angle in the horizontal plane (degrees) | ±60 |
| Base station vertical coverage range (degrees) | 90-120 |
| Mechanical down-tilt (degrees) | 10 |

The cdf of the array gain of the elevation pattern above the horizon is shown below.

We make the following observations:

The median gain difference is about 3 dB.

When more UEs are added to the ZF set, the placement of nulls in the azimuth pattern will have a consequential impact on the elevation pattern and will further increase this difference.

The example is for two UEs with a given azimuth placement. The median gain difference will also be sensitive to the actual azimuth angles at which the UEs are placed. The point of this exercise is just to demonstrate the impact of ZF beamforming using the correct formulation of pseudo inverse to accurately assess the aggregate antenna array gain above the horizon. The conclusions presented here can be replicated for a larger number of users in a given sector with the same methodology.

##### 7.1.4.1.2	MMSE based beamforming

In the case of MMSE beamforming, the MMSE beamforming matrix, W, is given by

W=HHHHH+βI-1,                                                      (5)

where β>0 is the regularisation constant/factor and I is the identity matrix of the same dimension as HHH. By maximizing the SINR at the UEs, the optimal regularisation constant/factor was derived in [6] as β=K/ρ, where ρ is the operating signal-to-noise ratio (SNR) typically computed by taking the ratio of the EIRP with the noise variance (power spectral density) of the UE. Note that optimal regulariser was only obtained for the case of equal conducted transmit power being split across the K user equipments and only applies to rank-1 transmission.

It is noteworthy that when ρ →∞ (holding the value in K as constant), reflecting high SNR scenarios, the MMSE beamforming matrix in (5) reduces to the ZF beamforming matrix in (2) as β→0.

The normalization of the beamforming vectors of the MMSE approach is identical to the ZF and hence the expressions in (3) and (4) apply with w1 and w2 now derived from the MMSE expression in (5).

# Text Proposals on PA nonlinearity impacts and ACLR

## 7.2	Modelling PA non-linearity impacts and ACLR

### 7.2.1	Overview

A question posed by WP 5D is:

To establish a model for array correlation factor (r) roll-off to describe how the array factor collapses moving away from the centre frequency. If the beamforming performance degrades towards a single element radiation pattern.

This section provides IMT base station and IMT user equipment techncial parameters for analyzing adjacent band compatibility of terrestrial IMT systems with other co-primary services in the ITU Radio Regulations.

### 7.2.2	ITU Radio Regulations on adjacent terrestrial IMT systems with other co-primary services

There are adjacent services in the bands being considered for IMT 2030.

For example: for the case of 4400-4800 MHz, the adjacent band services are:

On the lower edge below 4400 MHz:

- Aeronautical radionavigation;

- Aeronautical mobile;

- Additional possible allocation of EESS (passive) as per Agenda Item 1.19 of WRC-27;

On the upper edge above 4800 MHz:

- Fixed service;

- Radio Astronomy;

- Radio navigation satellite;

For the case of 7125-8400 MHz, the adjacent band services are:

On the lower edge below 7125 MHz:

- Fixed

- Fixed Service Satellite (Earth-to-space)

On the upper edge above 8400 MHz:

- Space Research Service (space-to-Earth)

- Additional possible allocations of EESS (passive) as per Agenda Item 1.19 of WRC-27

- Earth Exploration Satellite

- Radiolocation

- Space Research (active).

For the case of 14.8-15.35 GHz, the adjacent band services are:

On the lower edge below 14.8 GHz:

- Fixed Service;

- Fixed Satellite Service (Earth-to-space)

- Radionavigation

On the upper edge above 15.35 GHz:

- Space research (passive)

- EESS (passive)

- Radio Astronomy

In particular, the frequency range 15.35 – 15.4 GHz is marked as a footnote 5.340 band, which is stating “All Emissions are Prohibited” with the exception of those provided by RR 5.511.

It is therefore important that out of band emissions are correctly modelled so that appropriate guidance is provided for the above question to WP 5D.

### 7.2.3	Adjacent band compatibility analysis from BS perspective

Out of band emissions are well understood but often for a single transmitter.

It is necessary to distinguish between a single element ACLR, and an array ACLR. In a non AAS system, where the radiation pattern is “constant” over a sector, the single element ACLR makes sense, since the received power ratio at any point is the same as the transmitted. However, with AAS, the desired signal experiences also an array/beamforming gain that might be different from the array/beamforming gain of the interfering signal received in the adjacent band; this later observation is also dependent if SU or MU MIMO is used (ie the array may send distortions in unintended directions).

When an array in AAS base stations is used, the 3GPP specifications provide a method to compute the array ACLR (albeit in a TRP mode) [16]:

“The ACLR (CACLR) absolute basic limits in table 6.6.3.2-2 + X, 6.6.3.2-2a + X (where X = 9 dB) or the ACLR (CACLR) basic limit in table 6.6.3.2-1, 6.6.3.2-2a or 6.6.3.2-3, whichever is less stringent, shall apply.”

However, the above method is array pattern agnostic and does not account for beamforming variations due to MU MIMO.

Recent published results, discussed/summarized in this report, show that that array ACLR is spatially sensitive and is also dependent if SU or MU MIMO functionality is utilized. There is thus a need to accurately model the ACLR when AAS is used.

As part of additional information requested by ITU-R WP 5D establish a model for array correlation factor (r) roll-off to describe how the array factor collapses moving away from the centre frequency. If the beamforming performance degrades towards a single element radiation pattern, then the frequency when the continuity of array features is regarded as degraded, will also need to be determined.

To address the SI, the questions posed may be devolved into topics below:

- Establish a model for array correlation factor-ie the correlation of the in-band signal with the out of band signal.

There is no definition of an array correlation factor. Is it the correlation of the in-band signal with out of band signal. But the in-band signal, Pinband (θ,φ),   is spatially sensitive and as discussed below the out of band signal , P oob (θ,φ),  may also be spatially sensitive.  Then the array correlation  is also spatially sensitive may be defined as:

r (θ, φ) = E (Pinband (θ,φ) PT oob (θ,φ) ) ………………………….(6)

- How does the array factor collapse when moving away from the centre frequency?

This also means that r (θ, φ) defined above is a function of frequency separation from the band centre. In this case best to denote this as:

r f(θ, φ)………………………………………………………………..(7)

Where f is the frequency separation from band centre

- If the beamforming performance degrades towards a single element radiation pattern, then the frequency when the continuity of array features is regarded as degraded, will also need to be determined.

Therefore we would need to characterise and investigate the constituent components of P oob (θ,φ) as a function of f

The above topics are interrelated but need to be separately discussed as well. The discussion below addresses topics 2 and 3 respectively.

#### 7.2.3.1	Non-AAS BS perspective

#### 7.2.3.2	AAS BS perspective

Let us denote the three bands under study as:

- Band 1: 4400-4800 MHz

- Band 2: 7125-8400 MHz

- Band 3: 14.8-15.35 GHz.

Array performance in adjacent bands is governed by:

- Array performance at different frequencies arising due to adjacent bands.

- Modelling of the impact of PA non linearities

- Modelling of band pass filters

- Putting all of the above together to obtain an end-to-end model

Additionally, one must also consider if the array is supporting a single user or multi-user systems.

##### 7.2.3.2.1	Array performance at adjacent band frequencies

Assume that the lower and upper adjacent bands have a bandwidth of 100 MHz each respectively.  Arrays in a band are designed at the centre frequency of a given band ie vertical and horizontal spacing is determined by the centre frequency of the band which for example in band 1 is 4600 MHz shown in Table 7.2.3.2.1-1 below. Likewise for other bands. The lower and upper adjacent bands can be simulated via a proxy array with different spacings e.g for band 1, the lower adjacent band is 4300-4400 MHz with centre frequency 4350 MHz. Therefore, for band 1, changing the vertical spacing to 0.74 lambda and horizontal spacing to 0.53 lambda will effectively simulate the lower adjacent band. Likewise for the upper adjacent band (0.66, 0.474) will simulate the upper adjacent band.

Table 7.2.3.2.1-1: Array performance in adjacent bands

| Band | Centre Frequency | 0.7 Lambda, 0.5 Lambda (cm) | Centre freq of Lower adj band | Centre freq of Upper adj band | 0.7 Lambda, 0.5 Lambda for lower adj band | 0.7 Lambda, 0.5 Lambda for upper adj band |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 4600 MHz | 4.565/3.26 | 4350 MHz | 4850 MHz | 4.827/3.45 (0.74, 0.53) | 4.33/3.09 (0.66, 0.474) |
| 2 | 7762.5 MHz | 2.7/1.93 | 7075 MHz | 8450 MHz | 2.96/2.118 (0.767, 0.55) | 2.48/1.773 (0.64, 0.46) |
| 3 | 15.075 GHz | 1.39/0.99 | 14.75 GHz | 15.40 GHz | 1.42/1.012 (0.715, 0.511) | 1.36/0.97 (0.68, 0.49) |

The following parameter/metrics are impacted [1] when the inter element spacing is changed:

- Nulls

- Maxima

- Half power points

- Minor lobe maxima

- First null beamwidth

- Half power beamwidth

- First side lobe beamwidth

Exact expressions for the above are given below from [1] for a ULA along the z axis and for omni directional antennas. Considering the exact formulas below in Table 7.2.3.2.1-2 and Table 7.2.3.2.1-3, the changes due to inter element spacing in the proxy array will result in some changes to the values of the above metrics. See Figure 7.2.3.2.1-1, Figure 7.2.3.2.1-2 and Figure 7.2.3.2.1-3 for an eight element ULA for 0.5 lambda, 0.7 lambda and 0.8 lambda inter element spacing respectively. However these frequency/wavelength adjustments are no different to what is present today in systems that have a wide operating bandwidth and antennas are designed at the centre frequency of the band). Note that the results below are not for an array of sub arrays [9] and 3GPP antenna element patterns, but the trends will be similar for the array of sub array case as well.

Table 7.2.3.2.1-2: Broadside array antenna pattern equations

| Nulls, Maxima, Half-Power points and Minor Lobe maxima for Broadside Arrays |  |
| --- | --- |
| Nulls | θn = cos-1 (±nN λd), n = 1,2,3…., n≠N, 2N,3N.. |
| Maxima | θm = cos-1 (±m λd), m = 0,1,2,3…., |
| Half Power Points | θh = cos-1 (±1.391λπ Nd), πdλ ≪ 1 |
| Minor Lobe Maxima | θs= cos-1 (±λ2d 2s+1N), s = 1,2,3, πλd ≪ 1 |

Table 7.2.3.2.1-3: Broadside array beamwidth equations

| Beamwidths for Uniform Amplitude Broadside Arrays |  |
| --- | --- |
| First Null beamwidth (FNBW) | θn=π2-cos-1λNd |
| Half Power beamwidth (HPBW) | θh≅2π2-cos-11.391λπNd πdλ ≪ 1 |
| First sidelobe beamwidth (FSLBW) | θs≅2π2-cos-13λ2Nd πdλ ≪ 1 |

Figure 7.2.3.2.1-1:  Eight element ULA response with 0.5 lambda spacing

Figure 7.2.3.2.1-2:  Eight element ULA response with 0.7 lambda spacing

Figure 7.2.3.2.1-3:  Eight element ULA response with 0.8 lambda spacing

It is also to be noted that in case of band 2 where the band is very wide (operating bandwidth 1275 MHz) proxy antenna inter element spacings for a carrier at the lower end of the band will almost have a similar impact as the lower adjacent band. Likewise for the upper end of the band.

The above inferences will also remain valid for the second adjacent band.

The above observations will remain valid even if the array is made of sub arrays and the notion that an array may drop down to a sub array in adjacent bands is not valid.

##### 7.2.3.2.2	Modelling of PA non-linearities

Non linearities in hardware in the RF path cause signal distortions- both in band and out of band-(adjacent bands). The nonlinear components in the RF path consist of power amplifiers that are the main cause of nonlinear distortions but other components such as D/A converters, mixers, beam forming circuitry may also contribute to the nonlinear distortions. When antenna arrays are deployed, multiple users may be served by beam forming either in the analog domain or both in the analog and digital domain.  Digital beamforming is commonly employed in today’s systems- such as zero forcing, MMSE [5]. This results in the placing of nulls in the path of all users except the desired user, but this operation may also cause out of band distortions to fall in unintended spatial directions.

The modelling of PA non linearities has received a lot of interest in the published literature [see 8,10-14 and references therein]. PA behavioural models can be classified as:

- Volterra series based,

- Artificial neural networks,

- Table look up methods,

- Models with linear and nonlinear memory, with and without memory etc.

Ref [8] gives a very good comparison of PA behavioural models in terms of metrics like identification complexity, adaptation complexity, running complexity etc. Measurements on some PAs and the accuracy of the models are presented in [12]. Some commonly used models use polynomials or memory polynomials [10, 14], here PA units are modelled via 9th order polynomials whose coefficients are obtained from RF measurements. These papers conclude that the received signal with massive MIMO consists of a desired term with linear gain of the array and also a nonlinear distortion term. Additionally,  if the linear response coefficients of the different parallel PAs (in the massive MIMO unit) are different (and they do have small differences), then even the linear signal terms are not fully coherently combining at the receiver. In case of the nonlinear distortion terms, this effect is even more evident as here the phase characteristics of the constituent terms representing the component PAs are different. These two effects manifest in the decrease of the desired instantaneous SINR. When pre-coded multiuser MIMO is used, [13,14] derive analytical models for three different precoder types and for each case derives the received signal. The later now consists of a linear useful signal part and several distortion terms arising from multiuser interference and nonlinear distortion. The conclusion of this paper is:

The choice of a PA behavioural model validated by measurements is needed. Ref [10,11] provide a good framework to model these non-linearities via polynomial models.

The differences in the linear responses of the parallel PAs have a large impact on the system performance. The nonlinear terms do not necessarily adopt the digital predistortion as easily and efficiently. Therefore, the impact of nonlinear power amplifiers is nontrivial both in band and out of band. However, to do this we would need a model for the differences in the PA models in an AAS.

Power Amplifier non linearities should be carefully modelled when considering adjacent channel impacts when MIMO arrays are used.  The choice of a PA model should be agreed (say via polynomial models), then it may be validated by measurements and also agree on a model for the differences in the parameters for the distributed PAs.

We may assume for the sake of simplicity there are no differences in the PA model parameters in a distributed PA AAS.

Adjacent band impacts is however not the main focus of [10], but they are considered in [8,10,13]. These papers give the spatial distribution of out of band radiation in the presence of non-linear amplifiers that are modelled by orthogonal polynomials- a special case of the more general Volterra series as described in [8]. Refs [8,13] derive an expression for array ACLR when linear precoding is used with MIMO arrays and a spatial distribution of the ACLR is derived for 1, 2 and 10 users respectively. In particular, [13] develops a framework for rigorous analysis of the spatial characteristics of nonlinear distortion from arrays both in band and out of band. The framework is validated via measurements on a GaN class AB amplifier [15]. It is shown that in the immediate adjacent bands, the third order distortion terms are significant. For a two user MU MIMO case it is shown that the desired signal is beamformed in two directions but the adjacent band distortion signal (arising from PA non linearities) is not beamformed in the same direction as the desired signal. Consideration of this will be important in evaluating the impact of adjacent band victims due to PA non linearities in the operating band.

##### 7.2.3.2.3	Modelling of band pass filters

The filters modelled in some references [1, 4 ,9] use a root raised cosine filter with an excess bandwidth of 0.22.  Also, the adjacent channel has the same bandwidth as the operating bandwidth. This is clearly not the case, for all the bands under consideration the operating bandwidths are much larger than the carrier bandwidth. The adjacent channel bandwidth may be say 100 MHz. However, the framework in [15] can be used to change the bandwidth of the BPF to reflect the operating bandwidth. However, we must specify the bandwidth of the BPF and a model for the BPF.

##### 7.2.3.2.4	Modelling of SU and MU MIMO

The distortion impacts need to be analysed with assumptions regarding MU MIMO. Ref [13] shows that if K is the number of served users and L is the number of significant channel taps (given by the channel model), the distortions are beamformed in many directions and these scale with O(K3L2) and some of these directions will be out of band. Therefore, it is important to consider SU and MU MIMO and precoder type.

The paper also observes that the array ACLR and antenna ACLR are different quantities. It derives a ccdf of array ACLR for 1, 2 and 10 users respectively. When the number of users increases, the array ACLR becomes almost isotropic as the channel is now beamforming in many directions.

In summary, to address the question posed by WP 5D, attention to the questions below is needed.

Array performance in adjacent bands is governed by:

- Array performance at different frequencies arising due to adjacent bands.

The discussion in this document shows that the array continues to perform in adjacent bands and does not drop down to a single element or that of a sub array.

- Modelling of the impact of PA non linearities

This discussion in this document shows that a model of PA non linearities via a memory less polynomial model may be simple to use. It may also be assumed that in a distributed PA systems used in an AAS, all PAs have the same model parameters.

- Modelling of band pass filters

A model for the BPF is required. A root raised cosine filter with an excess bandwidth say of 0.22 may be used.

- Analysing the impact of MU MIMO beamforming

The ACLR may be spatially sensitive, and this sensitivity could vary with MU MIMO beamforming

- Putting all of the above together to obtain an end-to-end model for characterising the array ACLR and adjacent band impacts.

### 7.2.4	Adjacent band compatibility analysis from UE perspective

(TBC)

8.0 References

Note: The following references should be consolidated with all other References in the TR 38 922 and placed in section 2

[1] C A Balanis, “Antenna Theory Analysis and Design”, John Wiley and sons

[2] TR 38901, Study on channel model for frequencies from 0.5 to 100 GHz (Release 18)

[3] B. Sadhu, X. Gu and A. Valdes-Garcia, "The More (Antennas), the Merrier: A Survey of Silicon-Based mm-Wave Phased Arrays Using Multi-IC Scaling," in IEEE Microwave Magazine, vol. 20, no. 12, pp. 32-50, Dec. 2019.

[4] TR 38 921 “Study on International Mobile Telecommunications (IMT) parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5 GHz” March 2024.

[5] H. Jin et al., "Massive MIMO Evolution Toward 3GPP Release 18," in IEEE Journal on Selected Areas in Communications, vol. 41, no. 6, pp. 1635-1654, June 2023

[6] C. B. Peel, B. M. Hochwald, and A. L. Swindlehurst, “A vector-perturbation technique for near-capacity multiantenna multiuser communications - Part I: Channel inversion and regularization,” IEEE Transactions on Communications, vol. 53, no. 1, pp. 195-202, Jan. 2005.

[7] TR 38 922, Study on International Mobile Telecommunications (IMT) parameters for 4400 - 4800 MHz, 7125 - 8400 MHz and 14800 - 15350 MHz

[8] Christopher Mollen, Ulf Gustavsson, Thomas Eriksson, Erik G Larsson, “Out of Band radiation Measure for MIMO arrays with beamformed transmission “, Proc. IEEE Int. Conf. Commun., May 2016.

[9] R4 LS to ITU-R and CEPT on extension of IMT array antenna model to support sub-array structures, TSG-RAN Working Group 4 (Radio) meeting #99-E 19th – 27th May 2021

[10] Erik Larsson, Liesbet Van der Perre, “Out of band Radiation for Antenna arrays Clarified”, arXiv : 1802.02745v1, Feb 2018

[11] E. Sienkiewicz, N. McGowan, B. Göransson, T. Chapman and T. Elfström, "Spatially dependent ACLR modelling," 2014 IEEE Conference on Antenna Measurements & Applications (CAMA), Antibes Juan-les-Pins, France, 2014, pp. 1-4,

[12] A. S. Tehrani, H. Cao, S. Afsardoost, T. Eriksson, M. Isaksson and C. Fager, "A Comparative Analysis of the Complexity/Accuracy Trade off in Power Amplifier Behavioural Models," in IEEE Transactions on Microwave Theory and Techniques, vol. 58, no. 6, pp. 1510-1520, June 2010

[13] C. Mollén, U. Gustavsson, T. Eriksson and E. G. Larsson, "Spatial Characteristics of Distortion Radiated From Antenna Arrays With Transceiver Nonlinearities," in IEEE Transactions on Wireless Communications, vol. 17, no. 10, pp. 6663-6679, Oct. 2018.

[14] Y. Zou et al., "Impact of Power Amplifier Nonlinearities in Multi-User Massive MIMO Downlink,” 2015 IEEE Globecom Workshops, San Diego, CA, USA, 2015, pp. 1-7.

[15] P Nandin et al, “Web Lab: A Web based set up for PA digital distortion and Characterisation”, IEEE Microwave Magazine, vol 16, no1, p 138- 140, Feb 2015

[16] TS 38 104, NR; Base Station (BS) radio transmission and reception.

It should be note that the array response is both in the azimuth and elevation.

We assume a broadside array. An elevation angle of 90 degrees is when users are in the elevation boresight.

Whilst antenna numbers can increase due to advancements in semiconductor technology [3], the numbers of users to be nulled must be less than or equal to the number of RF chains

In practice, this is not the case, as Sounding Reference Signals (SRS) are used in time-division duplex (TDD) systems, where the UEs would send an uplink SRS signal based on which the BS will estimate the uplink channel and using channel reciprocity, will estimate the downlink channel. In frequency-division duplex (FDD) systems, Channel State Information-Reference Signal (CSI-RS) is transmitted from the BS periodically and the UEs will estimate the channel and feedback the channel response via a codebook to the BS [5].

Mathematical notation: We note that h1 denotes a vector, while H denotes a matrix. Furthermore, h is denoted as a real scalar value, where as ⋅Tand ⋅H are used to denote a transpose and a Hermitian transpose operation. Finally, H-1 is used to denote the matrix inverse of a square matrix (same number of rows and columns), H.

The beamforming matrix can also be derived without considering FRF1 and FRF2 but this will amount to ignoring the presence of RF beamfoming.

Note the notation w for the columns of W is different from the wm in Table 7.1.3-1 and care must be exercised.