3GPP TSG-RAN WG4 Meeting # 112bis	R4-2414928

Hefei, China, Oct. 14 – Oct. 18, 2024

Agenda Item: 6.24.2.1

Source: MediaTek Inc.

Title: Discussion on measurement enhancements for LTM

Document for: Discussion

# Introduction

RAN#105 agreed on the revised WID “NR mobility enhancements Phase 4” [1]. According to WID, RAN4 is supposed to define requirements for the following measurement enhancements:

- Event triggered L1 report,

- CSI-RS L1 measurements,

- CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch.

In the next, we would like to provide our views on the above three parts one by one.

# Discussion

## 2.1 Event triggered L1 report

In legacy, capabilities for Support of Event Triggered L3 measurement Reporting had been specified in section 9.1.4. Similarly, for event triggered L1 measurement reporting, the number of events supported shall be defined too. Table 9.1.4.2-1 is copied here for information.

38.133 Table 9.1.4.2-1: Requirements for reporting criteria per measurement category

| Measurement category | Ecat | Note |
| --- | --- | --- |
| Intra-frequency Note 1,2,3,4,5 | 9 | Events for any one or a combination of intra-frequency SS-RSRP, SS-RSRQ, SS-SINR, CSI-RSRP, CSI-RSRQ, and CSI-SINR for NG-RAN intra-frequency cells |
| Inter-frequency Note 2,3,4,5 | 10 | Events for any one or a combination of inter-frequency SS-RSRP, SS-RSRQ, SS-SINR, CSI-RSRP, CSI-RSRQ, and CSI-SINR for NG-RAN inter-frequency cells |
| Inter-RAT (E-UTRA FDD, E-UTRA TDD) Note 2,4,5 | 10 | Only applicable for UE with this (inter-RAT) capability. These reporting criteria apply for any E-UTRA carrier frequencies other than the carrier frequency of the E-UTRA PSCell or E-UTRA SCell. |
| Inter-RAT (E-UTRA FDD, E-UTRA TDD) RSTD Note 2,4,5 | 1 | Inter-RAT RSTD measurement reporting for UE supporting OTDOA; 1 report capable of minimum 16 inter-RAT cell measurements. Only applicable for UE with this (inter-RAT RSTD via LPP [22]) capability. These reporting criteria apply for any E-UTRA carrier frequencies other than the carrier frequency of the E-UTRA PSCell or E-UTRA SCell. |
| Inter-RAT (E-UTRA FDD, E-UTRA TDD) RSRP and RSRQ measurements for E-CID Note 2,4,5 | 1 | Inter-RAT RSRP and RSRQ measurements for E-CID reported to E-SMLC via LPP [22]. One report capable of at least in total 10 inter-RAT RSRP and RSRQ measurements. Applicable to UE capable of reporting inter-RAT RSRP and RSRQ to E-SMLC via LPP. These reporting criteria apply for any E-UTRA carrier frequencies other than the carrier frequency of the E-UTRA PSCell or E-UTRA SCell. |
| Intra-frequency RSSI and channel occupancy measurements with CCA Note 1,2,3 | 1 | One report capable of one RSSI and one channel occupancy measurements over a channel [TS 37.213] with CCA. Applicable for UE capable of performing and reporting RSSI and channel occupancy on carrier frequencies under CCA. |
| Inter-frequency RSSI and channel occupancy measurements with CCA Note 2,3 | 1 | One report capable of one RSSI and one channel occupancy measurements over a channel [TS 37.213] with CCA. Applicable for UE capable of performing and reporting RSSI and channel occupancy on carrier frequencies under CCA. |
| Intra-frequency SSB-based measurements for NR E-CID Note 1,2,3,4,5 | 1 | Intra-frequency SS-RSRP and SS-RSRQ measurements for NR E-CID reported to LMF via LPP [34]. One report capable of at least in total 9 intra-frequency SS-RSRP and SS-RSRQ measurements. Applicable to UE capable of reporting at least one of SS-RSRP and SS-RSRQ to LMF via LPP. |
| Intra-frequency CSI-RS based measurements for NR E-CID Note 1,2,3,4,5 | 1 | Intra-frequency CSI-RSRP and CSI-RSRQ measurements for NR E-CID reported to LMF via LPP [22]. One report capable of at least in total 9 intra-frequency CSI-RSRP and/or CSI-RSRQ measurements. Applicable to UE capable of reporting any of CSI-RSRP and CSI-RSRQ to LMF via LPP, as indicated in nr-ECID-MeasSupported-r16. |
| Inter-frequency SSB-based measurements for NR E-CID Note 2,3,4,5 | 1 | Inter-frequency SS-RSRP and SS-RSRQ measurements for NR E-CID reported to LMF via LPP [34]. One report capable of at least in total 10 inter-frequency SS-RSRP and SS-RSRQ measurements. Applicable to UE capable of reporting at least one of SS-RSRP and SS-RSRQ to LMF via LPP. |
| Inter-frequency CSI-RS based measurements for NR E-CID Note 2,3,4,5 | 1 | Inter-frequency CSI-RSRP and CSI-RSRQ measurements for NR E-CID reported to LMF via LPP [22]. One report capable of at least in total 10 inter-frequency CSI-RSRP and CSI-RSRQ measurements. Applicable to UE capable of reporting any of CSI-RSRP and CSI-RSRQ to LMF via LPP, as indicated in nr-ECID-MeasSupported-r16. |
| DL RSTD Note 2,4,5 | 1 | DL RSTD measurement reporting; 1 report capable of multiple (within the UE PRS measurement capability, nr-DL-TDOA-MeasCapability, indicated via LPP [34]) DL RSTD measurements and if supported also multiple corresponding DL PRS-RSRP measurements configured for DL-TDOA. Only applicable for UE capable of reporting measurements for DL-TDOA to LMF via LPP [34]. |
| UE Rx-Tx Note 2,4,5 | 1 | UE Rx-Tx measurement reporting; 1 report capable of multiple (within the UE PRS measurement capability, nr-DL-PRS-MeasCapability, indicated via LPP [34] for multi-RTT) UE Rx-Tx measurements and if supported also multiple corresponding DL PRS-RSRP measurements configured for multi-RTT. Only applicable for UE capable of reporting measurements for multi-RTT to LMF via LPP [34]. |
| DL PRS-RSRP Note 2,4,5 | 1 | DL PRS-RSRP measurement reporting; 1 report capable of multiple (within the UE PRS measurement capability, nr-DL-PRS-MeasCapability, indicated via LPP [34] for AoD) DL PRS-RSRP measurements configured for DL-AoD. Only applicable for UE capable of reporting measurements for DL-AoD to LMF via LPP [34]. |
| DL PRS-RSRPP Note 2,4,5 | 1 | DL PRS-RSRPP measurement reporting; 1 report capable of multiple (within the UE PRS measurement capability, nr-DL-PRS-MeasCapability, indicated via LPP [34] for AoD) DL PRS-RSRPP measurements configured for DL-AoD. Only applicable for UE capable of reporting measurements for DL-AoD to LMF via LPP [34]. |
| SRS-RSRP Note 2,3,4,5 | 1 | SRS-RSRP measurement reporting for CLI; 1 report capable of up to 32 SRS resources measurements. Only applicable for UE supporting cli-SRS-RSRP-Meas-r16. |
| CLI-RSSI Note 2,3,4,5 | 1 | CLI-RSSI measurement reporting for CLI; 1 report capable of up to 64 CLi-RSSI resources measurements. Only applicable for UE supporting cli-RSSI-Meas-r16. |
| NOTE 1: When the UE is configured with PSCell and SCell carrier frequencies, Ecat for Intra-frequency is applied per corresponding NR serving frequency. NOTE 2: Applicable for UE configured with SA NR operation mode. NOTE 3: Applicable for UE configured with EN-DC operation mode. NOTE 4: Applicable for UE configured with NE-DC operation mode. NOTE 5: Applicable for UE configured with NR-DC operation mode. |  |  |

Regarding how to define the capability for event triggered L1 measurement reporting, we think at least the following aspects shall be discussed.

Whether to share the capability with Event Triggered L3 measurement Reporting

In our understanding, the goal of event triggered L1 measurement reporting is to replace L3 report and used to trigger LTM. With this understanding, for the frequency layers configured with event triggered L1 measurement reporting, it is not necessary to have event triggered L3 measurement reporting.

But on the other hand, NW may configure and adjust event triggered L1 measurement reporting based on L3 report. Then there will be both L3 event and L1 event for the same frequency layer. We think it is reasonable to introduce extra UE capability to support event triggered L1 measurement reporting.

Proposal 1: Define a separate capability for supported number of events triggered L1 measurement reporting.

Whether and how to differentiate intra-frequency and inter-frequency events

As shown in Table 9.1.4.2-1 in 38.133, for L3 measurement, the events for intra-frequency and inter-frequency are viewed as different events. It is straightforward for L3 measurement, as each report configuration is linked to a measObject which is for a single frequency layer (as shown in the copied L3 measurement and report configurations from 38.331).

L3 measurement and report configurations MeasIdToAddMod ::= SEQUENCE { measId MeasId, measObjectId MeasObjectId, reportConfigId ReportConfigId } MeasObjectToAddMod ::= SEQUENCE { measObjectId MeasObjectId, measObject CHOICE { measObjectNR MeasObjectNR, ..., measObjectEUTRA MeasObjectEUTRA, measObjectUTRA-FDD-r16 MeasObjectUTRA-FDD-r16, measObjectNR-SL-r16 MeasObjectNR-SL-r16, measObjectCLI-r16 MeasObjectCLI-r16, measObjectRxTxDiff-r17 MeasObjectRxTxDiff-r17, measObjectRelay-r17 SL-MeasObject-r16, measObjectNR-SL-r18 MeasObjectNR-SL-r18 } } MeasObjectNR ::= SEQUENCE { ssbFrequency ARFCN-ValueNR OPTIONAL, -- Cond SSBorAssociatedSSB ssbSubcarrierSpacing SubcarrierSpacing OPTIONAL, -- Cond SSBorAssociatedSSB smtc1 SSB-MTC OPTIONAL, -- Cond SSBorAssociatedSSB smtc2 SSB-MTC2 OPTIONAL, -- Cond IntraFreqConnected refFreqCSI-RS ARFCN-ValueNR OPTIONAL, -- Cond CSI-RS referenceSignalConfig ReferenceSignalConfig, absThreshSS-BlocksConsolidation ThresholdNR OPTIONAL, -- Need R absThreshCSI-RS-Consolidation ThresholdNR OPTIONAL, -- Need R …… }

But for L1 measurement and report, if we follow the configurations in R18 LTM. That is: each LTM-CSI-ReportConfig is linked to a LTM-CSI-ResourceConfigId. In one LTM-CSI-ResourceConfig, the SSB to measure can be from any LTM candidate cell (intra-f or inter-f).

So LTM-CSI-ReportConfig may be linked to multiple cells on different frequency layers. It seems not so straightforward on how to count the number of events configured for intra-frequency and inter-frequency L1 measurement.

R18 LTM L1 measurement and report configurations LTM-CSI-ReportConfig-r18 ::= SEQUENCE { ltm-CSI-ReportConfigId-r18 LTM-CSI-ReportConfigId-r18, ltm-ResourcesForChannelMeasurement-r18 LTM-CSI-ResourceConfigId-r18, ltm-ReportConfigType-r18 CHOICE { periodic-r18 SEQUENCE { reportSlotConfig-r18 CSI-ReportPeriodicityAndOffset, pucch-CSI-ResourceList-r18 SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource }, semiPersistentOnPUCCH-r18 SEQUENCE { reportSlotConfig-r18 CSI-ReportPeriodicityAndOffset, pucch-CSI-ResourceList-r18 SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource }, semiPersistentOnPUSCH-r18 SEQUENCE { reportSlotConfig-r18 CSI-ReportPeriodicityAndOffset, reportSlotOffsetList-r18 SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128), reportSlotOffsetListDCI-0-2-r18 SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128), reportSlotOffsetListDCI-0-1-r18 SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128), p0alpha P0-PUSCH-AlphaSetId }, aperiodic-r18 SEQUENCE { reportSlotOffsetList-r18 SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128), reportSlotOffsetListDCI-0-2-r18 SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128), reportSlotOffsetListDCI-0-1-r18 SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128) } }, ltm-ReportContent-r18 LTM-ReportContent-r18, ... } LTM-CSI-ResourceConfig-r18 ::= SEQUENCE { ltm-CSI-ResourceConfigId-r18 LTM-CSI-ResourceConfigId-r18, ltm-CSI-SSB-ResourceSet-r18 LTM-CSI-SSB-ResourceSet-r18, ... } LTM-CSI-SSB-ResourceSet-r18 ::= SEQUENCE { ltm-CSI-SSB-ResourceList-r18 SEQUENCE (SIZE (1..maxNrofLTM-CSI-SSB-ResourcesPerSet-r18)) OF SSB-Index, ltm-CandidateIdList-r18 SEQUENCE (SIZE (1..maxNrofLTM-CSI-SSB-ResourcesPerSet-r18)) OF LTM-CandidateId-r18, ... }

There may be two ways to solve the issue if we only consider SSB based L1 measurement. One is that if the LTM-CSI-ReportConfig is linked to multiple cells on different frequency layer, then the number of events should be counted no less than the number of frequency layers provided in this LTM-CSI-ReportConfig. Another way is to inform RAN2 to take this issue into consideration when design the signaling, e.g., when configuring event triggered L1-RSRP events, all candidate cells in the same LTM-CSI-ReportConfig are limited to the same frequency layer.

RAN1 has agreed to support both SSB based and CSI-RS based L1-RSRP measurement for event triggered reporting. We think it is too early to consider CSI-RS based L1-RSRP measurement for event triggered reporting as whether to define intra-frequency and inter-frequency is still in discussion, and which type of CSI-RS based L1-RSRP measurement (only consider the case that CSI-RS resources are within active BWP or not) is also in discussion. We propose to focus on SSB based L1-RSRP measurement for event triggered reporting at this time.

RAN1#118 Agreement - SSB based L1-RSRP measurements is supported for event triggered reporting - CSI-RS based L1-RSRP measurements is supported for event triggered reporting - FFS: CSI-RS based L1-SINR measurements is supported for event triggered reporting

Proposal 2: Focus on SSB based L1-RSRP measurement for event triggered reporting at first.

Proposal 3: For SSB based event triggered L1-RSRP measurement reporting, regard the events related to different candidate cells on different frequency layers as different events. Discuss how to differentiate intra-frequency and inter-frequency events. Further discuss the following options:

- Option 1: if the LTM-CSI-ReportConfig is linked to multiple cells on different frequency layer, then the number of events related to this LTM-CSI-ReportConfig should be counted no less than the number of frequency layers.

- Option 2: inform RAN2 that the events related to different candidate cells on different frequency layers are regarded as different events and ask RAN2 to take this into consideration when design the signaling.

How to coordinate with R19 MIMO

RAN2#126 Agreement - Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events. - Event LTM2: Beam of serving cell becomes worse than absolute threshold; - Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell; - Event LTM4: Beam of candidate cell becomes better than absolute threshold; - Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

RAN1#117 Agreement Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam Above feature is subject to UE capability. - Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs FFS: Whether the above is captured in RAN1 or RAN2 specification. RAN1#118 Working Assumption On UE-initiated/event-driven beam reporting, regarding trigger events, besides for Event-2, Event-1 and Event-7 are both supported. - Event-1: Quality of the current beam is worse than a certain threshold. - Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the M-th best quality. - M is RRC configured with subjective to UE capability signalling - UE may only indicate a single candidate value or not support Event-7. - The additionally supported events will reuse the same design as event 2 – unless there is consensus to do otherwise - The additionally supported events will be lower priority compared to event 2.

As shown above, Event LTM2-5 are supported in R19 mobility. Similar events are also supported in R19 MIMO. It is likely that NW configures separate events for mobility and MIMO. As far as we know, the evaluation method of events configured in mobility and MIMO may be different, so the events configured for mobility and MIMO should be considered as different events even when they are configured for the same beams and the same cell.

We believe the capability of supported number of events for L1 measurement will be discussed in R19 MIMO too. The question is to define separate capabilities for R19 mobility and MIMO or define a unified capability. We prefer to define a capability that indicates the total number of L1 events supported for mobility and MIMO. If to define separate capabilities, the number of supported events for both mobility and MIMO should be smaller as the efforts to support events for mobility and MIMO are similar for UE. Defining a single one for both can make the best use of UE capability. In addition, although the evaluation methods and reporting mechanisms for mobility and MIMO are different, it is a waste of UL resources as well as UE buffer and power to configure basically the same two events for both mobility and MIMO.

Proposal 4: Events configured for mobility and MIMO should be considered as different events even when they are configured for the same beams and the same cell with the same threshold.

Proposal 5: Define the capability for L1 events shared by mobility L1 events and MIMO L1 events.

Last meeting, there were some discussions on how to handle L1 filtering in RAN4. Although RAN2 assumes filtering of the L1 measure results is needed for LTM event, the detail is up to RAN1. It seems RAN1 has no further agreement on this yet, RAN4 can wait for more progress in RAN1 before discussing the detailed requirements.

RAN2#126 - RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation.

Proposal 6: Wait for more RAN1 progress to discuss how to handle L1 filtering in RAN4 for event triggered L1 report.

## 2.2 CSI-RS L1 measurements

RAN4#112 discussed CSI-RS based L1 measurement on neighbor cells. Basically, all the issues are open. In this section, we would like to further discuss the open issues.

Before discussing how to define CSI-RS L1 measurement requirements, we would like to recall the past discussions on CSI-RS based L3 measurement. For CSI-RS based L3 measurement, some information is listed here:

- associatedSSB is configured in CSI-RS-Resource-Mobility and detectable,

- Define requirements for Density=3 with (≥) 48 PRBs. No tighter requirements for more 48 PRBs,

- Single FFT is assumed for multiple cell measurements per frequency layer for both intra- and inter-frequency measurements,

- Only the requirements for intra-f without gap and inter-f with MG are specified.

- Intra-frequency and inter-frequency layer are defined. A measurement is defined as a CSI-RS based intra-frequency measurement provided that:

  - the SCS is the same as the SCS of CSI-RS on the serving cell indicated for measurement, and

  - the CP is the same as the CP type of the CSI-RS resource of the serving cell indicated for measurement,

  - the centre frequency is the same as the centre frequency of the CSI-RS resource of the serving cell indicated for measurement.

- Only one MO is configured per CSI-RS frequency layer

  - all CSI-RS resources in the same MO are configured with the same csi-rs-MeasurementBW,

  - all CSI-RS resources in the same MO are configured with the same periodicity,

  - All CSI-RS in the same MO are confined in the same 5ms window.

For CSI-RS based L1 measurement on neighbor cells, we think RAN4 also needs to discuss whether each of the above items is applicable. In the next, we would like to discuss the above items one by one.

Whether measurement on associated SSB is performed before measuring CSI-RS

Similar as SSB based L1-RSRP measurement, CSI-RS based L1-RSRP measurement should be performed only on known cells. In Rel-18 SSB based L1-RSRP, the cell is considered as known if the following conditions are met:

-	The UE has performed L3 measurement on the target cell during the last 5 seconds, and

-	The SSB from the target cell configured for L1 measurement remains detectable according to the cell identification requirements in clause 9.2.

Similar definition of known cell for SSB based L1-RSRP measurement also apply here. For CSI-RS based L1-RSRP measurement, the cell is considered as known if the following conditions are met,

-	The UE has performed L3 measurement on the target cell during the last 5 seconds, and

-	The CSI-RS from the target cell configured for L1 measurement remains detectable.

Proposal 7: CSI-RS based L1-RSRP measurement is performed only on known cells.

Proposal 8: For CSI-RS based L1-RSRP measurement, the cell is considered as known if the following conditions are met:

-	The UE has performed L3 measurement on the target cell during the last 5 seconds, and

-	The CSI-RS from the target cell configured for L1 measurement remains detectable.

For serving cell, in FR2 there are two kinds of CSI-RS for L1-RSRP measurement, with higher layer parameter repetition OFF or ON. In legacy CSI-RS based L1-RSRP measurement requirements for serving cell, the requirements for CSI-RS resources with repetition ON and OFF are different. If the parameter repetition is set to OFF, these CSI-RS are usually used in P-2 and UE is supposed to use the same Rx beam as the associated SSB to measure the CSI-RS. NW uses these CSI-RS resources with repetition OFF to refine Tx beam. If the parameter repetition is set to ON, these CSI-RS are usually used in P-3, i.e., NW fixes Tx beam and UE is supposed to refine Rx beam.

Pending on whether UE has performed L1 measurement on the associated SSB, there will be four cases:

| Case | Pre-requisite SSB measurements before measuring CSI-RS | CSI-RS repetition | Whether to sweep beams when measuring CSI-RS | Rx beam type for measuring CSI-RS and receiving target PDSCH | Whether to define requirement |
| --- | --- | --- | --- | --- | --- |
| 1 | SSB based L3 measurement only | OFF | No | Rough beam | No |
| 2 |  | ON | Yes | Fine beam | No |
| 3 | SSB based L1 measurement | OFF | No | Fine beam | Yes |
| 4 |  | ON | Yes | Fine beam | No |

For CSI-RS resources with repetition off:

- If UE has performed L1 measurement on the associated SSB, UE can use the same fine Rx beam as SSB to measure CSI-RS resources. The same fine Rx beam can be used for PDSCH reception afterwards. (Case 3)

- If UE has not performed L1 measurement on the associated SSB, UE has to use the rough Rx beam refined at L3 measurement. The same rough Rx beam will be used for PDSCH reception afterwards. (Case 1)

As the motivation to support L1-RSRP measurement on neighbor cell is to use fine beam directly after cell switch, Case 1 is not aligned with the goal. So we propose not to support case 1.

Proposal 9: For CSI-RS resources with repetition OFF in FR2, L1-RSRP measurement is performed only after UE has performed L1-RSRP measurement on the associated SSB.

For CSI-RS resources with repetition ON:

- (Case 2) If UE has not performed L1 measurement on the associated SSB, UE has to fine tune Rx beam on the basis of the rough Rx beam refined at L3 measurement. Beam sweeping factor may be needed. Fine Rx beam will be used for PDSCH reception afterwards.

- (Case 4) If UE is already performing L1 measurement on the associated SSB (with fine beam already), it seems unnecessary to further use CSI-RS with repetition ON to re-fine the Rx beam again. Even if such an opportunity is provided, we doubt whether its extra Rx beamforming gain can justify the additional RS overhead for transmitting CSI-RS with repetition ON.

As far as we know, CSI-RS resources with repetition ON were seldom used even for serving cell. We prefer not to support it for neighbor cell.

Proposal 10: Not to support CSI-RS resources with repetition ON in FR2 for L1 measurement on neighbor cell.

Define requirements for Density=3 with (≥) 48 PRBs

For CSI-RS based L1-RSRP measurement on serving cell and CSI-RS based L3 measurement, RAN4 only defined the requirements for Density=3 with (≥) 48 PRBs. There are no tighter requirements for more 48 PRBs. To avoid introducing too much complexity, for CSI-RS based L1-RSRP measurement on neighbor cell, we propose to define the requirements for Density=3 with (≥) 48 PRBs for CSI-RS based L1-RSRP measurement on neighbor cell and not to define tighter requirements for more 48 PRBs too.

Proposal 11: For CSI-RS based L1-RSRP measurement on neighbor cell, define the requirements for Density=3 with (≥) 48 PRBs and not to define tighter requirements for more 48 PRBs.

Whether to support RTD>CP case?

Single FFT is assumed for multiple cell measurements for CSI-RS based L3 measurement. For SSB based L1 measurement, RTD>CP case is supported with UE capability. If RAN4 can reach consensus on Proposal 11 above, to support RTD>CP case for CSI-RS based L1 measurement, another FFT is needed to cover as wide as 48PRBs, while SSB only occupies 20PRBs. The cost will be much higher to support RTD>CP for CSI-RS compared to SSB. In addition, compared to SSB based L1 measurement, we don’t see much gain to support CSI-RS based L1 measurement. It is sufficient to use SSB based L1 measurement. We propose not to support RTD>CP case for CSI-RS based L1-RSRP measurement.

Proposal 12: Not to define CSI-RS based L1-RSRP measurement requirements on neighbor cell for RTD>CP case.

Whether to support CSI-RS based inter-f L1-RSRP measurement on neighbor cell?

To discuss this issue, the definition of intra-f and inter-f shall be discussed at first. Different from L3 CSI-RS measurement, there is no concept of “Serving MO” that can be used as a reference to define intra-f and inter-f. Different from SSB based L1-RSRP measurement, different CSI-RS resources of the serving cell may have varied center frequency and BW, which means there is no specific CSI-RS resource to be used as a reference to define intra-f and inter-f. For SSB based L1-RSRP measurement, RAN4 defined the requirements for intra-f without gap, inter-f without gap and inter-f with MG. As well known, the requirements for SSB within active BWP, i.e., inter-f without gap and intra-f without gap, are exactly the same. But we have to capture the same requirements in two different chapters. Similarly, even we define intra-f and inter-f for CSI-RS based L1-RSRP measurement, the requirements of intra-f without gap and inter-f without gap will be the same. It may be better to categorize CSI-RS based L1-RSRP measurement into CSI-RS based L1-RSRP measurement within active BWP and outside active BWP for further discussion, instead of defining intra-f and inter-f.

Proposal 13: Categorize CSI-RS based L1-RSRP measurement into CSI-RS based L1-RSRP measurement within active BWP and outside active BWP for further discussion.

To perform CSI-RS based L1-RSRP measurement outside active BWP, gap shall be configured. Typical MGL are 6ms, 3ms, 4ms, 5.5ms and 1.5ms. To minimize the RS overhead, one CSI-RS resource is used for both serving and neighboring cell measurements, e.g., CSI-RS resources for L1-RSRP measurement on neighbor cell for UE#1 are in fact the resources for L1 measurement of serving cell for UE#2. For serving cell, there is no specific restriction in time domain on configurations of CSI-RS resources for L1-RSRP measurement. It makes difficult to ensure all the CSI-RS of the same frequency layer are within a single gap window. To solve this issue, for CSI-RS based L3 measurement, all CSI-RS resources in the same MO are confined in the same 5ms window. If to support CSI-RS based L1-RSRP measurement outside active BWP, similar restriction is also necessary. If so, it may pose a strong limitation on CSI-RS configuration. Generally, MG is configured for SSB based L3 measurement. SSB of each cell are overlapped or partially overlapped with MG. Due to measurement restriction in FR2, UE is not supposed to perform SSB based L3 measurement and CSI-RS based L1 measurement simultaneously. So the reasonable configuration is that SSB and CSI-RS for L1 measurement in FR2 are not overlapping. As shown below, most of the slots are occupied by SSB and only the white ones are not occupied by SSB. And SSB takes most of the symbols in the slots that contain SSB. If not to use concurrent gap, NW shall squeeze all the CSI-RS resources to the remaining symbols. It seems difficult to confine SSB and CSI-RS of the same frequency layer in the same 5ms window. In other words, it is difficult to use one gap to cover both SSB based L3 or L1 measurement and CSI-RS based L1 measurement, so concurrent gap is needed.

Also, to support CSI-RS based L1-RSRP measurement outside active BWP, it is necessary to define frequency layer is as different frequency layers should be measured TDM-ly. If not to define frequency layer, it will be difficult to discuss how to share the gap occasions by different CSI-RS resources.

Observation 1: If to support CSI-RS based L1-RSRP measurement outside active BWP, all CSI-RS in the same frequency layer shall be confined in the same 5ms window.

Observation 2: If to support CSI-RS based L1-RSRP measurement outside active BWP, concurrent gap shall be used.

Observation 3: If to support CSI-RS based L1-RSRP measurement outside active BWP, the definition of frequency layer should be discussed at first.

In summary, to support CSI-RS based L1-RSRP measurement outside active BWP, there would be much more restriction on CSI-RS configurations, and an optional feature concurrent gap should be supported at first. In addition, compared to SSB based L1 measurement, we don’t see much gain to support CSI-RS based L1 measurement. It is sufficient to use SSB based L1 measurement for inter-f. We propose to deprioritize CSI-RS based L1-RSRP measurement outside active BWP in R19.

Proposal 14: Deprioritize CSI-RS based L1-RSRP measurement outside active BWP in R19.

Whether and how to define “frequency layer”?

If RAN4 concludes not to define requirements for CSI-RS based L1-RSRP measurement outside active BWP and also RTD>CP case, maybe it is not necessary to define “frequency layer”.

In legacy, UE only performs L1-RSRP measurement on CSI-RS resources within active BWP in serving cell and RAN4 didn’t define any frequency layer concept for CSI-RS based L1 measurement. If only to consider CSI-RS based L1 measurement within active BWP and within CP, it will be similar as CSI-RS based L1 measurement in serving cell. It is not necessary to define frequency layer.

If RAN4 will define requirements for RTD>CP case or outside active BWP case, frequency layer shall be first defined. It is straight forward for CSI-RS based L1-RSRP measurement outside active BWP. In the next, we would like to discuss why frequency layer shall be first defined if to support RTD>CP case. Our proposal is not to define tighter requirements for more 48 PRBs, so another FFT supporting 48 PRBs is needed to support RTD>CP. As shown in Fig. 1, CSI-RS #1,2,3 are three resources from the same neighbor cell. It is obvious that CSI-RS #1 and #2 should be measured TDM-ly as they can not be covered by a single FFT window. If not defining the concept frequency layer, there will be ambiguity on how to measure CSI-RS#3. Or another much wider FFT is needed.

Fig. 1

Observation 4: If to define requirements for RTD>CP case, the definition of frequency layer should be discussed at first.

Although we prefer not to define requirements for RTD>CP case in R19, we still suggest to define frequency layer considering potential support of RTD>CP in future releases. Regarding how to define, we can follow CSI-RS based L3 measurement, i.e., for CSI-RS resources of the same frequency layer, SCS, CP type, center frequency, BW and periodicity are the same.

Proposal 15: Define the concept frequency layer for CSI-RS based L1-RSRP measurement.

Proposal 16: Following CSI-RS based L3 measurement, for CSI-RS resources of the same L1 frequency layer, SCS, CP type, center frequency, BW and periodicity are the same.

## 2.3 CSI acquisition on candidate cell(s) before or during LTM cell switch

According to the note in the WID, RAN1 shall decide on whether to support CSI report before or during the LTM cell switch at first. RAN4 can start the discussion after RAN1 concludes.

Proposal 17: Wait for more RAN1 progress to start the discussion on CSI acquisition in RAN4.

# Summary

In this paper, we provide some views on the R19 mobility. We have the following observation and proposals:

Proposal 1: Define a separate capability for supported number of events triggered L1 measurement reporting.

Proposal 2: Focus on SSB based L1-RSRP measurement for event triggered reporting at first.

Proposal 3: For SSB based event triggered L1-RSRP measurement reporting, regard the events related to different candidate cells on different frequency layers as different events. Discuss how to differentiate intra-frequency and inter-frequency events. Further discuss the following options:

- Option 1: if the LTM-CSI-ReportConfig is linked to multiple cells on different frequency layer, then the number of events related to this LTM-CSI-ReportConfig should be counted no less than the number of frequency layers.

- Option 2: inform RAN2 that the events related to different candidate cells on different frequency layers are regarded as different events and ask RAN2 to take this into consideration when design the signaling.

Proposal 4: Events configured for mobility and MIMO should be considered as different events even when they are configured for the same beams and the same cell with the same threshold.

Proposal 5: Define the capability for L1 events shared by mobility L1 events and MIMO L1 events.

Proposal 6: Wait for more RAN1 progress to discuss how to handle L1 filtering in RAN4 for event triggered L1 report.

Proposal 7: CSI-RS based L1-RSRP measurement is performed only on known cells.

Proposal 8: For CSI-RS based L1-RSRP measurement, the cell is considered as known if the following conditions are met:

-	The UE has performed L3 measurement on the target cell during the last 5 seconds, and

-	The CSI-RS from the target cell configured for L1 measurement remains detectable.

Proposal 9: For CSI-RS resources with repetition OFF in FR2, L1-RSRP measurement is performed only after UE has performed L1-RSRP measurement on the associated SSB.

Proposal 10: Not to support CSI-RS resources with repetition ON in FR2 for L1 measurement on neighbor cell.

Proposal 11: For CSI-RS based L1-RSRP measurement on neighbor cell, define the requirements for Density=3 with (≥) 48 PRBs and not to define tighter requirements for more 48 PRBs.

Proposal 12: Not to define CSI-RS based L1-RSRP measurement requirements on neighbor cell for RTD>CP case.

Proposal 13: Categorize CSI-RS based L1-RSRP measurement into CSI-RS based L1-RSRP measurement within active BWP and outside active BWP for further discussion.

Observation 1: If to support CSI-RS based L1-RSRP measurement outside active BWP, all CSI-RS in the same frequency layer shall be confined in the same 5ms window.

Observation 2: If to support CSI-RS based L1-RSRP measurement outside active BWP, concurrent gap shall be used.

Observation 3: If to support CSI-RS based L1-RSRP measurement outside active BWP, the definition of frequency layer should be discussed at first.

Proposal 14: Deprioritize CSI-RS based L1-RSRP measurement outside active BWP in R19.

Observation 4: If to define requirements for RTD>CP case, the definition of frequency layer should be discussed at first.

Proposal 15: Define the concept frequency layer for CSI-RS based L1-RSRP measurement.

Proposal 16: Following CSI-RS based L3 measurement, for CSI-RS resources of the same L1 frequency layer, SCS, CP type, center frequency, BW and periodicity are the same.

Proposal 17: Wait for more RAN1 progress to start the discussion on CSI acquisition in RAN4.

# 4 Reference

RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom