3GPP TSG-RAN WG4 Meeting # 112bis	                                                R4-2414929

Hefei, China, Oct. 14 – Oct. 18, 2024

Agenda item:	6.17.4.1

Source:	MediaTek Inc.

Title:	Discussion on core requirements of AI/ML BM use case

Document for:	Discussion

# Introduction

In this contribution, we provide our views on core requirements of AI/ML BM use case.

# Discussion

## 2.1 Input and output of AI/ML BM models

During the discussion last meeting, we don’t think RAN4 have common understanding on the input and output of AI/ML BM models. As this is the basis to further discuss other issues, we would like to provide our view here.

As captured in TR38.843 [1], in R18 SI, both DL Tx beam prediction and beam pair prediction are studied.

TR38.843 For both sub-use cases, the following alternatives are studied for the predicted beams: - Alt.1: DL Tx beam prediction - Alt.2: DL Rx beam prediction (deprioritized) - Alt.3: Beam pair prediction (a beam pair consists of a DL Tx beam and a corresponding DL Rx beam) Using BM-case1 as an example:

In R19 WI, as pointed out in WID, only DL Tx beam prediction is in scope and beam pair prediction is not in the scope.

RP-242399 - Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”) - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”) - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2

Observation 1: Both DL Tx beam prediction and beam prediction are studied in R18 AI/ML SI, but only DL Tx beam prediction is in the scope of R19 AI/ML WI.

In our understanding and also as the input of different companies in R18 SI (R18 AI/ML BM evaluation in RAN1), the model input and output of DL Tx beam prediction and beam pair prediction are shown in Fig.1 and Fig.2.

Fig.1 input and output of DL Tx beam prediction AI/ML model

Fig.2 input and output of beam pair prediction AI/ML model

Observation 2: According to the submitted input from different companies in R18 SI, for DL Tx beam prediction, the model inputs are L1-RSRP of Tx beams in Set B with Tx beam ID information as input or implicitly indicated. The outputs are probability of each beam in set A to be the Top-1 beam or predicted L1-RSRP of beams in Set A.

Observation 3: AoA information is not used at least in all the submitted simulations in R18 SI (R18 AI/ML BM evaluation in RAN1). Rx beam information is not used in DL Tx beam prediction.

Proposal 1: RAN4 to confirm Rx beam info is not part of the AI/ML model inputs.

      - 2.2 Definition of known TCI state

As AI/ML beam prediction is for the purpose of beam management, there may be some impact on beam management related requirements. For instance, after UE reports the predicted beams or RSRP to NW, NW may activate corresponding reported best-predicted beam. In RAN4, we defined TCI state switching delay requirements. Take the requirements of MAC CE based TCI state activation as an example. The TCI state switch delay is THARQ + 3ms + TL1-RSRP +TOuk*(Tfirst-SSB+ TSSB-proc) / NR slot length, where TL1-RSRP = 0 for known TCI state and TL1-RSRP equals to L1 measurement delay in FR2 for unknown TCI state. If the TCI state is unknown, the switching delay would be long. One of the conditions of known TCI state is that UE has sent at least 1 L1-RSRP report for the target TCI state before the TCI state switch command in 1280ms. The question here is whether the predicted beam is known or unknown if a prediction report is sent recently. Next, we would like to provide our views.

As UE is the one who performs TCI state switching, when talking about known condition, the corresponding TCI state should be actually known from UE side. At least two conditions shall be met in FR2. One is that SINR of the corresponding RS cannot be too low. Another is that UE knows its Rx beam for reception (for DL TCI state activation). To meet the 2nd condition, a TCI state is known only if UE has measured the corresponding RS and reported the measured result in last 1280ms before TCI state switching command.

As shown in Observation 1-3, only DL Tx beam prediction is in scope and the output of DL Tx beam prediction AI/ML BM model does not include Rx beam information. So if the beam to switch to does not belong to set B, even there is a prediction report but UE still does not know which Rx beam to use for DL reception.

Proposal 2: The TCI state QCL to an RS that is not in Set B is unknown even if this RS was included in the L1-RSRP report by UE within 1280ms before TCI state activation.

For the TCI state whose QCL source RS belongs to Set B, if it is spatial-domain beam prediction, the TCI state can be viewed as known if the corresponding predicted beam is reported in 1280ms. If it is temporal beam prediction, we should use the last observation occasion (T3) instead of the reported occasion as shown in Fig 6. The reason lies in that the time gap between T3 and T4 may be too long to ensure Rx beam unchanged. The report occasion may be close to T4.

Fig 6. Illustration for AI/ML temporal beam prediction

Proposal 3: The TCI state which is QCL to an RS in Set B is known:

- if the corresponding predicted beam is reported in 1280ms before the TCI state switch command and SNR of the RS is above -3dB for spatial-domain beam prediction.

- if the last observation occasion is within 1280ms before the TCI state switch command and SNR of the RS is above -3dB for temporal beam prediction.

      - 2.3 Impact of measurement error

Issue 2-6: Measurement error impact Agreement: RAN4 will study the impact of measurement error for the input to inference and dataset for training(set B measurement error) on prediction accuracy, companies should bring proposals on how to proceed with such a study. impact on different prediction accuracy metrics should be taken into account studies already performance by other groups (RAN1) could also be taken into account Note: intention of the study is just to understand the impact of measurement error on prediction accuracy.

As guided by the agreement last meeting, we would like to provide some simulation results and views on how to deal with measurement error.

We wonder the prediction requirements defined in RAN4 are used to reflect the potential performance of AI/ML BM models in the field or just as a reference for the test. If it is the former, in reality, the L1-RSRP used during training and inference are all measured by UE and will anyway include measurement error. The impact on the prediction accuracy shall be simulated by adding measurement error on both training data and inference input. If the performance requirements defined in RAN4 is based on synthetic channel, errorless training dataset can be obtained based on the agreed channel and settings. We have compared prediction accuracy of the two ways to add measurement error, i.e., adding error at both training and inference or adding error only on inference. It can be observed that the prediction accuracy degradations are similar for both ways. We are open on how to add the measurement error, i.e., adding error at both training and inference or adding error only on inference. We slightly prefer to add the measurement error based on test goal, i.e., reflect the potential performance of AI/ML BM models in the field or verify UE can train a model that works well under a certain channel and scenario.

| Adding error | BB error [dB] | RF error [dB] | top-1 prediction accuracy (%) | top-3 prediction accuracy (%) | top-5 prediction accuracy (%) |
| --- | --- | --- | --- | --- | --- |
| No error | 0 | 0 | 60.37 | 88.2 | 95.4 |
| Train & Inference | 0 | 4 | 59.52 | 87.78 | 95.13 |
| Train &Inference | 2 | 2 | 56.88 | 86.88 | 94.29 |
| Train & Inference | 4 | 0 | 54.13 | 85.34 | 93.54 |
| only Inference | 0 | 4 | 59.15 | 88.04 | 95.08 |
| only Inference | 2 | 2 | 56.93 | 87.35 | 94.44 |
| only Inference | 4 | 0 | 54.23 | 84.71 | 93.39 |

Proposal 4: Discuss how to add the measurement error, i.e., adding error at both training and inference or adding error only on inference. How to add the measurement error shall be aligned with the test goal, i.e., reflect the potential performance of AI/ML BM models in the field or verify UE can train a model that works well under a certain channel and scenario.

Regarding measurement error, there are both BB errors and RF errors. BB error is assumed different for different measurement results.  For RF error, it is assumed that if UE uses the same Rx chain to calculate L1-RSRP over different Tx beam, then the RF error assumed to be the same. In the simulation, we investigate different modelling of measurement error. We consider the following error distribution in our simulation.

  - BB error

    - BB error is sampled from single Gaussian distribution with 95% samples in ±2dB or ±4dB

- RF error

    - RF error (specific bias for each Rx) is sampled from single Gaussian distribution with 95% samples in ±2dB or ±4dB

It can be observed that different measurement error model will lead to different simulation results. If the test goal is to verify UE can train a model that works well under a certain channel and scenario, and single AoA test system is used, it is possible that RF error can be assumed to be the same during the test and we only need to consider the impact of BB error.

Proposal 5: Discuss error modelling when defining the prediction accuracy requirements. How to model the measurement error shall be aligned with the test goal, i.e., reflect the potential performance of AI/ML BM models in the field or verify UE can train a model that works well under a certain channel and scenario.

# Conclusion

In this contribution, we provide our views for AI/ML BM use case. Observations and proposals are summarized as follows.

Observation 1: Both DL Tx beam prediction and beam prediction are studied in R18 AI/ML SI, but only DL Tx beam prediction is in the scope of R19 AI/ML WI.

Observation 2: According to the submitted input from different companies in R18 SI, for DL Tx beam prediction, the model inputs are L1-RSRP of Tx beams in Set B with Tx beam ID information as input or implicitly indicated. The outputs are Probability of each beam in set A to be the Top-1 beam or predicted L1-RSRP of beams in Set A.

Observation 3: AoA information is not used at least in all the submitted simulations in R18 SI (R18 AI/ML BM evaluation in RAN1). Rx beam information is not used in DL Tx beam prediction.

Proposal 1: RAN4 to confirm Rx beam info is not part of the AI/ML model inputs.

Proposal 2: The TCI state QCL to an RS that is not in Set B is unknown even if this RS was included in the L1-RSRP report by UE within 1280ms before TCI state activation.

Proposal 3: The TCI state which is QCL to an RS in Set B is known:

- if the corresponding predicted beam is reported in 1280ms before the TCI state switch command and SNR of the RS is above -3dB for spatial-domain beam prediction.

- if the last observation occasion is within 1280ms before the TCI state switch command and SNR of the RS is above -3dB for temporal beam prediction.

Proposal 4: Discuss how to add the measurement error, i.e., adding error at both training and inference or adding error only on inference. How to add the measurement error shall be aligned with the test goal, i.e., reflect the potential performance of AI/ML BM models in the field or verify UE can train a model that works well under a certain channel and scenario.

Proposal 5: Discuss error modelling when defining the prediction accuracy requirements. How to model the measurement error shall be aligned with the test goal, i.e., reflect the potential performance of AI/ML BM models in the field or verify UE can train a model that works well under a certain channel and scenario.

# Reference

- 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface