3GPP TSG-RAN WG4 Meeting #112bis                      R4-2414955

Hefei, Anhui, China, 14th – 18th October, 2024

Source:	China Telecom

Title:	Discussion on UE demodulation performance requirements for 8Rx with MMSE-IRC

Agenda Item:	6.16.2

Document for:	discussion

Introduction

In RAN#104 meeting, the Rel-19 new WID on NR demodulation performance: Phase 5 is approved.

The following objectives are approved for UE with 8Rx:

- Define the following UE performance requirements for 8Rx CPE/FWA/vehicle/industrial devices:

  - PDSCH demodulation and CQI reporting requirements with inter-cell interference with MMSE-IRC receiver

    - Reuse the Rel-17 interference model for 2/4Rx UE as a baseline

  - PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver

    - Reuse the existing interference model for 2/4Rx UE as a baseline

RAN4#112 is the first meeting for the discussion on the above objectives, as an outcome, the WF was approved in [1]. In this contribution, our views on the open issues are given.

Discussion

In the following table, we have summarized the key parameters used for MMSE-IRC requirements defined for 2/4Rx UE in Rel-17:

Table 1. Key parameters used for Rel-17 MMSE-IRC requirements for 2/4Rx UE

- PDCSH requirements with inter cell interference - Receiver type: - MMSE-IRC - Network type: - Synchronized network only - Cover both HomNet and Hetnet scenarios with the following applicability rule [2] - ‘If UE supporting both TDD and FDD with same Rx number, UE will pass test case under homogenous scenario with FDD mode, and pass test case under HetNet scenario with TDD mode’ - Interference profile: INR based inter cell interference modeling with the following interference level: - INR1 = 7.77dB and INR2 = 2.29dB for HomNet - INR1 = 7.58dB for HetNet - Random rank with 70% and 30% probability for rank 1 and rank 2 for the interference cell(s) - Test and PDSCH configuration - TDLC300-100, MCS 13, 2x2 ULA Low, rank1 for HomNet - TDLA-30-10, MCS 13, 2x2 ULA Low, rank1 for HetNet - PDCSH requirements with intra cell inter user interference - Receiver type: - MMSE-IRC and R-ML - 1 co-scheduled UE with random 16QAM symbols for MMSE-IRC test - Rank configuration: - Both rank 1+1 and rank 2+2 are covered - Precoding method: - Target UE: Random precoding - Co-scheduled UE: - Random precoding for MMSE-IRC tests with rank 1+1; - Orthogonal precoding with the target UE for MMSE-IRC tests with rank 2+2 and for R-ML tests - Test and PDSCH configuration for the target UE - TDLC300-100, MCS 13, 2x2 and 2x4 ULA Low for MMSE-IRC tests with rank 1+1 - TDLA30-10, MCS 13, 4x4 ULA Low for MMSE-IRC tests with rank 2+2 - DMRS port allocation between target and co-scheduled UEs: - Same DMRS group for rank 1+1 test, i.e., {1000} for target and {1001} for co-UE - Different DMRS groups for rank 2+2 test, i.e., {1000, 1001} for target and {1002, 1003} for co-UE - Wideband CQI reporting requirements with inter-cell interference - Receiver type: - MMSE-IRC - Test criteria: - the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥ ; - when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR, the average BLER for the indicated transport formats shall be greater than or equal to 0.02. - Configuration for the target cell: - TDLA30-5, 2x2 and 2x4 ULA Low - Configuration for the target cell: - AWGN 1x2 and 1x4, INR = 10.04dB - Tested SINR = -2dB

In general, we think most of the test settings and parameters can be reused for 8Rx UEs. At the same time, we would also like to share our views on the following parameters.

General

Test scope

Status in the WF [1]: - RAN4 to introduce UE demodulation requirements for both TDD and FDD - Not to cover PDSCH absolute physical layer throughput requirements for 8Rx - Candidate options on whether to cover CQI requirements for ICI scenario: - Option 1: CQI reporting requirements under ICI should be cover for 8Rx - Option 2: No need to cover such requirements

No matter whether CQI reporting requirement under fading channel is defined for 8Rx UE in Rel-18, for MMSE-IRC processing verification point of view, we believe both PDSCH and CQI reporting requirements are necessary. The UE with MMSE-IRC should be able to report correct CQI with interference suppression, which can only be verified under ICI tests.

Proposal 1: CQI reporting requirements under ICI should be covered for 8Rx.

Receiver assumptions

Status in the WF [1]: - Candidate options - Option 1: Cover both Baseline SU-MIMO 8Rx receiver and Simplified SU-MIMO 8Rx receiver - Option 2: Define requirements only for SU-MIMO baseline receiver - Interested companies are encouraged to bring simulation results to show the gap between the two types of receivers.

In Rel-18, 2 types of 8Rx receivers are defined in TS38.306:

SU-MIMO 8Rx receiver - Baseline SU-MIMO 8Rx receiver: 8Rx receivers for SU-MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector in FR1 - Simplified SU-MIMO 8Rx receiver: 8Rx receivers for SU-MIMO transmissions with support of up to 4 layers with two joint 4Rx MIMO detectors in FR1.

In general, we believe the new MMSE-IRC requirements should cover both receiver types.

Different implementations of 8Rx receivers have resulted in significant performance difference, e.g., the performance gap between Baseline SU-MIMO 8Rx receiver and simplified SU-MIMO 8Rx receiver is 2.2dB for FDD rank 2 requirements. Therefore, we may also need to define 2 sets of requirements for the baseline and simplified 8Rx receiver respectively.

In addition, even if the final simulation results show similar performance between the baseline and simplified 8Rx receiver, we can consider define one set of requirements which is applicable for both receiver types.

Proposal 2: UE performance requirements should cover both Baseline SU-MIMO 8Rx receiver and Simplified SU-MIMO 8Rx receiver. Define 2 sets of requirements for the baseline and simplified 8Rx receiver respectively if large performance difference is observed. Otherwise, consider define one set of requirements which is applicable for both receiver types.

PDSCH mapping type

Status in the WF [1]: - Candidate options: - Option 1: Include both PDSCH type A and type B requirements. - Option 2: Reuse the same configuration for Rel-17 2/4Rx, i.e., PDSCH mapping type A only

Neither the Rel-17 2/4Rx MMSE-IRC requirements nor the Rel-18 8Rx UE performance requirements have covered PDSCH type B. Hence, we do not see the reason to cover PDSCH type B for 8Rx new requirements for MMSE-IRC.

Proposal 3: Reuse the same configuration for Rel-17 2/4Rx, i.e., PDSCH mapping type A only.

Test applicability rule and release independent

Test applicability rule and release independent

Status in the WF [1]: - Candidate options: - Option 1: The new requirements for 8Rx MMSE-IRC receiver should be mandatory requirements for all 8Rx UEs without additional test applicability rule - Option 2: Applicability rule to skip 2/4Rx requirements if 8Rx tests are passed, if same test scenarios/configurations as 2RX/4RX are defined

MMSE-IRC is a mandatory without capability signaling feature for 2/4Rx UEs from Rel-17. As a new feature for Rel-18, we believe all 8Rx UEs should at least implement MMSE-IRC receiver. Therefore, the new requirements for 8Rx MMSE-IRC receiver should also be mandatory requirements for all 8Rx UEs without additional test applicability rule.

For UEs supporting both 8Rx and 2/4Rx, in Rel-18, we have already introduced test applicability rules to skip the 2/4Rx requirements if the corresponding 8Rx requirements are passed as shown below. I believe the same approach can be reused for the Rel-19 new requirement to be defined.

Table 5.1.1.2-1: Requirements applicability Supported RX antenna ports; Test type; Test list. UE supports 8Rx, 4Rx and 2Rx; PDSCH; All tests in Clause 5.2.3 (Note 2, 3) All tests in Clause 5.2.4 (Note 2); If UE passes tests in Clause 5.2.4, UE can skip Test 2-1 and Test 2-2 in Clause 5.2.3 Table 5.2.3.1.1-4, Table 5.2.3.2.1-4 and Test 4-1 in Clause 5.2.3 Table 5.2.3.1.1-6, Table 5.2.3.2.1-6. or; PDCCH; All tests in Clause 5.3.3 (Note 2,3). UE supports both 8Rx and 4Rx; PBCH; All tests in Clause 5.4.3 (Note 1). UE supports both 8Rx and 2Rx; PDSCH; All tests in Clause 5.2.2 (Note 2, 4) All tests in Clause 5.2.4 (Note 2); If UE passes tests in Clause 5.2.4, UE can skip Test 2-1 and Test 2-2 in Clause 5.2.2 Table 5.2.2.1.1-4, Table 5.2.2.2.1-4. PDCCH; All tests in Clause 5.3.2 (Note 2,4). PBCH; All tests in Clause 5.4.2.

For release independent, the 8Rx UE feature is introduced in Rel-18 and we have already discussed the possibility to cover MMSE-IRC requirements during the performance part discussion under WI ‘NR_ENDC_ RF_FR1_enh2’. Such requirements are not introduced in Rel-18 mainly because of the limited time and huge workload. Therefore, we think the new requirements for 8Rx should be release independent from Rel-18.

Proposal 4: The new requirements for 8Rx MMSE-IRC receiver should be mandatory requirements for all 8Rx UEs without additional test applicability rule

Proposal 5: For UEs supporting both 8Rx and 2/4Rx, the 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped if the corresponding 8Rx requirements are passed.

Proposal 6: The new requirements for 8Rx should be release independent from Rel-18.

PDCSH requirements with inter cell interference

Interference modelling and power levels

Status in the WF [1]: - Candidate options: - Option 1: Use the Intercell Interference model, specified in Clause B.6.2 of TS 38.101-4 - Option 1A: Reuse the same interference profiles (i.e., cover HomNet and Hetnet scenarios) and same power levels (i.e., INR values) defined for Rel-17 2/4Rx - Option 1B: Only consider HomNet scenario with 2 interference cells and reuse the same power levels (i.e., INR values) defined for Rel-17 2/4Rx - Option 2: RAN4 to discuss if it is still reasonable to use 16QAM based randomly modulated data for each interfering cell

As this WI targets to define requirements for 8Rx CPE/FWA/vehicle/industrial devices, the actual interference power level may be different from the values we have agreed for Rel-17 handheld UEs due to changing of deployment scenarios.

However, since the purpose of these requirements targets to verify the MMSE-IRC processing itself, we are also ok to reuse the existing INR values for Rel-17 2/4Rx to avoid extra system level simulation work. And we prefer to cover both HomNet and Hetnet scenarios which is align with 2/4Rx definition.

As for the interference signal modulation order, by using MMSE-IRC, we expect similar performance by using 16QAM or higher orders. From the test purpose point of view, we are more preferable to reuse the same interference signal modulation order as 2/4Rx requirements.

Proposal 7: Reuse the same interference profiles (i.e., cover HomNet and Hetnet scenarios) and same power levels (i.e., INR values) defined for Rel-17 2/4Rx.

Proposal 8: Reuse the same interference signal modulation order as 2/4Rx requirements, i.e., random 16QAM symbols.

Network Type and SCS/CBW

Status in the WF [1]: - Candidate options: - Option 1: Reuse the same propagation condition defined for Rel-17 2/4Rx, i.e., 15kHz/10MHz for FDD, and 30kHz/40MHz for TDD with 7D1S2U(S=6D+4G+4U) - Other options are not precluded

We propose to reuse the same Network Type and SCS/CBW defined for Rel-17 2/4Rx, i.e., 15kHz/10MHz for FDD, and 30kHz/40MHz for TDD with 7D1S2U(S=6D+4G+4U).

Proposal 9: Reuse the same Network Type and SCS/CBW defined for Rel-17 2/4Rx, i.e., 15kHz/10MHz for FDD, and 30kHz/40MHz for TDD with 7D1S2U(S=6D+4G+4U)

Propagation condition

Status in the WF [1]: - Candidate options: - Option 1: Reuse the same propagation condition defined for Rel-17 2/4Rx, i.e., TDLC300-100 for 2 interfering cells (HomNet) and TDLA30-10 for 1 interfering cell (HetNet) - Option 2: Consider TDLA30-10 as the starting point

We propose to reuse the same propagation condition defined for Rel-17 2/4Rx, i.e., TDLC300-100 for 2 interfering cells (HomNet) and TDLA30-10 for 1 interfering cell (HetNet) as a start point.

Proposal 10: Reuse the same propagation condition defined for Rel-17 2/4Rx, i.e., TDLC300-100 for 2 interfering cells (HomNet) and TDLA30-10 for 1 interfering cell (HetNet) as a start point.

Antenna configuration and rank

Status in the WF [1]: Antenna and MIMO correlation - Candidate options: - Option 1: 2T8R ULA low - Option 2: 4T8R - Option 3: - 2T8R ULA medium B - 4T8R ULA low - 8T8R ULA low Rank - Candidate options: - Option 1: Consider higher rank for 8Rx - Option 1A: RAN4 to investigate rank 1, 2, 4 and choose a reasonable requirement value - Option 1B: Rank 2 and Rank 4 as the starting point - Option 1C: Rank 2 and Rank 4 and Rank 8 - Option 1D: Rank 4 - Option 1E: Consider 2 layers, 4 layers and 6 layers - Option 2: Same as Rel-17 2/4Rx, i.e., Rank 1

The existing requirements with ICI for 4Rx rank1 are around 10dB for TDLC300-100 and 7dB for TDLA30-10.

By increasing the Rx number from 4 to 8, around 3dB performance gain is expected due to pure diversity gain. In addition, more performance gain of the MMSE-IRC receiver over the baseline receiver can be further expected due to more spatial domain freedom. For example, the requirements for MMSE-IRC with 4Rx has reduced to 7.4dB from 12.5dB with 2Rx.

In that case, under the same test configuration with 2/4Rx, the SNR requirement value for 8Rx may be reduced to around 2-3 dB, which is not a common SNR requirement for 8Rx.

Observation 1: Under the same test configuration with 2/4Rx, the SNR requirement value for 8Rx may be reduced to around 2-3dB due to larger diversity gain and more MMSE-IRC performance gain.

Considering the above, it is proposed to investigate simulation results for rank 1 (2x8), 2 (2x8) and 4 (4x8) and choose a reasonable requirement value.

Proposal 11: Investigate simulation results for rank 1 (2x8), 2 (2x8) and 4 (4x8) and choose a reasonable requirement value.

MCS

Status in the WF [1]: - Candidate options: - Option 1: Reuse the same configuration for Rel-17 2/4Rx, i.e., MCS 13 - Option 2: MCS 16 - Option 3: MCS 19 for rank 2, MCS 17 for rank 4 and 8

We think the MCS for requirement definition should be decided based on simulation results by taking performance gain and reasonable SINR into consideration. We propose to perform simulation results for all candidate MCS and make decision based on the results.

Proposal 12: Perform simulation results for all candidate MCS and make decision based on the results.

PDCSH requirements with intra cell inter user interference

Network Type and SCS/CBW

Status in the WF [1]: - Candidate options: - Option 1: Reuse the same propagation condition defined for Rel-17 2/4Rx, i.e., 15kHz/10MHz for FDD, and 30kHz/40MHz for TDD 7D1S2U - Other options are not precluded

We propose to reuse the same propagation condition defined for Rel-17 2/4Rx, i.e., 15kHz/10MHz for FDD, and 30kHz/40MHz for TDD 7D1S2U.

Proposal 13: Reuse the same propagation condition defined for Rel-17 2/4Rx, i.e., 15kHz/10MHz for FDD, and 30kHz/40MHz for TDD 7D1S2U

Interference modelling

Status in the WF [1]: - Candidate options on MU-MIMO Beamforming Model: - Option 1: Reuse the same MU-MIMO Beamforming Model for Rel-17 2/4Rx in B.4.2 in TS38.101-4 - Option 2: Include in a new clause of Annex B.6 of TS 38.101-4 - Candidate options on co-UE number: - Option 1: Same as Rel-17 2/4Rx requirements, i.e., 1 target UE + 1 co-UE - Option 2: Consider 1 target UE and more than 1 co-UEs

We believe the existing MU-MIMO Beamforming Model in B.4.2 in TS38.101-4 is also applicable for 8Rx and we prefer to reuse it for new 8Rx requirements.

For the co-scheduled UE number, since we only consider 1 co-scheduled UE for all existing 2/4Rx requirements, we propose to also consider 1 co-UE for new 8Rx requirements and at the same time, we can carefully design the rank allocation between UEs.

Proposal 14: Reuse the same MU-MIMO Beamforming Model for Rel-17 2/4Rx in B.4.2 in TS38.101-4

Proposal 15: Consider 1 co-UE for new 8Rx requirements and at the same time, we can carefully design the rank allocation between UEs.

Propagation condition

Status in the WF [1]: - Candidate options: - Option 1: Reuse the same precoding method as MMSE-IRC, i.e., TDLC300-100 with rank 1+1 and TDLA30-10 with rank 2+2 - Option 2: Consider TDLA30-10

We propose to reuse the same propagation condition as MMSE-IRC, i.e., TDLC300-100 with rank 1+1 and TDLA30-10 with rank 2+2

Proposal 16: Reuse the same propagation condition as MMSE-IRC, i.e., TDLC300-100 with rank 1+1 and TDLA30-10 with rank 2+2.

Precoding method for the co-scheduled UE

Status in the WF [1]: - Candidate options: - Option 1: Reuse the same precoding method as MMSE-IRC, i.e., TDLC300-100 with rank 1+1 and TDLA30-10 with rank 2+2 - Option 2: Consider TDLA30-10

For 2/4Rx MMSE-IRC tests, random precoding was used with rank 1+1 and orthogonal precoding was used for rank 2+2 tests. Same time, orthogonal precoding was used for all R-ML tests.

In our understanding, RAN4 has chosen to use only orthogonal precoding for R-ML tests because the test configurations for R-ML is stricter in terms of MCS and antenna correlation. Higher modulation order or higher antenna correlation makes it hard for the network to use random precoding for co-UE anymore.

Therefore, if we consider using the same test setting and PDSCH configuration as MMSE-IRC, we propose to reuse the same precoding method as MMSE-IRC, i.e., random precoding was used with rank 1+1 and orthogonal precoding was used for rank 2+2 tests.

Proposal 17: Reuse the same precoding method as MMSE-IRC, i.e., random precoding was used with rank 1+1 and orthogonal precoding was used for rank 2+2 tests, if we consider using the same test setting and PDSCH configuration as MMSE-IRC.

Antenna, rank (target UE + co-scheduled UEs) configuration and MIMO correlation

Status in the WF [1]: - Candidate options: - Option 1: 2T8R ULA low - Option 2: Rank 4+2 with 8T8R - Option 3: - Rank 2+2 for 2T8R - Rank 4+4 for 4T8R - Rank 8+8 for 8T8R when 1 paired UE scheduled - Option 4: - Rank 1+1 for 2T8R - Rank 2+2 for 4T8R - Rank 2+6 or rank 6+6 for 8T8R - Option 5: - Rank 2+2+2 for 8T8R - Rank 4+4 for 4T8R - Option 6: - Rank 2+2 - Rank 4+4

We support to cover rank 2+2, 4+4 and 8+8 as a start point and further check whether these configurations are feasible based on simulation results.

Proposal 18: Cover rank 2+2, 4+4 and 8+8 as a start point and further check whether these configurations are feasible based on simulation results.

MCS

Status in the WF [1]: - Candidate options for the target UE: - Option 1: Same as Rel-17 2/4Rx requirements for MMSE-IRC, i.e., MCS 13 - Option 2: - MCS 19 Table 1 for Rank 2 - MCS 17 Table 1 for Rank 4 - MCS 17 Table 1 for Rank 8 - Option 3: Consider MCS 17 and MCS 13 - Option 4: MCS 19 for rank 2, MCS 13 for rank 4

Similar as above, we propose to perform simulation results for all candidate MCS and make decision based on the results.

Proposal 19: Perform simulation results for all candidate MCS and make decision based on the results.

Wideband CQI reporting requirements with inter-cell interference

CQI calculation algorithm

Status in the WF [1]: - Candidate options: - Option 1: RAN4 to consider whether to introduce two CQI calculation assumptions to match two 8Rx receiver assumptions. - Baseline CQI calculation: UE perform CQI calculation with 8Rx joint processing - Simplified CQI calculation: UE perform CQI calculation with separate 4Rx processing - Other options not precluded

We have already defined 2 types of 8Rx receivers in TS38.306:

SU-MIMO 8Rx receiver - Baseline SU-MIMO 8Rx receiver: 8Rx receivers for SU-MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector in FR1 - Simplified SU-MIMO 8Rx receiver: 8Rx receivers for SU-MIMO transmissions with support of up to 4 layers with two joint 4Rx MIMO detectors in FR1.

We believe the UE reported CQI should be matched with the PDSCH receiving algorithm. We cannot imagine a UE can report CQI by ‘Baseline CQI calculation’ and receiving PDSCH by ‘Simplified SU-MIMO 8Rx receiver’, which will cause large system degradation.

We support to cover both types of 8Rx receivers thus we propose to define CQI reporting requirements as below:

  - Baseline SU-MIMO 8Rx receiver with Baseline CQI calculation

  - Simplified SU-MIMO 8Rx receiver with Simplified CQI calculation.

Proposal 20: Define CQI reporting requirements as below:

  - Baseline SU-MIMO 8Rx receiver with Baseline CQI calculation

  - Simplified SU-MIMO 8Rx receiver with Simplified CQI calculation.

Test metric

Status in the WF [1]: - Candidate options: - Option 1: Reuse the same test metric for Rel-17 2/4Rx a) the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥  b) when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR, the average BLER for the indicated transport formats shall be greater than or equal to X%.. - Other options are not precluded

We support to reuse the same test metric for Rel-17 2/4Rx.

Proposal 21: Reuse the same test metric for Rel-17 2/4Rx:

a)	the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥ 

b)	when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR, the average BLER for the indicated transport formats shall be greater than or equal to X%.

Propagation condition

Status in the WF [1]: - Candidate options: - Option 1: Reuse the same configuration for Rel-17 2/4Rx, i.e., TDLA30-5 for the serving cell and AWGN for the interference cell - Option 2: Use AWGN static channel as the baseline, further study whether to involve fading channel

According to the existing test metric, both AWGN and fading channel are involved in the test. Unless we are trying to define a new test metric for CQI reporting with MMSE-IRC, we propose to reuse the same configuration for Rel-17 2/4Rx, i.e., TDLA30-5 for the serving cell and AWGN for the interference cell.

Proposal 22: Reuse the same configuration for Rel-17 2/4Rx, i.e., TDLA30-5 for the serving cell and AWGN for the interference cell.

Interference power level

Status in the WF [1]: - Candidate options: - Option 1: Reuse the same configuration for Rel-17 2/4Rx, i.e., INR = 10.04dB with rank 1 - Other options are not precluded

We support to reuse the same configuration for Rel-17 2/4Rx, i.e., INR = 10.04dB.

Proposal 23: Reuse the same configuration for Rel-17 2/4Rx, i.e., INR = 10.04dB with rank 1

Antenna configuration, rank and MIMO correlation for the serving cell

Status in the WF [1]: Antenna configuration and MIMO correlation for the serving cell - Candidate options: - Option 1: 2T8R ULA Low - Option 2: Consider 4T8R Rank - Candidate options: - Option 1: Reuse the same configuration for Rel-17 2/4Rx, i.e., rank 1 - Option 2: Rank 2 - Option 3: Rank 4

We are supportive to consider higher rank compared with Rel-17 2/4Rx, we propose to cover rank 1,2 and 4 for initial study and further make decision based on simulation results.

Proposal 24: Cover rank 1,2 and 4 for initial study and further make decision based on simulation results.

Tested SINR value

In Rel-17, the tested SINR value is chosen based on the simulation results with reasonable performance gain. Therefore, we propose to decide the detailed SINR value based on simulation results.

Proposal 25: The tested SINR value should be decided based on simulation results with reasonable performance gain.

Besides the above proposal, for the other parameters for 8Rx UE new requirements, we propose to reuse the same configuration for Rel-17 2/4Rx as summarized in Table 1.

Proposal 26: For the other parameters for 8Rx UE new requirements, we propose to reuse the same configuration for Rel-17 2/4Rx as summarized in Table 1.

Conclusion

Proposal 1: CQI reporting requirements under ICI should be covered for 8Rx.

Proposal 2: UE performance requirements should cover both Baseline SU-MIMO 8Rx receiver and Simplified SU-MIMO 8Rx receiver. Define 2 sets of requirements for the baseline and simplified 8Rx receiver respectively if large performance difference is observed. Otherwise, consider define one set of requirements which is applicable for both receiver types.

Proposal 3: Reuse the same configuration for Rel-17 2/4Rx, i.e., PDSCH mapping type A only.

Proposal 4: The new requirements for 8Rx MMSE-IRC receiver should be mandatory requirements for all 8Rx UEs without additional test applicability rule

Proposal 5: For UEs supporting both 8Rx and 2/4Rx, the 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped if the corresponding 8Rx requirements are passed.

Proposal 6: The new requirements for 8Rx should be release independent from Rel-18.

Proposal 7: Reuse the same interference profiles (i.e., cover HomNet and Hetnet scenarios) and same power levels (i.e., INR values) defined for Rel-17 2/4Rx.

Proposal 8: Reuse the same interference signal modulation order as 2/4Rx requirements, i.e., random 16QAM symbols.

Proposal 9: Reuse the same Network Type and SCS/CBW defined for Rel-17 2/4Rx, i.e., 15kHz/10MHz for FDD, and 30kHz/40MHz for TDD with 7D1S2U(S=6D+4G+4U)

Proposal 10: Reuse the same propagation condition defined for Rel-17 2/4Rx, i.e., TDLC300-100 for 2 interfering cells (HomNet) and TDLA30-10 for 1 interfering cell (HetNet) as a start point.

Observation 1: Under the same test configuration with 2/4Rx, the SNR requirement value for 8Rx may be reduced to around 2-3dB due to larger diversity gain and more MMSE-IRC performance gain.

Proposal 11: Investigate simulation results for rank 1 (2x8), 2 (2x8) and 4 (4x8) and choose a reasonable requirement value.

Proposal 12: Perform simulation results for all candidate MCS and make decision based on the results.

Proposal 13: Reuse the same propagation condition defined for Rel-17 2/4Rx, i.e., 15kHz/10MHz for FDD, and 30kHz/40MHz for TDD 7D1S2U

Proposal 14: Reuse the same MU-MIMO Beamforming Model for Rel-17 2/4Rx in B.4.2 in TS38.101-4

Proposal 15: Consider 1 co-UE for new 8Rx requirements and at the same time, we can carefully design the rank allocation between UEs.

Proposal 16: Reuse the same propagation condition as MMSE-IRC, i.e., TDLC300-100 with rank 1+1 and TDLA30-10 with rank 2+2.

Proposal 17: Reuse the same precoding method as MMSE-IRC, i.e., random precoding was used with rank 1+1 and orthogonal precoding was used for rank 2+2 tests, if we consider using the same test setting and PDSCH configuration as MMSE-IRC.

Proposal 18: Cover rank 2+2, 4+4 and 8+8 as a start point and further check whether these configurations are feasible based on simulation results.

Proposal 19: Perform simulation results for all candidate MCS and make decision based on the results.

Proposal 20: Define CQI reporting requirements as below:

  - Baseline SU-MIMO 8Rx receiver with Baseline CQI calculation

  - Simplified SU-MIMO 8Rx receiver with Simplified CQI calculation.

Proposal 21: Reuse the same test metric for Rel-17 2/4Rx:

a)	the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥ 

b)	when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR, the average BLER for the indicated transport formats shall be greater than or equal to X%.

Proposal 22: Reuse the same configuration for Rel-17 2/4Rx, i.e., TDLA30-5 for the serving cell and AWGN for the interference cell.

Proposal 23: Reuse the same configuration for Rel-17 2/4Rx, i.e., INR = 10.04dB with rank 1

Proposal 24: Cover rank 1,2 and 4 for initial study and further make decision based on simulation results.

Proposal 25: The tested SINR value should be decided based on simulation results with reasonable performance gain.

Proposal 26: For the other parameters for 8Rx UE new requirements, we propose to reuse the same configuration for Rel-17 2/4Rx as summarized in Table 1.

Reference

R4-2413570, WF on NR demodulation performance: Phase 5, China Telecom, RAN4#112, Aug 2024.

R4-2120707, WF on general and PDSCH demodulation requirements for inter-cell interference MMSE-IRC, Intel, 3GPP TSG-RAN WG4 Meeting #101-e, Nov 2021