3GPP TSG-RAN WG4 Meeting #112bis                      R4-2414957

Hefei, Anhui, China, 14th – 18th October 2024

Source:	China Telecom

Title:	Views on spatial channel modeling study

Agenda Item:	6.14.1

Document for:	discussion

Introduction

RAN4#112 is the first meeting for the discussion on the spatial channel modeling study, as an outcome, the WF was approved in [1]. In this contribution, our views on the open issues are given.

Discussion

Test Cases for SU-MIMO

Way Forward:

Status in the WF [1]: As a starting point: - 8Rx: - 8 Layer (8T8R) - 8 Layer PDSCH TPUT (Options on precoder choice to be presented at next meeting) - FFS: 8 Layer PMI Reporting (CB Type 1, single panel) - FFS: 4 Layer (4T8R) - FFS: 4 Layer PDSCH TPUT - FFS: 4 Layer PMI Reporting (CB Type 1, single panel) - FFS: 4Rx: - FFS: 4 Layer (4T4R) - FFS: 4 Layer PDSCH TPUT Interested companies to bring views at the next meeting (for comparison and initial alignment).

With the purpose to identify the limitations for legacy TDL modeling, we support to cover PMI reporting in the study item. In Rel-16, RAN4 has compared the performance difference between wideband and sub-band PMI reporting, however, negligible performance gain was observed under TDL channel modeling. It remains doubtful whether sub-band PMI reporting feature is useful for 5G NR.

In this study, we are considering enhanced TDL or CDL channel model which batter reflect the spatial domain propagation condition. Thus, we propose to also cover PMI reporting in the study to better study the limitation of TDL modeling.

Observation 1: In Rel-16, RAN4 has compared the performance difference between wideband and sub-band PMI reporting, however, negligible performance gain was observed under TDL channel modeling.

Proposal 1: Cover PMI reporting in the study to better study the limitation of TDL modeling.

For 8Rx, there are 2 types of receiver implementations, i.e., baseline and simplified. Simplified receiver does not support 8 layers for PDSCH transmission. If we exclude PDSCH transmission with 4 layers, simplified receiver will also be excluded. Thus, we propose to cover 4 Layer (4T8R) in the study.

Proposal 2: Cover 4 Layer (4T8R) in the study.

Test Cases for MU-MIMO

Status in the WF [1]: Companies to bring proposals at the next meeting.

MU-MIMO is widely used in 5G NR deployments and UE demodulation tests under intra cell inter user interference are mandatory for all UEs. We would like to study the performance difference between CDL and TDL under such popular scenarios.

Proposal 3: Cover MU-MIMO scenario in the study.

Conclusion

Observation 1: In Rel-16, RAN4 has compared the performance difference between wideband and sub-band PMI reporting, however, negligible performance gain was observed under TDL channel modeling.

Proposal 1: Cover PMI reporting in the study to better study the limitation of TDL modeling.

Proposal 2: Cover 4 Layer (4T8R) in the study.

Proposal 3: Cover MU-MIMO scenario in the study.

Reference

R4-2413570, Way Forward for [112][327] FS_NR_demod_SCM, Nokia, RAN4#112, Aug 2024.