3GPP TSG-RAN WG4 Meeting #112bis	R4-2414960

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	6.17.4.2

Source:	CAICT

Title:	Discussion on testability and interoperability issues for beam management

Document for:	Discussion

1	Introduction

In this contribution, we are going to discuss the following issue from last meeting.

- Issue 2-1: RSRP prediction accuracy

2	Discussion

2.1 Issue 2-1: RSRP prediction accuracy

Issue 2-1: RSRP Prediction accuracy - Proposals - Option 1: The predicted L1-RSRP difference is similar to the legacy, the absolute RSRP accuracy and the relative RSRP accuracy can be: - Absolute RSRP accuracy= predicted L1-RSRP of beam index i – ideal measured L1-RSRP of beam index I (ground truth as agreed in RAN4#111) - Relative RSRP accuracy= predicted L1-RSRP of beam index i – predicted L1-RSRP of beam index n, where the beam index n owns the largest predicted value. - Accuracy requirements should apply to all predicted beams that satisfy the predefined side conditions, including SNR. - Option 2: others

Regarding to the definition of absolute RSRP accuracy in option 1 (predicted L1-RSRP of beam index i – ideal measured L1-RSRP of beam index I): in legacy tests for measurement accuracy, the power level of SSBs and testing conditions are specified, which allows us to determine benchmark for the actual L1-RSRP of SSBs. Once the benchmark is established, we can define a range of reported measurement values that meet the accuracy requirements. Following this understanding, the latter part, i.e., “ideal measured L1-RSRP of beam index i” in the definition should also serve as the benchmark for prediction accuracy. Therefore, the ideal measurement should meet a more stringent measurement requirement compared to legacy.

Proposal 1: Consider to define a more stringent requirements for ideal measurement used to generate ground truth and discuss how to verify the ideal measurement performance.

Regarding to the definition of relative RSRP accuracy in option 1 (predicted L1-RSRP of beam index i – predicted L1-RSRP of beam index n, where the beam index n owns the largest predicted value): it is inherited from the definition of relative RSRP accuracy for measurement in the specification. The main content of this test is to verify the difference between reported DIFF-RSRP and expected DIFF-RSRP. To obtain the expected DIFF-RSRP, we also see the need of a tighter measurement accuracy as being proposed in proposal 1. As for the requirements of relative RSRP accuracy, it seems there is nothing special compared to legacy, thus legacy relative RSRP requirements could be considered as starting pointing.

Proposal 2: If following the definition of relative RSRP accuracy for prediction in option 1, legacy relative RSRP accuracy requirements could be considered as starting point.

Besides the two listed accuracy, we think another RSRP accuracy metric could also be considered in the test, specifically,

the ideal measured L1-RSRP of beam index i – ideal measured L1-RSRP of beam index k, where the beam index i owns the largest predicted value and beam index k owns the largest measured value.

The metric can reflect the potential performance difference when the best predicted beam is different from the actual best beam based on measurement.

Proposal 3: Suggest to add a new RSRP accuracy metric, specifically, the ideal measured L1-RSRP of beam index i – ideal measured L1-RSRP of beam index k, where the beam index i owns the largest predicted value and beam index k owns the largest measured value.

Conclusion

In this contribution, testability and interoperability issues for beam management are discussed with following proposals:

Proposal 1: Consider to define a more stringent requirements for ideal measurement used to generate ground truth and discuss how to verify the ideal measurement performance.

Proposal 2: If following the definition of relative RSRP accuracy for prediction in option 1, legacy relative RSRP accuracy requirements could be considered as starting point.

Proposal 3: Suggest to add a new RSRP accuracy metric, specifically, the ideal measured L1-RSRP of beam index i – ideal measured L1-RSRP of beam index k, where the beam index i owns the largest predicted value and beam index k owns the largest measured value.