3GPP TSG-RAN WG4 Meeting #112bis	R4-2414963

# Hefei, China,14 – 18 Oct., 2024

Source: 	Huawei, HiSilicon

Title: 	Discussion on A-IoT deployment scenarios and spectrum usage

Agenda Item:	6.21.2.1

Document for:	Approval

Introduction

The SID on solutions for Ambient IoT was revised in RAN#103 [1]. During the previous meeting, the A-IoT deployment scenarios and spectrum usage for device 1 and device 2a were agreed [2]. This contribution further discusses some remaining issues.

Discussion

Deployment scenarios for D1T1

Issue 2-1-1: deployment scenarios for D1T1

Option 2-1: Legacy NR gNB are co-located with AIoT reader and CW. All of NR and AIoT BS/UE/Reader/Device/CW are indoors. AIoT reader /CW and Legacy gNB share same hardware

Option 2-2: Legacy NR gNB are co-located with AIoT reader and CW. All of NR and AIoT BS/UE/Reader/Device/CW are indoors. AIoT reader /CW and Legacy NR gNB do not share same hardware. (less limitation on the power boosting)

Agreement in RAN4#112 [2]

- Feasibility of option 2-1/2-2 for device 1 and 2a can be evaluated with LLS. Parameters for LLS are based on company report.

  - Encourage companies to provide the co-existence simulation results.

Option 2-1 is that NR BS and A-IoT BS share the same hardware, and option 2-2 is that NR BS and A-IoT BS are co-located on the same site.

As discussed in our previous paper [3], assuming a 10MHz RF transmission bandwidth and 180kHz BB LPF, simulation shows that a PSD difference of 24dB is required to avoid affecting envelope detection demodulation. It’s is very challenge for implementation.

Furthermore, option 2-1 and option 2-2, may result in problematic interference issues. In this scenario, there may be a significant sensitivity difference between A-IoT devices (specifically Device 1 and Device 2a) and legacy NR UEs. Interference between indoor NR UEs and indoor A-IoT BS may be problematic due to mutual blocking. When the interference source is the A-IoT micro-BS transmitter, and the victim is the indoor NR UE receiver, the interference signal from A-IoT BS transmitter can reach levels about -36dBm.However, the blocking level for the NR UE receiver is -56dBm, which means that some NR UE receivers may be blocked by the A-IoT micro-BS transmitting signal. Conversely, a few NR UE receivers that receive strong NR signals and are not blocked by the A-IoT may interfere with the A-IoT micro-BS receiver.

Proposal 1: It is difficult to implement Option 2-1 and Option 2-2 due to the significant difference in PSD and interference.

D1T1-A2 vs D1T1-A1

Last meeting it was agreed that D1T1-A2, that is, case 1-1, case 1-2 are included as formal Evaluation cases for co-existence evaluation, while D1T1-A1 is not included.

We believe that D1T1-A2 is stricter than D1T1-A1, because D1T1-A1 offers the advantage of bistatic flexibility and easier attainment of spatial isolation. If D1T1-A2 passes the co-existence evaluation, then it can be assumed that D1T1-A1 will also perform well.

Proposal 2: If D1T1-A2 passes the co-existence evaluation, then it can be assumed that D1T1-A1 with the same spectrum usage will also perform well.

R2D SINR vs BLER

The open issue regarding that RAN4 requires an SINR value as a performance metric for system level simulation.

Assuming BLER=10%, our simulation shows that the Input SNR for a RF envelope detector is required to be around 4 dB, given that the RF BW=10MHz.

Proposal 3: Assuming BLER=10%, the Input SNR for a RF envelope detector is required to be around [4] dB, given that the RF BW=10MHz.

Conclusion

Based on the analysis in this contribution, we have the following observations and proposals:

Proposal 1: It is difficult to implement Option 2-1 and Option 2-2 due to the significant difference in PSD and interference.

Proposal 2: If D1T1-A2 passes the co-existence evaluation, then it can be assumed that D1T1-A1 with the same spectrum usage will also perform well.

Proposal 3: Assuming BLER=10%, the Input SNR for a RF envelope detector is required to be around [4] dB, given that the RF BW=10MHz.

Reference

[1] RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA, RAN #103, Mar. ,2024

[2]R4-2414304, WF on Co-existence study for ambient IoT, CMCC, RAN4#112, Aug, 2024

[3]R4-2412880, Discussion on A-IoT deployment scenarios and spectrum usage, Huawei, HiSilicon, RAN4#112, Aug, 2024