3GPP TSG-RAN WG4 Meeting #112bis	R4-2414964

# Hefei, China,14 – 18 Oct., 2024

Source: 	Huawei, HiSilicon

Title:	On the RF Requirements for Ambient IoT intermediate UE

Agenda Item:	6.21.3.3

Document for:	Approval

1	Introduction

The SID on solutions for Ambient IoT was revised in RAN#103 [1]. The RAN4-led objectives are copied as below:

RAN4-led:

  - Coexistence study of Ambient IoT and NR/LTE.

  - RF requirements study for Ambient IoT:

    - Ambient IoT BS transmission and reception

    - Ambient IoT Device, as per the General Scope, transmission and reception

    - Intermediate node (UE), as per the General Scope, transmission and reception

This paper we further discuss on the potential RF requirements impact on A-IoT intermediate UE based on the WF [2] agreed in RAN4#112.

2	Discussion

For Ambient IoT intermediate UE requirements, we can look at both the legacy NR UE and RFID reader (or Interrogator). We can study if these requirements are necessary to meet the coverage target and ensure optimal coexistence performance.

2.1 TX

To ensure adequate DL coverage and comply with spectrum emission regulations, certain TX requirements are potentially needed. The following was agreed last meeting:

(Copied from R4-2414305 [2])

Agreement in RAN4#112:

- Requirements potentially needed:

  - Maximum output power

  - Transmitted signal quality (Frequency error, EVM)

  - Occupied bandwidth

  - Spectrum emission mask

  - Transmitter Intermodulation

  - ACLR

  - Operating band unwanted emissions

  - Transmitter spurious emissions

- FFS whether requirements are needed or not:

  - Output power dynamics

  - Transmit ON/OFF power

  - Transmit ON/OFF time mask

  - IBE

Here we further discuss these issues.

- Maximum output power

The maximum output power should help to achieve the coverage target and co-existence performance. Our goal is to reuse the legacy NR UE hardware as much as possible and also take advantage of the RFID ecosystem. Considering the widespread use of PC3 NR UE, the maximum output power of PC3 NR UE can serve as a baseline for intermediate UE using UL spectrum.

However, if simulation results show that higher power scenarios can coexist while improving coverage, a maximum output power greater than 23dBm may be not excluded.

Proposal 1: 23dBm of PC3 NR UE can be considered as a start point of maximum output power of intermediate UE.

- Output power dynamics

For legacy NR UE, Output power dynamics includes minimum output power, transmit OFF power, transmit ON/OFF time mask and power control.

Minimum output power: Not necessary for iUE.

Transmit OFF power: May not necessary.

Transmit ON/OFF time mask: May not be necessary. More discussion can be covered for signal quality.

Power control: No need to define power control capability for iUE.

- Transmitted signal quality (Frequency error, EVM)

For frequency error，the legacy NR UE transmitted frequency error requirement could be reused for A-IoT intermediate UE.

For legacy NR UE，transmit modulation quality includes EVM, carrier leakage and  In-band emissions. EVM may not be suitable for R2D OOK waveform. As a reference, the EPC RFID uses RF envelope to measure the transmitted signal quality [3], which could be a viable alternative.

Figure 1 in [3]: Interrogator-to-Tag RF envelope

Table 1 in [3] RF envelope parameters

Proposal 2: RF envelope parameters could be used as a transmit signal quality metric for A-IoT intermediate UE.

Carrier leakage: No need to define.

In-band emissions: No need to define.

- Output RF Spectrum emissions

Output RF spectrum emissions for legacy NR UE includes occupied bandwidth, out of band emission (ACLR), spurious emissions and transmit intermodulation.

Occupied bandwidth: Potentially needed for intermediate UE. The specific value is dependent on the physical layer design of R2D, which has not yet been finalized at this stage.

ACLR: ACLR can be defined based on the co-existence study.

Spurious emissions: Existing spurious emission requirement for legacy NR UE can be applicable.

Transmit Intermodulation: Generally, the transmitter intermodulation requirement is to protect against interference from a nearby Reader. However, in indoor deployments, it is not possible to have co-located Readers on the same band without causing interference to the device. Therefore, there is no need to define transmitter intermodulation for A-IoT intermediate UE.

Proposal 3: No need to define transmit intermodulation for A-IoT intermediate UE.

Based on the above discussion, a summary is captured in the Table 3.

Table 3 RF Requirement for A-IoT intermediate UE- TX part

|  |  |  |  | RF Requirement for A-IoT intermediate UE- TX part |
| --- | --- | --- | --- | --- |
| TX requirement | Maximum output power |  |  | 23dBm of PC3 NR UE can be considered as a start point of maximum output power of intermediate UE. |
|  | Output power dynamics | Minimum output power |  | Not necessary |
|  |  | Transmit OFF power |  | May not necessary. |
|  |  | Transmit ON/OFF time mask |  | May not be necessary. More discussion can be covered for signal quality. |
|  |  | Power control |  | No need to define. |
|  | Transmitted signal quality | Frequency error |  | The legacy NR UE transmitted frequency error requirement could be reused for A-IoT intermediate UE. |
|  |  | Transmit modulation quality | EVM | RF envelope parameters could be used as a transmit signal quality metric for A-IoT intermediate UE. |
|  |  |  | Carrier leakage | No need to define. |
|  |  |  | In-band emissions | No need to define. |
|  | Output RF spectrum emissions | Occupied bandwidth |  | Potentially needed, and is dependent on the physical layer design of R2D. |
|  |  | ACLR |  | ACLR can be defined based on the co-existence study. |
|  |  | Spurious emissions |  | Existing spurious emission requirement for legacy NR UE is applicable. |
|  |  | Transmit intermodulation |  | No need to define transmit intermodulation for A-IoT intermediate UE. |

To summarize, regarding TX requirements, here is our opinion on what may or may not be necessary.

- Requirements potentially needed:

  - Maximum output power

  - Transmitted signal quality (Frequency error, EVM)

  - Occupied bandwidth

  - ACLR

  - Spurious emissions

- Requirements not needed:

  - Output power dynamics (include minimum output power, transmit OFF power, transmit ON/OFF time mask and power control)

  - Transmit Intermodulation

2.1 RX

Certain Rx requirements are needed to ensure UL coverage and meet the coexistence requirements. The following was agreed last meeting:

(Copied from R4-2414305 [2])

Agreement in RAN4#112:

- Requirements potentially needed:

  - Reference sensitivity power level

  - Maximum input power

  - Receiver intermodulation

- FFS whether requirements are needed or not:

  - ACS

  - In-band blocking

  - Out-of-band blocking

  - Narrow band blocking

  - Receiver spurious response

  - Rx spurious emission

- Requirements not needed:

  - ICS

Here we further discuss these items.

- Reference sensitivity power level

Reference sensitivity requirement for intermediate UE could be derived based on the following equation:

PREFSENSdBm=-174dBm+10×log10(BW)+NF+IM+SNR

- Maximum input power

Requirements potentially needed.

- ACS

ACS requirement and the interference level depend on co-existence study.

- In-band blocking

In-band blocking requirement and the interference level depend on co-existence study.

- Out-of-band blocking

Existing out-of-band blocking requirement is applicable.

- Narrow band blocking

FFS.

- spurious response

Existing RX spurious emission requirement is applicable.

- Receiver intermodulation

Requirements potentially needed.

- Spurious emissions

The legacy NR UE receiver spurious emission requirement is applicable.

Based on the above discussion, a summary for RX part is captured in the following Table

Table 4 RF Requirement for A-IoT intermediate UE- RX part

| RF Requirement for A-IoT intermediate UE- RX part |  |  |
| --- | --- | --- |
| RX requirement | Reference sensitivity | Requirement necessary |
|  | Maximum input power | Potentially needed |
|  | ACS | ACS can be defined based on the co-existence study. |
|  | In-band blocking | In-band blocking can be defined based on the co-existence study. |
|  | Out-of-band blocking | Existing out-of-band blocking requirement is applicable. |
|  | Narrow band blocking | FFS |
|  | Spurious response | Existing RX spurious emission requirement is applicable. |
|  | Receiver intermodulation | Needed |
|  | RX spurious emissions | The legacy NR UE receiver spurious emission requirement is applicable |

To summarize, regarding RX requirements, here is our opinion on what may or may not be necessary.

- Requirements potentially needed:

  - Reference sensitivity power level

  - Maximum input power

  - Out-of-band blocking

  - Spurious response

  - Receiver intermodulation

  - Rx spurious emission

- FFS whether requirements are needed or not:

  - ACS

  - In-band blocking

  - Narrow band blocking

Proposal 4: Propose to capture the Table 3 and Table 4 impact of RF Requirements on A-IoT intermediate UE in TR 38.769.

3	Conclusion

In this paper, we have shared our views on the potential RF requirements for ambient IoT intermediate UE. The following proposals are made:

Proposal 1: 23dBm of PC3 NR UE can be considered as a start point of maximum output power of intermediate UE.

Proposal 2: RF envelope parameters could be used as a transmit signal quality metric for A-IoT intermediate UE.

Proposal 3: No need to define transmit intermodulation for A-IoT intermediate UE.

Proposal 4: Propose to capture the Table 3 and Table 4 impact of RF Requirements on A-IoT intermediate UE in TR 38.769.

References

- RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA, RAN #103, Mar. ,2024

- R4-2414305, WF on impacts of A-IoT on RF requirements, Moderator (Huawei), RAN4#112, Aug.,2024

- EPC® Radio-Frequency Identity Generation-2 UHF RFID Standard: Specification for RFID Air Interface Protocol for Communications at 860 MHz – 930 MHz, Jan., 2024