3GPP TSG-RAN WG4 Meeting #112bis	R4-2414976

# Hefei, China,14 – 18 Oct., 2024

Source: 	Huawei, HiSilicon

Title: 					Discussion on BS requirements for ATG enhancements

Agenda Item:	6.10.3

Document for:	Discussion

Introduction

The new WID on Enhancements for Air-to-ground network for NR was approved at the RAN#103 meeting [1] and the revised WID was approved at RAN#105 [2]. The RAN4 BS RF relative objectives are copied as below:

- Specify the RF and RRM core requirements for intra-band co-located and inter-band co-located DL CA [RAN4]:

  - FR1 intra-band contiguous CA

    - Example band combination: CA_n79C

  - FR1+FR1 inter-band CA

    - Example band combination: CA_n3A-n39A

- Specify the RF requirements for support of UL MIMO with 2TX for single CC for UE. [RAN4]

In this paper we provide our view on R19 ATG BS RF requirements.

Discussion

# In the previous RAN4#112 meeting, a WF was agreed as following: [3]

#### Issue 1-1:  Whether to preclude DL MIMO for TAE requirements for ATG BS supports CA?

Way Forward：

- DL MIMO is not included for ATG BS supports CA based on the current WID.

  - RAN4 can re-visit it if there are revised WID to include DL MIMO.

According to the revised WID in RAN#105[2], DL MIMO is not included for ATG BS enhancements. Therefore, we suggest not considering DL MIMO for ATG BS enhancements.

Proposal 1: Preclude DL MIMO for TAE requirements for ATG BS, since DL MIMO is not included in the updated WID.

Another open issue in the WF is as following: [3]

#### Issue 1-2: How to handle CA TAE requirements for ATG BS in TS38.104 based on the current WID?

Way Forward：

- FFS on the wordings on the TAE requirements for ATG BS CA in the TS38.104.

  - Only focus on TS38.104 in RAN4 #112-bis meeting.

In the current TS38.104, the TAE requirements for ATG BS, both conducted and radiated, in sections 6.5.3.1 and 9.6.3 respectively, are described as following：

There is no TAE requirement for ATG BS.

Giving that ATG BS does not support DL MIMO, but support intra-band contiguous CA and inter-band CA, it would be more suitable to revise these sentences to: There is no TAE requirement related to MIMO transmission for ATG BS.

Proposal 2: Revise the sentences “There is no TAE requirement for ATG BS” in 6.5.3.1 and 9.6.3 to “There is no TAE requirement related to MIMO transmission for ATG BS”, i.e.

There is no TAE requirement of MIMO transmission for ATG BS.

Conclusion

In the contribution, we provide our opinions on R19 ATG BS RF requirements and have the following proposals:

Proposal 1: Preclude DL MIMO for TAE requirements for ATG BS, since DL MIMO is not included in the updated WID.

Proposal 2: Revise the sentences “There is no TAE requirement for ATG BS” in 6.5.3.1 and 9.6.3 to “There is no TAE requirement related to MIMO transmission for ATG BS”, i.e.

There is no TAE requirement of MIMO transmission for ATG BS.

Reference

[1] RP-240839, New WID on Enhancements for Air-to-ground network for NR, Moderator (Apple), RAN#103, Mar., 2024

[2] RP-242381, Revised WID on Enhancements for Air-to-ground network for NR, CMCC, CATT, RAN#105, Sept., 2024

[3] R4-2413510, Way Forward for [112][306] NR_ATG_enh, ZTE, RAN4#112, Aug., 2024