3GPP TSG-RAN WG4 Meeting # 112bis      	R4-2415002

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	6.10.4

Source: 	CMCC

Title: 			 (NR_ATG_enh-Core) Discussion on RRM core requirements for R19 ATG

Document for:	Discussion

- Introductions

In last meeting, some general issues has been discussed for Air-to-ground network enhancement, the WF has been approved [1]. In this contribution, we further discuss the RRM impact and potential enhancement considering ATG specific CA scenario.

- Discussion

  - General

Issue 1-1-3: Co-location signaling

- Proposal 1: FFS whether co-location information in inter-band CA configuration should be provided to UE.

Considering only co-located scenario is considered in Rel-19 which is the common deployment, it can be the basic assumption for ATG UE. If non co-located scenario will be specified in future release, the related signaling or information can be introduced by then.

Proposal 1: No need to provide co-location information to ATG UE at least in Rel-19.

Issue 1-1-5: UE antenna type

Agreement:

- For intra-band contiguous CA, same antenna type should be applied on each carrier.

- For inter-band CA, either different or same antenna type can be applied on different carriers.

- For inter-band CA, consider the following cases:

  - 1. ATG UE with omnidirectional antennas on both PCell and SCell.

  - 2. ATG UE with an omnidirectional antenna on PCell and an antenna array on SCell.

  - 3. ATG UE with an antenna array on PCC and an omnidirectional antenna on SCell.

  - [4. ATG UE with an antenna array on both PCell and SCell with only one antenna panel.]

  - 5. ATG UE with an antenna array on both PCell and SCell with separate antenna panels.

  - Note: If some of the cases will be precluded in RF discussion, they will be removed for RRM requirement discussion.

Agreement from RAN4#111 of UE RF:

- No limitation on antenna types for ATG CA

- FFS on whether to assume omni-antenna type can be assumed for both band n3 and n39 in DL CA_n3-n39

- FFS on whether new capability is needed for ATG CA

Agreement from RAN4#112 of UE RF:

- Omni-antenna type can be assumed for both band n3 and n39.

Based on the current progress of RF discussion, for inter-band CA Case#1 to Case#5, no case is precluded. Especially, for Case#4, even for inter-band CA, when the component CCs are near to each other, e.g., CA_n39-n34, CA_n40-n41, one antenna panel can be shared between two CCs.

Proposal 2: For inter-band CA, consider the following cases:

- 1. ATG UE with omnidirectional antennas on both PCell and SCell.

- 2. ATG UE with an omnidirectional antenna on PCell and an antenna array on SCell.

- 3. ATG UE with an antenna array on PCC and an omnidirectional antenna on SCell.

- 4. ATG UE with an antenna array on both PCell and SCell with only one antenna panel. (can only form one Rx beam)

- 5. ATG UE with an antenna array on both PCell and SCell with separate antenna panels. (can form two simultaneous Rx beams)

UL Tx switching

For the purpose of UL capacity enhancement, 3GPP introduced UL Tx switching. During the discussion in last meeting, it seems necessary to clarify whether UL Tx switching shall be introduced for ATG or not.

In order to support UL Tx switching, UE is mandatory to support 2Tx and UL-MIMO on SCC, while UL-MIMO related requirement is being defined in Rel-19.

Further considering UL capacity is not the bottleneck for ATG scenario, we propose not to consider UL carrier switching at least in this release.

Proposal 3: No need to introduce UL carrier switching for ATG at least in Rel-19.

  - RRC IDLE/INACTIVE mobility requirements

Issue 1-2-1: Cell re-selection Idle/Inactive mode CA measurements

- Proposal 1: RAN4 to reuse NR cell re-selection as a baseline for ATG cell re-selection to support CA.

- Proposal 2: For cell re-selection, define the exceptions of side conditions for UE supporting CA in FR1. The legacy methodology exception applicability defined in Clause 4.2 and Clause B.3.2.1 can be reused with update of the reference clause of ΔRIB,c.

Although RF defined ΔRIB,c = 0 for NR inter-band CA_n3A-n39A, considering other CA combination may be introduced in future release, for forward compatibility, we propose to add the side conditions exceptions for cell re-selection, and reuse the legacy methodology exception applicability defined in Clause 4.2 and Clause B.3.2.1 with update of the reference clause of ΔRIB,c. We copy it for information:

In the requirements of clause 4.2, the exceptions for side conditions apply as follows: - for the UE capable of CA, the applicable exceptions for side conditions are specified in Annex B, clause B.3.2.1 for UE supporting CA in FR1;

B.3.2.1 Receiver sensitivity relaxation for UE supporting CA in FR1 For a UE configured with inter-band carrier aggregation with active uplink in NR band, if there is a relaxation of receiver sensitivity ΔRIB,c>0 dB as defined in clause 7.3A.3 of TS 38.101-1 [18], the relevant side conditions specifying received power levels (SSB_RP and Io) shall be increased by the amount Δ=ΔRIB,c defined for the corresponding downlink NR bands.

Proposal 4: For cell re-selection, define the exceptions of side conditions for UE supporting CA in FR1. The legacy methodology exception applicability defined in Clause 4.2 and Clause B.3.2.1 can be reused with update of the reference clause of ΔRIB,c.

Issue 1-2-2: Idle/Inactive mode CA measurements (R16)

- Option 1: Introduce idle/inactive mode CA measurements requirements for ATG UE in Rel-19

- Option 2: No need to introduce idle/inactive mode CA measurements requirements in ATG CA.

Issue 1-2-3: Measurement report for fast CA/DC setup (R18)

- Option 1: RAN4 to define EMR for ATG DL CA by using the Rel-18 NR CA EMR as a baseline

- Option 2: No need to introduce Rel-18 NR CA EMR in ATG CA.

- Option 3: Consider whether to define the requirements of measurement report for fast CA setup in idle/inactive mode for R19 ATG UE

The feature EMR (early measurement report) was introduced to facilitate the SCell addition, help network to configure a proper SCell by reporting the configured SCC measurement result in RRC_IDLE/INACTIVE mode.

However, in ATG co-located scenario, network could derive coarse SCell quality by PCell quality and known frequency gap information, we fail to see strong demand and significant gain to introduce this feature.

Observation 1: Fail to see strong demand and significant gain to introduce EMR in ATG co-located deployment.

Proposal 5: No need to introduce EMR in ATG CA if no strong demand and significant gain observed.

  - Timing requirements

Issue 1-3-1: MTTD

- Option 1: FFS whether MTTD requirement is needed or not

As defined in TS 38.133, for inter-band CA, when UE needs to transmit among different carriers, The UE shall be capable of handling at least a relative transmission timing difference between slot timing of all pairs of TAGs (pTAG and sTAG).

In ATG inter-band DL CA scenario, if we assume there is only one UL carrier during whole transmission, then the MTTD requirement is not needed at all. If we assume UL Tx switching will happen during the transmission, then need to define the MTTD requirement.

As we analyzed for proposal 3, since no need to introduce UL Tx switching for ATG, neither is there any need to introduce MTTD requirement.

Proposal 6: No need to define MTTD requirement.

Issue 1-3-2: MRTD

Agreement:

- For ATG intra-band contiguous CA, not define MRTD requirement.

- For ATG inter-band CA, MRTD requirement is:

  - Option 1: A little larger than 3us, i.e., 3.545 µs

  - Option 2: 33us

  - Option 3: 3us

  - Option 4: further discuss

For inter-band CA, the legacy FR1 MRTD requirement is 33µs which including 3µs TAE and propagation delay difference. Although propagation delay difference between PCell and SCell is near to 0 in ATG co-located deployment, we think 33µs MRTD can be reused for ATG UE which is more powerful than legacy hand-held UE, such approach is also more friendly to forward compatibility.

Proposal 7: Define MRTD requirement for ATG inter-band CA, legacy 33µs requirement can be reused.

  - Signalling characteristics requirements

Issue 1-4-2: SCell Activation and Deactivation Delay

- Proposal 1: SCell Activation and Deactivation Delay requirements should be introduced for ATG

  - Proposal 1-1: SCell activation requirements shall be further investigated for potential impact, taking into account the band combinations stated in the WID and the antenna types agreed in RF

  - Proposal 1-2: Reuse the principle from existing SCell activation / deactivation delay requirements as baseline for ATG, and scellWithoutSSB capability can be considered.

  - Proposal 1-3: Introduce following features for ATG, reuse the current FR1 requirements as the starting point, and further study whether further improvement is needed or not due to co-located deployment:

    - SCell Activation Delay Requirement for Deactivated SCell

    - SCell Deactivation Delay Requirement for Activated SCell

    - Direct SCell Activation at SCell addition

    - Direct SCell Activation at Handover

    - Direct SCell Activation at RRC Resume

    - Fast SCell Activation Delay Requirement for Deactivated SCell

    - SCell Activation Delay Requirement for Deactivated SCell with the L3 reporting during activation.

  - Proposal 1-4: RAN4 to define SCell activation for single-SCell for at least:

    - SCell activation

    - Direct SCell activation at SCell addition

    - Direct SCell activation at Handover

  - Proposal 1-5: Due to co-located deployment, similar beam coverage shared between multiple serving cells, the component of cell search can be ignored, whether L1 meas&report can be ignored, which can be discussed.

In our perspective, the following features can be introduced for ATG, the current FR1 requirement can be the starting point. To be mentioned, the requirements related to multiple downlink SCells are deprioritized since example bands are 2CCs in WID.

- SCell Activation Delay Requirement for Deactivated SCell

- SCell Deactivation Delay Requirement for Activated SCell

- Direct SCell Activation at SCell addition

- Direct SCell Activation at Handover

- Direct SCell Activation at RRC Resume

- Fast SCell Activation Delay Requirement for Deactivated SCell

- SCell Activation Delay Requirement for Deactivated SCell with the L3 reporting during activation.

Proposal 8: For SCell activation and deactivation, following features can be introduced for ATG, reuse the current FR1 requirements as the starting point.

- SCell Activation Delay Requirement for Deactivated SCell

- SCell Deactivation Delay Requirement for Activated SCell

- Direct SCell Activation at SCell addition

- Direct SCell Activation at Handover

- Direct SCell Activation at RRC Resume

- Fast SCell Activation Delay Requirement for Deactivated SCell

- SCell Activation Delay Requirement for Deactivated SCell with the L3 reporting during activation.

For SCell activation, the SSB-less SCell related requirement for intra-band CA and inter-band CA can be considered, considering the side conditions are easily fulfilled in co-located scenario, network energy saving gain can be achieved and activation delay can be decreased. The existing UE capability and requirement can be reused.

Proposal 9: Define SSB-less SCell related requirement for intra-band CA and inter-band CA. Reuse the existing UE capability and requirement.

Here, considering ATG specific scenario including co-located CA, large cell coverage, LOS propagation condition, we further consider SCell addition and activation improvement methodology.

With LOS propagation condition and co-located CA deployment, the coverage of PCell and SCell are relatively fixed, as shown in figure below.

In this case, SCell quality is highly related with distance between UE and cell center. We propose to consider distance triggered SCell addition and/or activation, which has following advantages:

1. helping UE to reduce power consumption from measurement for SCell addition and/or activation.

2. For ATG UE with an antenna array on both PCell and SCell with only one antenna panel, due to measurement for SCell addition can be omitted, no need to apply scheduling restriction for neighbour cell measurement on candidate SCC, throughput gain can be achieved.

Proposal 10: Further consider distance triggered SCell addition and/or activation.

Issue 1-4-3: Active BWP switch delay on multiple CCs

- Proposal 1: For Active BWP switch delay on multiple CCs, introduce following requirements, which can reuse the legacy:

  - Simultaneous DCI based BWP switch delay on multiple CCs

  - Simultaneous RRC based BWP switch delay on multiple CCs

  - Simultaneous and non-simultaneous Timer based BWP switch delay on multiple CCs

Active BWP switch has been introduced for single CC case. It is natural to also introduce the feature in multiple CCs case.

For DCI based BWP switch delay and RRC based BWP switch delay on multiple CCs , due to NR-DC is not in the scope, only simultaneous BWP switch delay scenario need to be considered, and the legacy requirement can be reused.

For Timer based BWP switch delay on multiple CCs, both simultaneous and non-simultaneous BWP switch delay scenario should be considered, and the legacy requirement can be reused.

Proposal 11: For Active BWP switch delay on multiple CCs, introduce following requirements, which can reuse the legacy:

- Simultaneous DCI based BWP switch delay on multiple CCs

- Simultaneous RRC based BWP switch delay on multiple CCs

- Simultaneous and non-simultaneous Timer based BWP switch delay on multiple CCs

Issue 1-4-4: Beam failure recovery in SCell

- Proposal 1: Introduce Beam failure recovery in SCell for ATG CA, the legacy procedure and requirement can be reused.

The delay requirement for beam failure recovery in SCell shall be considered, the legacy procedure and requirement can be reused.

Proposal 12: Introduce Beam failure recovery in SCell for ATG CA, the legacy procedure and requirement can be reused.

Issue 1-4-5: Pre-configured measurement gap activation/deactivation delay upon SCell activation/deactivation

- Proposal 1: Introduce Pre-configured measurement gap activation/deactivation delay requirement upon SCell activation/deactivation, the legacy requirement can be reused.

In Rel-18 ATG single carrier scenario, the pre-configured measurement gap is supported. When CA is supported Rel-19 ATG, it is natural to introduce the SCell activation/deactivation as an event for triggering pre-configured measurement gap activation/deactivation, the legacy requirement can be reused.

Proposal 13: Introduce Pre-configured measurement gap activation/deactivation delay requirement upon SCell activation/deactivation, the legacy requirement can be reused.

Issue 1-4-1: Interruption requirements

- Proposal 1: Interruption requirements should be introduced for ATG UE

  - Proposal 1-1: Interruption requirements should be defined for following features, legacy requirement can be reused by focusing on the applicable part for ATG by considering the same corresponding scenario

    - Requirement applicability, some adaptation is needed depending on the supported feature by ATG UE

    - Interruption at Scell addition/release

    - Interruptions at SCell activation/deactivation

    - Interruptions during measurements on deactivated SCC

    - Interruptions at UL carrier RRC reconfiguration

    - Interruptions due to Active BWP switching Requirement

    - Interruptions at inter-frequency SFTD measurement

    - Interruptions due to UE-specific CBW change

    - Interruptions at NR SRS carrier based switching

    - Interruptions at direct SCell activation

    - Interruptions due to SCell dormancy

    - Interruptions at NR SRS antenna port switching

    - Interruptions at fast SCell activation

    - Interruptions due to PUCCH SCell activation/deactivation

    - Interruptions due to measurements without gap carried out by UE supporting [NeedForInterruptionInfoNR-R18]

    - FFS:

      - DL Interruptions at UE switching between two uplink carriers

      - DL Interruptions at UE switching between two uplink carriers with two transmit antenna connectors

      - DL Interruptions at UE switching between two uplink bands with two transmit antenna connectors

      - DL Interruptions at UE switching across three or four uplink bands

      - Interruptions when identifying CGI of an NR cell with autonomous gaps

  - Proposal 1-2: Reuse the principle from existing interruption requirements as baseline for ATG for:

    - Interruptions at SCell activation/deactivation

    - Interruptions during measurement on deactivated SCell

  - Proposal 1-3: Introduce the following interruption requirement, the legacy requirement can be reused.

    - Interruptions at SCell addition/release

    - Interruptions at SCell activation/deactivation

    - Interruptions at direct SCell activation

    - Interruptions at fast SCell activation

    - Interruptions during measurements on deactivated SCC

    - Interruptions due to SCell dormancy

    - Interruptions due to Active BWP switching Requirement

    - Interruptions due to UE-specific CBW change

    - Interruptions when identifying CGI of an NR cell with autonomous gaps

    - Interruptions at NR SRS antenna port switching

  - Proposal 1-4: Existing interruption requirements can also apply to ATG supporting CA. RAN4 could discuss whether to introduce new interruption requirements if there are ATG new procedures which are not covered in current interruption requirements.

  - Proposal 1-5: For both omni-directional antenna and antenna array assumption, reuse existing interruption requirements of SCell addition/release.

For the following cases, the interruption will occur on PCell and activated SCell.

- SCell addition/release

- SCell activation/deactivation

- Direct SCell activation

- Fast SCell activation

- Measurements on deactivated SCC

- SCell dormancy

- Active BWP switching

- UE-specific CBW change

- When identifying CGI of an NR cell with autonomous gaps

- NR SRS antenna port switching

The interruption requirement shall be defined, the legacy requirement can be reused for ATG.

Proposal 14: Introduce the following interruption requirement, the legacy requirement can be reused.

- Interruptions at SCell addition/release

- Interruptions at SCell activation/deactivation

- Interruptions at direct SCell activation

- Interruptions at fast SCell activation

- Interruptions during measurements on deactivated SCC

- Interruptions due to SCell dormancy

- Interruptions due to Active BWP switching Requirement

- Interruptions due to UE-specific CBW change

- Interruptions when identifying CGI of an NR cell with autonomous gaps

- Interruptions at NR SRS antenna port switching

  - Measurement procedure requirement

Issue 1-5-1: Measurement gap

- Proposal 1: RAN4 to reuse NR carrier aggregation measurement gap as the baseline for ATG DL CA

- Proposal 2: The measurement gap applicability rule and scheduling applicability for SA single carrier shall be also applied for NR CA configuration.

- Proposal 3: When define the SCC interruption time, only consider aligned frame boundaries scenario, the interruption requirement in single carrier case can be reused for CA configuration.

Due to only FR1 CA configured, the measurement gap applicability rule and scheduling applicability for SA single carrier shall be also applied for NR CA configuration.

Proposal 15: The measurement gap applicability rule and scheduling applicability for SA single carrier shall be also applied for NR CA configuration.

Besides, since R19 only investigate co-located CA, then only aligned frame boundaries scenario will be existed. The interruption time on an SCC is same as the single carrier case.

Proposal 16: When define the SCC interruption time, only consider aligned frame boundaries scenario, the interruption requirement in single carrier case can be reused for CA configuration.

Issue 1-5-2: CSSF

- Proposal 1: Study and accommodate CSSF requirements for ATG DL CA

- Proposal 2: Reuse the legacy scheme when defining the CSSFoutside_gap for ATG FR1 only CA, that PCC occupy one measurement searcher resource, SCC and other inter-frequency MO with no measurement gap share another measurement searcher resource.

Due to both PCC and SCC need to be measured under CA configuration, the CSSFoutside_gap shall be updated to cover SCC measurement. The legacy scaling scheme can be reused, that PCC occupy one measurement searcher resource, SCC and other inter-frequency MO with no measurement gap share another measurement searcher resource.

Proposal 17: Reuse the legacy scheme when defining the CSSFoutside_gap for ATG FR1 only CA, that PCC occupy one measurement searcher resource, SCC and other inter-frequency MO with no measurement gap share another measurement searcher resource.

Issue 1-5-3: Capabilities for Support of Event Triggering and Reporting Criteria

- Proposal 1: For UE capability of supporting NR reporting criteria of category intra-frequency, the SCell serving frequency shall be counted in the number of configured NR serving frequencies carrier frequency.

For UE capability of supporting NR reporting criteria of category intra-frequency, the SCell serving frequency shall be counted in the number of configured NR serving frequencies carrier frequency.

Proposal 18: For UE capability of supporting NR reporting criteria of category intra-frequency, the SCell serving frequency shall be counted in the number of configured NR serving frequencies carrier frequency.

Issue 1-5-4: Pre-configured measurement gap requirement

- Proposal 1: Introduce the SCell related triggering conditions as an event for triggering pre-configured measurement gap, and the legacy procedure and requirement can be reused.

When define the requirements and the requirement applicability for network-controlled and autonomous activation/deactivation mechanism, it is natural to introduce the SCell related triggering conditions as an event for triggering pre-configured measurement gap, and the legacy procedure and requirement can be reused.

Proposal 19: Introduce the SCell related triggering conditions as an event for triggering pre-configured measurement gap, and the legacy procedure and requirement can be reused.

Issue 1-5-5: SCell activation triggered measurement reporting requirement

- Proposal 1: If SCell Activation for Deactivated SCell with the L3 reporting during activation is supported for ATG, then the measurement reporting requirement shall also be introduced, legacy requirement can be reused.

If SCell Activation for Deactivated SCell with the L3 reporting during activation is supported for ATG, then the measurement reporting requirement shall also be introduced, legacy requirement can be reused.

Proposal 20: If SCell Activation for Deactivated SCell with the L3 reporting during activation is supported for ATG, then the measurement reporting requirement shall also be introduced, legacy requirement can be reused.

Issue 1-5-7: Deactivated SCell measurement

- Proposal 1: Define intra-frequency measurement requirements for deactivated SCell

  - Option 1-1: Reuse the principle from the existing cell identification and measurement requirements with measCycleScell as baseline for ATG, for example

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | ceil( 5 x Kp x N1 Note 1 x Klayer1_measurement) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | ceil(5 x Kp x N1 Note 1 x Klayer1_measurement) x max(measCycleSCell, 1.5 x DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil( 5 x Kp x N1 Note 2 x Klayer1_measurement) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: For ATG UE with the antenna array, N1 = 3 when network assistance on ATG cells reference locations is provided, otherwise N1 = 4. Otherwise, N1 = 1. |  |

  - Option 1-2: Introduce Time period for PSS/SSS detection, Time period for time index detection, Measurement period for intra-frequency measurements without gaps for FR1 deactivated SCell. The legacy requirement can be reused, for ATG UE with one or more omni-directional antenna and ATG UE with antenna array on SCC.

  - Option 1-3: For the deactivated SCell measurement, whether need to introduce smaller candidate of measCycleSCell to facilitate the small ISD ATG deployment, which can be considered.

The deactivated SCell measurement requirement shall be introduced, including Time period for PSS/SSS detection, Time period for time index detection, Measurement period for intra-frequency measurements without gaps for FR1 deactivated SCell. The requirements shall be discussed case by case according to different antenna type.

For ATG UE with one or more omni-directional antenna on SCC, the legacy requirement can be reused, which defined in Table 9.2.5.1-4, Table 9.2.5.1-6, and Table 9.2.5.2-3 in TS 38.133.

For ATG UE with antenna array on SCC, due to co-located deployment, UE could determine beam direction by serving gNB location, thus, the legacy requirement can be reused.

Proposal 21: Introduce Time period for PSS/SSS detection, Time period for time index detection, Measurement period for intra-frequency measurements without gaps for FR1 deactivated SCell. The legacy requirement can be reused, for ATG UE with one or more omni-directional antenna and ATG UE with antenna array on SCC.

Issue 1-5-8: Scheduling Availability/Restrictions

- Proposal 1: the scheduling restriction extension to intra-band/inter-band CA, further study the impact of antenna pattern on scheduling availability ATG CA

- Proposal 2: For scheduling availability of UE performing measurement with a different subcarrier spacing than PDSCH/PDCCH for RLM, BFD, CBD, intra/inter-frequency measurement without MG, L1-RSRP measurement, and L1-SINR measurement

  - When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions on FR1 serving PCell should apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols.

  - When inter-band carrier aggregation within FR1 is configured, there are no scheduling restrictions on FR1 serving cell(s) configured in other bands than the bands in which PCell is configured.

- Proposal 3: For scheduling availability of UE performing measurements on FR1 TDD band for intra/inter-frequency measurement without MG and CSI-RS based intra-frequency measurements

  - When TDD intra-band carrier aggregation is performed, the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with aforementioned restricted symbols.

  - When TDD inter-band carrier aggregation is performed, the scheduling restrictions due to a given serving cell should also apply to another serving cell in a different band on the symbols that fully or partially overlap with the aforementioned restricted symbols, if UE does not have the capability of supporting simultaneousRxTxInterBandCA for this band pair.

- Proposal 4: For scheduling availability of UE with the antenna array performing measurements on FR1 for intra/inter-frequency measurement without MG and CSI-RS based intra-frequency measurements:

  - When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with aforementioned restricted symbols.

  - When inter-band carrier aggregation is performed:

    - For ATG UE with the antenna array on PCC and one or more omni-directional antenna on SCC, the scheduling restrictions due to PCell will not apply to another serving cell in a different band.

    - For ATG UE with the one or more omni-directional antenna on PCC and antenna array on SCC, the scheduling restrictions due to SCell will not apply to another serving cell in a different band.

    - For ATG UE with the antenna array on both PCC and SCC, and only one Rx beam can be formed, the scheduling restrictions due to a given serving cell should also apply to another serving cell in a different band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

    - For ATG UE with the antenna array on both PCC and SCC, and two Rx beams can be formed simultaneously, he scheduling restrictions due to a given serving cell will not apply to another serving cell in a different band.

Scheduling availability of UE performing measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1

For RLM, BFD, CBD, intra/inter-frequency measurement without MG, L1-RSRP measurement, L1-SINR measurement, scheduling availability of UE performing measurement with a different subcarrier spacing than PDSCH/PDCCH shall be discussed for CA configuration.

When intra-band carrier aggregation in FR1 is configured, due to one set of receiver will be applied on each carrier, the scheduling restrictions on FR1 serving PCell should apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols.

When inter-band carrier aggregation within FR1 is configured, due to UE mandatory capable of two sets of receiver, there are no scheduling restrictions on FR1 serving cell(s) configured in other bands than the bands in which PCell is configured.

Proposal 22: For scheduling availability of UE performing measurement with a different subcarrier spacing than PDSCH/PDCCH for RLM, BFD, CBD, intra/inter-frequency measurement without MG, L1-RSRP measurement, and L1-SINR measurement

- When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions on FR1 serving PCell should apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols.

- When inter-band carrier aggregation within FR1 is configured, there are no scheduling restrictions on FR1 serving cell(s) configured in other bands than the bands in which PCell is configured.

Scheduling availability of UE performing measurements on FR1 TDD band

For intra/inter-frequency measurement without MG and CSI-RS based intra-frequency measurements, scheduling availability of UE performing measurements on FR1 TDD band shall be discussed for TDD CA configuration.

When TDD intra-band carrier aggregation is performed, due to one set of antenna and receiver will be applied on each carrier, the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with aforementioned restricted symbols.

When TDD inter-band carrier aggregation is performed, the scheduling restrictions due to a given serving cell should also apply to another serving cell in a different band on the symbols that fully or partially overlap with the aforementioned restricted symbols, if UE does not have the capability of supporting simultaneousRxTxInterBandCA for this band pair.

Proposal 23: For scheduling availability of UE performing measurements on FR1 TDD band for intra/inter-frequency measurement without MG and CSI-RS based intra-frequency measurements

- When TDD intra-band carrier aggregation is performed, the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with aforementioned restricted symbols.

- When TDD inter-band carrier aggregation is performed, the scheduling restrictions due to a given serving cell should also apply to another serving cell in a different band on the symbols that fully or partially overlap with the aforementioned restricted symbols, if UE does not have the capability of supporting simultaneousRxTxInterBandCA for this band pair.

Scheduling availability of UE with the antenna array performing measurements on FR1

For intra/inter-frequency measurement without MG and CSI-RS based intra-frequency measurements, scheduling availability of UE with the antenna array performing measurements on FR1 shall be discussed.

When intra-band carrier aggregation in FR1 is configured, due to one set of antenna will be applied on each carrier, the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with aforementioned restricted symbols.

When inter-band carrier aggregation is performed:

For ATG UE with the antenna array on PCC and one or more omni-directional antenna on SCC, the scheduling restrictions due to PCell will not apply to another serving cell in a different band.

For ATG UE with the one or more omni-directional antenna on PCC and antenna array on SCC, the scheduling restrictions due to SCell will not apply to another serving cell in a different band.

For ATG UE with the antenna array on both PCC and SCC, and only one Rx beam can be formed, the scheduling restrictions due to a given serving cell should also apply to another serving cell in a different band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

For ATG UE with the antenna array on both PCC and SCC, and two Rx beams can be formed simultaneously, the scheduling restrictions due to a given serving cell will not apply to another serving cell in a different band.

Proposal 24: For scheduling availability of UE with the antenna array performing measurements on FR1 for intra/inter-frequency measurement without MG and CSI-RS based intra-frequency measurements:

- When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with aforementioned restricted symbols.

- When inter-band carrier aggregation is performed:

  - For ATG UE with the antenna array on PCC and one or more omni-directional antenna on SCC, the scheduling restrictions due to PCell will not apply to another serving cell in a different band.

  - For ATG UE with the one or more omni-directional antenna on PCC and antenna array on SCC, the scheduling restrictions due to SCell will not apply to another serving cell in a different band.

  - For ATG UE with the antenna array on both PCC and SCC, and only one Rx beam can be formed, the scheduling restrictions due to a given serving cell should also apply to another serving cell in a different band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

  - For ATG UE with the antenna array on both PCC and SCC, and two Rx beams can be formed simultaneously, the scheduling restrictions due to a given serving cell will not apply to another serving cell in a different band.

- Conclusion

In this contribution, we identify the potential RRM impact of ATG features. The proposals are:

Proposal 1: No need to provide co-location information to ATG UE at least in Rel-19.

Proposal 2: For inter-band CA, consider the following cases:

- 1. ATG UE with omnidirectional antennas on both PCell and SCell.

- 2. ATG UE with an omnidirectional antenna on PCell and an antenna array on SCell.

- 3. ATG UE with an antenna array on PCC and an omnidirectional antenna on SCell.

- 4. ATG UE with an antenna array on both PCell and SCell with only one antenna panel. (can only form one Rx beam)

- 5. ATG UE with an antenna array on both PCell and SCell with separate antenna panels. (can form two simultaneous Rx beams)

Proposal 3: No need to introduce UL carrier switching for ATG at least in Rel-19.

Proposal 4: For cell re-selection, define the exceptions of side conditions for UE supporting CA in FR1. The legacy methodology exception applicability defined in Clause 4.2 and Clause B.3.2.1 can be reused with update of the reference clause of ΔRIB,c.

Observation 1: Fail to see strong demand and significant gain to introduce EMR in ATG co-located deployment.

Proposal 5: No need to introduce EMR in ATG CA if no strong demand and significant gain observed.

Proposal 6: No need to define MTTD requirement.

Proposal 7: Define MRTD requirement for ATG inter-band CA, legacy 33µs requirement can be reused.

Proposal 8: For SCell activation and deactivation, following features can be introduced for ATG, reuse the current FR1 requirements as the starting point.

- SCell Activation Delay Requirement for Deactivated SCell

- SCell Deactivation Delay Requirement for Activated SCell

- Direct SCell Activation at SCell addition

- Direct SCell Activation at Handover

- Direct SCell Activation at RRCResume

- Fast SCell Activation Delay Requirement for Deactivated SCell

- SCell Activation Delay Requirement for Deactivated SCell with the L3 reporting during activation.

Proposal 9: Define SSB-less SCell related requirement for intra-band CA and inter-band CA. Reuse the existing UE capability and requirement.

Proposal 10: Further consider distance triggered SCell addition and/or activation.

Proposal 11: For Active BWP switch delay on multiple CCs, introduce following requirements, which can reuse the legacy:

- Simultaneous DCI based BWP switch delay on multiple CCs

- Simultaneous RRC based BWP switch delay on multiple CCs

- Simultaneous and non-simultaneous Timer based BWP switch delay on multiple CCs

Proposal 12: Introduce Beam failure recovery in SCell for ATG CA, the legacy procedure and requirement can be reused.

Proposal 13: Introduce Pre-configured measurement gap activation/deactivation delay requirement upon SCell activation/deactivation, the legacy requirement can be reused.

Proposal 14: Introduce the following interruption requirement, the legacy requirement can be reused.

- Interruptions at SCell addition/release

- Interruptions at SCell activation/deactivation

- Interruptions at direct SCell activation

- Interruptions at fast SCell activation

- Interruptions during measurements on deactivated SCC

- Interruptions due to SCell dormancy

- Interruptions due to Active BWP switching Requirement

- Interruptions due to UE-specific CBW change

- Interruptions when identifying CGI of an NR cell with autonomous gaps

- Interruptions at NR SRS antenna port switching

Proposal 15: The measurement gap applicability rule and scheduling applicability for SA single carrier shall be also applied for NR CA configuration.

Proposal 16: When define the SCC interruption time, only consider aligned frame boundaries scenario, the interruption requirement in single carrier case can be reused for CA configuration.

Proposal 17: Reuse the legacy scheme when defining the CSSFoutside_gap for ATG FR1 only CA, that PCC occupy one measurement searcher resource, SCC and other inter-frequency MO with no measurement gap share another measurement searcher resource.

Proposal 18: For UE capability of supporting NR reporting criteria of category intra-frequency, the SCell serving frequency shall be counted in the number of configured NR serving frequencies carrier frequency.

Proposal 19: Introduce the SCell related triggering conditions as an event for triggering pre-configured measurement gap, and the legacy procedure and requirement can be reused.

Proposal 20: If SCell Activation for Deactivated SCell with the L3 reporting during activation is supported for ATG, then the measurement reporting requirement shall also be introduced, legacy requirement can be reused.

Proposal 21: Introduce Time period for PSS/SSS detection, Time period for time index detection, Measurement period for intra-frequency measurements without gaps for FR1 deactivated SCell. The legacy requirement can be reused, for ATG UE with one or more omni-directional antenna and ATG UE with antenna array on SCC.

Proposal 22: For scheduling availability of UE performing measurement with a different subcarrier spacing than PDSCH/PDCCH for RLM, BFD, CBD, intra/inter-frequency measurement without MG, L1-RSRP measurement, and L1-SINR measurement

- When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions on FR1 serving PCell should apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols.

- When inter-band carrier aggregation within FR1 is configured, there are no scheduling restrictions on FR1 serving cell(s) configured in other bands than the bands in which PCell is configured.

Proposal 23: For scheduling availability of UE performing measurements on FR1 TDD band for intra/inter-frequency measurement without MG and CSI-RS based intra-frequency measurements

- When TDD intra-band carrier aggregation is performed, the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with aforementioned restricted symbols.

- When TDD inter-band carrier aggregation is performed, the scheduling restrictions due to a given serving cell should also apply to another serving cell in a different band on the symbols that fully or partially overlap with the aforementioned restricted symbols, if UE does not have the capability of supporting simultaneousRxTxInterBandCA for this band pair.

Proposal 24: For scheduling availability of UE with the antenna array performing measurements on FR1 for intra/inter-frequency measurement without MG and CSI-RS based intra-frequency measurements:

- When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with aforementioned restricted symbols.

- When inter-band carrier aggregation is performed:

  - For ATG UE with the antenna array on PCC and one or more omni-directional antenna on SCC, the scheduling restrictions due to PCell will not apply to another serving cell in a different band.

  - For ATG UE with the one or more omni-directional antenna on PCC and antenna array on SCC, the scheduling restrictions due to SCell will not apply to another serving cell in a different band.

  - For ATG UE with the antenna array on both PCC and SCC, and only one Rx beam can be formed, the scheduling restrictions due to a given serving cell should also apply to another serving cell in a different band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

  - For ATG UE with the antenna array on both PCC and SCC, and two Rx beams can be formed simultaneously, the scheduling restrictions due to a given serving cell will not apply to another serving cell in a different band.

- Reference

- R4-2413959, WF on RRM requirements for NR_ATG_enh, Moderator (CMCC).