3GPP TSG-RAN WG4 Meeting #112bis			R4-2415010

# Hefei, Anhui, China, 14th - 18th October, 2024

Agenda Item:	6.26.4.2

Source: 	CMCC

Title: 			   (NR_NTN_Ph3-Core) Discussion on the RRM requirement for NTN phase3

Document for:	Discussion

- Introduction

In last meeting, RAN4 first identified that the objectives DL coverage enhancement and support of regenerative payload have RAN4 impact. The WF has been approved in [1]. In this contribution, we further discuss the RRM impact and requirements.

- Downlink coverage enhancements

Issue 2-1-1: RRM impact of SSB periodicity enhancement

<Way Forward> : Further discuss the following proposal

- Proposal 1 (CATT, Apple, Xiaomi, Samsung, HW, QC):

  - RAN4 to wait for more progress from RAN1/2 on SSB periodicity extension and other possible system level enhancements to discuss the RRM impacts of DL coverage enhancement.

  - Proposal 1a (Samsung):

    - If SSB periodicity is extended, almost all RRM requirements should be impacted including idle mode/inactive mode and connected mode such as: cell (re)-selection, L3 measurement/L1 measurement and so on. Need further RAN1 conclusion on whether SSB periodicity is changed.

  - Proposal 1b (Ericsson): RAN4 to check if RRM requirements could be impacted by scaled measurement time delay due to extension of SSB/SMTC periodicity.

    - RAN4 to check if RRM requirements could be impacted by SSB periodicity extension to a value larger than 20ms during cell search, including the remaining physical channels and signals (e.g., SIB, paging, etc.).

For downlink coverage enhancements, following agreement from RAN1 and RAN2 has been achieved in last meeting, we list here for information:

Agreement from RAN1 As part of the NTN DL coverage enhancements at both system level and link level, RAN1 to consider: - Extending the periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - Default value[s] with extended periodicity assumed by NTN UE for initial access can be: - One [or more] values from the list {40ms, 80 ms, 160 ms, 320ms, 640ms} - Potential enhancements for transmitting the DL common channels using a wider beam footprint, while DL/UL dedicated channels (incl. PRACH) may be transmitted using a narrower beam footprint - Link-level enhancements for the following channels: - PDCCH - PDSCH with Msg 4 - PDSCH with SIB1/SIB19. - Note: link-level enhancements for PDSCH with SIB1/SIB19 may be applicable to other SIBs, without additional specification impact. - Note: the above does not imply that all the channels above will be enhanced, but all of them should be considered based on this agreement Agreement from RAN2 - 1. From RAN2 point of view, if the SSB periodicity is no larger than 160ms, there is no RAN2 impact on SSB configuration (there might still be impacts on DTX aspects) - 2. From RAN2 point of view, If the SSB periodicity is larger than 160ms, for example ssb-PeriodicityServingCell, measurement gap periodicity, SMTC configuration, ssb-Periodicity-r17 for NonCellDefiningSSB-r17 may need to be extended. And the field description of nAndPagingFrameOffset may need to be enhanced to consider the SSB periodicity higher than 160ms. 3. RAN2 can further consider SMTC impacts due to beam-hopping / larger SSB periodicity 4. If there is a need to bar pre-Rel19 NTN UEs from accessing a cell operating with DL coverage enhancement (e.g. because of extreme SSB periodicity) the existing NTN bar bit can be used. FFS about the behaviour for Rel-19 UEs not supporting DL coverage enhancement when the existing NTN bar bit is set.

RAN1 agreed to consider extending the periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access, default value[s] with extended periodicity assumed by NTN UE for initial access can be one [or more] values from the list {40ms, 80 ms, 160 ms, 320ms, 640ms}. If 320ms and/or 640ms will be finally chosen by RAN1, RAN2 will extend the SMTC, MG and so on correspondingly.

If RAN1/2 will extend the SSB/SMTC/MG periodicity larger than 320ms (the smallest DRX cycle), then almost all RRM requirements will be impacted including IDLE/Inactive mode and connected mode, such as: cell (re)-selection, L3 measurement, L1 measurement and so on. While if the extended SSB/SMTC/MG periodicity will not larger than 320ms, we think most of current requirements can be reused.

Observation 1: Almost all RRM requirements will be impacted if RAN1/2 will extend the SSB/SMTC/MG periodicity larger than 320ms (the smallest DRX cycle).

Since the extension of the SSB periodicity is still under RAN1’s discussion, RAN4 can wait for more progress from RAN1/2.

Proposal 1: RAN4 to wait for more progress from RAN1/2 on SSB periodicity extension

- Support of regenerative payload

Issue 5-2-1: Timing requirements for regenerative payload

<Way Forward>

- The common understanding is that NTA,adjcommon is configured by network.

- Further discuss the impact on timing requirements in TS 38.133 for regenerative mode of NTN.

Regarding common TA, RAN2 sent a LS [2] to RAN4 and RAN1 to check: Whether in a regenerative payload scenario it would be a problem to stick to 0 as the minimum possible value for TA-Common or whether we should e.g. introduce negative values for ta-Common.

Question: Whether in a regenerative payload scenario it would be a problem to stick to 0 as the minimum possible value for TA-Common or whether we should e.g. introduce negative values for ta-Common. Additionally, RAN2 understands in any case legacy UEs would have to rely on existing signaling and then for legacy UEs, minimum value of Common TA is equal to 0.

Recall to the RAN1 R17, to avoid the UE over pre-compensates its TA during RACH procedure, which may cause interruption to the UL transmission before RACH, there are some discussion about TA margin and TA common, following agreements had been achieved:

Agreement from RAN1#102: - FFS: The TA margin, if needed and indicated by the network (in order to account for the TA estimation uncertainty) Agreement from RAN1#106: In NTN, to avoid that the UE over pre-compensates its TA during RACH procedure, down-select one option from below: - Option 1: PRACH transmission is delayed by minCP2,GP2 - Option 2: TA margin can be considered and it is explicitly indicated to the UE - Option 3: TA margin can be considered and it is included within the Common TA - Option 4: UE handles it via implementation Conclusion from RAN1#106b: Do not define a TA margin.

Although the TA margin was not finally defined, if we check the FL summary from RAN1#106b i.e., R1-2110602, the reason behind is that TA margin can be implicitly covered in TA common. For example, if the ideal TA common = X, network could configure TA common = Y, which Y = X - TA margin.

Observation 2: RAN1 implicitly support TA margin through TA common configuration, e.g., if the ideal TA common = X, network could configure TA common = Y, which Y = X - TA margin.

Through this method, the practical TA for PRACH using by UE will be smaller, which means the PRACH transmission time will lag behind. Due to the GP structure behind, the delayed PRACH transmission will not interrupt the following UL transmission. As illustrated in Figure 1, if A>CP/2, while A-GP<CP/2, the TA margin method could prevent interruption between PRACH and other UL transmission.

Figure 1

However, after the discussion and design from RAN1, RAN4 defines the UL transmit timing error Te_NTN as following:

Table 7.1C.2-1: Te_NTN Timing Error Limit

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| FR1-NTN | 15 | 15 | 29*64*Tc |
|  |  | 30 | 24*64*Tc |
|  |  | 60 | N/A |
|  | 30 | 15 | 24*64*Tc |
|  |  | 30 | 22*64*Tc |
|  |  | 60 | N/A |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

Table 7.1C.2-2: Te_NTN Timing Error Limit for fixed VSAT is served by GSO and fixed VSAT is served by NGSO

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| FR2-NTN | 120 | 60 | 13*64*Tc |
|  |  | 120 | 7.5*64*Tc |
|  | 240 | 60 | 13*64*Tc |
|  |  | 120 | 7.5*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

Table 7.1C.2-3: Te_NTN Timing Error Limit for mobile VSAT is served by GSO

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| FR2-NTN | 120 | 60 | 13*64*Tc |
|  |  | 120 | [10]*64*Tc |
|  | 240 | 60 | 13*64*Tc |
|  |  | 120 | [10]*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

As we can see, the Te_NTN, which is the UL transmit timing error, is smaller than CP/2 in FR1, FR2-NTN fixed VSAT, and FR2-2 NTN mobile VSAT UL 60kHz SCS case. While in FR2-2 NTN mobile VSAT UL 120kHz SCS case, the Te_NTN is larger than CP/2.

Thereby, for the minimum possible value for TA-common, in FR2-2 NTN mobile VSAT UL 120kHz SCS case, we think it would be a problem to stick to 0, and it is meaningful to introduce negative values for ta-Common, which can leave more implementation flexibility for network.

Proposal 2: It is meaningful to introduce negative values for ta-Common, especially for FR2-2 NTN mobile VSAT UL 60kHz SCS case.

Based on above analysis, we prepare the reply LS in our companion paper [3].

Issue 5-3-2: RRM requirements for RRC Connection Mobility Control with regenerative payload

<Way Forward> : Further discuss the following proposal

- Whether to update the specific RRM requirements for RRC re-establishment with regenerative payload should wait for RAN2/RAN3 progress.

- Whether to update the specific RRM requirements for satellite switch and RACH-less handover with regenerative payload should wait for RAN2 progress.

- FFS: RAN4 to clarify requirement of satellite switching without PCI change is not applied for regenerative mode of NTN.

According to the RAN plenary progress, satellite switching with re-sync over Regenerative Architecture will not be pursued in REL-19. Therefore, the requirement of satellite switching with re-sync will not be applied for NTN regenerative mode.

Proposal 3: The requirement of satellite switching with re-sync will not be applied for NTN regenerative mode.

- Conclusion

Based on the discussion above, the following observations and proposals are concluded.

Observation 1: Almost all RRM requirements will be impacted if RAN1/2 will extend the SSB/SMTC/MG periodicity larger than 320ms (the smallest DRX cycle).

Proposal 1: RAN4 to wait for more progress from RAN1/2 on SSB periodicity extension

Observation 2: RAN1 implicitly support TA margin through TA common configuration, e.g., if the ideal TA common = X, network could configure TA common = Y, which Y = X - TA margin.

Proposal 2: It is meaningful to introduce negative values for ta-Common, especially for FR2-2 NTN mobile VSAT UL 60kHz SCS case.

Proposal 3: The requirement of satellite switching with re-sync will not be applied for NTN regenerative mode.

- Reference

- R4-2414037, The WF on RRM requirements for Rel-19 NR NTN phase3, CATT

- R2-2407623, LS on common TA in a regenerative payload scenario, RAN2, CMCC

- R4-2415011, Reply LS on common TA in a regenerative payload scenario, CMCC