3GPP TSG-RAN WG4 Meeting # 112bis                                                           R4-2415013

Hefei, China, 14th – 18th October, 2024

Agenda item:	6.10.3

Source: 	CMCC

Title: 	Discussion on BS RF requirements for ATG with CA

Document for:	Discussion

# 1. Introduction

RAN#105 meeting approved revised WID on Enhancements for Air-to-ground network for NR in Rel-19 [1]. After discussions of several meetings, we have made good progress on ATG BS with CA. This paper will focus on the remaining issues.

# 2. Discussion

## 2.1. ATG UE RF requirements with DL CA

The description for ATG BS with DL CA in the WID is shown as below:

- Specify the RF and RRM core requirements for intra-band co-located and inter-band co-located DL CA [RAN4]: - FR1 intra-band contiguous CA - Example band combination: n79 - FR1+FR1 inter-band CA - Example band combination: n3+n39

In Rel-18, considering ATG did not support CA, there is no TAE requirement for ATG as below.

#### 6.5.3.1 General This requirement shall apply to frame timing in MIMO transmission, carrier aggregation and their combinations. Frames of the NR signals present at the BS transmitter antenna connectors or TAB connectors are not perfectly aligned in time. The RF signals present at the BS transmitter antenna connectors or transceiver array boundary may experience certain timing differences in relation to each other. The TAE is specified for a specific set of signals/transmitter configuration/transmission mode. For BS type 1-C, the TAE is defined as the largest timing difference between any two signals belonging to different antenna connectors for a specific set of signals/transmitter configuration/transmission mode. For BS type 1-H, the TAE is defined as the largest timing difference between any two signals belonging to TAB connectors belonging to different transmitter groups at the transceiver array boundary, where transmitter groups are associated with the TAB connectors in the transceiver unit array corresponding to MIMO transmission, carrier aggregation for a specific set of signals/transmitter configuration/transmission mode. There is no TAE requirement for ATG BS.

Since ATG supports intra-band contiguous and inter-band CA in Rel-19, the TAE requirements in TS 38.104 could be reused as a baseline.

#### Issue 1-1: Whether to preclude DL MIMO for TAE requirements for ATG BS supports CA? Way Forward： - DL MIMO is not included for ATG BS supports CA based on the current WID. - RAN4 can re-visit it if there are revised WID to include DL MIMO. #### Issue 1-2: How to handle CA TAE requirements for ATG BS in TS38.104 based on the current WID? Way Forward： - FFS on the wordings on the TAE requirements for ATG BS CA in the TS38.104. - Only focus on TS38.104 in RAN4 #112-bis meeting.

After the last plenary meeting, a discussion paper considering DL MIMO requirements for ATG BS is submitted to update the work item scope to include DL MIMO performance, however, the scope expansion related to RAN4 had not been discussed in RAN#105 meeting, and this paper is not treated. In our opinion, removing the limitation for TAE is the requirement for ATG support CA, there is not necessary to define new requirements for ATG BS with MIMO as other BS RF requirements. Considering options from other companies, we could wait for the conclusion of the December plenary meeting.

Proposal 1: Wait for the conclusion of December plenary meeting to revise the wording on the TAE requirements.

# 3. Conclusions

In this contribution, ATG BS requirements are discussed with the following proposals:

Proposal 1: Wait for the conclusion of December plenary meeting to revise the wording on the TAE requirements.

# 4. References

[1] RP-242381	Revised WID on Enhancements for Air-to-ground network for NR

[2] RP-241935	Considerations on R19 ATG enhancement