3GPP TSG-RAN WG4 Meeting #112bis										R4-2415021

Hefei, China, 14th – 18th October, 2024

Agenda item:	6.20.1

Source: 	CMCC

Title: 	Discussion on SBFD general part

Document for:	Discussion

# 1. Introduction

In TR 38.858, the impacts on SBFD requirements have been analyzed. RAN4 has identified which legacy requirements are still applicable, which legacy requirements are not applicable and which new requirements are needed.

In last meeting, WF on SBFD BS requirement is approved with several agreements. [2]

In this contribution, we continue the general part discussion based on the summary approved in last meeting.

- Discussion

  - BS RF Specification structure for SBFD requirements

In last meeting, the candidate options are listed as below.

- Way forward: - RAN4 shall study how to introduce BS RF new requirements for SBFD-capable BS: - Option 1: Embedding the corresponding SBFD RF requirements in the corresponding section of TS 38.104 (similar as NB-IoT RF requirement in TS36.104) - Option 2: Creating new sub-clauses in TS 38.104 for RF requirements that applies for SBFD - Option 2a: details (like new suffix or not) can be FFS when RF requirements are stable. - Option 2b: Set up new sub-clause with suffix-B for modified existing RF requirements and SBFD-dedicated new RF requirements - Option 3: A new specification for SBFD BS RF requirements

Different from other BS spec, there are candidate new requirements for SBFD, e.g. ASCS, transient period. For such new RF requirements, it’s better to create new sub-clauses in TS 38.104 since such requirements only apply for SBFD gNB, which is much similar as UE feature in TS 38.101. For the legacy RF requirements, some of them are still applicable for SBFD with the same limit but others may need to be updated with new limit, e.g. Tx inter-modulation. for spec consistency, it’s also capture all legacy RF requirements applicable for SBFD into the new sub-clause. For the RF requirements that are the same for legacy gNB and SBFD, it’s better just state that the RF requirements in sub-clause still applies instead of simple copy-paste.

Proposal 1: for RF spec structure of SBFD requirements, it’s better to create new sub-clauses in TS 38.104 which capture all new and existing RF requirements that applies for SBFD.

  - Operating bands for SBFD

In last meeting, the candidate options are listed as below.

Issue 1-1-1: Band/channel bandwidth support for SBFD - Agreement: - Declaration-based method, and SBFD is possible to be used on any TDD band, with the following principle: - The supported channel bandwidth(s) is declared by gNB vendors - The supported channel bandwidth(s) is from existing channel bandwidth defined in TS 38.104 - The supported channel bandwidth shall be no less than X MHz - X = [20] - For next meeting, we will further discuss the potential to increase the value of X beyond X = [20]. Companies are encouraged to bring t-docs to discuss the benefits or drawback to increase this value. - FFS for some specific band if this principle rules out this band, which is required for SBFD by operators. - FFS how to implement in the spec: - Option 1: To include the following text in Clause 5.2 in TS 38.104: subband full duplex can be applied to TDD bands with channel bandwidth >= X MHz, given in Table 5.2-1. - Option 2: Add SBFD entry in Table 5.2-1 in Clause 5.2 in TS 38.104 for the TDD bands with channel bandwidth >= X MHz.

Unlike the UE specification, the BS specification does not include band-specific RF requirements for most bands, with the exception of system parameters. The exceptions are the unlicensed band and band n104.

Considering the deployment of the SBFD, it is advisable to define a general principle for SBFD. Given that some middle TDD bands do not support large bandwidths, we suggest that the channel bandwidth should be equal to or larger than 20 MHz.

Proposal 2: The channel bandwidth should be equal or larger than 20MHz.

  - Guard band and subband configurations

In last meeting, the candidate options are listed as below.

- Agreement: - To specify DL/UL subband configuration in BS RF requirements, the following 3 options for DL/UL subband configuration are identified: - Option-1 (with the least flexibility): - Only a number of selected DL/UL subband configurations are specified for RAN4 BS RF requirement. - Example (given in Nokia Tdoc R4-2413238): - For FR1 DUD configuration in 100 MHz channel and 30 kHz SCS: < NRB,SBFD,DL, NRB,SBFD,UL, NRB,SBFD,GB > = <106, 51, 5> - For FR1 DU or UD configuration in 100 MHz channel and 30 kHz SCS: < NRB,SBFD,DL, NRB,SBFD,UL, NRB,SBFD,GB > = <217, 51, 5> - Option-2 (with the middle flexibility): - Option-2a: - DL subband configuration shall be selected from the existing values of BS transmission bandwidth configuration, as provided in Table 5.3.2-1 for FR1 and Table 5.3.2-2 for FR2-1. - UL subband size and guardband size(s) depends on BS implementation - Example (FR1 DU or UD in 100MHz and 30kHz SCS, 80-20MHz case): As long as BS transmission configuration for SBFD shall be 273 (NRB,SBFD for 100MHz and 30kHz SCS) and NRB,SBFD,DL is selected from Table 5.3.2-1 (according to the 30kHz SCS), from BS RF requirement perspective, no limitation on the values NRB,SBFD,UL and NRB,SBFD,GB. - Option-2b: - UL subband configuration shall be selected from the existing values of BS transmission bandwidth configuration, as provided in Table 5.3.2-1 for FR1 and Table 5.3.2-2 for FR2-1. - DL subband size(s) and guardband size(s) depends on BS implementation - Example (FR1 DU or UD in 100MHz and 30kHz SCS, 80-20MHz case): As long as BS transmission configuration for SBFD shall be 273 (NRB,SBFD for 100MHz and 30kHz SCS) and the value of NRB,SBFD,UL is selected from Table 5.3.2-1 (according to the 30kHz SCS), from BS RF requirement perspective, no limitation on the values NRB,SBFD,DL and NRB,SBFD,GB. - Option-3 (with the highest flexibility): - No restriction on the subband sizes, i.e., RF requirement shall be specified to enable all DL and UL subband configurations (and implicitly guardband configurations). - Example (FR1 DU or UD in 100MHz and 30kHz SCS): From BS RF requirement perspective, no limitation on the values of NRB,SBFD,DL, NRB,SBFD,UL, NRB,SBFD,GB

As we understand, one of the key advantages of SBFD is its flexible frame structure. To preserve this advantage, we believe that there should be no limitations imposed on the size of subbands. Consequently, the specification should not fix the value of the Downlink or Uplink sub-band size. Option 2, which offers a moderate level of flexibility, is a suitable approach to maintain the benefits of SBFD.

Proposal 3: To keep the flexibility of SBFD, option 2(with the middle flexibility) could be used as baseline.

- Agreement: - Restriction on guardband size for SBFD: - Option 1: Definition based on existing guardbands requirement in TS38.104 (i.e., minimum guardband) - Option 2: Typical values of guardband defined for requirement, but the guardband should not be restricted to those in RAN4 specifications. - Option 3: No restriction (manufacturer declaration-based), but with restriction on maximum allowed number of RBs for guardband: - Option 3a: No restriction (manufacturer declaration-based), and no restriction on maximum/minimum number of RBs for guandband:

Since the sub-band size is not fixed in the spec, the guard band also should not be fixed. The maximum value of the guard band size should be specified in the spec, and some guard RB could be used as DL or UL in actual operation.

Proposal 4: The maximum value of the guard band size should be specified in the spec.

# 3. Conclusions

In this contribution, SBFD RF requirements are discussed with following observations and proposals.

Proposal 1: for RF spec structure of SBFD requirements, it’s better to create new sub-clauses in TS 38.104 which capture all new and existing RF requirements that applies for SBFD.

Proposal 2: The channel bandwidth should be equal or larger than 20MHz.

Proposal 3: To keep the flexibility of SBFD, option 2(with the middle flexibility) could be used as baseline.

Proposal 4: The maximum value of the guard band size should be specified in the spec.

# 4. Reference

[1] TR 38.858,

[2] R4-2413514, Way Forward for [112][307] NR_duplex_evo_General