3GPP TSG-RAN WG4 Meeting #112bis								     	R4-2415027

Hefei, China, 14th – 18th October, 2024

Agenda item:	7.11.2

Source: 	CMCC

Title: 	Discussion on simultaneous Rx/Tx for CA_n40-n41

Document for:	Discussion

# 1. Introduction

In RAN4 #105, simultaneous Rx/Tx operation for CA_n40-n41 were discussed and no consensus reached. The contribution provides continue discussion for this band combination.

- Discussion

Last meeting, we have reached the following WF

<Way forward> The way to handle the simultaneous Rx-Tx for CA_n40-n41 in Rel-19. - Option 1: Keep the requirements with n41 using 4Rx in simultaneous Rx-Tx for CA_n40-n41 - Option 2: Analyse the UE architecture with n41 using 2Rx in simultaneous Rx-Tx for CA_n40-n41, and try to define the separate requirement including MSD optionally from that in Rel-18, taking the implementation difficulties into consideration. - However, the UE should support 4Rx for n41 when the UE does not indicate simultaneous Rx-Tx for CA_n40-n41, i.e single n41 or other CA combo. - Option 3: Study whether new UE signalling(s) could be introduced for a certain band combination to indicate that: - concurrent support of 4x4 MIMO and simultaneous Rx/Tx is not supported - concurrent support of UL MIMO, and/or TxD and/or SRS antenna switching and simultaneous Rx/Tx is not supported - Other options are not precluded

During the last meeting, three options were discussed. Option 2 suggests that when a UE supports simultaneous Rx-Tx for bands n40 and n41 in CA, the requirement for n41 to have 4 Rx could be relaxed to 2Rx. In our view, the root of this issue lies in the implementation of antennas on handheld UEs. Since bands n40 and n41 are close in frequency, their antennas might share the same physical elements. This architecture reduces deployment complexity and the number of required antennas but also introduces the problem we are discussing. From the network's perspective, there is no clear benefit to relaxing the 4Rx requirement to support simultaneous Rx-Tx, and we do not dismiss the possibility that some legacy UEs could implement simultaneous Rx-Tx with n41's 4Rx capability.

As for Option 3, we believe there is no need to introduce new UE signaling for this matter. Initially, this situation arises only when CA_n40-n41 supports simultaneous Rx-Tx. Moreover, 4Rx is a mandatory requirement for n41, and whether a UE can concurrently support Uplink MIMO and simultaneous Rx/Tx should be based on the antenna deployment.

Based on the above analysis, it is preferable to adhere to Option 1 and maintain the requirement for n41 to use 4Rx in simultaneous Rx-Tx for CA_n40-n41.

Proposal 1: Keep the requirements with n41 using 4Rx in simultaneous Rx-Tx for CA_n40-n41.

# 3. Conclusions

In this contribution, question have been discussed with following observations and proposals.

Proposal 1: Keep the requirements with n41 using 4Rx in simultaneous Rx-Tx for CA_n40-n41.

# 4. Reference

R4-2411157, WF on simultaneous Rx-Tx for CA_n40-n41.