# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2415028

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	6.17.2.1

Source: 	OPPO

Title: 	Testability and interoperability issues for CSI compression

Document for:	Approval

- Discussion

For a two-sided AI/ML model, the information transmitted through air-interface (e.g. PMI) is generated by AI/ML models (e.g. CSI encoder), rather than codebooks predefined by protocol. Regarding the testability of two-sided model, it is necessary to consider bring in a test model to cooperate with the model under test.

In the previous RAN4 meetings, 4 options on test decoder have been proposed and discussed, i.e.,

- Option 1: test decoder is provided by the vendor of the encoder

- Option 2: test decoder is provided by the vendor of the decoder

- Option 3: The test decoder(s) are fully specified and captured in RAN4 spec.

- Option 4: The test decoder(s) are partially specified and captured in RAN4 spec.

In RAN4 #110 meeting, RAN4 agree to further discuss only options 3 and 4, i.e., fully or partially specify test decoder(s).

From our understanding, to define a test decoder in specification, following aspects should be taken into account.

-  Principles to define test decoder(s)

- To meet the minimum performance requirement

The test model is used to meet the minimum performance requirement in RAN4 tests, not directly to be used for commercial utilization and deployment.

- To be a simple design

As a test model to be specified in RAN4 spec, we may have a lot of proposals from different companies. How to do the merge and compromise among different designs and views from different companies would be a big challenge and a tough work.

From the beginning, we can try to design the test model(s) as simple as possible, to facilitate calibration and promote consistency among different companies.

Proposal 1: Principles to define test decoder(s)

- to meet the minimum performance requirement in RAN4 tests

- to be a simple design

CSI encoder and CSI decoder that proposed after RAN4 #111 email discussion are shown as below:

CNN based CSI encoder 								CNN based CSI decoder

Figure 1  CSI encoder and CSI decoder that proposed after RAN4 #111 email discussion

Observation 1: FLOPs, Number of parameters, model size for CNN based CSI encoder and CNN based CSI decoder are evaluated as below:

| CSI feedback bits | FLOPs |  | Num. of Trainable Parameters |  | Model Size |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Encoder | Decoder | Encoder | Decoder | Encoder | Decoder |
| 64 bits | 94.1M | 1481.6M | 0.13M | 1.80M | 638KB | 7.03MB |
| 112 bits | 94.2M | 1481.7M | 0.14M | 1.82M | 677KB | 7.11MB |

Observation 2: Based on the CNN based CSI encoder and CNN based CSI decoder proposed by RAN4#111, performance evaluation results are shown as below(train by UMa channel data as agreed in the last meeting, test by different channel model)

| CSI feedback bits | Training dataset | Test dataset (SGCS) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | TDL-A30 | CDL-A30 | CDL-C30 | UMa |
| 64 bits | UMa | 0.759 | 0.705 | 0.603 | 0.725 |
| 112 bits | UMa | 0.784 | 0.768 | 0.645 | 0.782 |

However, even after adopting the same model structure and the same simulation assumptions (based on RAN4 conclusion), different companies still obtained different simulation results(e.g., in SGCS), as summarized in RAN4#112 meeting.

Here, we first want to clarify an issue: for RAN4 test model alignment, what exactly needs to be aligned? Is it expected that the alignment of minimum performance understanding can be achieved through results from different companies? Or is it expected to strictly align the output results of each company's AI/ML models through simulation?

Proposal 2: For RAN4 test model alignment, the motivation should be further clarified

- Option1: It is expected that the alignment of minimum performance understanding can be achieved through results from different companies

- Option2: It is expected to strictly align the output results of each company's AI/ML models through simulation

For option2 (strict alignment of each company's model output), from our understanding, it is difficult to achieve by simply unifying the settings of some parameters (without strict calibration alignment of the simulation platform among different companies).

As for the first type of alignment, companies can relatively judge and reach a consensus on minimum performance understanding. On one hand, considering the impact of different datasets and model training schemes on test metrics, and on the other hand, serving as a reference basis for formulating RAN4 test performance metrics.

In RAN4#112 meeting, companies agreed to perform a second round of alignment and refined more simulation parameters as captured in R4-2414447. A second-round check could be done at the RAN4#112bis meeting. Simulation results from our side are shown as follows:

Observation 3: According the refined simulation parameters in R4-2414447, corresponded second-round alignment results could be found as below:

| CSI feedback bits | Quantization of encoder output | SGCS (UMa channel) |
| --- | --- | --- |
| 64 bits | without quantization of the FC layer outputs for validation purposes only, i.e., float32 is sent in CSI report | 0.717 |
| 32 floats | 2 bits quantization for CSI report | 0.771 |

In addition, based on our preliminary research, a complex encoder design is not necessary for a CSI feedback design. Especially in RAN4, if the test goal is to meet the minimum performance requirements, a simpler reference model design facilitates easier alignment of testing and evaluation results among different companies.

A simple MLP CSI encoder design and paired CNN based CSI decoder(proposed after RAN4 #111 email discussion) are shown as below:

MLP based CSI encoder 								CNN based CSI decoder

Figure 2  MLP based CSI encoder and CNN based CSI decoder

FLOPs, Number of parameters, model size for above MLP based CSI encoder and CSI decoder are evaluated as below.

In comparison to the CNN based CSI encoder, there's a substantial decrease in FLOPs in the MLP based CSI encoder, dropping from 94M to a mere 0.22M. This simplification lowers the challenges of aligning and implementing standardized CSI encoders across different companies, without compromising the CSI compression feedback performance.

| CSI feedback bits | FLOPs |  | Num. of Trainable Parameters |  | Model Size |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Encoder | Decoder | Encoder | Decoder | Encoder | Decoder |
| 64 bits | 0.22M | 1481.6M | 0.11M | 1.80M | 455KB | 7.03MB |
| 112 bits | 0.23M | 1481.7M | 0.11M | 1.82M | 467KB | 7.11MB |

Based on above MLP based CSI encoder and CSI decoder, some evaluation results are shown as below(train by UMa channel data as agreed in the last meeting, test by different channel model)

| CSI feedback bits | Training dataset | Test dataset (SGCS) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | TDL-A30 | CDL-A30 | CDL-C30 | UMa |
| 64 bits | UMa | 0.724 | 0.674 | 0.554 | 0.696 |
| 112 bits | UMa | 0.765 | 0.744 | 0.618 | 0.742 |

Observation 4: In comparison to the CNN-based CSI encoder, there's a substantial decrease in FLOPs in the MLP-based CSI encoder, dropping from 94M to a mere 0.22M

Observation 5: For the reference encoder, a simplified MLP-based CSI encoder could lower the challenges of aligning and implementing standardized CSI encoders across different companies, without compromising the CSI compression feedback performance.

MLP based CSI encoder 								CNN based CSI decoder

- Conclusions

In this contribution, we have discussed the testability and interoperability issue on AI/ML based CSI enhancement and got following proposals:

Proposal 1: Principles to define test decoder(s)

- to meet the minimum performance requirement in RAN4 tests

- to be a simple design

Proposal 2: For RAN4 test model alignment, the motivation should be further clarified

- Option1: It is expected that the alignment of minimum performance understanding can be achieved through results from different companies

- Option2: It is expected to strictly align the output results of each company's AI/ML models through simulation

Observation 1: FLOPs, Number of parameters, model size for CNN based CSI encoder and CNN based CSI decoder are evaluated as below:

| CSI feedback bits | FLOPs |  | Num. of Trainable Parameters |  | Model Size |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Encoder | Decoder | Encoder | Decoder | Encoder | Decoder |
| 64 bits | 94.1M | 1481.6M | 0.13M | 1.80M | 638KB | 7.03MB |
| 112 bits | 94.2M | 1481.7M | 0.14M | 1.82M | 677KB | 7.11MB |

Observation 2: Based on above CNN based CSI encoder and CNN based CSI decoder, performance evaluation results are shown as below(train by UMa channel data as agreed in the last meeting, test by different channel model)

| CSI feedback bits | Training dataset | Test dataset (SGCS) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | TDL-A30 | CDL-A30 | CDL-C30 | UMa |
| 64 bits | UMa | 0.759 | 0.705 | 0.603 | 0.725 |
| 112 bits | UMa | 0.784 | 0.768 | 0.645 | 0.782 |

Observation 3: According the refined simulation parameters in R4-2414447, corresponded second-round alignment results could be found as below:

| CSI feedback bits | Quantization of encoder output | SGCS (UMa channel) |
| --- | --- | --- |
| 64 bits | without quantization of the FC layer outputs for validation purposes only, i.e., float32 is sent in CSI report | 0.717 |
| 32 floats | 2 bits quantization for CSI report | 0.771 |

Observation 4: In comparison to the CNN-based CSI encoder, there's a substantial decrease in FLOPs in the MLP-based CSI encoder, dropping from 94M to a mere 0.22M

Observation 5: For the reference encoder, a simplified MLP-based CSI encoder could lower the challenges of aligning and implementing standardized CSI encoders across different companies, without compromising the CSI compression feedback performance.

- References

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface