# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2415033

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	6.18.2

Source: 	OPPO

Title: 	Study of impacts on RAN4 requirements for AI mobility

Document for:	Approval

- Introduction

In RAN#102 meeting, the SID on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR is approved. In RAN#105 meeting, the related work plan in RAN4 has been further checked and updated as follows:

Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation

In this contribution, we will discuss the RAN4 impacts based on current RAN2 assumptions.

- Discussion

  - Use case

To evaluate potential benefits and gains of AI/ML aided mobility, following aspects are agreed in SID[1] for study:

- Measurement prediction (including UE sided and NW sided model)

- HO failure/RLF prediction (only UE sided model)

- Measurement events prediction (only UE sided model)

And as clarified in RAN2 TP on TR 38.744, the study of the use cases is driven mainly by two study goals.

- The 1st study goal is to reduce measurement efforts in temporal, spatial or frequency domain by using predicted measurements.

- The 2nd study goal is to improve the handover performance (e.g., Ping-pong HO, HOF/RLF, short time of stay, Handover interruption).

    - RRM measurement prediction

- Sub Use Case

Regarding the RRM measurement prediction, 3 sub-use cases are considered for cell-level RRM measurement prediction:

- Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated

- Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s)

- Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

- Temporal/frequency/spatial domain measurement prediction

- For temporal domain measurement prediction,

In RAN2#125bis meeting, it has been agreed as:

- For temporal domain measurement prediction, we will consider the AI-PHY beam management Case A and Case B from the RAN1 AI/ML PHY TR and it applies to both beam level and cell level. As baseline we will focus on pure temporal predication.

In addition, based on the post email discussion after RAN2#127 meeting, temporal domain case A and case B have been further clarified as:

Intra-frequency temporal domain case A: continuous measurement results in prediction window are predicted by continuous historical measurement result(s) in observation window. Then observation window and prediction window slide forward with either one or more sampling periods (with sliding L1/L3 filtering option) or one or more measurement periods (with non-sliding L1/L3 filtering option), where measurement result(s) are actually measured before sliding.

Figure 1

Intra-frequency temporal domain case B: measurement results in prediction window are predicted by historical measurement result(s) in observation window. Then observation window and prediction window slide forward with either sampling period(s) (with sliding L1/L3 filtering option) or measurement period(s) (with non-sliding L1/L3 filtering option) and measurement result(s) in previous prediction window is/are skipped during window sliding

Figure 2-1 temporal domain case example 1

Figure 2-2 temporal domain case example 2

- For frequency domain measurement prediction, typical feature is to use the measurement result from one frequency to predict the measurement result at another frequency, which can be applicable to FR1 to FR1 cases in R19 evaluation.

- For spatial domain measurement prediction, typical feature is to predict cell-level measurements through measurements of partial beams, which can be applicable to sub-use case 1 and 3 in R19.

- Prioritization for different RRM measurement prediction scenarios

However, different sub-use cases and different scenario assumptions mentioned above may involve a lot of combinations such as FR1 to FR1, FR2 to FR2, intra/inter-frequency, intra/inter-cell, case A/B, test goal 1/2, etc. In R19, to simplify the scope, RAN2 has decided to focus on the following scenarios and conditions, and provided an explanation of the priorities in RAN2#126.

The prioritization among evaluation scenarios in RAN2 is captured as follows:

| scenario number | Priority | Evaluation scenario | Target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

- Performance metrics and assumptions

- Metrics for measurement prediction accuracy

In RAN2, measurement prediction accuracy for cell-level RRM measurement prediction is defined as:

Average L3 RSRP difference between predicted L3 filtered cell-level measurement result and actual L3 filtered cell-level measurement result of the same cell for all RRM sub-use cases, and the RSRP difference values should be an absolute value before they are averaged.

- Assumptions on measurement reduction rate

Measurement reduction rate for intra-frequency scenario in RAN2 is defined in the temporal domain (called MRRT) by assuming same length of measurement time instances and spatial domain respectively(called MRRS):

MRRT = skipped measurement time instances / total measurement time instances

MRRS = skipped beams to be measured/ total beams to be measured

For cases that need to be evaluated for the 1st study goal(reduce measurement efforts) for both FR1 and FR2 scenario. MRRT(s) should be aligned among companies.

- Impacts on RAN4 requirements for AI mobility

Although 1)Measurement prediction, 2)RLF prediction, and 3)Measurement events prediction are all included in RAN2's research, given the dependency of RLF prediction and Measurement events prediction on Measurement prediction itself, and RAN2's current progress on each use case, we suggest that in RAN4's discussion, priority should be given to the RRM measurement prediction as the starting point.

Furthermore, considering different RRM measurement prediction scenarios and the conclusions drawn by RAN2 regarding scenario prioritization, RAN4 should prioritize the study of high-priority scenarios in its research as well. Prioritized scenarios of RRM measurement prediction in RAN4 are shown as below:

| Priority | Evaluation scenario | Target study goal | Methodology |
| --- | --- | --- | --- |
| High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |

Proposal 1: In RAN4, prioritize the study on RRM measurement prediction.

Proposal 2: For RRM measurement prediction, prioritize following scenarios(align with the prioritized scenarios of RRM measurement prediction in RAN2 evaluation)

| Priority | Evaluation scenario | Target study goal | Methodology |
| --- | --- | --- | --- |
| High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |

For each scenario, further consideration is needed on how to define it in RAN4 tests. This includes the configuration of the observation window (e.g., window length, measurement sample configuration within the window), the configuration of the prediction window (e.g., window length, samples to be predicted within the window), and other test settings/conditions. The above aspects can be referenced based on the progress of RAN2's research, particularly the evaluation assumptions.

Based on the post email discussion after RAN2#127 meeting[2], for temporal domain case A, following limitations have been made in RAN2 and could be considered in RAN4 discussion:

- The ratio between observation window and prediction window is at least limited to value range {5,4,3,2,1,1/2,1/3,1/4,1/5}

- The window length should be multiple times of sampling period or measurement period of corresponding frequency range.

Based on the post email discussion after RAN2#127 meeting[3], for FR1 intra-frequency temporal domain case B, following recommended value range have been suggested in RAN2 and could be also considered in RAN4 discussion:

- MRRT: {50%~80%}

- OW: {40ms ~ 2000ms}

- PW: {40ms~800ms}

Proposal 3: For each scenario, further consideration is needed on how to define it in RAN4 tests, e.g., configurations of the observation window and prediction window.

-	above aspects can be referenced based on the progress of RAN2's research, particularly the evaluation assumptions

Proposal 4: For temporal domain case A, following limitations have been made in RAN2 and could be considered in RAN4:

- the ratio between observation window and prediction window is at least limited to value range {5,4,3,2,1,1/2,1/3,1/4,1/5}

- the window length should be multiple times of sampling period or measurement period of corresponding frequency range.

Proposal 5: For FR1 intra-frequency temporal domain case B, following recommended value range have been suggested in RAN2 and could be also considered in RAN4 discussion:

- MRRT: {50%~80%}

- OW: {40ms ~ 2000ms}

- PW: {40ms~800ms}

Regarding the performance metrics, RAN4 may focus on the intermediate KPI and follow RAN2’s definition, e.g.,

- Metrics for measurement prediction accuracy

Measurement prediction accuracy for cell-level RRM measurement prediction is defined as Average L3 RSRP difference between predicted L3 filtered cell-level measurement result and actual L3 filtered cell-level measurement result of the same cell for all RRM sub-use cases, and the RSRP difference values should be an absolute value before they are averaged.

- Assumptions on measurement reduction rate

MRRT and MRRS that defined in RAN2 can be reused:

MRRT = skipped measurement time instances / total measurement time instances

MRRS = skipped beams to be measured/ total beams to be measured

For cases that need to be evaluated for the 1st study goal(reduce measurement efforts) for both FR1 and FR2 scenario. MRRT(s) should be aligned among companies.

Proposal 6: In RAN4, reuse the intermediate KPIs that defined in RAN2 as candidate performance metrics for further study,

- Metrics for measurement prediction accuracy

Measurement prediction accuracy for cell-level RRM measurement prediction is defined as Average L3 RSRP difference between predicted L3 filtered cell-level measurement result and actual L3 filtered cell-level measurement result of the same cell for all RRM sub-use cases, and the RSRP difference values should be an absolute value before they are averaged.

- Assumptions on measurement reduction rate

MRRT and MRRS that defined in RAN2 can be reused:

MRRT = skipped measurement time instances / total measurement time instances

MRRS = skipped beams to be measured/ total beams to be measured

For cases that need to be evaluated for the 1st study goal(reduce measurement efforts) for both FR1 and FR2 scenario. MRRT(s) should be aligned among companies.

    - Measurement event prediction

As agreed in RAN2, indirect measurement event prediction(e.g., the assessment of measurement event is based on the predicted L3 measurement result) will be studied. Direct measurement event prediction is also allowed.

According to the agreements that made in RAN2#125bis, measurement event prediction study can start after some further progress on RRM measurement prediction has been made.

Proposal 7: Postpone the discussion on Measurement event prediction to Feb. 2025, details(e.g. sub use cases, KPIs, etc.,) depending on the progress in RAN2.

    - HO failure/RLF prediction

As agreed in RAN2, RLF can be obtained by:

- Directly RLF prediction based on actual measurement result(s),

- Indirectly RLF prediction based on actual measurement result(s) e.g. SINR of PCell measurement result(s) is predicted at first, based on which RLF is derived.

For directly RLF prediction, candidate KPI could be: missed RLF detection/false RLF detection/F1 score. Whether the RLF will happen based on the following methodology:

- the probability of predicted RLF should above a probability threshold

- the predicted RLF will be compared with true RLF, to calculate the KPIs

For indirectly RLF prediction, candidate KPI could be: SINR difference/missed RLF detection/false RLF detection/F1 score/time difference of true time RLF and predicted RLF/true RLF prediction.

FR2 study will be prioritized for RLF prediction.

From our understanding. the research on RLF prediction depends on the progress and conclusions of RRM measurement prediction, and the discussion of this part in RAN4 can be postponed until April 2025.

As agreed in RAN2 #126 meeting, HOF prediction is down prioritized.

Proposal 8: Postpone the discussion on RLF event prediction to April 2025, details(e.g. sub use cases, KPIs, etc.,)depending on the progress in RAN2.

Proposal 9: Down prioritized the discussion on HOF in RAN4 until new progress in RAN2.

- Conclusions

In this contribution, we discussed the Rel-19 AI/ML impacts to RAN4 and got following proposals

Proposal 1: In RAN4, prioritize the study on RRM measurement prediction.

Proposal 2: For RRM measurement prediction, prioritize following scenarios(align with the prioritized scenarios of RRM measurement prediction in RAN2)

| Priority | Evaluation scenario | Target study goal | Methodology |
| --- | --- | --- | --- |
| High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |

Proposal 3: For each scenario, further consideration is needed on how to define it in RAN4 tests, e.g., configurations of the observation window and prediction window.

-	above aspects can be referenced based on the progress of RAN2's research, particularly the evaluation assumptions

Proposal 4: For temporal domain case A, following limitations have been made in RAN2 and could be considered in RAN4:

- the ratio between observation window and prediction window is at least limited to value range {5,4,3,2,1,1/2,1/3,1/4,1/5}

- the window length should be multiple times of sampling period or measurement period of corresponding frequency range.

Proposal 5: For FR1 intra-frequency temporal domain case B, following recommended value range have been suggested in RAN2 and could be also considered in RAN4 discussion:

- MRRT: {50%~80%}

- OW: {40ms ~ 2000ms}

- PW: {40ms~800ms}

Proposal 6: In RAN4, reuse the intermediate KPIs that defined in RAN2 as candidate performance metrics for further study,

- Metrics for measurement prediction accuracy

Measurement prediction accuracy for cell-level RRM measurement prediction is defined as: Average L3 RSRP difference between predicted L3 filtered cell-level measurement result and actual L3 filtered cell-level measurement result of the same cell for all RRM sub-use cases, and the RSRP difference values should be an absolute value before they are averaged.

- Assumptions on measurement reduction rate

MRRT and MRRS that defined in RAN2 can be reused:

MRRT = skipped measurement time instances / total measurement time instances

MRRS = skipped beams to be measured/ total beams to be measured

For cases that need to be evaluated for the 1st study goal(reduce measurement efforts) for both FR1 and FR2 scenario. MRRT(s) should be aligned among companies.

Proposal 7: Postpone the discussion on Measurement event prediction to Feb. 2025, details(e.g. sub use cases, KPIs, etc.,) depending on the progress in RAN2

Proposal 8: Postpone the discussion on RLF event prediction to April 2025, details(e.g. sub use cases, KPIs, etc.,)depending on the progress in RAN2.

Proposal 9: Down prioritized the discussion on HOF in RAN4 until new progress in RAN2.

- References

RP-234055 Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

R2-2407849 summary of [POST127][030][AI mobility] RRM simulation assumptions - Phase 1

R2-2407885 summary of [POST127][030][AI mobility] RRM simulation assumptions - phase 2