# 3GPP TSG-RAN WG4 Meeting #112 bis	                                 R4-2415045

# Hefei, China, 14 – 18 October, 2024

Source:                Tejas Networks Pvt. Ltd.

Title:                     Discussion on existing Tx requirements for SBFD operation

Agenda Item:       6.20.2.2

Document for:     Discussion

# Introduction

In this contribution, we present overview of impact on existing BS RF TX requirements for SBFD slots for total dynamic range and unwanted emissions.

# Discussion

## Output power dynamics

### Total dynamic range

In the RAN4 #112 meeting, RAN4 agreed to formulate the total power dynamic range requirement for SBFD slots using an equation based on the declaration (of rated output power with all DL sub-band RBs active for SBFD) instead of the table used for the existing total power dynamic range requirement [1].

Also, as discussed in [2] the agreement reached in RAN4 in the SI phase, the total power dynamic range requirement for SBFD is defined as the ratio of the declared rated output power with all DL sub-band RBs active for SBFD (maximum) and the same single RB power as non-SBFD (minimum).

Proposal 1: We propose the following equation for the total dynamic range calculation.

Total power dynamic range=10*log10(Maxactive(NRB))

OR

Total power dynamic range=10*log10CBW-2*GB1RB

where,

CBW is the Channel BW

GB: Guard BW

NRB= Maximum transmission bandwidth configuration

1RB=1 Resource block BW

## Unwanted emissions

In the RAN4 #112 meeting, RAN4 agreed to further study on ACLR and OBUE with following agreement and WF [1].

Agreement: - When coexisting with legacy TDD system in adjacent channel, RAN4 shall apply the legacy ACLR and OBUE requirement for SBFD-capable BS. WF: - FFS on the followings: - For FR1 macro deployments, when coexisting with new SBFD system in adjacent channel, RAN4 to define additional ACLR and/or OBUE requirement to ensure feasible coexistence with SBFD-capable gNB. - The assumption for the victim BS DESENS and CL of BS2BS is FFS.

### OBUE

We would like to present our views on FR1 macro deployments when coexisting with a new SBFD system in an adjacent channel, RAN4, to define additional ACLR and/or OBUE requirements to ensure feasible coexistence with SBFD-capable gNB. In [3], the feasibility analysis of SBFD BS co-location has been discussed, where for FR1 aggressor WA BS, the output power is 49 dBm, the bandwidth of the channel is 100 MHz, and if we assume the degradation of victim BS sensitivity is 1 dB, the required total isolation between the two BSs is 144 dB, that is, the sum of ACLR, couple loss of co-location, and the rejection of RF BPF used after PA must be larger than 144 dB. For FR1 macro-BS with the output power of 49 dBm and a 100 MHz bandwidth of channel, the adjacent +1 ACLR of 55 dB can be easily achieved with the help of the PA linearization technique. Moreover, due to the advancement in the Digital Predistortion (DPD) techniques for PA linearization, it is practically possible to achieve better than 55 dB of ACLR in adjacent +1 channels.

In the case of location, to achieve this value, we additionally propose the gNB-gNB CLI handling scheme to be introduced. Therefore, we revisit the feasibility analysis of SBFD BS co-location as discussed in [3].

Table 1: Feasibility analysis of SBFD BS co-location

| Parameters | Value |
| --- | --- |
| Output power(dBm) | 49 |
| BW(MHz) | 100 |
| Degradation of victim BS sensitivity(dB) | 1 |
| Required total isolation(dB) | 144 |
| Adjacent +1 (2nd) ACLR (dB) | 55 |
| Coupling loss of co-location (dB) | 30 |
| Rejection of RF BPF used after PA (dB) | 59 |

Proposal 2: We would like to propose that the minimum requirement for ACLR in the adjacent +1 channel should be 55 dBc for 100 MHz channel BW; therefore, rejection of RF BPF used after PA can be practically realized.

### Co-location ACLR

Based on the above OBUE discussion the minimum ACLR required as mentioned in TR 38.858 [4] “For ACLR requirement, it shall be defined outside of the whole carrier instead of sub-band for SBFD DL symbols/slots and ACLR requirement is still defined as the ratio of sum of TX power within the whole carrier to the adjacent carrier.”

The manufacturer will declare whether support co-location requirements in SBFD symbols/slots.

Proposal 3: The minimum colocation ACLR can be -50dBc for 100MHz considering applicability of CLI scheme in adjacent channel or deployment restrictions.

# Conclusion

In this contribution, we want to share our views on RF TX requirements for SBFD BS are made as following:

Proposal 1: We propose the following equation for the total dynamic range calculation.

Total power dynamic range=10*log10(Maxactive(NRB))

Proposal 2: We would like to propose that the minimum requirement for ACLR in the adjacent +1 channel should be 55 dBc for 100 MHz channel BW; therefore, rejection of RF BPF used after PA can be practically realized.

Proposal 3: The minimum Co-location ACLR can be -50 dBc for 100 MHz signal by considering CLI handling schemes or handled through deployment.

# Reference

- R4-2413515: Way Forward for [112][308] NR_duplex_evo_BSRF, Huawei, HiSilicon

- R4-2412915, “Impact on SBFD BS RF TX requirements”, Ericsson.

- R4-2411080, “Discussion on modification of existing Tx requirements for SBFD operation”, CATT

- 3GPP TR 38.858: “Study on Evolution of NR Duplex Evolution”.