# 3GPP TSG-RAN WG4 Meeting #112 bis	                                 R4-2415046

# Hefei, China, 14 – 18 October 2024

Source:                Tejas Networks Pvt. Ltd.

Title:                     Views on modification of existing Rx requirements for SBFD

Agenda Item:       6.20.2.3

Document for:     Discussion

# Introduction

In the RAN4 #112 meeting, RAN4 agreed to further study, SBFD in-band blocking, receiver dynamic range, OTA sensitivity degradation.

# Discussion

## SBFD in-band blocking

In the RAN4 #112 agreed to further study BS receiver in-band blocking with following agreement and WF [1].

Agreement: - On contributing source for in-band blocking requirement, it is only the DL adjacent channel interference. - RAN4 to not consider any CLI handling scheme effects when defining the in-band blocking requirements. - For co-existence study, - The reference point for the power level which should be before array gain WF: - FFS on the followings: - RAN4 needs to discuss on how in-band blocking requirement are derived: - Option 1: MCL assumption for BS2BS CLI interference - Option 2: Co-existence study - Option 3: Both - For co-existence study, RAN4 need firstly agree on the open parameters for simulation assumption: - FFS grid-shift values should be considered. - Option 1: 10% - Option 2: 20% - Option 3: 50% - Option 4: 100% - 99% of the UL SBFD wideband received power CDF

The SBFD in-band blocking simulation findings from our company are included in this contribution, in the case of co-existence (Option 2) and down-selected grid-shift values of 10% and 100%.

### Simulation Assumptions

Based on the WF captured in [1], we have studied the following scenario for the case with TDD DL (aggressor) to SBFD UL subband. The objective is to measure the in-band blocking power at the SBFD base stations. Table 1 shows the co-existence scenario considered, and Table 2 shows the configuration used for the adjacent channel co-existence case for the simulation.

Table 1:  co-existence Scenario considered for the simulation

| Scenario | FR | Aggressor | Victim | Corresponding BS class |
| --- | --- | --- | --- | --- |
| 1 | FR1 | Urban Macro | Urban Macro | Wide area BS |

Table 2:  Adjacent channel co-existence case considered for the simulation

| Aggressor | Victim | Slot allocation Aggressor Victim |
| --- | --- | --- |
| TDD DL | SBFD |  |

Following simulation assumption have been selected from Annex E in [2].

- Grid shift: We have selected the worst-case scenario with small grid-shift (10%) for uncoordinated deployments and best-case grid shift of 100%.

- LOS probability: Option 2: If the 2D distance between two BS is less than or equal to the ISD, set the LOS probability to X; X = 0.75. Otherwise, UMa (h_UT = 25 m).

- UE distribution: uniform, 20% indoor and 80% outdoor.

- BS transmit power: TDD 49 dBm and SBFD 49 dBm.

- Antenna configuration: TDD 32×32, and SBFD config 32×32.

Moreover, for the performance evaluation, RAN4 had only considered the case of {D, U} as an SBFD configuration as it is comparable to the {D, U, D} SBFD configuration. Throughput loss due to the adjacent channel CLI was studied, and Figs. 1 through Fig-3 are box plots that represent the summary of a median throughput loss from performance reports of the adjacent channel coexistence study [3]. Plots include the minimum, first (lower) quartile, median, third (upper) quartile, and maximum of the median throughput losses published in the coexistence study.

Figure1: Adjacent Channel CLI median throughput loss (%) summary for Scenario-1 (FR-1) concluded in Rel-18 CLI

Figure 2: SBFD UL median throughput loss summary at cell edge and cell centre for Scenario-1

Figure 3: SBFD UL median throughput loss summary for Cell centre (Left) & Cell edge (Right) users for varying ACIR for Scenario-1

The following observations were made from the performance results with respect to both legacy TDD and SBFD as victims.

- The cell edge throughput degradation is worse than the average throughput degradation.

- The throughput degradation is due to the inter-BS ACI introduced by TDD-UL over SBFD, which increases as grid-shift (BS-to-BS distance) decreases in scenario 1.

- SBFD BS antenna configuration slightly impacts the throughput degradation.

- Major SBFD UL throughput degradation is observed cell edge uplink.

- For higher BS Tx power and lower grid-shifts the degradation is increased for cell edge throughput and average throughput.

- No throughput degradation is observed on the SBFD DL for both average throughput and cell edge throughput, respectively, for different BS Tx powers (ranging from 46 dBm to 53 dBm for FR1 and 30 dBm for FR2-1), grid-shifts (5% to 100%), and SBFD BS antenna configurations.

RAN4 had considered the case of {D, U} as an SBFD configuration instead of {D, U, D} SBFD configuration. But when we consider {D, U, D} with {40MHz,20MHz,40MHz} bandwidth, there is an additional protection of 40MHz for the leakage from the adjacent channel CLI. The study done as part of TR 38.858 [2] discussed the feasibility of analog filtering options for self-interference and co-channel interference due to co-site/co-sector in sections 9.3.1 and 9.3.2, respectively. It was dropped due to design complexity, other insertion loss due to a narrow roll-off from a few kHz to a few MHz, and limitations in supporting multi-carrier. However, this feasibility was not studied from an adjacent channel blocker perspective, considering the benefit of 40MHz filter roll-off due to {D, U, D} configuration for SBFD. For the SBFD symbol represented by {D, U, D}, Adjacent channel gNB-gNB CLI is seen in the UL sub-band of the SBFD symbol due to DL slots of legacy TDD from neighbouring gNBs overlapping with SBFD symbols.

For the Legacy TDD symbols, the Adjacent channel gNB-gNB CLI is seen in UL symbols in the Flexible slot overlapping with the SBFD slot of neighbouring gNB. Since the configurations considered for the adjacent channel CLI for throughput analysis are different from what has been studied in TR 38.858 [2] there is a need for further study on network throughput impact due to adjacent channel co-existence.

Proposal 1: For in-band blocking requirement, select option 2 (Co-existence study) and for grid shift consider option 1 (10% grid shift) and Option 4(100% grid shift).

Following BS Receiver Blocker Models and Receiver noise figure models for FR1 wide area BS considered for simulation:

- Baseline BS Receiver Blocking with baseline noise figure model for FR1 wide area BS: P1=-43 dBm, P2=-25 dBm, F1=5 dB, F2=14 dB (default option in 38.858 Annex E)

- Reduced blocker-based BS receiver blocking with modified noise figure P1=-37 dBm, P2=-25 dBm, F1=8 dB, F2=14 dB.

Since we have already considered {D, U, D} with {40MHz, 20MHz, 40MHz} bandwidth, there is an additional protection of 40MHz for the leakage from the adjacent channel CLI. Therefore, we have considered analog filtering options for adjacent interference reduction. Moreover, we have also considered an additional 3 dB noise figure to consider the noise introduced by this filter.

- Reduced blocker-based BS receiver blocking with modified noise figure P1=-37 dBm, P2=-25 dBm, F1=8 dB, F2=14 dB, same simulation scenario as CASE-B above with additional beam nulling. Beam nulling is implemented at the co-channel gNBs of the SBFD network as proposed in RAN1 discussions as one of the gNB-gNB CLI handling schemes.

Observation 1: Along with Baseline Blocker Model, reduced blocker model with {D,U,D} SBFD configuration, analog filtering of adjacent interference and modified noise figure – {P1=-37 dBm, P2=-25 dBm, F1=8 dB, F2=14 dB.} and the same reduced blocker model with CLI handling schemes should be considered and further studied  for SBFD performance evaluations and simulations

Proposal 2: For in-band blocking simulations the reduced blocker model and reduced blocker model with CLI handling mechanisms should be further studied.

### Simulation results

#### Simulation results for Baseline BS Receiver Blocker Model with Baseline Noise Figure [CASE-A]

We consider a coexistence scenario where UMa deployments of two different operators are deployed in the same area and in adjacent channels. The first operator (victim) deploys a SBFD network, while the second operator (aggressor) deploys a TDD network. We are interested in evaluating the wideband interference power received by the UL of the SBFD operator from the DL of the TDD operator. The default noise figure model defined in the 38.858 Annex E [2] is considered as baseline.

Figure4: FR1, UMa -> UMa, in-band blocking

Table 3: FR1 SBFD UMa -> UMa conducted in-band blocking simulation results

|  | 100% grid-shift | 10% grid-shift |
| --- | --- | --- |
|  | 99% CDF | 99% CDF |
| Conducted in-band blocking (dBm) | -27.47 | -0.82 |

The in-band blocking requirement is defined for the 99%-tile of the CDF of the received UL wideband interference power. From Figure 4, it can be observed that the in-band blocking requirement is higher as the grid shift decreases. It can also be observed that for the 100% grid shift case, the 99 percentile of the in-band blocking power is 15.54 dBm, and for the 10% grid shift case, it is approximately 42.18 dB above the currently specified in-band blocking requirement (-43 dBm) for Wide Area BS, respectively.

Observation 2: For UMa deployments, 100% grid shift case, the 99 percentile of the in-band blocking power is 15.54 dBm, and for the 10% grid shift case, it is approximately 42.18 dB above the currently specified in-band blocking requirement (-43 dBm).

Figure 5: FR1, UMa--, in-band blocking

In Figure 5, we have plotted the distributions of received power levels, considering 10% grid shift for this simulation scenario. We have captured adjacent channel gNB DL power level, co-channel gNB DL power level, co-channel UE-UL power level, and wanted signal level. The results show that for 10% grid shift, adjacent channel interference power is dominating the total interference.

Observation 3: For 10% grid shift, the adjacent channel CLI power dominates the total in-band blocking power. Hence, we require gNB-gNB CLI handling schemes or deployment restrictions to mitigate the impact of adjacent channel CLI power.

- (b)

Figure 6: CDF of (a) SINR (b)Spectral Efficiency

Figures 6(a) and 6(b) show the comparison of the SINR CDFs and spectral efficiency, respectively, at the victim receiver, including total noise and total interference (38.858 Annex E, E. 2.3.2.1) between the 10% and 100% grid shift scenarios. Figure 6(a) shows that at the 40 percentiles of SINR CDF, 100% grid shift shows a better SINR compared to 10% of grid shift by 16 dB. Similarly, Figure 6 (b) shows that for the 10% grid shift, only 22% of UEs have a spectral efficiency above 0.5 compared to the 51% of UEs for the 100% grid shift.

Observation 4: SINR and Spectral Efficiency of the SBFD system decreases with increasing grid shift percentage. The adjacent channel in band interference dominates significantly for 10% grid shift cases, resulting in performance degradation of victim SBFD gNBs.

Proposal 3: For the FR1 Urban Macro Scenario, the existing in-band blocking requirement of (-43 dBm) should be increased for SBFD Wide area BS to cope with the elevated levels of in-band blocking power.

Proposal 4:  For 10% Grid Shift requirement, the interference power level and in-band blocking requirement should be considered after RAN1 studying the applicability of CLI handling mechanism on adjacent channel or consideration of any deployment restrictions.

#### Simulation results for Reduced Blocker based BS Blocker Model with Modified Noise Figure [CASE-B]

In this simulation scenario, we consider additional adjacent channel interference reduction using an analog filter. To consider the noise introduced by this filter, we have added an additional 3 dB noise figure (compared to the baseline noise figure in 38.858 Annex E) as P1 = -37 dBm, P2 = -25 dBm, F1 = 8 dB, and F2 = 14 dB. The objective here is to see the impact of the reduced blocker model on the improvement of SINR and spectral efficiency.

Figure 7: In band Blocking Power Levels in the Reduced Blocker Model

In Fig. 7, we can observe that due to additional filter in the analog domain, the adjacent channel interference power level reduces significantly compared to the baseline model. The 99-percentile value of the in-band blocking power CDF reduces significantly for the reduced blocker model, as depicted in Table 4.

Table 4: FR1 SBFD UM-UMa conducted in-band blocking simulation results with reduced blocker

|  | 10% grid-shift |  |
| --- | --- | --- |
|  | 99% CDF [Reduced Blocker] | 99% CDF [Baseline] |
| Conducted in-band blocking (dBm) | -36.09 | -0.82 |

- (b)

Figure 8: CDF of (a) SINR     (b) Spectral Efficiency

Figures 8(a) and 8(b) show the comparison of the SINR CDFs and spectral efficiency, respectively, at the victim receiver, including total noise and total interference (38.858 Annex E, E. 2.3.2.1) between the reduced blocker model and baseline model for the same grid shift of 10% in both the models. Figure 8 (a) shows that at the 40 percentiles of SINR CDF, the reduced blocker model gives better SINR of 8.23 dB compared to the baseline model. Similarly, for the baseline model, only 22.5% of UEs have a spectral efficiency above 0.5 compared to the 41% of UEs in the reduced blocker model. The performance of the reduced blocker model increases significantly as the adjacent channel interference is blocked by the extra analog filters.

Observation 5: The 99-percentile value of the in-band blocking power is -36.09 dBm for the 10% grid shift scenario for the reduced blocker model. The in-band blocking power requirement was reduced significantly for the Reduced Blocker Model.

Observation6:  Improvement of 8dB in SINR seen in simulations with reduced blocker compared to baseline model. With reduced blocker about ~18-20% more UEs are having spectral efficiency above 0.5.

Observation 7: The throughput loss of the Reduced Blocker Model is less than the baseline model. but is not fully eliminated using the analog filters, as adjacent channel interference leakage will also be the dominant factor causing the interference.

Proposal 5: RAN-4 should further do the feasibility study of the extra analog BPF filter and its configuration for the DUD to consider for simulations, considering the performance impact in 10% grid shift.

Proposal 6: The throughput of the SBFD network with analog filter for DUD configuration should be further studied considering the applicability of CLI handling mechanisms for Adjacent channel or deployment restrictions.

#### Simulation results for Reduced Blocker based BS Blocker Model with Modified Noise Figure with Beam Nulling [CASE-C]

##### Only Co-Channel gNB Beam Nulling

In this scenario, a beam nulling-based CLI handling scheme as proposed by RAN1 is used at the co-channel gNBs to reduce the interference caused by them to the SBFD network. We would like to see the gain in SINR and spectral efficiency due to beam nulling and the impact of adjacent CLI leakage.

(a)                                                              (b)

Figure 9: CDF of (a) SINR     (b) Spectral Efficiency

Figures 9(a) and 9(b) show the comparison of the SINR CDFs and spectral efficiency, respectively, at the victim receiver, including total noise and total interference (38.858 Annex E, E. 2.3.2.1) between the reduced blocker model and reduced blocker model with beam nulling at Co-Channel gNBs for the same grid shift of 10% in both the models. Figure 9(a) shows that at the 40 percentiles of SINR CDF, the reduced blocker model with beam nulling gives better SINR of 6.18 dB compared to the same model without beam nulling. Similarly, for the reduced blocker model, 41% of UEs have a spectral efficiency of above 0.5 compared to the 57% of UEs in the reduced blocker model with beam nulling. The CLI handling scheme for the co-channel gNBs reduces the throughput loss percentage and increases the SINR as the co-channel interference is reduced by beam nulling.

Observation 8: Improvement of 6dB in SINR seen and ~16-18% more UEs with more than 0.5 Spectral Efficiency in simulations with reduced blocker with CLI handling mechanism compared to reduced blocker without CLI mechanism

Observation 9: The throughput loss for the model with beam nulling is reduced. But the results indicate that around 20% of the UEs are still in throughput loss, as it indicates adjacent channel interference leakage is still dominating even with the extra blocker.

Proposal 7: For 10% Grid Shifts, the co-channel gNB CLI handling mechanism should be considered for SBFD networks, although there is throughput loss due to adjacent channel leakage.

##### Co-Channel and Adjacent-Channel gNB Beam Nulling

In this scenario, a beam nulling-based CLI handling scheme as proposed by RAN1 is used at the co-channel gNBs to reduce the interference caused by them to the SBFD network. We would like to see the gain in SINR and spectral efficiency due to beam nulling and the impact of adjacent CLI leakage.

(a)                                                          (b)

Figure 10: CDF of (a) SINR (b) Spectral Efficiency

Figures 10(a) and 10(b) show the comparison of the SINR CDFs and spectral efficiency between the reduced blocker model with and without beam nulling at Adjacent Chanel gNBs for the same grid shift of 10% in both the models (co-channel Beam Nulling considered). Figure 10(a) shows that at the 40 percentiles of SINR CDF, the reduced blocker model with adjacent channel beam nulling gives better SINR of 4.85 dB compared to the same model without beam nulling. Similarly, for the reduced blocker model, 40% of UEs have a spectral efficiency of above 1 compared to the 54% of UEs in the reduced blocker model with beam nulling. The CLI handling scheme for the adjacent channel gNBs reduces the throughput loss percentage and increases the SINR as the adjacent channel leakage interference is reduced by beam nulling.

Observation 10 : Improvement of 4.85dB in SINR seen and ~15% more UEs with more than 1 Spectral efficiency in simulations with reduced blocker with CLI handling mechanism compared to reduced blocker without CLI mechanism for adjacent Channel gNBs.

Observation 11: Improvement of 16dB in SINR seen and ~45% more UEs with more than 0.5 Spectral efficiency in simulations with reduced blocker with CLI handling mechanism compared to baseline model (CASE-A) without analog filter or CLI handling mechanism.

Proposal 8: RAN4 should consider Further study on the BS Receiver Blocker Model with analog filter and applicability of CLI handling mechanism at the Adjacent Channel gNBs for 10% Grid Shift.

## Receiver Dynamic Range

WF: - FFS on the followings: - Option 1: RAN4 to discuss if the IoT level and wanted signal power level will be derived based on simulation work. - Option 2: The existing dynamic range requirement, RAN4 assume 20dB interference over thermal noise, which is enough to cover the co-channel interference from other base stations. - Option 3: for receiver dynamic requirement, both uplink signals and BS2BS CLI signal should be considered for IoT levels.

In this contribution, we will discuss the IoT level and wanted signal power level for the dynamic range requirement. The scenarios for Rx dynamic range simulation are the same as those for SBFD in-band blocking discussed above,

Observation 12: When the grid shift is 10%, the IoT is significantly greater than the 20dB and when grid shift is 100%, the 20dB interference over thermal noise defined is enough to cover the co-channel interference from other base stations without considering any gNB2gNB CLI handling mechanism

Observation 13: For 10% grid shift, with reduced blocker model, the IoT is approximately 20 dB without considering any CLI handling mechanism.

Proposal 9: For 10% Grid Shift, the 20dB interference over thermal noise defined is not enough to cover the interference from other base stations without considering any gNB2gNB CLI handling mechanisms, restrictions on deployment scenarios or extra analog filter as proposed.

## OTA sensitivity degradation

Agreement: - OTA sensitivity is measured with only self-interference. - FFS on the OTA sensitivity degradation within [0.5~1.0] dB.

As agreed, the OTA sensitivity is measured with only self-interference considering it we have summarised the table below

Table 5: Calculation of sensitivity degradation

| Quantity | SBFD | Units | Remark |
| --- | --- | --- | --- |
| Noise Figure (Noise Figure + SI) | 6 | dB | self-interference can be modelled as additive white gaussian noise with fixed INR = - 6 dB targeting 1 dB desense. Hence, it can be added as a 1 dB part of Noise figure. Original, NF = 5 dB |
| Required SNR | -2 | dB | Assuming QPSK AWGN requirements for TDD/SBFD |
| BW | 5 | MHz |  |
| Transmit Power | 49 | dBm |  |
| Tx BW | 100 | MHz |  |
| TxdBm/Hz | -31 | dBm/Hz |  |
| Receiver Signal Sensitivity | -93 | dBm | The Receiver signal sensitivity in TDD is high because the Required SNR is high. |
| Interference | -99 | dBm | Fixed INR for SI of -6dB |
| Interference Level/Hz | -165.9897 | dBm/Hz |  |
| Total Rejection Required | 134.9897 | dB |  |
| Desensitivity | 0.97 | dB |  |
| Isolations | 50 |  |  |
| DPD | 50 | dB |  |
| Antenna SIC | 65 | dB |  |
| Digital SIC | 30 | dB |  |
| Total Cancellation (Ant. SIC+Dig. SIC+DPD) | 145 | dB |  |

Observation 14: Considering the total self-interference (both in analog and digital domain) cancellation of ~145dB, NF of 6 dB, the wide area base station results in a desensitivity value of ~1dB.

Proposal 10:  The target value for OTA desensitivity can be 1 dB in the presence of SBFD self-interference.

# Conclusion

In this contribution, we discuss about in-band blocking, receiver dynamic range and OTA sensitivity degradation with the following proposals and observations.

Proposal 1: For in-band blocking requirement, select option 2 (Co-existence study) and for grid shift consider option 1 (10% grid shift) and Option 4(100% grid shift).

Observation 1: Along with Baseline Blocker Model , reduced blocker model with {D,U,D} SBFD configuration, analog filtering of adjacent interference and modified noise figure – {P1=-37 dBm, P2=-25 dBm, F1=8 dB, F2=14 dB} and the same reduced blocker model with CLI handling schemes should be considered and further studied  for SBFD performance evaluations and simulations

Proposal 2: For in-band blocking simulation the reduced blocker model and reduced blocker model with CLI handling mechanism should be further studied.

Observation 2: For UMa deployments, 100% grid shift case, the 99 percentile of the in-band blocking power is 15.54 dBm, and for the 10% grid shift case, it is approximately 42.18 dB above the currently specified in-band blocking requirement (-43 dBm).

Observation 3: For 10% grid shift, the adjacent channel CLI power dominates the total in-band blocking power. Hence, we require gNB-gNB CLI handling schemes or deployment restrictions to mitigate the impact of adjacent channel CLI power.

Observation 4: SINR and Spectral Efficiency of the SBFD system decreases with increasing grid shift percentage. The adjacent channel in band interference dominates significantly for 10% grid shift cases, resulting in performance degradation of victim SBFD gNBs.

Proposal 3: For the FR1 Urban Macro Scenario, the existing in-band blocking requirement of (-43 dBm) should be increased for SBFD Wide area BS to cope with the high levels of in-band blocking power.

Proposal 4:  For 10% Grid Shift requirement, the interference power level and in-band blocking requirement should be considered after RAN1 studying the applicability of CLI handling mechanism on adjacent channel or consideration of any deployment restrictions.

Observation 5: The 99-percentile value of the in-band blocking power is -36.09 dBm for the 10% grid shift scenario for the reduced blocker model. The in-band blocking power requirement is reduced significantly for the Reduced Blocker Model.

Observation 6:  Improvement of 8dB in SINR seen in simulations with reduced blocker compared to baseline model. With reduced blocker about ~18-20% more UEs are having spectral efficiency above 0.5.

Observation 7: The throughput loss of the Reduced Blocker Model is less than the baseline model. It is not fully eliminated using the analog filters, as adjacent channel interference leakage will also be the dominant factor causing the interference.

Proposal 5: RAN-4 should further do the feasibility study of the extra analog BPF filter and its configuration for the DUD to consider for simulations, considering the performance impact in 10% grid shift.

Proposal 6: The throughput of the SBFD network with analog filter for DUD configuration should be further studied considering the applicability of CLI handling mechanisms for Adjacent channel or deployment restrictions.

Observation 8: Improvement of 6dB in SINR seen and ~16-18% more UE’s with more than 0.5 Spectral Efficiency in simulations with reduced blocker with CLI handling mechanism compared to reduced blocker without CLI mechanism.

Observation 9: The throughput loss for the model with beam nulling is reduced. But the results indicate that around 20% of the UEs are still in throughput loss, as it indicates adjacent channel interference leakage is still dominating even with the extra blocker.

Proposal 7: For 10% Grid Shifts, the co-channel gNB CLI handling mechanism should be considered for SBFD networks, although there is throughput loss due to adjacent channel leakage.

Observation 10: Improvement of 4.85dB in SINR seen and ~15% more UEs with more than 1 Spectral efficiency in simulations with reduced blocker with CLI handling mechanism compared to reduced blocker without CLI mechanism for adjacent Channel gNBs.

Observation 11: Improvement of 16dB in SINR seen and ~45% more UEs with more than 0.5 Spectral efficiency in simulations with reduced blocker with CLI handling mechanism compared to baseline model (CASE-A) without analog filter or CLI handling mechanism.

Proposal 8: RAN4 should consider Further study on the BS Receiver Blocker Model with analog filter and applicability of CLI handling mechanisms at the Adjacent Channel gNBs for 10% Grid Shift.

Observation 12: When the grid shift is 10%, the IoT is significantly greater than the 20dB and when grid shift is 100%, the 20dB interference over thermal noise defined is enough to cover the co-channel interference from other base stations without considering any gNB2gNB CLI handling mechanism

Observation 13: For 10% grid shift, with reduced blocker model, the IoT is approximately 20 dB without considering any CLI handling mechanism.

Proposal 9: For 10% Grid Shift, the 20dB interference over thermal noise defined is not enough to cover the interference from other base stations without considering any gNB2gNB CLI handling mechanisms, restrictions on deployment scenarios or extra analog filter as proposed.

Observation 14: Considering the total self-interference (both in analog and digital domain) cancellation of ~145dB, NF of 6 dB, the wide area base station results in a desensitivity value of ~1dB.

Proposal 10:  The target value for OTA desensitivity can be 1 dB in the presence of SBFD self-interference.

# Reference

- R4-2413515: Way Forward for [112][308] NR_duplex_evo_BSRF.

- TR 38.858, “Study on evolution of NR duplex operation”, 3GPP.

- R1-2403894, Discussion on gNB-gNB CLI handling, Tejas Networks Ltd.